# APPENDIX A5.	Network layer exception handling

## A5.1.	Scope

This section presents situations which may develop during interworking between a DTE and a radio unit and which are not fully covered by section 5.3. Proposals presented here are examples of how a designer may prevent or deal with these situations.

## A5.2.	DTE in the idle state

In the idle state the DTE may receive an OPERATING CONDITION message with 'radio transmitting' indication. In this case the radio is being controlled locally, independently of the DTE, and the DTE does not know anything about the call and may not control it by using messages which require the called party address.

| Solutions | a) DTE does not interfere with the call at all. |
| --- | --- |
|  | b) The radio unit informs the DTE, by SETUP PROGRESS(Call connected) message, that it has set up a call before sending an OPERATING CONDITION(radio transmitter on) message. |