# Appendix A2	SDL representation of MAP27 data link layer

## A2.1.	General

This Specification and Description Language (SDL) presentation [7] of the data link layer protocol is designed to help implementations. In it certain assumptions have been made and certain values have been assigned to variables. This does not mean that other values may not be used. Protocol states are for example only and a designer may choose to have a different number of states. If there is any discrepancy between the textual presentation and the SDL presentation, the text in the normative section 4 should have a higher priority. There is after this textual version a graphical presentation of this SDL. That SDL defines an optional procedure for an implementation of extended link establishment timeout.

## A2.2.	SDL definitions

/*	This is the data_link process described in 'linear SDL' by subdivision into the following states:

reset_wait:	This is the default state at power-up and this state is used for connection reset. The act of powering-on causes the data_link process to immediately activate the 'transition' initiated by the input 'power_on'. The data_link process remains in this state until it is satisfied that its opposite number has also started link establishment. On completion of the transition it changes state to 'link_wait'.

link_wait:	The data_link process remains in this state until it is satisfied that it can communicate with its opposite number at the remote end of the RS-232 cable. Whenever it enters this state, the Link-establishment timer (T0) must be started. Whenever it leaves this state, the 'Network Layer' must be informed.

ready:	The data_link process resides in this state as long as it believes it is in communication with its opposite number. If it loses communication, it must inform the Network Layer, attempt to re-establish the link, and return to the reset_wait and link_wait processes. Whenever it enters this state, the 'credit_report_timer' (T3) must be started. When in this state, data packets may be exchanged on the RS-232 link. When the data_link process exits this state, no timers except the link-establishment timer may be left running.

### A2.2.1.	Timers

The following timers exist and those timers may have a higher priority than the other messages:

T0	link_establishment_timer

T1	retry_timer

T2	acknowledgement_timer

T3	activity_timer

T4	link_failure_detection_timer

### A2.2.2.	Inputs from operating system

At power-up, the following message is input to this process:

power_on

### A2.2.3.	Inputs from timers

Expired timers input the following messages:

link_establishment_timeout(T0)

retry_timeout(T1)

acknowledgement_timeout(T2)

activity_timeout(T3)

link_failure_detection_timeout(T4)

### A2.2.4.	Inputs from RS-232 line

The following input messages may be received from the RS-232 port:

link_request(maximum_length, window_size, version)

link_acknowledge(rx_sequence_number, rx_credit_number)

link_transfer(tx_sequence_number, acknowledgement_request, network_layer_message)

### A2.2.5.	Inputs from network layer

The following input messages may be received from the network layer:

network_layer_reset

network_layer_packet(network_layer_message)

-	note that in this case, the message passing mechanism must wrap the network-layer message in an envelope labelled 'network_layer_packet'.

credit_value(receive_credit)

-	the network layer should send this message to the data-link layer whenever it removes a packet from its data_link layer input buffer.

### A2.2.6.	Outputs to RS-232 line

The following outputs may be sent along the RS-232 link:

link_request(maximum_length, window_size, version)

-	This message is sent to the RS-232 port with its parameters set to the current values of maximum_length, window_size and version.

link_acknowledge(rx_sequence_number, rx_credit_number)

-	When this output is initiated, the data-link layer first stores the value of 'receive_credit' in variable send_credit'. The link_acknowledge message is then sent to the RS-232 port with parameter 'rx_sequence_number' given the value of 'receive_state' and parameter 'rx_credit_number' given the value of 'receive_credit'.

link_transfer(tx_sequence_number,acknowledgement_request, network_layer_message)

-	When this message is sent to the RS-232 port, its parameters are set to 'send_state' and 'LT_acknowledge_request' respectively, and its data area is filled with a network layer message from the queue of messages waiting to be transmitted. The correct message is identified by selecting the one whose stored_packet_number is equal to the value of the variable 'send_state'.

### A2.2.7.	Outputs to Network_layer

The following outputs may be sent to the network layer:

link_ready

link_failure

packet_accepted

packet_rejected

network_layer_packet(network_layer_message)

-	note that in this case, the message passing mechanism must present the packet to the network layer as a variable message whose type depends upon the first octet of the packet's contents).

### A2.2.8.	Constants

The following constants are defined:

N2	Maximum number of re-transmissions

N3	Maximum number of inactivity timeouts

T0	Link establishment time

T1	Retry time

T2	Acknowledgement time

T3	Activity time

T4	link_failure_detection_time

tx_messages_buffer 	A buffer >= maximum possible value ( k * 16 * (N1+1)).

tx_message_pointer 	A pointer to next message to be transmitted

unit_maximum_length 	The maximum message length this unit can accommodate. 	unit_maximum_window_size	The maximum window size this unit can accommodate.

unit_version	The highest MAP27 version number this unit knows.

CV	Current Version

### A2.2.9.	Variables

The following variables are defined for interfacing between the TASKs and IFs:

AR	LT_acknowledge_request

N1	maximum_length (number of octets is 16 * (N1+1))

C1	retransmission_count

k	window_size

N(k)	rx_credit_number

N(R)	rx_sequence_number

R(k)	receive_credit

S(k)	send_credit

SN(R)	stored_acknowledged_rx_sequence_number

SR(k)	stored_receive_credit

V(S)	send_state_variable

V(R)	receive_state_variable

### A2.2.10.	TASKs

The following TASKS are defined

adjust_link_parameters

-	Sets values of variables maximum_length, window_size and current_version to the minima of unit_maximum_length, unit_maximum_window_size, and unit_version, and the values of the corresponding parameters received in the most recently received 'link_request' message.

decrement retransmission_counter(C1)

-	Decrements the variable 'retransmission_count' (C1).

decrement_send_credit

-	Decrements the variable 'send_credit' (S(k)).

delete_acknowledged_packets

-	Removes acknowledged messages from the queue of network layer messages awaiting transmission by deleting all messages with 'stored_packet_number' less than the value of 'rx_sequence_number' (N(R)) in the last received link_acknowledgement message.

increment_send_state_variable

-	Increments the variable 'send_state_variable' (V(S)).

initialise_variables

-	Sets the following:

send_state = 1
receive_state = 1
receive_credit = k
send_credit = 0
stored_acknowledged_rx_sequence_number = 1
clears data link layer message buffers
clears RS-232 port buffers
retransmission_count = N2

initialise_rs232_port

-	This task sets the RS-232 port ready to operate at the pre-selected speed (9600 bps), with 8 bits, no parity, 1 start bit and 1 stop bit.

maximise_link_parameters

-	Set values of variables 'maximum_length' (N1), 'window_size' (k) and 'current_version' (VERSION) to the maximum permissible for this unit, i.e., unit_maximum_length, unit_maximum_window_size, and unit_VERSION.

record_send_credit

-	Store r the 'send_credit', S(k) as:

=N(k) - (V(s)-N(r))

= rx_credit_number - (send_state_variable - rx_sequence_number).

rewind_packet_number

-	Set 'send_state', V(S), to the value of the rx_sequence_number parameter (N(R)) of the last received link_acknowledgement message, and set tx_message_pointer to the corresponding message in the tx_messages_buffer, so that it will be the next message to be transmitted.

set_retransmission_counter

-	Sets the variable 'retransmission_count' (C1) to its maximum value, N2.

store_acknowledged_rx_sequence_number

-	Store latest received rx_sequence_number as stored_acknowledged_rx_sequence_number.

store_packet

-	Stores most recently received packet from network layer, and marks it with a 'stored_packet_number' equal to the current value of the variable 'transmit_buffer_top' and increments the variable 'transmit_buffer_top'.

Store_receive_credit

-	Stores the current 'receive_credit' (R(k)).as 'stored_receive_credit' SR(k).

update_receive_credit

-	Calculates from the last received 'credit_value' message from network layer the new 'receive_credit' (R(k)).

### A2.2.11.	IFs:

The following tests are defined:

acknowledgement_outside_window

-	Returns 'true' if the rx_sequence_number (N(R)) of the last received 'link_acknowledgement' message is outside the range of stored_acknowledged_rx_sequence_number SN(R) to send_state_variable V(S).

all_transmitted_packets_acknowledged

-	Returns 'true' if the 'rx_sequence_number' (N(R)) parameter of the last received 'link_acknowledgement' message is one greater than the 'tx_sequence_number' parameter of the last transmitted 'link_transfer' message (equal to current V(S)).

immediate_reply_requested

-	Returns 'true' if the 'acknowledgement_request' (AR) field of the most recently received link_transfer message is set to '1'.

packet_out_of_sequence

-	Returns 'true' if the sequence_number (N(S)) of the last received link transfer message is not equal to the 'receive_state_variable' (V(R)).

packet_outside_window

-	Returns ´true´ if the tx_sequence_number of the last received ´link transfer´message is outside the range of values given by the expression [(rx_sequence_number - stored receive_credit) to (rx_sequence_number-1 + rx_credit_number)], these variables being the values of parameters in the last transmitted ´link_acknowledge´messages.

receive_credit_available

-	Returns 'true' if there is enough buffer space available in the network layer to pass the network_layer_message content of the last received 'link_transfer' message to the network layer.

received_parameters_acceptable

-	Returns 'true' if the parameters in the last received 'link_request' message (maximum_length, window_size, version) are less than or equal to the maximum permissible values for this unit.

repeated_link_acknowledge

-	Returns 'true' if the rx_sequence_number of the last received 'link_acknowledge' message is equal to the rx_sequence_number of the last but one received 'link_acknowledge' message, and is not equal to 'send_state'.

retransmission_count_zero

-	Returns 'true' if the variable 'retransmission_count'  (C1) is zero.

send_credit_available

-	Returns 'true' if send_credit is greater than zero.

NOTE: Unless otherwise commented, INPUTs and OUTPUTs travel along the RS-232 link to the co-operating data_link process.*/

## A2.3.	SDL presentation

PROCESS data_link

STATE reset_wait

INPUT network_layer_reset	/* -- from the network layer -- */ /* or */

INPUT power_on					/* -- from operating system -- */

TASK initialise_rs232_port

TASK maximise_link_parameters

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT link_request

TASK adjust_link_parameters

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT link_wait

INPUT link_acknowledge

INPUT link_transfer

INPUT link_establishment_timeout(T0)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT network_layer_packet	/* -- from network layer -- */

OUTPUT packet_rejected	/* to network_layer -- */

EXIT reset_wait

STATEND reset_wait

/*********************/

STATE link_wait						/* The link establishment timer must be running in this state */

INPUT link_request

TASK adjust_link_parameters

IF received_parameters_acceptable THEN

TASK initialise_variables

TIMESTOP link_establishment_timer(T0)

OUTPUT link_acknowledge

OUTPUT link_ready		/* -- to network layer -- */

TIMESTART activity_timer(T3)

TIMESTART link_failure_detection_timer(T4)

EXIT ready

ELSE

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT link_wait

FI

INPUT link_acknowledge

TASK initialise_variables

TASK record_send_credit

TIMESTOP link_establishment_timer(T0)

OUTPUT link_acknowledge

OUTPUT link_ready			/* -- to network layer -- */

TIMESTART activity_timer(T3)

TIMESTART link_failure_detection_timer(T4)

EXIT ready

INPUT link_establishment_timeout(T0)

INPUT link_transfer

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT network_layer_reset	/* -- from network layer -- */

TASK initialise_rs232_port

TASK maximise_link_parameters

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT network_layer_packet	/* -- from network layer -- */

OUTPUT packet_rejected	/* Optional message to network_layer -- */

EXIT link_wait

INPUT power_on			/* formally copied from ready state, must be discussed */

TASK initialise_rs232_port

TASK maximise_link_parameters

OUTPUT link_failure		/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

STATEND link_wait

/*********************/

STATE ready

INPUT network_layer_packet	/* -- from network layer -- */

IF send_credit_available THEN

OUTPUT packet_accepted	/* -- to network layer -- */

TASK store_packet	/* in case we have to repeat it */

TASK set_retransmission_counter(C1)

OUTPUT link_transfer

TASK increment_send_state_variable

TASK decrement_send_credit_value

TIMESTART retry_timer(T1)

/* don't reset activity_timer - other party may need a credit_report

while we are sending a stream of link_transfer messages */

ELSE

OUTPUT packet_rejected

/* -- to network_layer, because no credit available to transmit it -- */

FI

EXIT ready

INPUT retry_timeout(T1)

TASK decrement_retransmission_counter(C1)

IF   retransmission_count_zero THEN

TASK maximise_link_parameters

OUTPUT link_failure	/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

ELSE

TASK rewind_packet_number

OUTPUT link_transfer

TIMESTART retry_timer(T1)

EXIT ready

FI

INPUT link_acknowledge

IF acknowledgement_inside_window THEN	/* The link is still working!*/

TIMESTART link_failure_detection_timer(T4)

TASK delete_acknowledged_packets

TASK record_send_credit

IF all_transmitted_packets_acknowledged THEN

TIMESTOP retry_timer(T1)

TASK set_retransmission_counter(C1)

TASK store_acknowledged_rx_sequence_number

EXIT ready

ELSE

IF repeated_link_acknowledge THEN

/* this is a request to send some packets again */

TASK decrement_retransmission_counter(C1)

IF retransmission_count_zero   THEN

TASK maximise_link_parameters

OUTPUT link_failure	/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

ELSE

TASK rewind_packet_number

OUTPUT link_transfer 	/* starting at requested packet number*/

TIMESTART retry_timer(T1)

EXIT ready

FI

FI

FI

ELSE 						/* acknowledgement is outside window */

TASK maximise_link_parameters

OUTPUT link_failure	/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)					TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

FI

INPUT link_request

TASK adjust_link_parameters

OUTPUT link_failure		/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT link_wait

INPUT power_on

TASK initialise_rs232_port

TASK maximise_link_parameters

OUTPUT link_failure		/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT network_layer_reset	/* -- from the network layer -- */

TASK initialise_rs232_port

TASK maximise_link_parameters

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait	/* Don't send 'link_failure' to network layer, as network layer initiated this transition*/

INPUT link_transfer

TIMESTART  link_failure_detection_timer(T4)

/* the link is working */

IF packet_outside_window THEN	/* Optional check; this is a data link error */

TASK maximise_link_parameters

OUTPUT link_failure	/* -- to network layer -- */

TIMESTOP acknowledgement_timer	/* Optional, not used in 'link-wait' state */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait		/* end of optional check */

ELSE

IF packet_out_of_sequence THEN	/* Packet is inside window */

TIMESTOP acknowledgement_timer(T2)

OUTPUT link_acknowledge	/* with expected paccket number */

TIMESTART activity_timer(T3)

ELSE

IF receive_credit_available THEN

OUTPUT network_layer_packet	/* -- to network layer */

TASK increment_receive_state

TASK decrement_receive_credit	/* if network_layer is slow to update credit */

IF immediate_reply_requested THEN

TASK store_receive_credit

TIMESTOP acknowledgement_timer(T2)

OUTPUT link_acknowledge

TIMESTART activity_timer(T3)

ELSE

TASK store_receive_credit

TIMESTART acknowledgement_timer(T2)

FI

ELSE					/* no space to store the packet */

TASK store_receive_credit

TIMESTOP acknowledgement_timer(T2)

OUTPUT link_acknowledge

TIMESTART activity_timer(T3)

FI

FI

EXIT ready

FI

INPUT acknowledgement_timeout(T2)

OUTPUT link_acknowledge

TIMESTART activity_timer(T3)

EXIT ready

INPUT activity_timeout(T3)

TIMESTOP acknowledgement_timer(T2)

OUTPUT link_acknowledge

TIMESTART activity_timer(T3)

Exit ready

INPUT link_failure_detection_timeout(T4)

TASK maximise_link_parameters

OUTPUT link_failure	/* -- to network layer -- */

TIMESTOP retry_timer(T1)

TIMESTOP acknowledgement_timer(T2)

TIMESTOP activity_timer(T3)

TIMESTOP link_failure_detection_timer(T4)

OUTPUT link_request

TIMESTART link_establishment_timer(T0)

EXIT reset_wait

INPUT credit_value			/* -- from network layer -- */

TASK update_receive_credit

IF receive_credit_available THEN

TIMESTOP acknowledgement_timer

OUTPUT link_acknowledge

TIMESTART activity_timer(T3)

FI

EXIT ready

STATEND ready

/*********************/

PROCEND data_link