# 2.	Definitions, abbreviations and symbols

## 2.1.	Definitions

Character:	Eight bits (an octet) of data combined with Start and Stop bits.

Control message:	A message intended for layer control.

Data message:	A message intended for the transmission of higher layer data (user data).

DLE stuffing:	A method to ensure transparent data transmission and reliable packet start and end indication.

Header:	The layer control part of a message.

Message:	A generic name of control and data messages.

Modem call:	A data call using an external or internal modem and data transfer protocol, which are not defined in MPT 1327. The equivalent expression in the MPT 1327 standard is the 'non-prescribed data call'.

Packet body:	The section of the data link control and higher layer information between start and stop sequences.

Start bit:	The bit which signifies the beginning of an asynchronous character.

Stop bit:	The bit which signifies the end of an asynchronous character.

Start flag:	A unique data link packet start sequence used for receiver synchronisation.

Stop flag:	A unique data link packet end sequence used for receiver synchronisation.

## 2.2.	Abbreviations

7-bit ASCII:	International Alphabet No. 5

8-bit ASCII:	Personal computer (PC) character set

ADESC:	Address descriptor

BCD:	Binary coded decimal

CRC:	Cyclic redundancy check

DCE:	Data communication equipment

DLE:	Data link escape (10h)

DTE:	Data terminal equipment

DUMMYI:	Dummy ident, all bits '0'

DUMMYP:	Dummy prefix, all bits '0'

ETX:	End of text (03h)

FCS:	Frame check sequence

H_byte:	Most significant Byte of a 16-bit value

IA2:	International Telegraph Alphabet No. 2

IA5:	International Alphabet No. 5

L_byte:	Least significant Byte of a 16-bit value

LSB:	Least significant bit

MSB:	Most significant bit

MST:	Multiple segment transaction (Extended data message)

NLB:	Number of last bits

NULL:	Null BCD character (0Fh)

NUL:	Null (00h)

RU:	Radio unit

RX:	Receiver

SST:	Single segment transaction (Short data message)

STX:	Start of text (02h)

SYN:	Synchronous idle (16h)

TX:	Transmitter

## 2.3.	Symbols

C1:	The retransmission counter.

h:	Hexadecimal notation, e.g. 2Ah is equal to 42 decimal.

AR:	Parameter for immediate acknowledgement request.

k:	The maximum number of unacknowledged sequentially numbered user data packets at a specific time.

N1:	The maximum information length of an LT packet.

N2:	The maximum number of retransmissions of a packet.

N3:	The number of activity timeouts before link failure is detected.

T0:	The retry waiting time in the establishment phase after whose expiry a retransmission of a packet is initiated.

T1:	The retry waiting time in the data transfer phase after whose expiry a retransmission of a packet is initiated.

T2:	The waiting time after whose expiry transmission of an acknowledgement is initiated.

T3:	The inactivity time after whose expiry transmission of an acknowledgement to report the receive state is initiated.

T4:	Link failure detection time.

VERSION	Protocol version number.

## 2.4.	Radio path definitions

This section is an incomplete and shortened extract from MPT 1327.

ACK:	A general positive acknowledgement message which terminates the transaction.

ACKB:	A call-back acknowledgement message which terminates the transaction.

ACKI:	An intermediate response message.

ACKQ:	A call request queued message.

ACKT:	A call diversion message which terminates the transaction.

ACKV:	A message which terminates a call or transaction attempt for operational reasons.

ACKX:	A message which terminates a call or transaction attempt for functional reasons.

AHY:	A radio availability check message.

AHYC:	A short data message invitation.

AHYQ:	A status message to a radio unit.

AHYX:	A message sent to a called radio unit to cancel an incoming traffic call addressed to the unit.

CLEAR:	A call disconnection message.

Codeword:	A radio path message containing 48 bits of data and/or control information, 15 bits for error detection and error correction and one bit parity.

Dataitem:	An element of a set of standard data which is used at the radio path protocol. The whole or part of a Tmessage.  A dataitem has a length of 10 bits + n * 47 bits, where n varies from 0 to 62, i.e. the maximum length of 2924 bits. The actual length of a dataitem used on the radio path is not controllable by the user.

Extended addressing:	Means of sending a call request where the PFIX of called party is different to that of the calling party.

GTC:	Go to the Traffic Channel message for a voice or modem call connection.

GTT:	Go To Transaction message for radio units which are to commence a standard data call.  HEAD + data:	A user data message also known as Dataitem.

HEAD + data:	A short data message sent by the radio unit in response to AHYC and by the system without prompting.

IDENT:	The least significant 13 bits of a radio protocol address.

MAINT:	A call maintenance message.

PFIX:	The most significant 7 bits of a radio protocol address.

RQC:	A short data request message.

RQE:	An emergency voice or modem call set-up request message.

RQQ:	A status message.

RQS:	A voice or modem call set-up request message.

RQX:	A request to cancel a previous call request or abort a transaction e.g. a status transaction.

SAMIS:	A message used to send, for example, extended addressing information or telephone dialling data.

Tmessage:	Standard data user message on the radio path interface. May be of any length. May be used also to indicate any user message.

TRANS:	A temporary address used in the standard data protocol.