# APPENDIX A8.	Values of system parameters

| ITEM NO. | ITEM | STORAGE TYPE | ITEM SIZE/ RANGE OF VALUES | NO. 0F ENTRIES | SOURCE OF DATA | M/O SO | COMMENTS | REFERENCE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Transmission speed | A | 2 bits | 1 | NP | M | 0 = 9600 bit/s 1 = 1200 bit/s 2 = customer defined | 3.4 |
| 2 | Maximum length N1 | A | 8 bit | 1 | NP | M |  | 4.3.2.1 |
| 3 | Maximum window size k | A | 8 bit | 1 | NP | M |  | 4.3.2.1 |
| 4 | Acknowledgement request | A | 2 bit | 1 | NP | M | 0 = set to 0 1 = set to 1 2 = variable | 4.3.2.3 |
| 5 | Link establishment timer T0 | A | 8 bit | 1 | NP | M | 100 ms resolution maximum 15 s | 4.4.1.1 |
| 6 | Transfer phase retry timer T1 | A | 8 bit | 1 | NP | M | 100 ms resolution | 4.4.2.1 |
| 7 | Acknowledgement timer T2 | A | 8 bit | 1 | NP | M | 100 ms resolution maximum 15 s | 4.4.2.2 |
| 8 | Activity timer T3 | A | 8 bit | 1 | NP | M | 100 ms maximum 15 s | 4.4.2.3 |
| 9 | Link failure detection time T4 | A | 8 bit | 1 | NP | M | T4 > N3 * T3 | 4.4.2.6 |
| 10 | Retransmission count N2 | A | 8 bit | 1 | NP | M | standard 10 | 4.4.2.6 |
| 11 | Link activity count N3 | A | 8 bit | 1 | NP | M | standard 5 N3 >=2 | 4.4.2.6 |