5.		CODEWORD STRUCTURES

This section lists the codewords used in the standardised messages and defines their structure.  A brief indication of the usage of the messages is given, but readers should refer to the procedures sections for a full definition of usage.  Readers may find it helpful to study the procedures sections together with this section rather than consecutively.

It is not a mandatory requirement on either a TSC or a radio unit to understand the meaning of all the standardised messages.  The messages that must be used will depend on the facilities implemented in a TSC and a radio unit; the procedures sections define which messages are required.

Standardised fields

The codewords are shown broken down into their constituent fields, with a definition of the meaning of each field.  The fields in the codewords shall be set to appropriate values.  Machine transmission of fields is most significant bit first.

In this standard, the numerical value of a field is referred to either by the decimal equivalent of the bit sequence concerned, with leading zeros suppressed, or in binary.  Binary values are shown enclosed in apostrophes, e.g. Type '11', except in the codeword diagrams in this section.

When the prefix is not required to complete an address (e.g. for special ident ALLI), it may be set to an arbitrary value and, on reception, its value shall be considered to have no significance.

Reserved fields

Some fields are designated as "reserved".  In the future development of this standard, the whole or any part of a reserved field may be designated for a specific purpose.  Any such designation will be made in a manner that does not cause any existing application of this standard to cease to comply with the standard or to suffer a reduction in its functionality.  Neither the existing protocol procedures nor the already designated meanings of messages will be changed in order to bring a reserved field into service.

Therefore, equipments complying with this issue of the standard shall, on transmission, set reserved fields to the specified default value and, on reception, shall consider the value of reserved fields to have no significance.  Equipments which understand the designation of any previously reserved field shall, on transmission, note that the recipient equipment may attribute no significance to that field or, on reception, shall be prepared to accept the default value of that field.

Spare fields and codewords

There are "spare" fields and codewords available for customisation of services (see section 5.2).  Spare fields and codewords will never be used within this standard, but may be designated for a specific purpose within any given application of this standard.  In applications where spare fields or codewords are employed, rules shall be generated governing their use. Any designation of spare fields and codewords shall not modify the meaning of standardised fields and codewords.

Unless a radio unit knows the meaning of spare fields and codewords on the system it is currently using, it shall not transmit spare messages to the TSC, nor take any action on receiving spare messages from the TSC, nor use the spare fields in standardised messages received from the TSC.

5.1		System Codewords

5.1.1	Control Channel System Codeword (CCSC)

The Control Channel System Codeword is transmitted on a control channel by a TSC in order to identify the system to radio units and to provide control channel slot synchronisation (see section 3.3.3).  It is a data codeword, structured as shown below.

|  | 0 | SYS | CCS | PREAMBLE 1010101010101010 | P 1100010011010111 |
| --- | --- | --- | --- | --- | --- |
| no. of bits | 1 | 15 | 16 | 16 | 16 |

SYS	-	System identity code of the transmitting system.

Values of SYS which result in production of the control channel codeword synchronisation sequence, SYNC, in any part of the 48 information bits of the CCSC are not permitted.

CCS	-	Codeword Completion Sequence, chosen so that the parity check bits P always form the control channel codeword synchronisation sequence.  The bit values of the CCS will  depend on the system identity code; an algorithm for generating the CCS is given in Appendix 3.

PREAMBLE	-	Preamble bit reversals, ending with a '0'.

P	-	Parity check bits.

These complete the codeword and also form the control channel codeword synchronisation sequence, SYNC (section 3.2.1.1).

5.1.2	Data Channel System Codeword (DCSC)

The DCSC is transmitted on a data channel by the TSC in order to identify the system to radio unit's and to provide data channel slot synchronisation.  It is a data codeword as shown below:

|  | 0 | SYS | DCS | PREAMBLE 1010101010101010 | P 0011101100101000 |
| --- | --- | --- | --- | --- | --- |
| no. of bits | 1 | 15 | 16 | 16 | 16 |

SYS	-	System identity code of the transmitting system. Values of SYS which result in production of the data channel codeword synchronisation sequence, SYNT, in any part of the 48 information bits of the DCSC are not permitted.

DCS	-	Data-codeword Completion Sequence, chosen such that the parity check bits form the data channel sychronisation sequence (SYNT).  An algorithm for determining the codeword completion sequence (DCS) of a data channel system codeword (DCSC) is given in appendix 3.

PREAMBLE	-	Preamble bit reversals, ending with a '0'.

P	-	Parity check bits.

These complete the codeword and also form the Standard Data channel codeword synchronisation sequence, SYNT (section 3.2.1.2).

5.2		General Address Codeword Structure

There is a general address codeword structure which is divided into 8 categories, and one special structure for a "Go To Channel" message (see 5.4).  The general and GTC structures are distinguished by Bit 22 of the codeword; Bit 22 of the general structure is always '1' whereas Bit 22 of the GTC codeword is set to '0'.

The most usual general structure is shown below.

| no. of bits | 1 | 20 | 1 | 3 | 23 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | PARAMETERS | 1 | CAT | other fields | PARITY |

:	:	:

:	:	2	:	3	:	18	:

| Category '000' Messages: | 000 | TYPE | FUNC | PARAMETERS |
| --- | --- | --- | --- | --- |

:	:		:		:

|  | : | 00 | FUNC | 8 Alohas |
| --- | --- | --- | --- | --- |
|  | : | 01 | FUNC | 8 Acknowledgments |
|  | : | 10 | FUNC | 8 Requests/Ahoys |
|  | : | 11 | FUNC | 8 Miscellaneous |

:	:		:

:	:	2	:	22	:

| Category '001' Messages: | 001 | TYPE | PARAMETERS |
| --- | --- | --- | --- |

:	:		:

|  | : | 0 | Single address messages |
| --- | --- | --- | --- |
|  | : | 1 | Short Data Message |

:	:		:

:	:	1	:	22	:

| Category '010' Standard Data |  |  | 010 |  |  | KIND | PARAMETERS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Messages: |  |  |  | : : |  |  |  |  |  |
|  |  | : |  |  |  | 0 | Go To Transaction and General Information |  |  |  |
|  |  | : |  |  |  | 1 | Request/Ahoy |  |  |  |

:	:	:

|  | 011 | spare |
| --- | --- | --- |
|  | 100 | spare |

:	:

:	:	1	:	4	:	18	:

| Category '101' Standard Data |  |  | 101 |  | KIND |  |  | JOB |  | PARAMETERS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Messages: |  |  |  |  |  | : : |  |  |  |  |
|  |  | : |  |  | 0 |  |  | JOB |  |  |  |

:	:	:	:

|  | : |  | 0xxxx | 6 Acknowledgements |
| --- | --- | --- | --- | --- |
|  | : |  | 1xxxx | 3 Requests, 4 Ahoys |

:	:		:		:

:	:	1	:	1	:	21	:

|  |  | 101 | KIND | TASK | PARAMETERS |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | : |  | 0 | TASK |  |  |

:	:	:	:

|  |  |  | 0 | 1 Selective Acknowledgement |
| --- | --- | --- | --- | --- |
|  | : |  | 1 | 1 Fragment address codeword |

:	:

|  | 110 | reserved |
| --- | --- | --- |
|  | 111 | reserved |

Categories zero, one two, and five (CAT = '000', '001', '010' and '101') contain standardised codewords.  The "reserved" codewords are intended for future expansion of the standard message set, whereas the "spare" codewords may be used for customisation of services.

It is anticipated that reserved categories could be used for the definition of polling and other data communication protocols etc. in a future phase of standardisation.

5.3	List of Address Codewords

Mnemonic 		Meaning 	Section

GTC Message:		GTC 		Go to channel command 		5.4

CAT '000' Messages:						5.5

TYPE '00'		Aloha invitations: 		5.5.1

ALH  		general

ALHS 		standard data excluded

ALHD 		"Simple" calls excluded

ALHE 		emergency only

ALHR 		registration or emergency

ALHX 		registration excluded

ALHF 		fall-back mode

reserved

TYPE '01'	Acknowledgements:		5.5.2

ACK 		general

ACKI		intermediate

ACKQ		call queued

ACKX		message rejected

ACKV		called unit unavailable

ACKE		emergency

ACKT		try on given address

ACKB		call-back / negative ack

TYPE '10		Requests (sent by RUs):		5.5.3.1

RQS		"Simple"		5.5.3.1.1

spare		5.5.3.1.2

RQX		cancel/abort 		5.5.3.1.3

RQT		divert		5.5.3.1.4

RQE		emergency		5.5.3.1.5

RQR		registration 		5.5.3.1.6

RQQ		status		5.5.3.1.7

RQC		short data		5.5.3.1.8

Ahoys (sent by TSC):		5.5.3.2

AHY	general availability check		5.5.3.2.1

spare for customisation		5.5.3.2.2

AHYX 		cancel alert/waiting state		5.5.3.2.3

reserved				5.5.3.2.4

reserved				5.5.3.2.5

AHYP	Called Unit Presence Monitoring		5.5.3.2.6

AHYQ 		status message 		5.5.3.2.7

AHYC 		short data invitation		5.5.3.2.8

TYPE '11'		Miscellaneous:	5.5.4

MARK 		control channel marker 		5.5.4.1

MAINT		call maintenance		5.5.4.2

CLEAR		call clear-down 		5.5.4.3

MOVE 		move control channel 		5.5.4.4

BCAST		broadcast 		5.5.4.5

reserved

reserved

reserved

contd.

List of Address Codewords, contd.

Mnemonic 		Meaning 	Section

TYPE '0'				Single address messages:		5.6.1

CAT '001' Messages: 						5.6

SAMO 		Outbound		5.6.1.1

Inbound:		5.6.1.2

SAMIU			inbound unsolicited		5.6.1.2.1

SAMIS			inbound solicited		5.6.1.2.2

TYPE '1'		HEAD 		Short data message		5.6.2

CAT '010' Messages:		Codewords applicable to Standard Data		5.7

Kind '1'		RQD 		Request for Standard Data		5.7.1

AHYD		Availability check		5.7.2

Kind '0'		GTT		Go To Transaction 		5.7.3

DRUGI		Radio Unit General Information		5.7.4

CAT '011' Messages:    	spare

CAT '100' Messages:    	spare

CAT '101' Messages:	Standard Data 			5.8

KIND '0'

JOB	FROM TSC	FROM radio unit

'0000' 	DACK+DAL 				5.8.2

'0001'	DACK+DALG				5.8.2

'0010'	DACK+DALN				5.8.2

'0011'	DACK+GO	DACK+GO			5.8.2

'0100'	DACKZ  	DACKZ			5.8.3

'0101'	DACKD  		DACKD			5.8.1

'0110'	reserved

'0111'	spare

'1000'	DAHY 	RSVD			5.8.4

'1001'	RSVD 	RSVD

'1010'	RSVD 	DRQG			5.8.8

'1011'	RSVD 	RSVD

'1100'	DAHYZ	DRQZ			5.8.9

'1101'	RSVD 	RSVD

'1110'	DAHYX	DRQX		5.8.6/5.8.10

'1111'	RLA  	RLA 			5.8.7

KIND '1'

TASK

'0'		SACK	SACK			5.8.11

'1'		SITH	SITH			5.8.12

CAT '110' Messages:		reserved

CAT '111' Messages:		reserved

5.4		Go To Traffic Channel Message, GTC

This message is transmitted on a control channel from a TSC to radio units.  It directs the addressed radio units to switch to a designated channel and proceed with communication.

This message may also be transmitted on a traffic channel to move radio units already in communication to a replacement traffic channel.  When the units have retuned to the replacement channel, communication may continue.

|  | 1 | PFIX | IDENT1 | 0 | D | CHAN | IDENT2 | (N) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| no. of bits | 1 | 7 | 13 | 1 | 1 | 10 | 13 | 2 | 16 |

PFIX	-	Unit or group prefix.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call, a call from a PABX extension or from the PSTN, or an Include call.

Ident	-	for an interprefix call when the message is sent to the called party.

IPFIXI 	-	for an interprefix call when the message is sent to the calling party.

ALLI	-	for a system-wide call.

PABXI	-	for a call to a PABX extension.

PSTNSIj	-	for a call to a prearranged PSTN destination.

PSTNGI	-	for a call to a general PSTN destination.

DUMMYI 	-	for an intersite call where the calling party is active on the same timeshared control channel.  (Option 1)

TSCI	-	for an intersite call where the calling party is active on the same timeshared control channel.  (Option 2)

D	-	'0' 	if the addressed radio units shall unmute the audio (for speech communication).

'1'	if the addressed radio units shall mute the audio (for data communication), and need not send maintenance messages within items unless required by the system by prearrangement.

CHAN	-	Designates the allocated channel number.

IDENT2	-	Calling party or gateway:

Ident	-	for a common-prefix call to a unit or group of units, a system-wide call, or a call to a PABX extension or to the PSTN.

Ident	-	for an interprefix call when the message is sent to the calling party.

IPFIXI	-	for an interprefix call when the message is sent to the called party.

PABXI	-	for a call from a PABX extension.

PSTNGI	-	for a call from the PSTN.

INCI	-	for an Include call.

DUMMYI	-	for an intersite call where the called party is active on the same control channel.  (Option 1)

TSCI	-	for an intersite call where the called party is active on the same control channel.  (Option 2)

(N)	-	Aloha number.

See random access protocol (section 7).

P	-	Parity check bits.

5.5		Category '000' Messages

5.5.1		Aloha Messages (Type '00')

These messages are transmitted on a control channel by a TSC - see section 7.  They invite radio units to transmit single codeword random access messages in the designated frame of timeslots.  An Aloha message may also be used to demand a response from an individually addressed radio unit.

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 00 | FUNC | CHAN4 | WT | RSVD | (M) | (N) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 4 | 3 | 2 | 5 | 4 | 16 |

PFIX	-	Prefix (see also IDENT1).

IDENT1	-	PFIX/IDENT1 specifies the radio units that are invited to transmit.  Only the (M) least significant bits of the 20-bit address are used; the remaining address bits may be set arbitrarily.

CAT	-	'000'.

TYPE	-	'00'.

FUNC	-	Specifies the function of the Aloha invitation:

'000'	ALH	Any single codeword message invited.

'001'	ALHS	Messages invited, except RQD.

'010'	ALHD	Messages invited, except RQS.

'011'	ALHE	Emergency requests (RQE) only invited.

'100'	ALHR	Registration (RQR) or emergency requests (RQE) invited.

'101'	ALHX	Messages invited, except RQR.

'110'	ALHF	Fall-back mode; messages invited only from radio units which know the fall-back method used by this system.

The fall-back mode is a customised mode of operation used only in the case of equipment malfunction.

'111'		Reserved for future use.

The rules defining the Aloha functions appropriate to customised random access messages are system-dependent.

CHAN4	-	Least significant four bits of the channel number of the control channel on which the message is sent;  (to protect against breakthrough).

contd.

WT	-	Delay parameter for repeat transmissions.

See random access protocol (section 7).

RSVD	-	Reserved for future definition.  Default value = '00'.

(M)	-	Address qualifier.

See random access protocol (section 7).

(N)	-	Aloha number.

See random access protocol (section 7).

P	-	Parity check bits.

5.5.2	Acknowledgement Messages (Type '01')

These messages may be sent by the TSC at various stages of call set-up, and by a radio unit in response to a TSC message that demands a reply. The meanings of these messages vary both according to the function of the messages they acknowledge, and according to the source.

The basic structure of the acknowledgements is illustrated below but, for clarity, it is shown separately for TSC source and radio unit source in subsections 5.5.2.1 and 5.5.2.2 respectively.

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 01 | FUNC | IDENT2 | QUAL | (N) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 4 | 16 |

PFIX	-	Prefix.

IDENT1	-	Ident of the called party or gateway.

(This is the called party or gateway in the call for which the acknowledgement is being sent.)

CAT	-	'000'.

TYPE	-	'01'.

FUNC	-	Specifies the function of the acknowledgement:

'000'	ACK	General acknowledgement

'001'	ACKI	Intermediate acknowledgement, more signalling to follow

'010'	ACKQ	Acknowledge, call queued

'011'	ACKX	Acknowledge, message rejected

'100'	ACKV	Acknowledge, called unit unavailable

'101'	ACKE	Acknowledge emergency call

'110'	ACKT	Acknowledge, try on given address

'111'	ACKB	Acknowledge, call-back, or negative acknowledgement.

IDENT2	-	Ident of the calling party or gateway.

(This is the ident of the party or gateway that originated the call for which the acknowledgement is being sent.)

QUAL	-	Qualifies the function (FUNC) of the acknowledgement.

(N)	-	Aloha number in messages transmitted by a TSC.

Reserved in messages transmitted by radio units; 
default value = '0000'.

P	-	Parity check bits.

5.5.2.1	Acknowledgement messages sent by the TSC

The acknowledgement messages may be sent by the TSC at various stages of call set-up (or during transactions), to indicate the progress of the call.  Data codeword(s) may be appended to an ACKT address codeword to convey additional information, depending on the value of IDENT1.

Acknowledgement address codeword:

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 01 | FUNC | IDENT2 | QUAL | (N) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 4 | 16 |

PFIX	-	Prefix of the calling radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call.

IPFIXI	-	for an interprefix call.

ALLI	-	for a system-wide call.

PABXI	-	for a call to a PABX extension.

PSTNSIj	-	for a call to a prearranged PSTN destination.

PSTNGI	-	for a call to a general PSTN destination.

TSCI	-	for an RQQ or RQC transaction with the TSC.

DIVERTI	-	for general cancellation by a recipient of diversions.

In acknowledgements to RQR, IDENT1 = REGI.

In ACKT(QUAL=0), IDENT1 is the diversion ident or gateway; if IDENT1 = IPFIXI, PABXI or PSTNGI, the diversion address is given in appended data codeword(s) - see below.  If a call to an individual address has been diverted to a group address, or vice versa, IDENT1 in ACKT is set to IPFIXI and the diversion address is given in an appended data codeword (with bit GF set appropriately).

In acknowledgements to RQR, IDENT1 = REGI.

CAT	-	'000'.

TYPE	-	'01'.

FUNC	-	ACK, ACKI, ACKQ, ACKX, ACKV, ACKE, ACKT or ACKB.

IDENT2	-	Ident of the calling radio unit.

QUAL	-	Qualifies the function (FUNC) of the acknowledgement. 
See below.

(N)	-	Aloha number.
See random access protocol (section 7).

P	-	Parity check bits.

contd.

Data codewords following ACKT(QUAL=0) address codeword:

When ACKT(QUAL=0) is transmitted by the TSC, up to three data codewords may be appended to convey the diversion address or dialling information. The form of these data codewords depends on the value of IDENT1.

a.  If IDENT1 = PSTNGI  then up to three data codewords with the following structure may be appended to ACKT(QUAL=0):

|  | 0 | RSA | FCW | eleven BCD digits | P |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | 2 | 11 x 4 | 16 |

RSA	-	Return Slot Access Flag.

When transmitted by the TSC on a control channel in the second half of a slot preceding an access slot:

'0'	-	radio units are not permitted random access in the following slot on the return control channel.

'1'	- 	radio units are permitted random access in the following slot on the return control channel.

In all other cases of transmission, the meaning of the RSA flag is reserved, default='0'.  See also 7.2.5.

FCW	-	Number of data codewords appended to this data codeword (in the same message):

'00'  no data codewords follow

'01'  one data codeword follows

'10'  two data codewords follow

'11'  reserved.

BCD	-	Eleven BCD groups representing the dialled digits of the diversion PSTN destination, coded in accordance with the table in Appendix 5.  The BCD digits are transmitted in the dialled order (i.e. the leftmost digit in the above diagram is the earliest in the dialling order; digits in any following codeword are later in the dialling order).

P	-	Parity check bits.

contd.

b.  If IDENT1 = PABXI 		then a single data codeword with the following       structure is appended to ACKT(QUAL=0):

| 0 | RSVD | SP | PARAMETERS | P |
| --- | --- | --- | --- | --- |
| 1 | 10 | 1 | 36 | 16 |

RSVD	-	Reserved for future definition.  Default value = all '0's.

SP	-	'0' for a "long" PABX extension number.

'1' for an extension number that can be represented by 13 bits.

PARAMETERS	-	See parameter formats below.

P	-	Parity check bits.

Parameter formats

| If SP='0' | BCD1 | BCD2 | BCD3 | BCD4 | BCD5 | BCD6 | BCD7 | BCD8 | BCD9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

BCDn	-	BCD groups representing the dialled digits of the diversion PABX destination, coded in accordance with the table in Appendix 5.  The BCD digits are  transmitted in the dialled order.

| If SP='1' | RSVD | EXCHANGE | Number |
| --- | --- | --- | --- |
|  | 21 | 2 | 13 |

RSVD	-	Reserved for future definition.  Default value = all '0's.

EXCHANGE	-	Indicates the appropriate PABX exchange.

Number	-	PABX extension number.

contd.

c.  If IDENT1 = IPFIXI 	then a single data codeword with the following structure is appended to ACKT(QUAL=0):

| 0 | RSVD | GF | PFIXT | IDENTT | P |
| --- | --- | --- | --- | --- | --- |
| 1 | 26 | 1 | 7 | 13 | 16 |

RSVD	-	Reserved for future definition.  Default value = all '0's.

GF	-	'0' if the diversion address is an individual address.

'1' if the diversion address is a group address.

PFIXT	-	Prefix of the diversion address.

IDENTT	-	Ident of the diversion address.

P	-	Parity check bits.

contd.

Acknowledgements sent to calling radio unit to indicate progress of Simple or emergency call (as requested by an RQS or RQE message):

ACKI (QUAL=0)	-	Called unit alerting but user/ data equipment not ready.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	All traffic channels in use.  TSC has queued the call.

ACKQ (QUAL=1)	-	Conflicting call in progress (e.g. called unit engaged), or higher in queue.  TSC has queued the call.

ACKX (QUAL=0)	-	Invalid call; request rejected.

ACKX (QUAL=1)	-	System overload; request rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or call set-up abandoned.

ACKV (QUAL=1)	-	Conflicting call in progress or higher in queue (and call has not been queued), or called user does not wish to receive this call.

ACKB (QUAL=0)	-	Called unit has accepted the call for call-back.

ACKT (QUAL=0)	-	Called party's calls have been diverted.

Acknowledgements sent to radio unit to indicate progress of Include call 
(as requested by an RQS on a traffic channel):

ACK (QUAL=0)	-	Include request accepted; called party will be directed to the traffic channel.

ACKI (QUAL=0)	-	Called party alerting but not yet ready.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	All traffic channels in use on called site; more signalling to follow.

ACKX (QUAL=0)	-	Invalid call; request rejected.

ACKX (QUAL=1)	-	System overload; request rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or Include call abandoned.

ACKV (QUAL=1)	-	Conflicting call in progress (e.g. called party engaged), or called user does not wish to receive this call.

ACKT (QUAL=0)	-	Called party's calls have been diverted.

Acknowledgements sent during set-up of standard data communication 
(as requested by an RQD message):

These acknowledgements are defined in Sections 17.1.1.1.1, 17.1.1.1.2 and 17.1.1.1.4.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	All data channels are busy.  Wait for further signalling.

ACKQ (QUAL=1)	-	Called party is engaged (and calling party requires interactive contact).  Wait for further signalling.

ACKT(QUAL=0)	-	Called party's calls have been diverted.

ACKX (QUAL=0)	-	Invalid call e.g. TSC or called party does not support standard data at least from this caller, or the radio unit is blacklisted, or called address is unobtainable.

ACKX (QUAL=1)	-	System overload, or for INTER='1' the called party is engaged or will not interact at this time and the TSC has not queued the call; request rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or call set-up abandoned.

ACKV (QUAL=1)	-	Call abandoned because the called party is unable to receive a call with the required facilities, e.g. the called radio unit does not support HADT or interaction or cannot accept the requested PORT.

Acknowledgements to cancel/abort message RQX:

ACK (QUAL=1)	-	Acknowledgement of RQX.

Acknowledgements to call diversion request RQT:

ACK  (QUAL=0)	-	Call diversion or cancellation has been accepted.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKX (QUAL=0)	-	Invalid call; request rejected.

ACKX (QUAL=1)	-	System overload; request rejected.

ACKV (QUAL=0)	-	Transaction abandoned.

Acknowledgements sent as response to emergency call request RQE:

ACKE (QUAL=0)	-	Acknowledgement, wait for further signalling.

Acknowledgements to registration request RQR:

ACK  (QUAL=0)	-	Registration accepted.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKX (QUAL=0)	-	Invalid request; registration denied.

ACKX (QUAL=1)	-	System overload; registration failed.

Acknowledgements sent to radio unit to indicate progress of status transaction 
(as requested by an RQQ message):

ACK  (QUAL=0)	-	Transaction has been successfully completed, i.e. the called destination has accepted the status information.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	System is busy.  Wait for further signalling.

ACKQ (QUAL=1)	-	Called unit engaged.  Wait for further signalling.

ACKX (QUAL=0)	-	Invalid call; message rejected.

ACKX (QUAL=1)	-	System or called unit overload; message rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or transaction abandoned.

ACKV (QUAL=1)	-	Called unit engaged (and TSC will not hold the request), or called unit does not wish to accept the information.

ACKT (QUAL=0)	-	Called unit's calls have been diverted.

Acknowledgements sent to radio unit to indicate progress of short data transaction 
(as requested by an RQC message):

ACK  (QUAL=0)	-	Transaction has been successfully completed.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	System is busy.  Wait for further signalling.

ACKQ (QUAL=1)	-	Called party engaged.  Wait for further signalling.

ACKX (QUAL=0)	-	Invalid call; message rejected.

ACKX (QUAL=1)	-	System or called unit overload; message rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or transaction abandoned.

ACKV (QUAL=1)	-	Called party engaged (and TSC will not hold the request) or called unit does not wish to accept the message.

ACKT (QUAL=0)	-	Called party's data calls have been diverted.

5.5.2.2		Acknowledgement messages sent by radio units

An acknowledgement may be sent by a radio unit in response to a TSC Ahoy or HEAD message that demands a reply.  An acknowledgement is also sent if a radio unit receives an individually addressed Aloha message and has no suitable random access message to send (see 7.4.1 and 8.3.2.1).

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 01 | FUNC | IDENT2 | QUAL | (N) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 4 | 16 |

PFIX	-	Prefix of the transmitting unit.

IDENT1	-	IDENT1 from the TSC's message, unless the acknowledgement is sent in response to an individually addressed Aloha message, in which case IDENT1 is set to TSCI.

CAT	-	'000'.

TYPE	-	'01'.

FUNC	-	ACK, ACKI, ACKX, ACKV or ACKB.

IDENT2	-	IDENT2 from the TSC's message, unless the acknowledgement is sent in response to an individually addressed Aloha message, in which case IDENT2 is the transmitting unit's ident.

QUAL	-	Qualifies the function (FUNC) of the acknowledgement.

(N)	-	Reserved for future definition.  Default value = '0000'.

P	-	Parity check bits.

Acknowledgements to AHY (i.e. general availability check):

i) From called unit (PFIX/IDENT1 from AHY):

ACK (QUAL=0)	-	General acknowledgement; unit/ user/ data equipment ready for call.

ACKI (QUAL=0)	-	Unit alerting but user/ data equipment not ready.

ACKX (QUAL=0)	-	Call cannot be accepted (e.g. no data equipment, for a data call).

ACKV (QUAL=1)	-	User does not wish to receive this call

(e.g. his "Busy control" is in the busy state).

ACKB (QUAL=0)	-	Call accepted for call-back.

ACKB (QUAL=1)	-	Data codeword appended to AHY was not decodeable and unit requires the message to be retransmitted.

ii) From requesting unit (PFIX/IDENT2 from AHY):

ACK  (QUAL=0)	-	Unit is waiting for signalling for a call.

ACKX (QUAL=0)	-	Unit is not waiting for signalling for a call.

contd.

Acknowledgement from called unit to AHYX (cancel alert/waiting state):

ACK  (QUAL=1)	-	Acknowledgement.

Acknowledgement from called unit to AHYP (called unit presence monitoring):

ACK (QUAL=1)	-	Acknowledgement

Acknowledgements from called unit to AHYQ (status message):

ACK (QUAL=0)	-	Unit has accepted the information in the AHYQ message.

ACKX (QUAL=0)	-	Unit is not equipped to accept the information.

ACKX (QUAL=1)	-	Unit cannot accept the information at this time (e.g. its queue is full).

ACKV (QUAL=1)	-	Unit does not wish to accept status information from this calling party.

ACKB (QUAL=1)	-	Data codeword appended to AHYQ was not decodeable and the unit requires the message to be retransmitted.

Acknowledgements to AHYC (instruction to send short data):

i)  From called unit  (PFIX/IDENT1 from AHYC):

ACKX (QUAL=0)	-	Unit is not equipped to send the required data.

ii)  From calling unit (PFIX/IDENT2 from AHYC):

ACKX (QUAL=0)	-	Unit is not waiting to send address information or short data message.

Acknowledgements from called unit to HEAD message (short data):

ACK  (QUAL=0)	-	Unit has accepted the information in the HEAD message.

ACKX (QUAL=0)	-	Unit is not equipped to accept the data message.

ACKX (QUAL=1)	-	Unit cannot accept the message at this time (e.g. its data store is full).

ACKV (QUAL=1)	-	Unit does not wish to accept a data message from this calling party.

ACKB (QUAL=1)	-	Not all the appended data codewords were decodeable and the unit requires the message to be retransmitted.

Acknowledgement to an individually addressed Aloha message (if addressed radio unit has no random access message to send):

ACKX (QUAL=0)	-	Acknowledgement, nothing to send.

Acknowledgement to an individually addressed AHYD message

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 01 | FUNC | IDENT2 | QUAL | RSVD | MODEM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 3 | 1 | 16 |

RSVD	-	Reserved. Default value = all '0's

MODEM	-	as a response to AHYD, availability of modem for Standard Data any other response, reserved for future definition.

Default value '0'

'0' - 1200 bits/sec FFSK only available

'1' - Both customised rate and 1200 bits/sec available

5.5.3		Type '10' Messages (Requests and Ahoys)

The Request messages sent by radio units and the Ahoy messages sent by the TSC have the same Category and Type.  For clarity, they are shown separately:

-	Section 5.5.3.1 defines the Request codeword structures.

-	Section 5.5.3.2 defines the Ahoy codeword structures.

5.5.3.1		Request Messages (Type '10')

These messages are transmitted to the TSC from a radio unit requesting a function.  Request messages on a control channel are sent using the random access protocol (see 7.3).

The most usual basic structure is illustrated below but, for clarity of definition, the message for each function is detailed separately in sections 5.5.3.1.1 to 5.5.3.1.8.

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 18 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Ident of the called party or gateway.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	Specifies the function of the request:

'000'	RQS  Request Simple call

'001'	Spare. Available for customisation

'010'	RQX  Request call cancel / abort transaction

'011'	RQT  Request call diversion

'100'	RQE  Request emergency call

'101'	RQR  Request to register

'110'	RQQ  Request status transaction

'111'	RQC  Request to send short data message.

PARAMETERS	-	See following pages.

P	-	Parity check bits.

5.5.3.1.1	Request "Simple" Call Message, RQS

This message is transmitted to the TSC on a control channel by a radio unit requesting a non-emergency conversation or a channel over which any appropriate audio signalling, even a non-standard modulation or format, can be sent.  See section 9 for the call procedures.

The RQS codeword contains all the information necessary to request a call to a unit or group of units with the same prefix, to all units in the system, to a prearranged PSTN destination or to a PABX extension that can be accommodated in the range 0-8191.  In addition, RQS may be used to request entry into the extended addressing mode for an interprefix call, a general call to the PSTN or a call to a PABX extension with a "long" number; in this case, after receiving the RQS message, the TSC demands the full called party information using the AHYC message (see 5.5.3.2.8).

The RQS message may also be sent to the TSC by a radio unit on its allocated traffic channel, to ask for another party to join the call.  See section 11 for the Include call procedures.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 000 | IDENT2 | DT | LEVEL | EXT | FLAG1 | FLAG2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 1 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system-wide call

number	-	for a short addressing call to a PABX extension (EXT='1')

PABXI	-	for a call to a "long" PABX extension number

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'000'.

IDENT2	-	Ident of the requesting radio unit.

DT	-	RQS message on a control channel:

'0' if the caller is requesting speech communication.

'1' if the caller wishes to send non-prescribed data.

RQS message on a traffic channel:

This bit shall be set equal to bit D from the GTC message that allocated the traffic channel.

contd.

LEVEL	-	RQS message on a control channel:

This bit specifies 2 levels of priority

(both non-emergency).

High priority is '0', non-priority call is '1'.

RQS message on a traffic channel:

This bit shall be set to '1'.

EXT	-	'0' if IDENT1 is not a short addressing PABX extension number.

'1' if IDENT1 is a short addressing PABX extension number.

FLAG1	-	For a group call (common-prefix or interprefix):

'0' - enable called users to reply

'1' - disable called users from replying.

For a short addressing call to a PABX extension, FLAG1/FLAG2 indicates the appropriate PABX exchange.

For a general call to the PSTN:

'0' - number has  up to 9 dialling digits

'1' - number has 10 to 31 dialling digits.

For all other types of call, FLAG1 is reserved for future definition.  Default value='0'.

FLAG2	-	For a short addressing call to a PABX extension,

FLAG1/FLAG2 indicates the appropriate PABX exchange.

For all other types of call, FLAG2 is spare for customisation.  reserved for future definition.  Default value='0'.

P	-	Parity check bits.

5.5.3.1.2	Request Codeword Spare for Customisation, Not defined

This message may be transmitted to the TSC on a control channel by a radio unit which is requesting a customised service.

5.5.3.1.3	Call Cancel / Abort Transaction Request Message, RQX

This message is transmitted to the TSC on a control channel by a radio unit in order to:

a.	cancel a previous call request, while waiting for its requested call to be set up, or

b.	abort a transaction e.g. a status transaction.

It may also be transmitted to the TSC on a traffic channel by a radio unit, in order to cancel an Include call request, while waiting for signalling for its Include call.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 010 | IDENT2 | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 5 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system-wide call

PABXI	-	for a call to a PABX extension

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination

TSCI	-	for an RQQ or RQC transaction with the TSC

DIVERTI	-	for aborting a general cancellation by a recipient of diversions.

Note: IDENT1 is the same as IDENT1 from the request being cancelled/aborted, except for a short addressing PABX call.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'010'.

IDENT2	-	Ident of the requesting radio unit.

RSVD	-	Reserved for future definition.  Default value = '00000'.

P	-	Parity check bits.

5.5.3.1.4	Request Call Diversion Message, RQT

This message is transmitted to the TSC on a control channel by a radio unit to request either that future calls addressed:

-	to itself, or

-	to another subscriber unit or group

be redirected to a specified alternative destination; the destination may be a radio or line unit, a group, a PABX extension or a PSTN number.

This message is also used by a radio unit to request:

-	cancellation of the diversion of its calls, or

-	cancellation of the diversion of another party's calls, or

-	cancellation of any existing diversions to it.

See section 12 for the diversion procedures.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 011 | IDENT2 | SD | DIV | FLAG1 | FLAG2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 2 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	For diversion requests:

party or gateway to which calls are to be redirected: ident, IPFIXI, PSTNSIj, PSTNGI or PABXI (for any PABX extension).

For "self" or "third-party" cancellation:

ident of the unit or group whose calls should be returned (or IPFIXI for an interprefix address).

For general cancellation by a recipient of diversions: set to DIVERTI.

SD	-	Specifies the types of calls to which the request to divert or cancel divert applies:

'00' 	if both speech and data calls are to be redirected.

'01' 	if only speech calls are to be redirected.

'10' 	if only data calls are to be redirected.

'11' 	reserved for future use.

For diversion purposes, "speech" calls are defined as calls requested using RQS(DT=0), RQE(D=0), RQQ(STATUS='00000') or RQQ(STATUS='11111').  "Data" calls are defined as calls requested using RQS(DT=1), RQE(D=1), RQQ('00001'-'11110'), RQC or RQD.

contd.

DIV	-	'0' for a request for call diversion.

'1' for a request for cancellation of call diversion.

FLAG1	-	For IDENT1 = PSTNGI,

'0' - number has  up to 9 dialling digits

'1' - number has 10 to 31 dialling digits.

For IDENT1 / PSTNGI, FLAG1 is reserved; default value='0'.

FLAG2	-	Specifies whether or not three addresses must be supplied:

For DIV='0', 	'0' for self-initiated diversion

'1' for third-party diversion.

For DIV='1', FLAG2 shall be set to '0'.

P	-	Parity check bits.

5.5.3.1.5	Request Emergency Call Message, RQE

This message is transmitted to the TSC by a radio unit requesting an emergency call.  The RQE codeword contains all the information necessary to request a call to a unit or group of units with the same prefix, to all units in the system, to a prearranged PSTN destination or to a PABX extension that can be accommodated in the range 0-8191.  In addition, RQE may be used to request entry into the extended addressing mode for an interprefix call, a general call to the PSTN or a call to a PABX extension with a "long" number.  See section 10 for the emergency call procedures.

RQE may also be used to request a special mode of service previously arranged with the system.

Usually emergency calls will take precedence over all other calls. Emergency calls may be pre-emptive, that is, another call may be terminated prematurely to free a channel for an emergency call.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 100 | IDENT2 | D | RSVD | EXT | FLAG1 | FLAG2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 1 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system-wide call

number	-	for a short addressing call to a PABX extension (EXT='1')

PABXI	-	for a call to a "long" PABX extension number

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination.

Note: If EXT='0'/FLAG2='1', the meaning of IDENT1 may be redefined.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'100'.

IDENT2	-	Ident of the requesting radio unit.

D	-	'0' if the caller is requesting speech communication.

'1' if the caller is requesting data communication.

Note: If EXT='0'/FLAG2='1', the meaning of bit D may be redefined.

contd.

RSVD	-	Reserved for future definition.  Default value='0'.

EXT	-	'0' if IDENT1 is not a short addressing PABX extension number.

'1' if IDENT1 is a short addressing PABX extension number.

FLAG1	-	For a group call (common-prefix or interprefix):

'0' - enable called users to reply

'1' - disable called users from replying.

For a short addressing call to a PABX extension,

FLAG1/FLAG2 indicates the appropriate PABX exchange.

For a general call to the PSTN:

'0' - number has  up to 9 dialling digits

'1' - number has 10 to 31 dialling digits.

For all other types of call, FLAG1 is reserved for future definition.  Default value='0'.

Note: If EXT='0'/FLAG2='1', the meaning of FLAG1 may be redefined.

FLAG2	-	For a short addressing call to a PABX extension,

FLAG1/FLAG2 indicates the appropriate PABX exchange.

For all other types of call (i.e. if EXT='0'):

'0' - caller is not requesting a special mode of service.

'1' - caller is requesting a special mode of service previously arranged with the system.  In this case, the meanings of fields IDENT1, D and FLAG1 may be redefined.  See also section 10.

P	-	Parity check bits.

5.5.3.1.6  Request to Register Message, RQR

This message is transmitted to the TSC on a control channel by a radio unit requesting to register.  See section 8 for the registration procedures.

Registration may be required for the tracking of roamers, for wide-area systems with multiple control channels and for polling systems.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 101 | INFO | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 15 | 3 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Ident of the requesting radio unit.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'101'.

INFO	-	Available for customisation by systems, to convey additional information to the TSC.  Null value = all '0's.

RSVD	-	Reserved for future definition.  Default value = '000'.

P	-	Parity check bits.

5.5.3.1.7	Request Status Transaction, RQQ

This message is transmitted to the TSC on a control channel by a radio unit:

-	to request that status information be relayed to the addressed line unit or radio unit, or

-	to send status information to the TSC.

The status field in an RQQ message consists of 5 bits, allowing 32 different status values.  Two of these values have been predefined (see below).

For a common-prefix status message, the RQQ message contains all the information necessary for the transaction.  For an interprefix status message, the RQQ message is used to request entry into the extended addressing mode.  See section 13 for the status procedures.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 110 | IDENT2 | STATUS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 5 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix transaction

IPFIXI	-	for an interprefix transaction

TSCI	-	for a transaction with the TSC.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'110'.

IDENT2	-	Ident of the requesting radio unit.

STATUS	-	For a transaction with a line unit or radio unit:

'00000' 	requests that the addressed unit call back with a speech call (no other status information indicated).

'00001' to '11110'	are user-defined status values.

'11111' 	cancels a previous speech call request (no other status information indicated).

For a transaction with the TSC:

'00000'	indicates "off-hook" or equivalent.

'00001' to '11110'	are system-defined status values.

'11111'	indicates  "on-hook" or equivalent.

P	-	Parity check bits.

5.5.3.1.8	Request to Transmit Short Data Message, RQC

This message is sent by a radio unit to request permission to transmit a short data message (comprising the HEAD address codeword and up to four data codewords).  After receiving the request, the TSC uses the AHYC message to instruct the requesting unit to transmit the data message on the control channel.   See section 14 for the short data message procedures.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 111 | IDENT2 | SLOTS | EXT | FLAG1 | FLAG2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 2 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the requesting radio unit.

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system-wide call

number	-	for a short addressing call to a PABX extension (EXT='1')

PABXI	-	for a call to a "long" PABX extension number

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination

TSCI	-	for a transaction with the TSC.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'111'.

IDENT2	-	Ident of the requesting radio unit.

SLOTS	-	The number of timeslots required for the data message:

'00'	reserved

'01'	reserved

'10'	two slots	(address codeword + 1 or 2 data codewords)

'11'	three slots	(address codeword + 3 or 4 data codewords)

EXT	-	'0' if IDENT1 is not a short addressing PABX extension number.

'1' if IDENT1 is a short addressing PABX extension number.

FLAG1	-	For a short addressing call to a PABX extension,

FLAG1/FLAG2 indicates the appropriate PABX exchange.

For a general call to the PSTN:

'0' - number has  up to 9 dialling digits

'1' - number has 10 to 31 dialling digits.

For all other types of call, FLAG1 is reserved for future definition.  Default value='0'.

FLAG2	-	For a short addressing call to a PABX extension,

FLAG1/FLAG2 indicates the appropriate PABX exchange.

For all other types of call, FLAG2 is reserved for future definition.  Default value='0'.

P	-	Parity check bits.

5.5.3.2	Ahoy Messages (Type '10')

These messages are transmitted by a TSC; they demand a response from an addressed radio unit.

The basic structure is illustrated below but, for clarity of definition, the message for each function is detailed separately in sections 5.5.3.2.1 to 5.5.3.2.8.

(Note that the request messages sent by radio units have the same Category and Type as the Ahoy messages sent by the TSC.)

|  | 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 7 | 13 | 1 | 3 | 2 | 3 | 18 | 16 |

PFIX	-	Prefix.

IDENT1	-	Ident of the called party or gateway.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	Specifies the function of the Ahoy:

'000'  AHY    General availability check

'001'  Spare Free for customisation

'010'  AHYX   Cancel alert/waiting state

'011'  Reserved for future use

'100'  Reserved for future use

'101'  AHYP Called Unit Presence Monitoring

'110'  AHYQ   Status message

'111'  AHYC   Short data invitation.

PARAMETERS	-	See following pages.

P	-	Parity check bits.

5.5.3.2.1	General Availability Check Message, AHY

This message is transmitted on a control channel by the TSC as follows.

a.	It may be transmitted to a called radio unit to establish the availability of the called unit/user prior to allocating a traffic channel for a call (see 9.1.1.5), or prior to including a unit in an    existing call (see 11.1.5).

b.	It may be sent to check the availability of a called radio unit before  the TSC transmits a short data message (HEAD); see 14.1.6.

c.	It may be sent to a requesting radio unit to check that it is still in  radio contact and to restart its waiting timer (see, for example, sections 8.2.1.3, 9.1.1.7 and 9.1.1.10).

AHY may also be sent by the TSC to a radio unit on a traffic channel, for example to check that the unit has reached, or is still on, the channel (see 6.1.2.1 and 9.1.2.2), or to restart the waiting timer of a radio unit which has requested an Include call (see 11.1.7).

One data codeword may be appended to the AHY address codeword, to convey additional information, depending on the value of bit AD. (In this issue of the standard, this facility is used only when the AHY is sent on a control channel to a called radio unit).

AHY demands an acknowledgement from the addressed unit:

i)	For AD = '0'	-	On a control channel, the addressed unit responds in the slot following the AHY.On a traffic channel, the unit times its response from the end of the AHY address codeword.

ii)	For AD = '1'	-	On a control channel, the addressed unit responds in the slot following the data codeword (i.e. in the slot following the slot that contains the data codeword).  On a traffic channel, the unit times its response from the end of the data codeword.

contd.

AHY address codeword:

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 000 | IDENT2 | D | POINT | CHECK | E | AD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 1 | 1 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the radio unit.

IDENT1	-	Called party or gateway:

a)	For POINT='0' (i.e. availability check on called unit), 
IDENT1 is the ident of the called radio unit.

b)	For POINT='1' (i.e. availability check on calling unit), 
IDENT1 is the called party or gateway as follows:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system-wide call

PABXI	-	for a call to a PABX extension

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination

TSCI	-	for an RQQ or RQC transaction with the TSC

DIVERTI	-	for general cancellation by a recipient of  diversions.

For restarting the waiting timer of a radio unit that has requested registration, IDENT1 = REGI.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'000'.

IDENT2	-	Calling party or gateway:

a)	For POINT='0' (i.e. availability check on called unit), 
IDENT2 is the calling party or gateway as follows:

INCI	-	for an Include call availability check

SDMI	-	for a short data message availability check

DUMMYI	-	for a "no-call" test availability check

For all other types of call,

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

PABXI	-	for a call from a PABX extension

PSTNGI	-	for a call from the PSTN.

If IDENT2 = IPFIXI or INCI, the address of the calling unit may be provided in an appended data codeword (see below).

b)	For POINT='1' (i.e. availability check on calling unit),
IDENT2 is the ident of the calling radio unit.

contd.

D	-	If the calling party requested speech communication 
(i.e. RQS(DT=0) or RQE(D=0)), then D='0'.

Otherwise, D='1'.

(For an AHY sent to restart the waiting timer of a radio unit sending "off-hook" or "on-hook" signalling, D='1').

POINT	-	'0' 	demands an acknowledgement from the unit whose individual address is PFIX/IDENT1.

'1' 	demands an acknowledgement from the unit whose individual address is PFIX/IDENT2.

See sections 9.2.2.2, 9.2.2.3 and 9.2.3.2 for the appropriate acknowledgement.

CHECK	-	AHY message on a control channel:

a)	For POINT='0' (i.e. availability check on called unit),

'0'	indicates that the TSC is checking only that the unit is in radio contact (and can accept this call).

'1'	indicates that the TSC is also checking:

-	for D='0',	whether the unit's user is ready for a speech call

-	for D='1', 	whether the unit's data equipment is ready.

b)	For POINT='1' (i.e. availability check on calling unit),CHECK is reserved for future definition.  Default value='0'.

AHY message on a traffic channel:

CHECK is reserved for future definition.  Default value='0'.

E	-	'0' 	if the calling party requested a non-emergency call or transaction.

'1' 	if the calling party requested an emergency call.

(For an AHY sent to restart the waiting timer of a radio unit requesting registration or an Include call, or sending "off-hook" or "on-hook" signalling, E='0').

AD	-	'0' 	if there is no appended data codeword.

'1' 	if there is a data codeword appended to the AHY.

P	-	Parity check bits.

contd.

Data codeword following AHY address codeword:

For a (control channel) availability check on the called unit, if bit AD in the AHY address codeword is set to '1', then a single data codeword with the following structure is appended to the AHY codeword.

| 0 | FORM | PARAMETERS | P |
| --- | --- | --- | --- |
| 1 | 3 | 44 | 16 |

FORM	-	Defines the format of the PARAMETERS field (see below).

'000'	-	Value used to convey the address of the calling unit in an interprefix or Include call.

'001' to '100' are reserved for future use.

'101' to '111' are spare for customisation.

PARAMETERS	-	See parameter formats below.

P	-	Parity check bits.

Parameter formats

PARAMETERS

| FORM = '000' | RSVD | PFIX2 | IDENT2 |
| --- | --- | --- | --- |
|  | 24 | 7 | 13 |

RSVD	-	Reserved for future use.

Default value = all '0's.

PFIX2	-	Prefix of the calling unit.

IDENT2	-	Ident of the calling unit.

5.5.3.2.2	This section is spare free for customisation.

5.5.3.2.3	Cancel Alert/Waiting State Message, AHYX

This message is transmitted on a control channel by the TSC, to inform a called radio unit of cancellation of an incoming traffic channel call e.g. if the calling unit no longer wants the call.  It demands a response ACK(QUAL=1) in the next slot from the called unit i.e. the unit whose individual address is PFIX/IDENT1 - see 9.2.2.4.  (Note that this message is used only for cancelled individual calls to radio units.)

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 010 | IDENT2 | POINT | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 5 | 16 |

PFIX	-	Prefix of the called radio unit.

IDENT1	-	Ident of the called radio unit.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'010'.

IDENT2	- 	Calling party or gateway:

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

PABXI	-	for a call from a PABX extension

PSTNGI	-	for a call from the PSTN

INCI	-	for an Include call.

RSVD	-	Reserved for future definition.  Default value = '00000'.

P	-	Parity check bits.

5.5.3.2.4	This section is reserved for future use.

5.5.3.2.5	This section is reserved for future use.

5.5.3.2.6  Called Unit Presence Monitoring Check Message, AHYP

This message is transmitted on a traffic channel by the TSC.

It may be transmitted to a called radio unit or group to establish if the unit is in radio contact.  A response is demanded if the PFIX/IDENT1 matches either -

a)	the individual identity of the Radio Unit or

b)	PFIX/IDENT matches the group address previously transmitted in the GTC message which directed this Radio Unit to the traffic channel or

c)	IDENT1=ALLI

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 101 | IDENT2 | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 5 | 16 |

PFIX	-	Unit or Group Prefix of the called radio unit.

IDENT1	-	Called party:

Ident	-	Individual or Group Identity of the called radio 			unit.

ALLI	-	for a system-wide call

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'101'.

IDENT2	- 	Calling party

Ident	-	TSCI

RSVD	-	Reserved for future definition.  Default value = '00000'.

P	-	Parity check bits.

5.5.3.2.7	Status Ahoy Message, AHYQ

This message is transmitted on a control channel by a TSC to inform a called radio unit of status information sent by a radio or line unit.  For an interprefix status message, IDENT2 in the AHYQ address codeword is set to IPFIXI and a data codeword is appended containing the calling unit's address.  AHYQ is also used to send a 5-bit status message to a radio unit from the TSC.

AHYQ demands an response ACK, ACKX, ACKV or ACKB from the called unit (i.e. the unit whose individual address is PFIX/IDENT1):

-	in the slot following the AHYQ address codeword, for a common-prefix status message (IDENT2 = ident) or a message from the TSC (IDENT2 = TSCI);

-	in the slot following the appended data codeword, for an interprefix status message (IDENT2 = IPFIXI).

See 13.2.1.5 and 13.2.3.1 for the Status Ahoy procedures.

AHYQ address codeword:

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 110 | IDENT2 | STATUS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 5 | 16 |

PFIX	-	Prefix of the called radio unit.

IDENT1	-	Ident of the called radio unit.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'110'.

IDENT2	-	Calling unit or gateway:

Ident	-	for a common-prefix transaction

IPFIXI	-	for an interprefix transaction

TSCI	-	for a message from the TSC.

If IDENT2 = IPFIXI, the address of the calling unit is provided in an appended data codeword (see below).

STATUS	-	For a status message from a radio or line unit, this field contains the status information sent by the calling unit:

'00000'			requests a speech call

'00001' to '11110' 	are user-defined status values

'11111'			cancels a previous speech call request.

For a status message from the TSC, the meaning of the STATUS field is system-dependent.

P	-	Parity check bits.

contd.

Data codeword following AHYQ address codeword:

For an interprefix status transaction, IDENT2 in the AHYQ address codeword is set to IPFIXI and a data codeword is appended containing the calling unit's address.

| 0 | RSVD | PFIX | IDENT2 | P |
| --- | --- | --- | --- | --- |
| 1 | 27 | 7 | 13 | 16 |

RSVD	-	Reserved for future use.  Default value = all '0's.

PFIX2	-	Prefix of the calling unit.

IDENT2	-	Ident of the calling unit.

P	-	Parity check bits.

5.5.3.2.8	Short Data Invitation Message, AHYC

This message is used by the TSC to instruct a radio unit to send a short data transmission (see sections 9.2.2.1, 11.3.1 and 15.2).

AHYC is used in two Modes:

-	In Mode 1, AHYC instructs a calling radio unit to send addressing information (see SAMIS, section 5.6.1.2.2) or RQC data (see HEAD, section 5.6.2), when a preceding request message from the radio unit has indicated the requirement.  Mode 1 is distinguished by setting PFIX/IDENT2 to a radio unit's individual address.

-	In Mode 2, AHYC demands that a radio unit transmits a data message of a prescribed type (see section 15); for example, the TSC may demand the serial number of a radio unit.  It is an interrogation mode, not part of a call requested by the radio unit.  Mode 2 is distinguished by setting PFIX/IDENT1 to a radio unit's individual address (with IDENT2 as a non-radio-unit ident).

The type of data to be transmitted by the radio unit is indicated by the DESC field and the non-radio-unit ident; the meaning of DESC can be different for the two modes.

AHYC may be sent on either a control channel or a traffic channel.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 111 | IDENT2 | SLOTS | DESC | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 13 | 2 | 3 | 16 |

PFIX	-	Prefix of the radio unit.

IDENT1	-	Called unit, gateway or special ident:

a. Mode 1	(instructing the unit with address PFIX/IDENT2 to send data):

IPFIXI	-	for inviting extended addressing information for an interprefix call

PSTNGI	-	for inviting dialled digits for a call to the PSTN

PABXI	-	for inviting address information for a call to a PABX extension

DIVERTI	-	for inviting the blocked address for third-party call diversion

SDMI	-	for inviting RQC data.

b. Mode 2 	(interrogation):	Ident of the radio unit.

CAT	-	'000'.

TYPE	-	'10'.

FUNC	-	'111'.

contd.

IDENT2	-	Calling party:

a. Mode 1:	Ident of the calling radio unit.

b. Mode 2 	(instructing the unit with address PFIX/IDENT1 to send data): Ident of the interrogator:

For DESC='000' (serial number check), IDENT2 = TSCI.

SLOTS	-	The number of slots reserved for the data message:

SLOTS	No. of slots 		No. of codewords

'00'	reserved

'01'	1		address codeword only

'10'	2		address codeword + 1 or 2 data codewords

'11'	3		address codeword + 3 or 4 data codewords

For Mode 1, SLOTS shall correspond to the request message from the radio unit as follows:

IDENT1 in AHYC 	SLOTS

IPFIXI	'01'

PSTNGI	'01' for up to 9 digits, or

'10' for 10 to 31 digits

PABXI	'01'

DIVERTI	'01'

SDMI	equal to SLOTS from the RQC

For Mode 2, SLOTS shall correspond to the data required from the radio unit as follows:

DESC			SLOTS

'000'	'01'

contd.

DESC	-	Data message codeword descriptor.  This field indicates the type of data message with which the radio unit shall respond:

i)	When the first codeword of the radio unit's data message is required to be SAMIS, DESC is set to the value of the DESC field to be used in the SAMIS as follows; (note that the meaning of the SAMIS message may be different for Modes 1 and 2):

a. 	Mode 1:

'000'	-	for inviting extended addressing information for an interprefix call, or the blocked address for third-party diversion

'001'	-	for inviting dialled digits for a call to the PSTN

'010'	-	for inviting address information for a call to a PABX extension

'011'	-	Reserved

'100 to 111' are spare

b. Mode 2:

'000'	-	for demanding the serial number of a radio unit.

'001 to 011' are reserved

'100 to 111' are spare

ii)	When the first codeword of the radio unit's data message is required to be HEAD (i.e. IDENT1=SDMI), DESC = '000'. 
DESC = '001' to '011' are reserved. 
DESC = '100' to '111' are spare

P	-	Parity check bits.

5.5.4		Miscellaneous Control Messages (Type '11')

These are various messages required for system control.  The basic structure is illustrated below, but the detailed structure for each message is defined separately on the following pages.

| 1 | PARAMS | 1 | CAT 000 | TYPE 11 | FUNC | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 1 | 3 | 2 | 3 | 18 | 16 |

PARAMS	-	Dependent on FUNC.

Where applicable, PARAMS is an address (PFIX/IDENT1);otherwise bits 2-21 are used for other purposes. See following pages.

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	Specifies the function of the message:

'000'	MARK	Control channel marker

'001'	MAINT	Call maintenance message

'010'	CLEAR	Clear down from allocated channel

'011'	MOVE 	Move to specified control channel

'100'	BCAST	Broadcast message for system parameters

'101'	Reserved for future use

'110'	Reserved for future use

'111'	Reserved for future use

PARAMETERS	-	See following pages.

P	-	Parity check bits.

5.5.4.1	Control Channel Marker, MARK

This message may be transmitted on a control channel by a TSC. For example, it may be sent as the first address codeword(s) on a newly designated control channel in order to accelerate control channel acquisition by radio units (see 3.3.3.1).  It does not need an address PFIX/IDENT1, so bits 2-21 are reused.

The message fields A and B are chosen so that:

i)	the parity check bits always form the control channel codeword synchronisation sequence, SYNC (see section 3.2.1.1), and

ii)	the number of bit transitions included between bits 33 and 49 is the maximum achievable, taking into account condition i) above.  The bit values of these fields will depend on CHAN4 and the system identity code.  An algorithm for generating these fields is given in Appendix 4.

| 1 | CHAN4 | A | SYS | 1 | CAT 000 | TYPE 11 | FUNC 000 | B | P 1100010011010111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 1 | 15 | 1 | 3 | 2 | 3 | 18 | 16 |

CHAN4	-	Least significant four bits of the channel number of the control channel on which the message is sent

A	-	See i) and ii) above and Appendix 4.

SYS	-	System identity code of the transmitting system.

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	'000'.

B	-	See i) and ii) above and Appendix 4.

P	-	Parity check bits.

These complete the codeword and also form the control channel codeword synchronisation sequence.

5.5.4.2	Call Maintenance Message, MAINT

These messages are transmitted on an allocated traffic channel during a call.

A radio unit sends MAINT messages (OPER = '000', '001', '010', '011') during a call; see 9.2.3.  The TSC may send MAINT (OPER = '110') to clear down from the channel any radio units that should not be there, and may send MAINT (OPER = '111') to disable radio units from user transmission.

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 10 | FUNC 001 | CHAN | OPER | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 10 | 3 | 5 | 16 |

PFIX	-	Prefix.

IDENT1	-	Transmitted by radio unit:

PFIX/IDENT1 is the unit's individual address if it was individually addressed by the GTC message; otherwise PFIX/IDENT1 is either its individual address or the group address (PFIX/IDENT1) from the GTC, as specified by the system - see 5.5.4.5c, 9.2.2.6 and 9.2.3.

Transmitted by TSC:

OPER = '110'  	PFIX/IDENT1 is the "call-labelling" address: either address from the GTC message.

OPER = '111'	Individual or group ident, or ALLI; see below.

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	'001'.

CHAN	-	Channel number of the channel on which the message is sent.

OPER	-	'000'	Pressel On

'001'	Pressel Off

'010'	Periodic message within an item

'011'	Disconnect message, end of channel use

'100'	Spare for customisation (eg radio transmitter power control)

'101'	Reserved for future use

'110'	Clear down radio units for which PFIX/IDENT1 is not a valid call-labelling address

'111'	Disable user transmission, as follows:

a)	an individual radio unit, if PFIX/IDENT1 is an individual address

b)	called radio units in a group, if PFIX/IDENT1 is the group address

c)	all radio units on the channel, if IDENT1 is ALLI.

RSVD	-	Reserved for future use.  Except for OPER = '100' when these bits are available for synchronisation when reserved, default value = '00000'.

P	-	Parity check bits.

5.5.4.3	Clear-Down Message, CLEAR

This message is transmitted by a TSC; it directs all radio units to clear down from a traffic channel.  It does not need an address PFIX/IDENT1, so bits 2-21 are reused.

| 1 | CHAN | CONT | 1 | CAT 000 | TYPE 11 | FUNC 010 | RSVD | SPARE | REVS 101010101010 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 2 | 3 | 4 | 2 | 12 | 16 |

CHAN	-	Channel number of the traffic channel on which the message is sent.

CONT	-	Channel number of the control channel to which radio units should move (unless CONT = '0000000000', in which case the channel movement is system-dependent).

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	'010'.

RSVD	-	Reserved for future use.  Default value = '0000'.

SPARE	-	These bits are available for customisation.

REVS	-	Bit reversals, ending with a '0'.

P	-	Parity check bits.

5.5.4.4	Move to Control Channel, MOVE

This message is transmitted on a control channel by a TSC; it directs selected radio units to a different control channel.  (See sections 6.1.1 and 7.4.2).

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 11 | FUNC 011 | CONT | (M) | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 10 | 5 | 2 | 1 | 16 |

PFIX	-	Prefix.

IDENT1	-	PFIX/IDENT1 specifies the radio units that should move.  Only the (M) least significant bits of the 20-bit address are used; the remaining address bits may be set arbitrarily.

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	'011'.

CONT	-	Channel number of the control channel to which the addressed radio units should move (unless CONT = '0000000000', in which case the channel movement is system-dependent).

(M)	-	Address qualifier.

RSVD	-	Reserved for future use.  Default value = '00'.

SPARE	-	This bit is available for customisation.

P	-	Parity check bits.

5.5.4.5	Broadcast Message, BCAST

This message is transmitted on a control channel by a TSC; it contains information about system parameters for either this system or another system.  It does not need an address PFIX/IDENT1, so bits 2-21 are reused.

32 different types of information can be broadcast using BCAST messages, by setting the SYSDEF and parameter fields appropriately. The parameter fields for each SYSDEF are detailed on the following pages.

| 1 | SYSDEF | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 18 | 16 |

SYSDEF	-	Specifies which system parameters are being broadcast:

'00000' 	Announce control channel

'00001' 	Withdraw control channel

'00010' 	Specify call maintenance parameters

'00011' 	Specify registration parameters

'00100' 	Broadcast adjacent site control channel number

'00101' 	Vote now advice

'00110' 	)

:    	)    Reserved for future use

'01111' 	)

'10000' 	)

:    	)    Spare for customisation of services

'11111' 	)

SYS	-	System identity code of the system to which the broadcast message refers.

CAT	-	'000'.

TYPE	-	'11'.

FUNC	-	'100'.

PARAMETERS	-	Parameter fields - see following pages.

P	-	Parity check bits.

Parameter fields in BCAST messages

a)	Announce control channel (SYSDEF = '00000')

This message announces a channel that may be used for control by the named system; radio units may then include it in their list of channels to scan.

| 1 | SYSDEF 00000 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | SPARE | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 2 | 6 | 16 |

CHAN	-	Channel number of the control channel being announced.

SPARE	-	These bits are available for customisation.

RSVD	-	Reserved for future definition.  Default = all '0's.

b)	Withdraw control channel (SYSDEF = '00001')

This message withdraws a channel that could previously be used for control by the named system; radio units may then delete it from their list of channels to scan.

| 1 | SYSDEF 00001 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | SPARE | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 2 | 6 | 16 |

CHAN	-	Channel number of the control channel being withdrawn.

SPARE	-	These bits are available for customisation.

RSVD	-	Reserved for future definition.  Default = all '0's.

c)	Specify call maintenance parameters (SYSDEF = '00010')

This message specifies:

(i)		whether this system requires radio units to send call maintenance messages on traffic channels periodically within speech items; if so, it specifies the maximum interval between the periodic messages;

(ii)		whether this system requires radio units on traffic channels to send NPON Pressel On messages at the start of speech items;

(iii)		whether this system requires that a called unit in a group shall set PFIX/IDENT1 in MAINT messages it sends to its individual address or to the group address from the GTC message.

This message shall be sent only by the system to which the broadcast refers.

Default requirements are specified in section 9.2.2.6.

| 1 | SYSDEF 00010 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | PER | IVAL | PON | ID | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 1 | 5 | 1 | 1 | 2 | 8 | 16 |

PER	-	'0' 	if radio units shall send call maintenance messages periodically within speech items.

'1' 	if radio units shall not send call maintenance messages periodically within speech items.

IVAL	-	If PER = 0, 	IVAL is the maximum interval (in seconds) between the start of the item and the first periodic message, and then between subsequent periodic messages.

If PER = 1, 	IVAL is reserved.  Default value = '00000'.

PON	-	'0' 	if radio units shall send NPON Pressel On messages at  the start of speech items.

'1' 	if radio units shall not send any Pressel On messages at the start of speech items.

ID	-	'0' 	if a called unit in a group shall set PFIX/IDENT1 in MAINT messages it sends to the group address from the GTC message.

'1' 	if a called unit in a group shall set PFIX/IDENT1 in MAINT messages it sends to its individual address.

RSVD	-	Reserved for future definition.  Default value = '00'.

SPARE	-	These bits are available for customisation.

d)	Specify registration parameters (SYSDEF = '00011')

This message is available for systems to specify parameters which radio units may require for implementing registration.

| 1 | SYSDEF 00011 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 4 | 14 | 16 |

RSVD	-	Reserved for future definition.  Default value = '0000'.

SPARE	-	These bits are available for customisation.

e)	Broadcast adjacent site control channel number (SYSDEF = '00100')

This message specifies a control channel currently being used for signalling on an adjacent site.  It gives the system identity code of the adjacent site and the channel number of the specified control channel, and may also give the local serial number of the adjacent site.

| 1 | SYSDEF 00100 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | SPARE | RSVD | ADJSITE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 2 | 2 | 4 | 16 |

CHAN	-	Channel number of the control channel being specified.

SPARE	-	These bits are available for customisation.

RSVD	-	Reserved for future definition.  Default value = '00'.

ADJSITE	-	For 	ADJSITE = 0, radio units shall ignore this field.

For 	ADJSITE = 1 to 15, ADJSITE is the local serial number of the adjacent site, as assigned by the transmitting site.

The use of site serial numbers is system-dependent.  The adjacent site number is useful for wide-area systems that provide more than one control channel with the same system identity code (SYS).  (Note that the same call information should be sent on control channels with the same SYS).

f)	Vote now advice (SYSDEF = '00101')

This message gives an opportunity to idle radio units to use the next slot for signal assessment of the control channel specified by the broadcast message.  It gives the system identity code of the adjacent site that is using the specified control channel and the channel number of the control channel, and may also give the local serial number of the adjacent site.

Note that the TSC should not use the next slot on the transmitting site to signal to units that are likely to be assessing the signal strength received from the adjacent site.

| 1 | SYSDEF 00101 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | SPARE | RSVD | ADJSITE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 2 | 2 | 4 | 16 |

CHAN	-	Channel number of the control channel being specified.

SPARE	-	These bits are available for customisation.

RSVD	-	Reserved for future definition.  Default value = '00'.

ADJSITE	-	For 	ADJSITE = 0, radio units shall ignore this field.

For 	ADJSITE = 1 to 15, ADJSITE is the local serial number of the adjacent site, as assigned by the transmitting site.

5.6		Category '001' Messages

5.6.1		Single Address Messages (Type '0')

5.6.1.1	Outbound Single Address Messages, SAMO

The SAMO messages are for the transmission of short data messages from the TSC to radio units.  They are not used in this issue of the standard, but are reserved for future definition.

| 1 | PFIX | IDENT1 | 1 | CAT 001 | TYPE 0 | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 22 | 16 |

PFIX	-	Prefix of the called radio unit or group.

IDENT1	-	Ident of the called radio unit or group.

CAT	-	'001'.

TYPE	-	'0'.

PARAMETERS	-	Reserved for future definition.

P	-	Parity check bits.

5.6.1.2	Inbound Single Address Messages

5.6.1.2.1	Inbound Unsolicited Single Address Message, SAMIU

The SAMIU messages are for the transmission of random access short data messages from radio units to the TSC.  They are not used in this issue of the standard, but are reserved for future definition.

| 1 | PFIX | IDENT1 | 1 | CAT 001 | TYPE 0 | SOL 1 | PARAMETERS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 1 | 21 | 16 |

PFIX	-	Prefix of the originating unit.

IDENT1	-	Ident of the originating unit.

CAT	-	'001'.

TYPE	-	'0'.

SOL	-	'1'.

PARAMETERS	-	Reserved for future definition.

P	-	Parity check bits.

5.6.1.2.2	Inbound Solicited Single Address Message, SAMIS

The SAMIS message is for the transmission of a short data message by a radio unit in response to an AHYC message from the TSC.  For example, it is used in the extended addressing procedures, in third-party call diversion (section 12) and for data interrogation (section 15).  When appropriate, data codewords are appended to a SAMIS address codeword.

The meaning of a SAMIS message is indicated by the DESC field.  The AHYC message which solicits a SAMIS is used in two different Modes (see 5.5.3.2.8); the meaning of the SAMIS message is specified independently for the two Modes.

The SAMIS message may be transmitted on a control channel and on a traffic channel.

SAMIS address codeword:

| 1 | PARAMETERS1 | 1 | CAT 001 | TYPE 0 | SOL 0 | DESC | PARAMETERS2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 1 | 3 | 1 | 1 | 3 | 18 | 16 |

PARAMETERS1	-	See parameter formats below.

CAT	-	'001'.

TYPE	-	'0'.

SOL	-	'0'.

DESC	-	Codeword descriptor:

a.	Mode 1: (AHYC with radio unit address as PFIX/IDENT2):

'000'	-	extended addressing information for an interprefix call, or blocked address for  third-party diversion

'001'	-	dialled digits for a call to the PSTN

'010'	-	address information for a call to a PABX extension

'011' 		is reserved

'100' to '111' are spare

b.	Mode 2: (AHYC with radio unit address as PFIX/IDENT1):

'000'	-	radio unit's serial number.

'001' to '011' are reserved.

'100' to '111' are spare.

PARAMETERS2	-	See parameter formats below.

P	-	Parity check bits.

contd.

Parameter formats for Mode 1

DESC 		PARAMETERS1 				PARAMETERS2

| '000' | PFIX | IDENT |  | RSVD |
| --- | --- | --- | --- | --- |
|  | 7 | 13 |  | 18 |

| '001' | BCD1 | BCD2 | BCD3 | BCD4 | BCD5 |  | L | BCD6 | BCD7 | BCD8 | BCD9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 4 | 4 | 4 | 4 |  | 2 | 4 | 4 | 4 | 4 |

| '010' | BCD1 | BCD2 | BCD3 | BCD4 | BCD5 |  | SP 0 | RSVD | BCD6 | BCD7 | BCD8 | BCD9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 4 | 4 | 4 | 4 |  | 1 | 1 | 4 | 4 | 4 | 4 |

or

| '000' | RSVD | EXCHANGE | Number |  | SP 1 | RSVD |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 2 | 13 |  | 1 | 17 |

PFIX	-	Prefix of unit or group.

IDENT	-	Ident of unit or group.

RSVD	-	Reserved.  Default value = all '0's.

L	-	Number of data codewords appended to SAMIS:

'00' - no data codewords

'01' - one data codeword

'10' - two data codewords

'11' - reserved

BCDn	-	BCD groups representing the dialled digits, coded in accordance with the table in Appendix 5.  BCD digits are transmitted in the dialled order (i.e. the leftmost digit in the above diagram is the earliest in the dialling order; digits in any following codeword are later in the dialling order).

EXCHANGE	-	Indicates the appropriate PABX exchange.

Number	-	PABX extension number.

SP	-	Indicates the format of the PABX address information:

'0'  -  BCD digits.

'1'  -  13-bit extension number plus 2-bit exchange number.

(Note that SP='1' is used only in the call diversion procedures).

contd.

Data codewords appended to SAMIS, Mode 1:

For DESC = '001', in response to AHYC inviting PSTN digits, one or two data codewords having the following format may be appended:

|  | 0 | RSVD | eleven BCD digits | P |
| --- | --- | --- | --- | --- |
|  | 1 | 3 | 11 x 4 | 16 |

RSVD	-	Reserved.  Default value = '000'.

BCD	-	Eleven BCD groups representing the dialled digits, coded in accordance with the table in Appendix 5.  BCD digits are transmitted in the dialled order.

P	-	Parity check bits.

Parameter formats for Mode 2

DESC 		PARAMETERS1 				PARAMETERS2

| '000' | 1st part of serial number |  | 2nd part of serial number |
| --- | --- | --- | --- |
|  | 20 |  | 18 |

The form of the serial number is system-dependent.

Data codewords appended to SAMIS, Mode 2:

Reserved for future definition.

5.6.2		Short Data Message Header, HEAD (Type '1')

This codeword is the address codeword in a short data message having up to four data codewords and transmitted on a control channel.  A radio unit may request to send a short data message using the RQC message (see 5.5.3.1.8).  The TSC instructs the radio unit to send its short data message (using AHYC), and then forwards the message to the called party (or the TSC may be the called destination).  The TSC may also transmit short data messages originated from a line unit, a PABX extension or the PSTN, or from the TSC itself.  See section 14 for the short data message procedures.

A HEAD message transmitted by the TSC to an individually addressed radio unit demands a response from the unit, in the slot following the last data codeword of the message.

HEAD address codeword:

| 1 | PFIX1 | IDENT1 | 1 | CAT 001 | TYPE 1 | LEN | PFIX2 | IDENT2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 2 | 7 | 13 | 16 |

PFIX1	-	Prefix of the called party (if applicable).

IDENT1	-	Called party or gateway:

Ident	-	for a common-prefix or interprefix call

ALLI	-	for a system-wide call

PABXI	-	for a call to a PABX extension

PSTNSIj	-	for a call to a prearranged PSTN destination

PSTNGI	-	for a call to a general PSTN destination

TSCI	-	for a call to the TSC.

CAT	-	'001'.

TYPE	-	'1'.

LEN	-	Indicates the number of data codewords appended to the HEAD address codeword:

'00'	one data codeword

'01'	two data codewords

'10'	three data codewords

'11'	four data codewords.

PFIX2	-	Prefix of the calling party (if applicable).

IDENT2	-	Calling party or gateway:

Ident	-	for a common-prefix or interprefix call

PABXI	-	for a call from a PABX extension

PSTNGI	-	for a call from the PSTN

TSCI	-	for a call from the TSC.

P	-	Parity check bits.

contd.

Data codeword(s) following HEAD address codeword:

| 0 | RSA | PARAMETERS | P |
| --- | --- | --- | --- |
| 1 | 1 | 46 | 16 |

RSA	-	Return Slot Access Flag.

When transmitted by the TSC on a control channel in the second half of a slot preceding an access slot:

'0' 	- 	radio units are not permitted random access in the following slot on the return control channel,

'1'	-	radio units are permitted random access in the following slot on the return control channel.  When transmitted in the first or third data codeword following HEAD, RSA = SPARE, default = '0'.

PARAMETERS	-	This field is available for free format data.

P	-	Parity check bits.

5.7 Codewords applicable to Standard Data Call Set-Up

5.7.1 Request Standard Data Communication RQD

This message is transmitted to the TSC on a control channel by a radio unit requesting to send a data message using the Standard Data Protocol

| 1 | PFIX | IDENT1 | 1 | CAT 010 | KIND 1 | PORT | FAD | IDENT2 | INTER | LEVEL | HADT | E | MODEM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 3 | 1 | 13 | 1 | 1 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the requesting radio unit

IDENT1	-	Called party or gateway

Ident	-	for a common-prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system wide call

PABXI	-	for a call to a PABX extension

PSTNSIj	-	for a call to a prearranged PSTN destination

NETSIj	-	for a call to to a pre-arranged data Network destination

PSTNGI	-	for call to a general PSTN destination

DNI	-	for a call to a data network

CAT	-	'010'

KIND	-	'1'

PORT	-	Logical Port number of the called party

FAD	-	Flag to indicate greater than 9 dialled digits for PSTN call

'0' - 9 or fewer dialled digits

'1' - greater than 9 dialled digits

IDENT2	-	Ident of the requesting radio unit

INTER	-	interactive contact required

'1' interactive contact with the called party is required

'0' interactive contact with the called party need not be provided

LEVEL	-	required priority

'1' non-priority

'0' high priority

HADT	-	'0' high accuracy data transfer not required

'1' high accuracy data transfer required

E	-	'0' if the calling party requested a non-emergency call or transaction.

'1' if the calling party requested an emergency call.

contd

MODEM	-	requested data rate

'0'	-	standard rate 1200 b/s only

'1'	-	customised rate and standard rate are supported.

P	-	parity check bits

5.7.2		Availability Check for Standard Data AHYD

| 1 | PFIX | IDENT1 | 1 | CAT 010 | KIND 1 | PORT | RSVD | IDENT2 | INTER | POINT | HADT | E | AD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 3 | 1 | 13 | 1 | 1 | 1 | 1 | 1 | 16 |

PFIX	-	Prefix of the radio unit

IDENT1	-	Called party or gateway:

Ident	-	for a common prefix call

IPFIXI	-	for an interprefix call

ALLI	-	for a system wide call

PABXI	-	for a call to a PABX extension

NETSIj	-	for a call to a pre-arranged Data Network

PSTNSIj	-	for a call to a pre-arranged PSTN destination

PSTNGI	-	for a call to the PSTN

DIVERTI	-	for general cancellation by a recipient of diversions

DNI	-	for a call to a Data Network

CAT	-	'010'

KIND	-	'1'

PORT	-	Logical port number

RSVD	-	Reserved. Default value = '0'

IDENT2	-	Calling party or gateway as follows:

DUMMYI	-	for a "no-call" test availability check for standard data

Ident	-	for a common prefix call

IPFIXI	-	for an interprefix call

PABXI	-	for a call from a PABX extension

PSTNGI	-	for a call from the PSTN

NETSIj	-	from a pre-arranged Data Network source

DNI	-	for a call from a Data Network

If IDENT2=IPFIXI, the address of the calling unit may be provided in an appended data codeword

INTER	-	interactive contact required

'1' interactive contact with the called party is required

'0' interactive contact with the called party need not be provided.

POINT	-	'0' 	demands an acknowledgement from the called unit i.e the unit whose individual address is PFIX/IDENT1

'1' 	demands an acknowledgement from the calling unit i.e the unit whose individual address is PFIX/IDENT2

contd.

HADT	-	'0' high accuracy data transfer not required

'1' high accuracy data transfer required.

E	-	'0' if the calling party requested a non-emergency data call.

'1' if the calling party requested an emergency data call.

AD	-	'0' if there is no appended data codeword

'1' if there is a data codeword appended to the AHYD

P	-	parity check bits

Data codeword following AHYD address codeword

|  | 0 | FORM | PARAMETERS | P |
| --- | --- | --- | --- | --- |
|  | 1 | 3 | 44 | 16 |

FORM	-	Defines the format of the PARAMETERS field (see below)

'000'	-	Value used to convey the address of the calling unit in an interprefix call

'001' to '100' are reserved for future use

'101' to '111' are spare for customisation

PARAMETERS	-	see parameter formats below

P	-	parity check bits

Parameter formats

| FORM='000' | RSVD | PFIX2 | IDENT2 |
| --- | --- | --- | --- |
|  | 24 | 7 | 13 |

RSVD	-	Reserved for future use. Default value = all '0's.

PFIX2	-	Prefix of the calling unit

IDENT2	-	Ident of the calling unit

5.7.3		Go To Transaction GTT

This message is transmitted from a TSC to radio units.  It directs addressed radio units to switch to a designated Standard Data channel in order to proceed with or continue a data call.

The message is used :

a)	On a control channel to send the RU to a data channel, and allocate its TRANS.  If TRANS='0000000000' then the TRANS will be allocated on the data channel using an additional GTT.

b)	On a data channel, with an IDENT set to a value in the range  1-8100, to allocate an additional TRANS to the RU.  In this case the channel number MUST be set to the current data channel

c)	On a data channel to move an individual RU to another data channel. This is only possible if the radio unit already has one TRANS assigned to it.  In this case, a new TRANS will replace the current TRANS on the new channel and the data call will continue.

d)	On a data channel to move ALL radio unit's to another data channel.  In this case IDENT is set to ALLI and TRANS='0000000000'.  All radio units moved shall retain their already allocated TRANS. The O/R and the RATE fields have no meaning and shall be set to '0'.

| 1 | PFIX | IDENT | 1 | CAT 010 | KIND 0 | CHAN | O/R | RATE | TRANS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 10 | 1 | 1 | 10 | 16 |

PFIX	-	Prefix of radio unit

IDENT	-	Ident of the radio unit

Ident	-	any ident applicable to the radio unit.

ALLI	-	for a system wide call

CAT	-	'010'

KIND	-	'0'

CHAN	-	Designates the allocated channel number

O/R	-	Originator or Recipient

'0'	-	radio unit is the originator

'1'	-	radio unit is the recipient

RATE	-	Transmission rate to be used

'0'	-	standard rate 1200 b/s

'1'	-	customised rate

TRANS	-	Transaction number

P	-	parity bits

5.7.4 	Standard Data Random access, Radio Unit General Information - DRUGI

This message shall be transmitted by a radio unit:

a) In response to DAHY

b) In random access when TDE or TDH has expired

c) When it wishes to resume data transfer after a pause

| 1 | PFIX | IDENT1 | 1 | CAT 010 | KIND 0 | RNITEL | TNITEL | TRANS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 6 | 6 | 10 | 16 |

PFIX	-	Prefix of the requesting radio unit

IDENT1	-	Ident of the requesting radio unit

CAT	-	'010'

KIND	-	'0'

RNITEL	-	Maximum number of data codewords which could be received in a New Fragment

TNITEL	-	The number of codewords proposed for the next dataitem transmitted by the radio unit.

TRANS	-	Transaction number

P	-	parity check bits

5.8 		Codewords applicable to Standard Data Traffic Handling

5.8.1 		Standard Data general purpose acknowlegement DACKD

This message may be transmitted by either a TSC or a radio unit as a general purpose acknowledgement on the data channel.

| 1 | PFIX | IDENT | 1 | CAT 101 | KIND 0 | JOB 0101 | RSVD | REASON | TRANS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 4 | 5 | 3 | 10 | 16 |

PFIX	-	Prefix of the radio unit

IDEN	-	Ident of the radio unit

CAT	-	'101'

KIND	-	'0'

JOB	-	'0101'

RSVD	-	Reserved for future use. Default value = all '0's.

REASON	-	Reason for this acknowledgement

'000'	 	response for DAHYX

'001'	 	as a response to a GO submessage if a pause in user data transmission is required

'010'-'101' reserved

'110'-'111' spare

TRANS	-	Transaction number to be closed

P	-	parity check bits

5.8.2 	Standard Data Codeword DACK containing Submessages 
DAL, DALG or GO

This message consists of an DACK acknowlegement submessage combined with either:

Layout No1.	A marker for a random access frame (codeword layout no.1) transmitted by the TSC only, or

Layout No2.	An invitation to transmit a 'GO' for a fragment (codeword layout no.2).

CODEWORD LAYOUT NUMBER 1

| 1 | ATRANS | RTRANS | 1 | CAT 101 | KIND 0 | JOB 00XX | W/F | P/N | RSVD | DN | TNITEL | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 3 | 1 | 2 | 5 | 6 | 1 | 16 |

ATRANS	-	TRANS number for the submessage for which this is the acknowledgment.  If ATRANS='0000000000' then this submessage has no significance.

RTRANS	-	Transaction number for random access  See 17.2.1.2.1.

CAT	-	'101'

KIND	-	'0'

JOB	- 	'0000'	-	DACK+DAL where DAL marks a general random access frame

'0001'	-	DACK+DALG where DALG marks a frame for requesting group message repeats

'0010' 	-	DACK+DALN where DALN marks a frame for all  except group message repeats

WF	-	Delay parameter for repeat transmissions i.e number of frame marks that must be counted before further random access of this type may be made.

DN	-	the Aloha Number for the random access frame

P/N	-	Positive (PACK) or Negative (NACK) Acknowledgement.

(PACK)	P/N = '1' indicates whole dataitem successfully received.

(NACK)	P/N = '0' indicates whole dataitem to be repeated.

TNITEL	-	Proposed Transmitted next dataitem length in the same direction as this message (ATRANS).

ITENUM	-	Dataitem number from the message to which this is the acknowledement (ATRANS).

RSVD	-	Reserved. Default value = all '0's

P	-	Parity check bits

- - - - - - - - - - - - - - - - - -

CODEWORD LAYOUT NUMBER 2

| 1 | ATRANS | RTRANS | 1 | CAT 101 | KIND 0 | JOB 0011 | RSVD | P/N | RSVD | RNITEL | TNITEL | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 3 | 1 | 1 | 6 | 6 | 1 | 16 |

ATRANS	-	TRANS number for the submessage for which this is the acknowledgment.  If ATRANS='0000000000' then this submessage has no significance.

RTRANS	-	Transaction number for the GO submessage

CAT	-	'101'

KIND	-	'0'

JOB	-	'0011' is for DACK+'GO'

P/N	-	Positive (PACK) or Negative (NACK) Acknowledgement.

(PACK) 		P/N = '1' indicates whole dataitem successfully received.

(NACK) 		P/N = '0' indicates whole dataitem to be repeated.

RSVD	-	Reserved. Default value = all '0's

RNITEL	-	to indicate the maximum length of fragment that can be accepted next (ATRANS).

TNITEL	-	Proposed Transmitted next dataitem length in the same direction as this message (ATRANS).

ITENUM	-	Dataitem number from the message to which this is the acknowledement (ATRANS).

RSVD	-	Reserved. Default value = all '0's

P	-	Parity check bits

5.8.3		Standard Data Acknowledgement for expedited data DACKZ

Acknowledgement for expedited data, transmitted by both the TSC and Radio Unit.

| 1 | ATRANS | SPRE | 1 | CAT | KIND | JOB | SX | SPRE | CAUSE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 3 | 7 | 8 | 16 |

ATRANS	-	Transaction number of the relevant TRANS for  the acknowledgement

SPRE	-	for SX = 000  to 101 - RSVD.

Default value = all '0's.

for SX = 110  to 111 - SPARE for customisation.

CAT	-	'101'

KIND	-	'0'

JOB	-	'0100'

SX	-	Type of expedited data.

'000' = RESET

'001' to '111' - RSVD. Default value = all '0's.

CAUSE	-	Reason for expedited data.

Default value = all '0's

P	-	parity check bits

5.8.4		Standard Data General ahoy DAHY

This message is transmitted by a TSC on a data channel to query availability relating to a particular TRANS.  It demands an immediate response.

| 1 | TRANS | RSVD | 1 | CAT 101 | KIND 0 | JOB 1000 | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 10 | 8 | 16 |

TRANS	-	Transaction number

RSVD	-	Reserved for future use. Default value = all '0's.

CAT	-	'101'

KIND	-	'0'

JOB	-	'1000'

SPARE	-	Spare for customisation

P	-	parity check bits

5.8.5		Standard Data ahoy containing expedited data DAHYZ

This message is transmitted by a TSC on a data channel to convey expedited data relating to a particular individual TRANS. It demands an immediate response.

| 1 | SPRE | RSVD | 1 | CAT 101 | KIND 0 | JOB 1100 | SX | SPRE | CAUSE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 3 | 7 | 8 | 16 |

TRANS	-	Transaction number

SPRE	-	for SX = 000  to 101 - RSVD.

Default value = all '0's.

for SX = 110  to 111 - SPARE for customisation.

RSVD	-	Reserved for future use. Default value = all '0's.

CAT	-	'101'

KIND	-	'0'

JOB	-	'1100'

SX	-	Type of expedited data

'000'		-  Reset the link to a known state

'001' to '101'	-  Reserved

'110' to '111' 	-  Spare

CAUSE	-	Reason for expedited data.

Default value = all '0's.

P	-	parity check bits

5.8.6		Standard Data ahoy for closing a TRANS. DAHYX

This message is transmitted by a TSC to close one or all TRANS' of a particular radio unit.

The message may also be used to clear ALL radio units from a data channel by setting IDENT='ALLI'.  In this case there will be no response and the fields PFIX, I/T, TOC, and TRANS shall have no meaning and default to all '0's with RESP='0'.

| 1 | PFIX | IDENT | 1 | CAT 101 | KIND 0 | JOB 1110 | I/T | RESP | SPRE | TOC | TRANS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 4 | 1 | 1 | 3 | 3 | 10 | 16 |

PFIX	-	Prefix of radio unit

IDENT	-	Ident of the radio unit

Ident  -  any ident applicable to the radio unit.

ALLI   -  to clear ALL radio units from a data channel.

CAT	-	'101'

KIND	-	'0'

JOB	-	'1110'

I/T	-	IDENT/TRANS

I/T='0' Close all TRANS associated with that PFIX/IDENT. 
See RESP.  (TRANS shall be set to '0000000000')

I/T='1' Close the specified TRANS. See RESP.

RESP	-	'0'	no response to the DAHYX is expected

-	'1'	An acknowledgement to the AHYX is required

SPRE	-	for TOC = 000  to 101 - RSVD.  Default value = all '0's.

for TOC = 110  to 111 - SPARE for customisation.

TOC	-	Type Of Clear

'000' 	ALLCLR Data transfer abandoned and incomplete

'001' 	ALLDONE Data Transfer for this link has been completed

'010' - '101'  Reserved

'110' - '111'  Spare

TRANS	-	Transaction number

P	-	parity check bits

5.8.7		Repeat last ACK - RLA

This message is transmitted on a data channel by a TSC or a radio unit to request a repeat of the last acknowledgement.

| 1 | TRANS | RSVD | 1 | CAT 101 | KIND 0 | JOB 1111 | RSVD | SPARE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 12 | 6 | 16 |

TRANS	-	Transaction number

RSVD	-	Reserved for future use. Default value = all '0's.

CAT	-	'101'

KIND	-	'0'

JOB	-	'1111'

SPARE	-	Spare for customisation

P	-	parity check bits

5.8.8		Repeat group message DRQG

This codeword is transmitted by radio unit's using random access procedures to request the retransmission of the relevant group message.

| 1 | TRANS | SPARE | RSVD | 1 | CAT 101 | KIND 0 | JOB 1010 | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 7 | 3 | 1 | 3 | 1 | 4 | 18 | 16 |

TRANS	-	Transaction number

SPARE	-	Spare for customisation

RSVD	-	Reserved. Default value = all '0's

CAT	-	'101'

KIND	-	'0'

JOB	-	'1010'

P	-	parity check bits

5.8.9		Request containing expedited data DRQZ

This codeword is transmitted by radio unit's using random access or 'GO' procedures as a request for expedited data to be transmitted to the other correspondant.

| 1 | TRANS | SPRE | 1 | CAT 101 | KIND 0 | JOB 1100 | SX | SPRE | CAUSE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 4 | 3 | 7 | 8 | 16 |

TRANS	-	Transaction number

SPRE	-	for SX = 000  to 101 - RSVD.

Default value = all '0's.

for SX = 110  to 111 - SPARE for customisation.

CAT	-	'101'

KIND	-	'10'

JOB	-	'1100'

SX	-	Type of expedited data

'000' 	Reset the link to a known state

'001' to '101 	Reserved

'110' to '111' 	Spare

CAUSE	-	Reason for expedited data

SX	-	Type of expedited data

P	-	parity check bits

5.8.10		Request to close a transaction DRQX

This codeword is transmitted by a radio unit to request the closure of one or all of its TRANS

| 1 | PFIX | IDENT1 | 1 | CAT 101 | KIND 0 | JOB 1110 | SPRE | TOC | TRANS | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 1 | 4 | 5 | 3 | 10 | 16 |

PFIX	-	Prefix of the requesting radio unit

IDENT1	-	Ident of the requesting radio unit

CAT	-	'101'

KIND	-	'0'

JOB	-	'1110'

SPRE	-	for TOC = 000  to 101 - RSVD.

Default value = all '0's.

-	for TOC = 110  to 111 - SPARE for customisation.

TRANS	-	Transaction number to be closed. If TRANS '0000000000' then close all transactions for this radio unit.

TOC	-	Type Of Clear

'000'  ALLCLR	Data transfer abandoned and incomplete

'001'  ALLDONE 	Data Transfer for this link has been completed

'010' - '101' Reserved

'110' - '111' Spare for customisation

P	-	parity check bits

5.8.11 	Standard Data Selective Acknowledgement Header SACK

This message is transmitted by a TSC or radio unit on a Standard Data channel as a selective acknowledgement to a user data message.

If there are more than 22 data codewords in the dataitem then a data codeword is appended to the SACK address codeword.

| 1 | ATRANS | EFLAGS | 1 | CAT 101 | KIND 0 | TASK 0 | RSVD | EFLAGS | ONES | AD | ITEMNUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 2 | 13 | 4 | 1 | 1 | 16 |

ATRANS	-	Transaction number of the original header to which this SACK refers

EFLAGS	-	Error Flags. See next page for rules of use

CAT	-	'101'

KIND	-	'1'

TASK	-	'0'

RSVD	-	Reserved. Default value = all '0's

ONES	-	the modulo 16 sum of all 23 EFLAG bits in this codeword

AD	-	'0' if there is no appended data codeword

'1' if there is an appended data codeword

ITENUM	-	itemnumber from the data header to which this acknowledgment refers.

P	-	parity check bits

Data codeword following Standard Data Acknowledgement Header SACK

| 0 | ONES | EFLAGS | RSVD | P |
| --- | --- | --- | --- | --- |
| 1 | 4 | 40 | 3 | 16 |

EFLAGS	-	Error Flags (unused Error Flags='0')

ONES	-	the modulo 16 sum of all 40 EFLAG bits in this codeword

RSVD	-	Reserved. Default value = all '0's

P	-	parity check bits

contd.

Rules governing the use of both EFLAG's fields

1.	Every data codeword in a user dataitem shall have an EFLAG assigned to it.  Each assigned EFLAG shall be set to '1' if its corresponding codeword is required to be repeated, otherwise it shall be set to '0'

2.	Within the EFLAG fields the assigned EFLAGs shall be arranged contiguously in the same order as their data codewords in the message to which they are assigned, starting with the first EFLAG i.e bit 12 of the SACK address codeword.

3.	The EFLAG bit following the last assigned EFLAG shall be used as a marker and set to '1' and any remaining EFLAG bits shall be set to '0'.

4.	The EFLAGs in the address codeword are assigned first and only if they are all assigned is a data codeword appended. Thus if there are 23 assigned EFLAGs then there will be only the marker and filler '0's in the appended data codeword.

5.8.12 	Standard Data Address Codeword Dataitem SITH

This message is transmitted by a TSC or radio unit as an address codeword for a fragment.

Individual Dataitem

| 1 | TRANS | USER DATA | 1 | CAT 101 | KIND 1 | TASK 1 | I/G 0 | MORE | LASTBIT | FRAGL | TNITEL | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 1 | 1 | 6 | 6 | 6 | 1 | 16 |

Group Dataitem - TSC to Group

| 1 | TRANS | USER DATA | 1 | CAT 101 | KIND 1 | TASK 1 | I/G 1 | MORE | LASTBIT | FRAGL | RSVD | ITENUM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 1 | 1 | 1 | 1 | 6 | 8 | 4 | 1 | 16 |

TRANS	-	Transaction number

USERDATA	-	10 bits of user data

CAT	-	'101'

KIND	-	'1'

TASK	-	'1'

I/G	-	'0' if this fragment is within an individual link

'1' if this fragment is within a group link

MORE	-	'0' if this dataitem is the last in the Tmessage.

'1' if more to follow

ITENUM	-	The number of the dataitem which includes the information in this message.

LASTBIT	-	Indicates the bit number (see 17.0.2.5) of the last bit of user information within the last data codeword holding user information.

FRAGL	-	Number of data codewords appended

FRAGLG	-	fragment length (8 bit field for group) number of data codewords appended

TNITEL	-	Proposed Transmitted  next dataitem length

RSVD	-	Reserved. Default value = all '0's

P	-	parity check bits