4.	ADDRESSING

The unit address enables the TSC to recognise the source of messages and/or to direct messages to a particular unit or group of units.  In addition, addresses may be used by the TSC to regulate access to the system.

This standard permits considerable flexibility in the way that unit addresses can be allocated, allowing each system full use of all available addresses.  However system operators shall not allocate addresses in such a way that two units, using the same individual address, could be active on a system concurrently.  Further, this standard does not support address reuse within interconnected systems.

The protocol allows over 32000 system identity codes and over one million addresses.  A unit may be allocated different addresses for each system within which it is required to operate, or its addresses can be common to more than one system.

Unit addresses can be used for individual units or for groups of units.  A group can be formed by allocating a common address to all members of the group.  All units shall have at least one individual address.

Individual and group addresses consist of a 7-bit prefix and a 13-bit ident.  Normally units within a fleet will share a common prefix, since this allows the short addressing procedures to be used during call set-up.  Idents allocated to units must be equal to the binary equivalent of decimal numbers in the range 1 to 8100, inclusive.

The ident value 0 shall not be allocated to any unit and is designated the "dummy" ident, DUMMYI; this ident may be used as a null value.

Values of ident above 8100 are designated special idents and are not available for allocation to units.  Use of these special idents allows a number of additional procedures and facilities to be achieved within this protocol standard.  Some special idents are designated as gateways.  These are used for calls which involve connection to communication facilities external to the system.

The arrangement of idents is summarised in Table 4-1.

Ident number 0 and special idents do not have a prefix associated with them; the prefix is only relevant to individual unit and group addresses.

The system-wide all-call ident applicable to all units (irrespective of prefix) is denoted by ALLI.  The individual ident of the Trunking System Controller (TSC) is denoted by TSCI; this ident is the same for all TSCs.

Special idents are also employed within certain call procedures described in this standard.  These include:

SDMI: 	which is used in the short data message procedures;

DIVERTI: 	which is used during call diversion procedures;

INCI: 	which is used by the TSC when checking the availability of a unit requested to be included in a call;

and	REGI: 	which is used by the TSC in the registration procedures.

Two methods, both of which employ gateway idents, are provided for radio units requesting calls to the PSTN, namely:

- "short-form" calls, to destinations previously nominated (eg the radio unit's head office).

- "general" calls, to any PSTN destination.

Radio units requesting calls to prearranged PSTN or Data Network destinations use the short addressing procedures, with the called ident set to an appropriate "short-form"  PSTN or Network ident.  These short-form idents are denoted by PSTNSIj or NETSIj respectively, for j = 1 to 15.  PSTNSIj is used for all appropriate call requests except RQD, and NETSIj is only used in RQD call requests.  Each short-form ident allocated to a radio unit shall represent a complete destination previously agreed between the system operator and the radio user.  A particular short-form ident may be reused for other radio units, each use having a distinct meaning.  Thus, when a short-form call is requested, the TSC shall determine the meaning of the particular short-form ident by reference to the calling radio unit's address. The same principle can be applied to incoming calls.

Radio units requesting a "general" PSTN call use the gateway ident, PSTNGI.  In this case, units are required to provide the full dialling information for the PSTN destination using the extended addressing procedures described in this standard.

Radio units requesting a "general" data network call use the gateway ident, DNI. In this case units will be allocated a data channel and TRANS. After this they supply the network addressing information on the data channel in a format appropriate to that network.

Radio units can request calls to PABX extensions using the short addressing procedures, provided that the extension number can be represented by 13 bits.  A call may be to any one of four PABX exchanges, as previously agreed between the system operator and the radio use - the TSC shall determine the appropriate exchange by reference to the calling radio unit's address.  Calls to PABX destinations that cannot be accommodated by the short addressing procedures use the PABX gateway ident, PABXI, and the extended addressing procedures.

Calls between units which do not share a common prefix also require use of the extended addressing procedures.  For such calls the appropriate special ident is IPFIXI.

Meaning	Notation	Ident

System-wide ident	ALLI	8191

Ident of TSC	TSCI	8190

Interprefix ident	IPFIXI	8189

Short data message ident	SDMI	8188

Divert ident	DIVERTI	8187

Include ident	INCI	8186

Registration ident	REGI	8185

Reserved for future allocation		8181 - 8184

Spare for customisation of systems		8136 - 8180

Short-form PSTN idents	PSTNSIj	8121 - 8135} Note:Common Short-form data Network idents	NETSIj	8121 - 8135}	range.

(j = 1 ... 15)

Reserved for future allocation		8104 - 8120

Data Network gateway ident	DNI	8103

PABX gateway ident	PABXI	8102

General PSTN gateway ident	PSTNGI	8101

User idents		1 - 8100

(individual and group idents)

Dummy ident	DUMMYI	0

Table 4-1 - Ident Numbering Scheme