14.		SHORT DATA MESSAGE PROCEDURES

This section defines the procedures for short data messages of up to 184 bits of free-format data, transmitted on a control channel.  The data is contained in up to four data codewords, appended to an address codeword (HEAD).

A radio unit requests to send a short data message using the RQC message (see 5.5.3.1.8).  The TSC then:

-	instructs the unit to send the short data message (and extended address information if appropriate)

-	forwards the message to the called party

-	indicates the outcome of the transaction to the calling unit.

A radio unit may send a short data message to the TSC or to a radio unit, a line unit, a group of units, all units in the system, a PABX extension (short or extended addressing) or a PSTN number (short-form or general).

The TSC may also transmit short data messages (addressed to a radio unit, a group or all units in the system), originated from the TSC itself or from a line unit, a PABX extension or the PSTN.

For calls from radio units, the TSC uses the AHYC message to demand:

a.	the called address information, for calls to:

-	interprefix addresses

(if appropriate: see below)

-	general PSTN destinations

-	"long" PABX extension numbers

b.	the short data message.

If both a. and b. are needed, the TSC obtains the full information in two steps, each step using the AHYC message.  The AHYCs are distinguished by the setting of IDENT1 (to IPFIXI, PSTNGI or PABXI for a., or to SDMI for b.), and so the order in which they are sent is not prescribed.

Note that, when a radio unit sends its short data message, it supplies the address (prefix/ident) of the called party in the data message header. Therefore, for an interprefix call, the TSC need not demand the called address separately unless it is required for operational convenience.

The format for data within a short data message is not prescribed. Also, further (system-dependent) specifications may be required to define:

-	timings for repeat messages, and/or

-	enumeration of data messages

to prevent duplicate messages (when a recipient accepts the repeat of a message as a new message).

A typical message sequence for a radio unit sending a short data message to another radio unit is illustrated in the example below.

|  |  |  |  |  |  |  |  |  |  |  | 1 slot <---> |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSC to RUs |  |  | 1 ALH (3) | ALH (0) | 3 AHYC | ALH (0) | ALH (4) | 5 HEAD | ¦5 data ¦ | ALH (0) | 7 ACK (1) | ACK (1) |  |  |

| RUs to TSC |  |  |  | 2 RQC |  | 4 HEAD | ¦4 data ¦ |  |  | 6 ACK |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | \____________________/ frame |  |  |  | \___________________________/ frame |  |  |  | \____/ frame |  |  |

Example	A message sequence on a control channel for sending a short data     message from one radio unit to another radio unit on the same site.  In this example, the data message comprises an address codeword and  two appended data codewords.

1.	ALH	:  General Aloha invitation (three-slot frame).

2.	RQC	:	Random access request to transmit a short data message. (The request indicates the number of timeslots required for the data message: in this case, two slots.)

3.	AHYC	:	Short data invitation message

-	acknowledges the RQC message

-	instructs the calling unit to send the data message in the next two slots

-	inhibits random access in the next slot.

4.	HEAD + data	:The calling radio unit sends its short data message to the TSC.  In this example the message comprises an address codeword (HEAD) and two appended data codewords.

5.	HEAD + data  :	The TSC forwards the short data message to the called radio unit.

The second data codeword contains a flag (RSA) which is set to '0' to inhibit random access in the following slot, thus reserving the slot for a response from the called unit.

6.	ACK	:	Acknowledgement ACK(QUAL=0) from the called radio unit - data message accepted.

7.	ACK	:	Acknowledgement ACK(QUAL=0) sent to the calling unit to indicate that the called unit has accepted the data message.  In this example the TSC immediately  repeats the ACK message, for added reliability.

14.1		TSC Procedures for Short Data Messages

14.1.1		Responses to a short addressing RQC message

A radio unit requests to send a short data message by generating an RQC message, complying with the random access protocol.  On receiving a short addressing RQC message (with EXT = 1, or with EXT = 0 and IDENT1 set to a valid called party ident), the TSC shall send one of the following responses:

a.	ACKI(QUAL=1), ACKQ(QUAL=1), ACKX or ACKV, with PFIX/IDENT2 as the calling unit's individual address and IDENT1 as the called ident (or PABXI for a call to a PABX extension).

b.	ACKT(QUAL=0), with PFIX/IDENT2 as the calling unit's individual address.

c.	An AHYC message instructing the calling unit to send its data message.

For acceptable delay, see 7.2.4.  See also 14.1.4 and 14.1.5.

14.1.2  Responses to an extended addressing RQC message

A radio unit requests to send a short data message by generating an RQC message, complying with the random access protocol.  On receiving an extended addressing RQC message (with EXT = 0 and IDENT1 = IPFIXI, PSTNGI or PABXI), the TSC shall send one of the following responses:

a.	ACKI(QUAL=1), ACKX or ACKV(QUAL=0), with the same prefix and idents as the RQC.

b.	An AHYC message instructing the calling unit to send the full called address information.

c.	An AHYC message instructing the calling unit to send its data message.

For acceptable delay, see 7.2.4.  See also 14.1.3 to 14.1.5.

14.1.3		Instruction to send extended address information

After receiving an extended addressing RQC message, the TSC may demand the full called address (if appropriate), by sending the AHYC message with:

-	the same prefix and idents as the RQC

(i.e. IDENT1 set to IPFIXI, PSTNGI or PABXI as appropriate, and PFIX/IDENT2 set to the calling unit's address)

-	DESC set to indicate the appropriate gateway (see 5.5.3.2.8)

-	SLOTS set to correspond to the RQC

(i.e. if IDENT1=PSTNGI and FLAG1=1 then SLOTS='10' else SLOTS='01').

The AHYC message instructs the calling unit to send the called party address information in the following SLOTS slot(s) (see 9.2.2.1).  If the TSC does not successfully decode the address information, it may repeat the AHYC message or transmit ACKV(QUAL=0) to indicate failure of the transaction.

Note that, when the radio unit sends its short data message, it supplies the called address (prefix/ident) in the data message header. Therefore, for an interprefix call, the TSC need not demand the called address separately unless it is required for operational convenience.

14.1.4		Instruction to send the short data message

After receiving an RQC message, the TSC may demand the short data message from the calling radio unit by sending the AHYC message, with:

-	IDENT1 set to SDMI

-	PFIX/IDENT2 set to the calling unit's address

-	DESC set to '000'

-	SLOTS equal to SLOTS from the RQC.

The AHYC message instructs the calling unit to send its short data message in the following SLOTS slots (see 9.2.2.1).  If the TSC does not successfully decode the short data message, it may repeat the AHYC message or transmit ACKV(QUAL=0) to indicate failure of the transaction.

Note that AHYC bars random access only in the first following return slot.  When demanding a short data message, the TSC shall take appropriate action to reserve the subsequent return slot(s) if they are within a frame (e.g. by sending the AHY message with both idents set to DUMMYI).

14.1.5		Acknowledgements sent to indicate progress of RQC transaction

The TSC may send acknowledgement messages to indicate to a calling radio unit the progress of its short data transaction - for idents, see 5.5.2.1.  (For an extended addressing call, acknowledgements ACKQ, ACKV(QUAL=1), ACKT(QUAL=0) and ACK(QUAL=0) are not appropriate until the called address has been obtained.  Acknowledgements ACKQ(QUAL=0) and ACK(QUAL=0) are not appropriate until the short data message has been obtained.)

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	System is busy.  Wait for further signalling.

ACKQ (QUAL=1)	-	Called party engaged.  Wait for further signalling.

ACKX (QUAL=0)	-	Invalid call e.g. TSC does not support short data messages, or called party is not equipped to accept the message.

ACKX (QUAL=1)	-	System or called unit overload; message rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or transaction abandoned.

ACKV (QUAL=1)	-	Called party engaged (and TSC will not hold the request) or called unit does not wish to accept the message.

ACKT (QUAL=0)	-	Called party's data calls have been diverted.

ACK (QUAL=0)		-	Transaction has been successfully completed.

For maximum acceptable delay of repeats of acknowledgements ACKX, ACKV, ACKT and ACK, see time-out TB in 14.2.4.

14.1.6	Availability check on called radio unit

Before transmitting a short data message to a radio unit, the TSC may check that the unit is in radio contact (and suitably equipped).  It uses the AHY message, with:

-	bit POINT set to '0'

-	bit CHECK set to '0'

-	bit D set to '1'

-	bit E set to '0'

-	bit AD set to '0'

-	PFIX/IDENT1 as the called unit's address

-	IDENT2 set to SDMI.

The AHY message demands a response in the following slot from the called unit (see 9.2.2.2B).

The TSC may indicate the result of the availability check to a calling radio unit by sending appropriate acknowledgement(s) (see 14.1.5).

14.1.7		Informing called party

The TSC transmits a short data message to a radio unit, a group or all units in the system by sending the HEAD message on a control channel (see 5.6.2).  The data message may have originated from the TSC itself, or from a radio unit (using RQC etc.), a line unit, a PABX extension or the PSTN.

The HEAD address codeword indicates the number of appended data codewords (up to four), and contains two 20-bit addresses: the called address and calling address (or gateway).  The user data is contained in the data codewords.  For an individually addressed short data message sent within a frame, the TSC shall set the RSA flag in the last data codeword (or in the "filler" data codeword) to '0', to inhibit random access in the next slot.

For an individually addressed short data message, the HEAD message demands a response from the called unit (see 14.3.1.1).  If the response is ACK(QUAL=0), ACKX or ACKV(QUAL=1), the TSC may send appropriate acknowledgement(s) to a calling radio unit (see 14.1.5).  If the TSC does not successfully decode a response, or if the response is ACKB(QUAL=1), it may repeat the HEAD message.  If the called unit cannot be contacted, the TSC may indicate the failure to the calling unit by sending ACKV(QUAL=0).

For a short data message addressed to a group (or system-wide), the called units do not respond; the TSC may repeat the data message, to increase the probability of successful receipt.  After transmitting the short data message, the TSC may send ACK(QUAL=0) to a calling radio unit.

14.1.8		Aborting the transaction

A calling radio unit may abort its short data transaction by generating an RQX message (see 5.5.3.1.3), complying with the random access protocol. On receiving an RQX message aborting a short data transaction, the TSC shall send a response: ACK(QUAL=1) with the same prefix and idents as the RQX.

14.1.9		TSC time-out

The TSC may operate a time-out on the maximum time for which it holds a short data message (for example, waiting for the called party to be free).

The TSC may instruct a calling radio unit to restart its waiting timer TJ or TW, by sending the AHY message with bit POINT set to '1'; see 9.1.1.7 and 9.2.2.3.  If a time TJ or TW, minus the tolerance on the radio unit's timer, elapses since the last message it received for a short data transaction (from the calling unit), the TSC shall not send any further signalling for the transaction.  See also 14.2.6.

14.2		Procedures for Radio Units Sending Short Data Messages

A radio unit shall make only one call attempt at a time (except in emergency); while attempting access or waiting for a terminating (i.e. end-of-transaction) acknowledgement to a short data message request, the unit shall not request another non-emergency call of any type (unless the user first aborts the original transaction).

14.2.1		Request for a short data transaction

A radio unit requests to transmit a short data message by sending an RQC message on a control channel, complying with the random access protocol (see 7.3).  The fields in the RQC message shall be set appropriately (see 5.5.3.1.8); however, note particularly that:

a.	Field SLOTS specifies the number of timeslots required for the data message (minimum two slots, maximum three slots).

b.	An extended addressing request is indicated by setting IDENT1 in the RQC message to the appropriate gateway (viz. IPFIXI, PSTNGI or PABXI).

The unit shall attempt access until it receives a valid response (see 14.2.2/3), or until its user aborts the transaction (see 14.2.7), or until the access attempt fails (i.e. the unit has sent the maximum number of transmissions NR and received no response, or its access time-out TC has expired (see 7.3.8)).  In the case of access failure, if the unit has not sent a request, it shall return to the idle state (and may indicate the failure to the user); otherwise, it shall wait for further signalling for the transaction - see 14.2.4 to 14.2.6.

14.2.2		Valid responses to a short addressing RQC

For a short addressing RQC, the calling unit shall accept the following messages as a response to its RQC and send no more requests:

a.	ACKI(QUAL=1), ACKQ(QUAL=1), ACKX or ACKV, with PFIX/IDENT2 as its individual address and IDENT1 as the called ident (or PABXI for a PABX call).

b.	ACKT(QUAL=0) with PFIX/IDENT2 as its individual address.

c.	AHYC, with PFIX/IDENT2 as its individual address and IDENT1 as SDMI.

For other actions on receiving these messages, see 14.2.4 and 9.2.2.1.

14.2.3		Valid responses to an extended addressing RQC

For an extended addressing RQC, the calling unit shall accept the following messages as a response to its RQC and send no more requests:

a.	ACKI(QUAL=1), ACKX or ACKV(QUAL=0), with the same prefix and idents as the RQC.

b.	AHYC, with the same prefix and idents as the RQC.

c.	AHYC, with PFIX/IDENT2 as its individual address and IDENT1 as SDMI.

For other actions on receiving these messages, see 14.2.4 and 9.2.2.1.

14.2.4		Acknowledgement received

If a radio unit attempting access or waiting for further signalling for a short data transaction receives an appropriate acknowledgement then it shall take action as indicated below.  (ACKQ, ACKV(QUAL=1), ACKT(QUAL=0) and ACK(QUAL=0) are not appropriate until the called address information has been sent - in the RQC, as extended address information or in the short data message.  ACKQ(QUAL=0) and ACK(QUAL=0) are not appropriate until the short data message has been sent.)  For idents, see 5.5.2.1.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKQ (QUAL=0)	-	System is busy.  Wait for further signalling.

ACKQ (QUAL=1)	-	Called party engaged.  Wait for further signalling.

ACKX (QUAL=0)	-	Invalid call; message rejected.

ACKX (QUAL=1)	-	System or called unit overload; message rejected.

ACKV (QUAL=0)	-	Called unit not in radio contact or transaction abandoned.

ACKV (QUAL=1)	-	Called party engaged (and TSC will not hold the request) or called unit does not wish to accept the message.

ACKT (QUAL=0)	-	Called party's data calls have been diverted.

ACK (QUAL=0)		-	Transaction has been successfully completed.

If ACKI(QUAL=1) or ACKQ is received, the unit shall wait for further signalling and may indicate to the user the progress of the transaction.

If ACKX or ACKV is received, the unit shall return to the idle state and may indicate to the user the reason for the failure of the transaction; it is recommended that receipt of ACKX(QUAL=0) be indicated in a distinct manner.

If a complete ACKT(QUAL=0) message is received, the unit shall either:

a.	return to the idle state (and may indicate to the user that the called party's data calls have been diverted), or

b.	attempt a new short data transaction to the diversion address given in the ACKT message.

If an incomplete ACKT(QUAL=0) message is received, then:

i)	If the unit does not require the diversion address, it shall return to the idle state (and may give an indication to the user).

ii)	If the unit does require the diversion address then:

-	if still attempting access for the transaction, it shall ignore the message and continue to attempt access;

-	otherwise it shall wait for a repeat ACKT, returning to the idle state if a time TB elapses (in which case, it may indicate the failure to the user).

If ACK(QUAL=0) is received, the unit shall return to the idle state and may indicate to the user that the transaction has been successfully completed i.e. that:

-		for an individual call, the called unit has accepted the short data message; (note that this does not imply user acceptance);

-		for a group (or system-wide) call, the short data message has been sent to the group.

After receiving ACKX, ACKV or ACK for the transaction, the unit shall not request another non-emergency call of any type to the same called ident (or gateway) for at least a time TB.  After receiving ACKT for the transaction, the unit shall not request another non-emergency call of any type for at least a time TB.

14.2.5		Sending the short data message

The calling unit shall transmit its short data message (a HEAD address codeword and appended data codeword(s) - see 5.6.2) on receipt of an appropriate AHYC from the TSC; see section 9.2.2.1.

14.2.6		Time-out after waiting

A calling radio unit waiting for further signalling for a short data transaction shall return to the idle state if a time TJ (for a data message addressed to the TSC) or TW (for other destinations) has elapsed since the last signalling message it sent for the transaction, viz.

RQC,   requesting the transaction (see 14.2.1)

or	SAMIS, providing extended address information for the call (see 9.2.2.1)

or	HEAD,  containing the short data message (see 14.2.5 and 9.2.2.1)

or	ACK(QUAL=0), sent in response to an AHY message with bit POINT = 1 and IDENT1 as the called ident or gateway (see 9.2.2.3).

It may also indicate to the user that the outcome of the transaction is unknown.

14.2.7		Aborting the transaction

A radio unit may abort a short data transaction (after sending an RQC and while still waiting to receive ACKX, ACKV, ACKT or ACK) by transmitting an abort transaction request RQX (see 5.5.3.1.3), complying with the random access protocol.  It shall attempt access until one of the following occurs:

a.	It receives ACK(QUAL=1) with the same prefix and idents as the RQX.  In this case, it may indicate to the user that the outcome of the transaction is unknown.

b.	It receives ACK(QUAL=0), ACKX, ACKV or ACKT(QUAL=0) for the transaction it is attempting to abort.  See also 14.2.4.

c.	It has sent the maximum number of transmissions NR and received no response, or its access time-out TC has expired (see 7.3.8).  In this case, it shall return to waiting for signalling for the short data transaction (see 14.2.4 to 14.2.6).

In cases a. and b., the unit shall return to the idle state.

14.3		Procedures for All Radio Units on a Control Channel

The procedures in this section shall be obeyed by all radio units that are equipped to recognise a received HEAD address codeword.  (The requirement to recognise HEAD will be system-dependent.)

14.3.1		Receiving short data message (HEAD)

14.3.1.1	Individually addressed HEAD message

If a radio unit on a control channel receives a HEAD message with PFIX1/IDENT1 matching its individual address then it shall respond with the appropriate acknowledgement (see below), with PFIX/IDENT1 as its individual address and IDENT2 set to IDENT2 from the HEAD.  The HEAD address codeword contains a field (LEN) which indicates the number of appended data codewords; the unit shall respond in the slot following the last data codeword.  For timing, see 6.2.1.3.

a.	If the unit is not equipped to accept the data message then it shall send ACKX (QUAL=0).

b.	Otherwise, the unit shall send one of the following acknowledgements:

ACKB (QUAL=1) if not all the appended data codewords were decodeable and the unit requires the message to be retransmitted

or	ACKX (QUAL=1) if it cannot accept the message at this time e.g. its data store is full)

or	ACKV (QUAL=1) if it does not wish to accept a data message from this calling party

or	ACK (QUAL=0) if it has accepted the data message.

14.3.1.2	HEAD message addressed to a group

If a radio unit on a control channel receives a HEAD message with PFIX2/IDENT2 not matching its individual address, and

PFIX1/IDENT1 matching one of its group addresses for this system   or

IDENT1 set to the system-wide all-call ident ALLI,

then it may accept the information contained in the HEAD address codeword and the appended data codewords, but shall transmit no response.