2.	      	DEFINITIONS

Note - Words appearing within asterisks within these definitions are defined terms.  (eg *defined term*)

Active on a Channel: A *radio unit* is *active on a channel* when, on  that channel, it is enabled to respond to *messages* addressed to it, or is transmitting, or is in transition between these two states.

Note - a *radio unit* becomes active on an assigned *traffic channel* as soon as it can receive on that channel, whereas, on a *control channel* it shall not become active until it has received a codeword containing an appropriate *system identity code*.

Address: A 20-bit number by which a unit or group of units is known within a *system*.  The *address* comprises two *fields*; a 7-bit *prefix* and a 13-bit *ident*.

Address Codeword: A 64-bit codeword, conforming to the requirements of this standard, where the first bit is set to '1'.  An *address codeword* is always the first codeword in any *message*, and defines the nature of the *message*.

Base Station: The entirety of transmitters and receivers operated by a *trunking system controller* at any one site.

Call: 	A complete information exchange between two or more *parties* which includes one or more *transactions* and may include direct user-to-user communication on a *traffic channel*.

Called Unit (or Group): The unit, or group of units, which a *calling unit* identifies as the desired recipient(s) of a *call*.  The *called unit (or group)* retains this designation for the duration of a *call* and this convention is used in *messages* relating to that particular *call*, irrespective of the origin of such *messages*.

Calling Unit: A *radio unit* or *line unit* which request a *call*. The *calling unit* retains this designation for the duration of a *call* and this convention is used in *messages* relating to that particular *call* irrespective of the origins of such *messages*.

Common Prefix Call: A *call* where the values of the *prefixes* in the calling and called *addresses* are the same.  *Common prefix calls* use the *short addressing* procedures.

Control Channel: A *forward channel* and *return channel* being used for the transmission of *messages* conforming to this standard with the primary purpose of enabling the *trunking system controller* to control radio units.

Data Codeword: A 64-bit codeword, conforming to the requirements of this standard, where the first bit is set to '0'.  *Data codewords* are concatenated to an *address codeword* and supplement the information in the *address* codeword*.

Dataitem: The whole, or a part of, a *Tmessage*. A dataitem may not include more than 62 data codewords.

Decodeable: A transmitted codeword shall be considered *decodeable* if, after receipt, and after any error correction (if used) has been applied, a valid codeword from the code defined in section 3.2.3 of this standard is formed.

Diversion: A procedure whereby a *party* may  request that future *calls* to a particular called address be redirected to an alternative destination.

Extended Addressing: A method which allows called *party* details to be conveyed to the *trunking system controller* when the *call* details cannot be accommodated in a single *address codeword*.  These called-party details may be an *address* or addressing information in a different form (eg PSTN dialling digits).

Field: A number of contiguous bits in a codeword which is specified in terms of the position within the codeword and the number of bits.

Forward Channel: A radio bearer where the direction of transmission is form the *base station* to *radio units*.

Fragment: A message which is either the whole of a *dataitem* or those codewords of a *dataitem* for which repetition has been requested by the receiving station.

Free Format Data: Data  within a codeword which, in this standard, is  constrained only by its position and length.

Gateway: A *special ident* which is used to identify a *message* relating to a *call* or *transaction* to or from a communications service outside of the *system* (eg the PSTN).  For the purposes of this standard the interprefix *ident*, IPFIXI, is also regarded as a *gateway*.

Group Address: An *address* which is common to more than one unit and which, when nominated as the called *address*, signifies a *group call*.  Units may be assigned any practicable number of *group addresses*.

Group Call: A *call* in which a *group address* is specified as the called *party* and, accordingly, provides a means of communication between more than two units.  The calling *party* in a *group call* may opt for a conversational mode, where all *parties* are able to speak, or for an announcement mode where only the caller may speak.

Ident: 	A 13-bit number used for identification purposes.  Values of *ident* between 1 and 8100 inclusive are assigned to individual units or groups, in which case they are associated with a *prefix* to form a 20-bit *address*.  Values of *ident* above 8100 are designated *special idents* and these are not associated with any particular *prefix*, neither is the *ident* value 0 (DUMMMYI).

Idle State: A *radio* unit* is in the *idle state* on a *system* when it is *active on a control channel* belonging to that *system*, is not currently within a *message* exchange and has no current *message* transfer requirement.

Include: A procedure whereby *parties* may be introduced into a *call* in progress at the request of an existing *party* to the *call*.

Individual Address: An *address* by which a single unit is known within a *system*, allowing that unit to be uniquely addressed by that *system*.  Units may be assigned any practicable number of *individual addresses* provides that at least one per *systems* is assigned to each unit.

Individual Call: A *call* between a calling *party* and a single called *party*.

Interprefix Call: A *call* where the values of the *prefixes* in the calling and called *addresses* are different.  *Interprefix calls* require *extended addressing* procedures.

Invoking message: A message from the TSC to a radio unit which requires or invites an immediate message from the radio unit according to the timing rules specified in section 6 if the transmission rate is 1200 bit/sec or the equivalent rules at any other transmission rate.

Item: 	A complete user transmission on a *traffic channel* by one *party* within a *call* at the conclusion of which that *party* rests from transmission.  It is possible for a *call* to contain only one *item*.

Line Unit (LU): A user station which is allocated an *individual address*, and is directly connected to the *trunking system controller* via a medium other than the radio spectrum to which this standard applies.

Link: 	Any transmission path in the communication chain between the end users in a Standard Data call, and particularly the radio connection between the TSC and its dependent radio unit in such a call.

Message: A single contiguous data transmission which consists of a codeword synchronisation sequence, an *address codeword* and (optionally) one or more *data codewords* conforming to this standard.

Non-prescribed data: Any data traffic which does not conform to the data protocols defined in this standard.

Party: 	A source and/or recipient of information within a *call*.  The term includes the totality of equipment at the user station and, where the context permits, the equipment user.  A party may be an individual or a group.

Prefix: The 7 most significant bits of an *address*.  Normally units within a fleet will be allocated the same *prefix* since *calls* between units and groups with the same *prefix* can be made without the use of *extended addressing* procedures.  A *prefix* is only relevant to *individual addresses* and *group addresses*.

Radio Unit (RU): A mobile or other user station contacting a *system*, by normal land mobile radio in accordance with this standard.

Random Access Attempt: The method by which a *radio unit* transmits an unsolicited *message* to the *trunking system controller* on a *control channel*.  The method requires that a *radio unit* repeats a random access *message* if a response *message* is not received within a designated waiting time.  Further  repeats are required, in the absence of an  appropriate acknowledgement, until a designated  number of repeats is reached.  In this standard a  *random access attempt* covers the period from  initiation of the *transaction* to the receipt of an  appropriate acknowledgement or the expiry of a  timeout.

Ready-for-Communication Control (RFCC): A device or system to inform a unit of the user's readiness to communicate, eg a switch-hook.

Registration: A procedure which confirms that a *radio  unit* is within a *session* on a *system*.  The  *registration* procedures may be initiated by a  demand from the *trunking system controller*, or at  the initiative of the *radio unit*, depending on the  circumstances of the *registration*.

Requested Unit (or Group): A unit, or group of units, which takes part in a *transaction* initiated by the *trunking system controllers* or another *party*.

Requesting Unit: A *radio unit* or *line unit* which initiates a *transaction* with the *trunking system controller* or another *party*, via the *trunking system controllers*.

Reserved: Codewords and *fields* which are designated as  *reserved* in this standard are intended for future  phases of standardisation and shall not be used in  the interim for the conveyance of information.   *Reserved fields* must be set to the default value  specified in this standard.

Return Channel: A radio bearer where the direction of transmission is from *radio Units* to the *base station*.

Session: A *session* is a period of operation associated  with one *system*.  A *session* on a *system* starts  when the *radio unit* becomes *active on a control  channel* of that *system*, either after switch-on or  after being *active on a control channel* of a  different *system*.  A *session* ends either when  the *radio unit* is switched off or when it starts  its next *session*.

Short Addressing: The method used when the *parties* to a  *call* can be completely specified by a single  *prefix* and two *idents*.  This form of addressing  minimises the signalling required.

Short Data: A procedure which allows a data *message* to  be exchanged between *parties*, or between *parties*  and the *trunking system controller*.  This  procedure does not support *messages* which include  more than four *data codewords*.

Short-Form PSTN Destination: A called PSTN *party*, previously agreed between the system operator and the user of the *calling unit*, which can be specified by a *special ident*, rather than the full stream of dialling digits representing the directory number.

Spare: Codewords and *fields* which are designated as *spare* are available for free use by *systems* (ie *system* customisation) provided that the conditions of this standard are not infringed.  The use of spare codewords and *fields* may vary from *system* to *system*.

Special Ident: An *ident* with a value greater than 8100. These *idents* are used for a variety of special purposes.  Some of these are specified in this standard, others may be nominated by system operators.  *Special idents*  are not associated with a *prefix* to form an *address*.

Standard Data: The procedure by which information exchange takes place using the data protocol defined in section 17 of this standard.

System: The totality of equipment required to provide the  communication facilities associated with a single  *system identity code*.  *Systems* may be combined  to form larger communications facilities, but the  delineation of *systems* and methods of combination  are not within the scope of this standard.

System Identity Code: A 15-bit number which contains a  unique identification of a *system*.  This code is  radiated on each *forward control channel* within  the *system* (in the SYS *field*).

Tmessage: A quantity of *user data* which the  correspondents by previous bilateral agreement have  mutually agreed is useful to them as a distinct  entity, and is marked as such by the originator for  end-to-end transmission.

Traffic Channel: A *forward channel* and *return channel*  being used primarily for user communication.

TRANS: A 10 bit transaction number allocated to a *link*  during set-up of a data call to replace the address  and port of the radio unit. The validity of a TRANS  ceases at the conclusion of the data call.

Transaction: A complete information exchange consisting of one or more *messages* between a *party* and the *trunking system controller*, or another *party*, via the *trunking system controller*.

Trunking System Controller (TSC): The central control intelligence necessary to enable the trunking system to function according to this standard.  The *trunking system controller* may control one or more *basestations*.

User Data: Data from or to the user which is either to or  from his correspondent, or is concerned with call  routing but is transmitted after a *TRANS* for the  call has been allocated.

User Data Message: A message headed by address codeword  "SITH" and containing user data.