12.		CALL DIVERSION PROCEDURES

This section defines the procedures for requesting and cancelling call diversion.  Requests may apply to speech calls, data calls or both.

Two types of call diversion are provided:

i)	Self-initiated diversion.

A radio unit may request that future calls addressed to it be redirected to a specified alternative destination.

ii)	Third-party diversion.

A radio unit may request that future calls addressed to another subscriber unit (or group) be redirected to a specified alternative destination.  For example, a despatcher unit may request diversion on behalf of a radio unit in its fleet.

In general, radio unit A (the requesting unit) may divert calls for address B (the blocked address) to alternative destination C (the diversion address); for self-initiated diversion, B = A.  The procedures permit blocked address B to be a radio unit, line unit or group address. Destination C may be a radio unit, line unit or group address, a PABX extension or a PSTN number (short-form or general).

Three types of call diversion cancellation are provided:

i)	Self-initiated cancellation.

A radio unit may request that its calls are no longer diverted.

ii)	Third-party cancellation.

A radio unit may request that another subscriber unit's (or group's) calls are no longer diverted.

iii)	General cancellation by recipient.

A radio unit may request that any existing diversions to it be cancelled.  (This is a general cancellation by the recipient of diversions; specific cancellation of diversions by the recipient is covered by third-party cancellation.)

It is recommended that requests for third-party diversion or third-party cancellation are accepted only from authorised units.

Call diversion is requested or cancelled using the Request Call Diversion Message RQT (see 5.5.3.1.4).  In this message:

- 	PFIX/IDENT2 is the address of the requesting radio unit.

- 	For diversion requests, IDENT1 is the diversion ident C

(or IPFIXI, PABXI or PSTNGI for an interprefix address, any PABX extension or a general PSTN number respectively).

For "self" or "third-party" cancellation of call diversion, IDENT1 specifies the unit or group whose calls should be returned (or IPFIXI for an interprefix address).

For general cancellation by a recipient, IDENT1 is set to DIVERTI.

-	Bit DIV indicates call diversion or cancellation.

-	For call diversion, FLAG2 indicates "self" or "third-party" diversion.

-	Field SD specifies the types of call (i.e. speech, data or both) to which the diversion or cancellation refers.

For diversion purposes, "speech" calls are defined as calls requested using RQS(DT=0), RQE(D=0), RQQ(STATUS='00000') or RQQ(STATUS='11111').  "Data" calls are defined as calls requested using RQS(DT=1), RQE(D=1), RQQ('00001' to '11110'), RQC or RQD.

For self-initiated diversion requests or any cancellation, two addresses (plus the other parameters) specify the requirement.  However, for third-party diversion requests, address B (the blocked address) must be supplied.

The TSC uses the AHYC message to demand:

a.  extended address information for IDENT1

b.  blocked address B

as appropriate.  If both a. and b. are needed, the TSC obtains the full information in two steps, each step using the AHYC message.  The AHYCs are distinguished by the setting of IDENT1 (to IPFIXI, PABXI or PSTNGI for a., or to DIVERTI for b.), and so the order in which they are sent is not prescribed.

In the procedures, a request for call diversion or cancellation is defined as an extended addressing request if IDENT1 is set to IPFIXI, PABXI or PSTNGI.  A request is defined as "complex" if it requires extended addressing or if three addresses must be provided; otherwise it is "simple".

Note that extended addressing procedures are used for requesting diversion to any PABX extension (with either a "long" or "short" extension number).  The radio unit sets IDENT1 in the RQT message to PABXI, and then sends the PABX address information in response to an AHYC message with IDENT1 set to PABXI and DESC set to '010'.  The unit sets bit SP in the SAMIS message to indicate whether it is sending BCD digits or a 13-bit extension number (plus 2-bit exchange number).  See 5.5.3.2.8 and 5.6.1.2.2.

If the TSC accepts a diversion request and then a call to the blocked address is requested by a radio unit, the TSC indicates the diversion address to the calling radio unit using the ACKT(QUAL=0) acknowledgement. The unit then either retries on the diversion address or returns to the idle state.  For example, see sections 9.1.1.4 and 9.2.1.4.

12.1		TSC Procedures for Call Diversion Requests

12.1.1		Responses to a simple diversion request

A radio unit requests simple call diversion by generating an RQT message (with FLAG2 = 0 and IDENT1 set to a valid called party ident or to DIVERTI), complying with the random access protocol.  On receiving a simple RQT message, the TSC shall send one of the following responses, with the same prefix and idents as the RQT:

ACKI(QUAL=1), ACKX, ACKV(QUAL=0) or ACK(QUAL=0).

For acceptable delay, see 7.2.4.  See also 12.1.5.

12.1.2		Responses to a complex diversion request

A radio unit requests complex call diversion by generating an RQT message (with FLAG2 = 1 and/or IDENT1 = IPFIXI, PABXI or PSTNGI), complying with the random access protocol.  On receiving a complex RQT message, the TSC shall send one of the following responses:

a.	An acknowledgement ACKI(QUAL=1), ACKX or ACKV(QUAL=0), with the same prefix and idents as the RQT.

b.	For an extended addressing RQT:

An AHYC message instructing the unit to send the full address information for IDENT1.

c.	For a request for 3-address diversion (i.e. RQT with FLAG2=1):

An AHYC message instructing the unit to send the blocked address.

For acceptable delay, see 7.2.4.  See also 12.1.3 to 12.1.5.

12.1.3		Instruction to send extended address information

After receiving an extended addressing RQT message, the TSC may demand the full address information for IDENT1 by sending the AHYC message, with:

-		the same prefix and idents as the RQT

(i.e. IDENT1 set to IPFIXI, PABXI or PSTNGI as appropriate, and PFIX/IDENT2 set to the requesting unit's address)

-		DESC set to indicate the appropriate gateway (see 5.5.3.2.8)

-		SLOTS set to correspond to the RQT

(i.e. if IDENT1=PSTNGI and FLAG1=1 then SLOTS='10' else SLOTS='01').

The AHYC message instructs the requesting radio unit to send the full address information for IDENT1 in the following SLOTS slot(s) (see 9.2.2.1). If the TSC does not successfully decode the address information, it may repeat the AHYC message or transmit ACKV(QUAL=0) to indicate failure of the transaction.

12.1.4		Instruction to send the blocked address

After receiving a request for 3-address diversion (i.e. FLAG2=1), the 0TSC may demand the blocked address by sending the AHYC message, with:

-	IDENT1 set to DIVERTI

-	PFIX/IDENT2 set to the requesting unit's address

-	DESC set to '000'

-	SLOTS set to '01'.

The AHYC message instructs the requesting radio unit to send the blocked address in the following slot (see 9.2.2.1).  If the TSC does not successfully decode the address information, it may repeat the AHYC message or transmit ACKV(QUAL=0) to indicate failure of the transaction.

12.1.5		Acknowledgements sent to indicate progress of RQT transaction

The TSC may send the following acknowledgement messages, with the same prefix and idents as the RQT, to indicate to a radio unit the progress of its diversion transaction.  (For a complex diversion request, ACK(QUAL=0) is not appropriate until the full diversion information has been obtained.)

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKX (QUAL=0)	-	Invalid call e.g. unauthorised diversion request, or TSC does not provide call diversion.

ACKX (QUAL=1)	-	System overload; request rejected.

ACKV (QUAL=0)	-	Transaction abandoned.

ACK  (QUAL=0)	-	Call diversion or cancellation has been accepted.

For maximum acceptable delay of repeats of acknowledgements ACKX, ACKV and ACK, see time-out TB in 12.2.4.

12.1.6		Aborting the transaction

A radio unit may abort its diversion transaction by generating an RQX message (see 5.5.3.1.3), complying with the random access protocol. On receiving an RQX message aborting a diversion transaction, the TSC shall send a response: ACK(QUAL=1) with the same prefix and idents as the RQX.

12.1.7		TSC time-out

The TSC may instruct a radio unit to restart its waiting timer TJ, by sending the AHY message with bit POINT set to '1' (and the same prefix and idents as the RQT); see 9.1.1.7 and 9.2.2.3.  If a time TJ, minus the tolerance on the radio unit's timer, elapses since the last message it received for a diversion transaction, the TSC shall not send any further signalling for the transaction.  See also 12.2.5.

12.2		Procedures for Radio Units Requesting Call Diversion

A radio unit shall make only one call attempt at a time (except in emergency); while attempting access or waiting for a terminating (i.e. end-of-transaction) acknowledgement to a diversion request, the unit shall not request another non-emergency call of any type (unless the user first aborts the original transaction).

12.2.1		Diversion request

A radio unit requests or cancels call diversion by transmitting an RQT message on a control channel, complying with the random access protocol (see 7.3).  The fields in the RQT message shall be set appropriately (see 5.5.3.1.4); however, note particularly that:

a.	Bit DIV specifies whether the unit is requesting call diversion or cancellation of call diversion.

b.	Bit FLAG2 specifies whether or not three addresses must be provided.

c.	An extended addressing diversion request is indicated by setting IDENT1 in the RQT message to the appropriate gateway ident viz. IPFIXI, PABXI or PSTNGI.  (Note that extended addressing procedures are used for requesting diversion to a PABX extension with either a "short" or "long" extension number.)

The unit shall attempt access until it receives a valid response (see 12.2.2/3), or until its user aborts the transaction (see 12.2.6), or until the access attempt fails (i.e. the unit has sent the maximum number of transmissions NR and received no response, or its access time-out TC has expired (see 7.3.8)).  In the case of access failure, if the unit has not sent a request, it shall return to the idle state (and may indicate the failure to the user); otherwise, it shall wait for further signalling for the transaction - see 12.2.4 and 12.2.5.

12.2.2		Responses to simple diversion request

For a simple diversion request, the radio unit shall accept the following messages (with the same prefix and idents as the RQT) as a valid response to its RQT and send no more requests:

ACKI(QUAL=1), ACKX, ACKV(QUAL=0) or ACK(QUAL=0).

For other actions on receiving these messages, see section 12.2.4.

12.2.3		Responses to complex diversion request

For a complex diversion request, the radio unit shall accept the following messages as a valid response to its RQT and send no more requests:

a.	ACKI(QUAL=1), ACKX or ACKV(QUAL=0), with the same prefix and idents as the RQT.

b.	For an extended addressing RQT:

AHYC, with the same prefix and idents as the RQT.

c.	For a request for 3-address diversion (i.e. RQT with FLAG2=1):

AHYC, with PFIX/IDENT2 as its individual address and IDENT1 as DIVERTI.

For other actions on receiving these messages, see 12.2.4 and 9.2.2.1.

12.2.4		Acknowledgement received

If a radio unit attempting access or waiting for further signalling for a diversion transaction receives an appropriate acknowledgement (with the same prefix and idents as the RQT) then it shall take action as indicated below.  For a complex diversion request, ACK(QUAL=0) is not appropriate until the full diversion information has been sent.

ACKI (QUAL=1)	-	Intermediate acknowledgement; more signalling to follow.

ACKX (QUAL=0)	-	Invalid call; request rejected.

ACKX (QUAL=1)	-	System overload; request rejected.

ACKV (QUAL=0)	-	Transaction abandoned.

ACK  (QUAL=0)	-	Call diversion or cancellation has been accepted.

If ACKI(QUAL=1) is received, the unit shall wait for further signalling for the transaction.

If ACKX or ACKV(QUAL=0) is received, the unit shall return to the idle state and may indicate to the user that the transaction has failed.

If ACK(QUAL=0) is received, the unit shall return to the idle state and may indicate to the user that the request for call diversion or cancellation has been accepted.

After receiving ACKX, ACKV or ACK for its diversion request, the unit shall not request another non-emergency call of any type to the same called ident (or gateway) for at least a time TB.

12.2.5		Time-out after waiting

A radio unit waiting for further signalling for a diversion transaction shall return to the idle state if a time TJ has elapsed since the last message it sent for the transaction, viz.

RQT, 		requesting the transaction (see 12.2.1)

or	SAMIS, 		providing address information for the call (see 9.2.2.1)

or	ACK(QUAL=0),	sent in response to an AHY message with bit POINT = 1 and the same prefix and idents as the RQT (see 9.2.2.3).

It may also indicate to the user that the outcome of the transaction is unknown.

12.2.6		Aborting the transaction

A radio unit may abort a diversion transaction (after sending an RQT and while still waiting to receive ACKX, ACKV or ACK) by transmitting an abort transaction request RQX (see 5.5.3.1.3), complying with the random access protocol.  It shall attempt access until one of the following occurs:

a.	It receives ACK(QUAL=1) with the same prefix and idents as the RQX.  In this case, it may indicate to the user that the outcome of the transaction is unknown.

b.	It receives ACK(QUAL=0), ACKX or ACKV(QUAL=0) for the transaction it is attempting to abort.  See also 12.2.4.

c.	It has sent the maximum number of transmissions NR and received no response, or its access time-out TC has expired (see 7.3.8).  In this case, it shall return to waiting for signalling for the diversion transaction (see 12.2.4 and 12.2.5).

In cases a. and b., the unit shall return to the idle state.