3.	SIGNALLING FORMATS

This section defines the basic signalling formats used by this standard.  The detailed structure of the codewords is defined in section 5, and the timing constraints for the transmission of messages are defined in section 6.

The provisions of this section do not preclude the use of other, non-prescribed formats on a traffic channel.

3.1	Basic Format

Signalling transmissions shall employ Fast Frequency Shift Keying (FFSK) at a bit rate of 1200 bit/s.  The basic components of the signalling formats are illustrated in Figure 3-1.

| LET | Preamble | Message | H |
| --- | --- | --- | --- |

Fig. 3-1.  Basic format

3.1.1	LET

Signalling transmissions shall be preceded by a Link Establishment Time (LET) within which a transmission of undefined modulation at not less than 90% of maximum power shall take place.  The duration of the LET shall be as specified in section 3.3.3.1 and section 6.

3.1.2	Preamble

Signalling transmissions shall begin with a preamble of bit reversals 1010 . . . 10 so that the receiver data demodulator can acquire bit synchronisation.  The preamble shall consist of a minimum of 16 bits and shall end with a binary zero.

3.1.3	Message

A message is a contiguous transmission consisting of a codeword synchronisation sequence, an address codeword and, where appropriate, one or more data codewords (see 3.2).

3.1.4	Hang-over Bit, H

Signalling transmissions shall be terminated by appending a "hang-over" bit of either binary zero or binary one to the last transmitted message.

3.2	Message Format

A message consists of a codeword synchronisation sequence, an address codeword and, where appropriate, one or more data codewords, as shown in Figure 3-2.  The address codeword defines the nature of the message, and data codewords supplement the information in the address codeword.

| Codeword Sync Sequence | Address Codeword | Data Codewords (where appropriate) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Fig. 3-2.  Message format

3.2.1	Codeword synchronisation sequence

The codeword synchronisation sequence shall be transmitted to enable decoders to establish codeword framing.  It consists of 16 bits.

3.2.1.1	Control channel codeword synchronisation sequence

The codeword synchronisation sequence for messages transmitted on a control channel, SYNC, is shown in Figure 3-3.  Bit number 1 shall be transmitted first.

| bit no | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |

Fig. 3-3.  Control channel codeword synchronisation sequence, SYNC

3.2.1.2	Traffic and Standard Data channel codeword synchronisation sequence

The codeword synchronisation sequence for messages transmitted on a traffic or standard data channel, SYNT, is shown in Figure 3-4.  Bit number 1 shall be transmitted first.  If a control channel is allocated for traffic, then SYNT shall be used for all messages transmitted while the channel is assigned for traffic.

| bit no | 1 |  |  |  |  |  |  |  |  |  |  |  |  | 16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 0 |

Fig. 3-4.	Traffic and Standard Data channel codeword synchronisation sequence, SYNT

3.2.2	Codewords

Messages shall be transmitted in 64-bit codewords.  Each codeword shall contain 48 information bits followed by 16 check bits.  There are two types of codeword, address and data codewords, which are distinguished by the first bit (A) within the codeword; see Figure 3-5.  Bit number 1 shall be transmitted first.

| bit no. | 1 | 2 48 | 49 64 |
| --- | --- | --- | --- |
|  | A | information field | check bits |
| no. of bits | 1 | 47 | 16 |

Bit 1 (A)	-	Binary one denotes an address codeword. Binary zero denotes a data codeword.

Bits 2 to 48	-	Information field; see section 5.

Bits 49 to 64	-	Check bits; see section 3.2.3.

Fig. 3-5.  Codeword structure

3.2.3	Encoding and error checking

The first 15 check bits are derived from a (63,48) cyclic code.  For encoding, the codeword bits 1 to 48 represent the coefficients of a polynomial having terms from X62 down to X15.  This polynomial is divided modulo-2 by the generating polynomial:

X15 + X14 + X13 + X11 + X4 + X2 + 1

The 15 check bits correspond to the coefficients of the terms from X14 to X0 in the remainder polynomial found at the completion of the division.  The final check bit of the (63,48) cyclic code (codeword bit 63) is then inverted.  Finally, one bit is appended to the 63-bit block (including the inverted bit number 63) to provide an even parity check of the whole 64-bit codeword.

Decoding algorithms are not prescribed in this standard; for the error control properties of the codeword, see Appendix 2.

3.3	Signalling Transmission Variants

3.3.1	Single message format

The format for signalling transmissions which contain a single message is shown in Figure 3-6.

| LET | Preamble | Message | H |
| --- | --- | --- | --- |

Fig. 3-6.  Single message format

3.3.2	Multiple message format on a traffic channel

The format for standardised signalling transmissions which contain more than one message is shown in Figure 3-7.  This format shall be used only on traffic channels.

| LET | Preamble | Message | bit reversals | Message |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Fig. 3-7.  Multiple message format

For multiple messages transmitted by a radio unit, there shall be 16 bits of bit reversals between messages.  For multiple messages transmitted by the TSC, bit reversals may be inserted between the messages as required. The final bit of any bit reversals (before the next message) shall be a binary zero.

3.3.3.	Forward control channel format

3.3.3.1	Basic control channel format

The start-up sequence for a base station commencing transmission on a control channel shall be as shown in Figure 3-8.

| LET | Preamble | SYNC | ADD1 | CCSC | ADD2 | CCSC | ADD3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¬¾¾¾¾start-up sequence¾¾¾¾® |  |  |  | ¬¾1 slot¾® |  | ¬¾1 slot¾® |  |  |  |  |  |

LET	-	Link establishment time of at least 6 bit periods (5 ms).

Preamble	-	At least 16 bits of bit reversals, ending with a binary zero.

SYN	-	Control channel codeword synchronisation sequence; see 3.2.1.1.

ADDn	-	Address codeword (any appropriate message); see section 5.

CCSC	-	Control Channel System Codeword; see 5.1.

Fig. 3-8.  Basic control channel format

Following the start-up sequence the TSC shall divide time into slots, each comprising two codewords.  The first codeword of a slot shall be the Control Channel System Codeword (CCSC), unless displaced by a data codeword from a previous message.  The second codeword of a slot shall be an address codeword, unless displaced by a data codeword (see 3.3.3.2).

Every address codeword in a slot shall be preceded by a CCSC.  The CCSC identifies the system to radio units and provides control channel slot synchronisation.  It is a data codeword in which the final 32 bits form the preamble and codeword synchronisation sequence for the following address codeword (see 5.1).

The MARK address codeword (see 5.5.4.1) may be transmitted by the TSC on a newly designated control channel during the period allowed for radio units to locate and identify the control channel (see 6.1.1).

3.3.3.2	Data codeword displacement

When data codewords are transmitted as part of a message, they displace CCSCs and address codewords, as illustrated in Figure 3-9.  Radio units must be capable of satisfactory operation despite this displacement (see section 6).  The TSC shall not displace more than two CCSCs in consecutive timeslots.

| CCSC | ADDn | DCW1 | DCW2 | DCW3 | DCW4 | CCSC | ADDn+1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ¬¾-1 slot-¾® |  | ¬¾-1 slot-¾® |  | ¬¾-1 slot-¾® |  | ¬¾¾¾1 slot¾¾® |  |  |  |  |  |  |  |  |

CCSC 	-	Control Channel System Codeword

ADDn 	- 	Address codeword (any appropriate message).

DCWm 	- 	Data codeword in message.

Fig. 3-9.   Example of data codeword displacement

When a message includes an odd number of data codewords, a "filler" data codeword shall be appended to the message (to maintain the slot structure); the content of the filler data codeword is not prescribed in this standard.  See also section 7.2.5.