MPT 1327

A Signalling Standard

for Trunked Private Land Mobile Radio

Systems

January 1988

Revised and reprinted October 1991

Revised and reprinted xxxxxx1996

A SIGNALLING STANDARD FOR

TRUNKED PRIVATE LAND MOBILE RADIO SYSTEMS

(C)	Crown Copyright 1988

First published January 1988

Reprinted and revised October 1990

Reprinted and revised October 1991

Reprinted and revised xxxxxxx1996

Amendments issued since publication 1988 edition

| Amendment Number | Date of issue | Text affected |
| --- | --- | --- |
| 1 | October 1990 | Incorporated in the version of October 1990 |

Amendments issued since publication 1991 edition

| Amendment Number | Date of issue | Text affected |
| --- | --- | --- |

Amendments issued since publication 1996 edition

| Amendment Number | Date of issue | Text affected |
| --- | --- | --- |

FOREWORD

This standard defines the rules for communication between radio units and trunking system controllers operating in trunked private land mobile radio systems.

Applications and test conditions for this standard, applicable to Band III, are contained in the following specifications prepared by the Department of Trade and Industry, Radiocommunications Agency.

MPT 1343	System interface specification for radio equipment to be used with commercial trunked networks operating in Band III, sub-bands 1 and 2.

MPT 1347	Radio interface specification for commercial trunked networks operating in Bank III, sub-bands 1 and 2.

MPT 1352	Test schedule for the approval of radio units to be used with commercial trunked networks operating in Band III, sub-bands 1 and 2.

Intellectual Property Rights

Firms intending to manufacture equipment which complies with the standard should be aware that certain features of the standard are subject to IPR claims.

All firms are therefore advised that they should make appropriate enquiries through their Patent Agents before proceeding.

CONTENTS

1.	INTRODUCTION

2.	DEFINITIONS

3.	SIGNALLING FORMATS

4.	ADDRESSING

5.	CODEWORD STRUCTURES

6.	CHANNEL DISCIPLINE

7.	RANDOM ACCESS PROTOCOL

8.	REGISTRATION PROCEDURES

9.	BASIC CALL PROCEDURES

10.	EMERGENCY CALL PROCEDURES

11.	INCLUDE CALL PROCEDURES

12.	CALL DIVERSION PROCEDURES

13.	STATUS MESSAGE PROCEDURES

14.	SHORT DATA MESSAGE PROCEDURES

15.	DATA INTERROGATION PROCEDURES

16.	Section reserved for additional short data procedures  e.g. SAMs.

17.	STANDARD DATA PROCEDURES

APPENDIX 1	Suggested values for parameters.

APPENDIX 2	The error control properties of the codewords.

APPENDIX 3.	An algorithm for determining the codeword completion sequence of a control channel system codeword.

APPENDIX 4.	An algorithm for generating fields A and B of the MARK codeword.

APPENDIX 5.	BCD coding.

APPENDIX 6.	Reserved for Timing of responses for standard data at a customised rate.

APPENDIX 7.	Other ideas considered during the drafting of section 17 (standard data).

1.	INTRODUCTION

MPT1327 is a signalling standard for trunked private land mobile radio systems.  It defines the protocol rules for communication between a trunking system controller (TSC) and users' radio units.

The standard can be used to implement a wide variety of systems, from small systems with only a few radio channels (even single-channel systems), through to large networks which may be formed by the interconnection of TSCs.

The protocol offers a broad range of user facilities and system options.  However, it is not necessary to implement all of the facilities available; an appropriate subset of the protocol could be implemented, according to the user requirements.  Also, there is scope for customisation for special requirements, and provision has been made for further standardised facilities to be added to the protocol in the future.

The standard defines only the over-air signalling and imposes only minimum constraints on system design.  Additional specifications will be required for specific implementations, for example, to define:

-  the facilities that must be implemented

-  parameter values

-  a channel plan

-  for a network, criteria for when a radio unit should register.

Section 1.1 of this introduction describes the user facilities which are explicitly provided by the protocol.  (It does not describe additional facilities which may be offered in a radio unit but which do not require any specific protocol.)

Section 1.2 describes some protocol features, indicating the options available to system designers.

Section 1.3 provides an introduction to the operation of the protocol.

Subsequent sections of this document contain the protocol definition. In most of these sections, the protocol rules for the TSC and for radio units are specified separately, but with cross-referencing where convenient.

1.1	User Facilities

The facilities available to users are outlined below.  For a full definition of the facilities, see the sections indicated.

1.1.1		Types of call

The standard protocol enables radio units to make the following types of call.

a.	Speech call.  (See section 9.)

Speech calls may be requested with normal or high priority.  For group calls, the calling party may opt for a conversational mode, where all   parties are able to speak, or for an announcement mode where only the caller may speak.

b.	Data call, for the transmission of non-prescribed signalling.  (See section 9.)

Parameters are available to specify either normal or high priority and, for a group call, whether the called group members can reply.

c.	Emergency call.  (See section 10.)

Parameters are available to specify either a speech or a data call and, for a group call, whether the called group members can reply.  Also, a radio unit may request a special mode of emergency service previously arranged with the system; the TSC determines the required action by reference to the calling unit's address.

d.	Include call.  (See section 11.)

During a call, a unit may request that another party joins the call.  This facility may be used to implement a Conference Call or Call Transfer.

e.	Status message.  (See section 13.)

Thirty-two different status messages may be conveyed between units.  The meanings of two of these messages are prescribed as a "call-me-back request" and "cancel previous call-me-back request".  The remaining thirty messages have user-defined meanings.  (Status messages can also be sent between radio units and the TSC.)

f.	Short Data Message.  (See section 14.)

Messages of up to 184 bits of free format data can be sent between units, or between units and the TSC.

g)	Standard Data Call (See section 17)

A standard data channel is defined which has the capacity to sustain 1023 links, though not all need be active simultaneously.  The section defines procedures for setting up data calls and then transmitting messages in a standard manner on one or more standard data traffic channels on a base station.  Data may be transferred between radio units, or between radio units and other data devices connected to the base station infrastructure and other networks.  Errors on the data channel are corrected as necessary by automatic request for repitition (ARQ) before the data is passed on to any other data link or equipment, i.e operation is "store and forward"

1.1.2		Making calls

A radio unit may request a call to any of the following called parties (except for status messages, which cannot be addressed to PABX or PSTN destinations or to groups):

-  an individual radio unit or line-connected unit

-  a group, or all units in the system

-  a PABX number, up to nine digits

-  a PSTN number, up to 31 digits.

In addition, status messages and short data messages may be sent to the TSC.

During call set-up, the TSC may pass a wide variety of information to the caller, to indicate the progress of the call.  For example, it may indicate the reason for any delays in call set-up or the reason for a call failure.

A call request may be cancelled at any time.

1.1.3		Receiving calls

A radio unit may receive calls from a radio unit or line unit, or (except for status messages) from a PABX extension or the PSTN.  In addition, status messages and short data messages may be received from the TSC.  For a call from a radio unit, a line unit or the TSC, the calling address may be supplied to the called unit.  For a call from a PABX extension or from the PSTN, the calling gateway is indicated as the source of the call but the caller's number is not conveyed to the called unit.

Incoming calls may be addressed to the unit individually or to a group to which it belongs.  A radio unit may be a member of an arbitrary number of groups; its group addresses can be chosen independently of its individual address.

A radio unit may refuse to accept all incoming calls, for example by means of a "busy" or "out-of-vehicle" control, or incoming calls could be refused selectively, depending on the source of the call.  If a user does not wish to proceed with an incoming call immediately, he can indicate that he will call back later.

Systems may be configured to alert a called individual and require him to indicate that he is ready, before a traffic channel is allocated for a call.

1.1.4		Diverting Calls

If a radio unit does not wish to receive calls, it may request that future calls addressed to it be redirected to a specified alternative destination.  A radio unit may also request redirection on behalf of a third party, for example, for a unit which is not equipped for call diversion. A radio unit calling a diverted party will be informed of the alternative destination to try; it may then re-make the call automatically, or it may give the user the option of deciding whether to call the alternative destination.  See section 12 for the full diversion facilities.

1.2	System Features and Facilities

1.2.1	System dimensions

The numbering range of the protocol accommodates:

-  1,036,800 addresses per system

-  1024 channel numbers

-  32768 system identity codes.

1.2.2	System control

The protocol uses signalling at 1200 bit/s with Fast Frequency Shift Keying (FFSK) subcarrier modulation.  It is designed for use by two-frequency half-duplex radio units and a duplex TSC.

The signalling for setting up calls is transmitted on a "control" channel.  A TSC can be operated using either of two control channel strategies: dedicated or non-dedicated.  A dedicated system has a control channel permanently available for signalling, whereas a non-dedicated system may assign the control channel for traffic (speech or data communication) if all the other channels are in use.  The use of a dedicated control channel is appropriate for a TSC with many channels, whereas a non-dedicated control channel may be more appropriate for a TSC with only a few channels.  The protocol allows the use of either strategy.

Broadcast messages are available to inform radio units of system information, such as the channels which the system may use for control signalling.

One of the problems of mobile radio signalling systems is the clashing of messages from different radio units transmitting at the same time.  The problems of clashing are controlled by an access protocol which offers high efficiency, stability and flexibility.  (See section 1.3.3 and section 7.)

Protection against interference is provided by labelling the signalling with a system identity code and, in some messages, the channel number.  If heavy interference is encountered, control can be changed to a different channel.

To cope with system malfunction, a customised fall-back mode of operation may be defined by the system designer.

1.2.3	Call handling

The protocol is designed for use by systems which queue calls that cannot be set up immediately, for example, if no channel is currently available for traffic.

Before a traffic channel is assigned for a call to an individual radio unit, the TSC checks that the called unit is in radio contact, in order to avoid wasted channel assignments.  It may also check that the radio unit's operator is ready for the call, to avoid a traffic channel being assigned to an unmanned unit.

Call maintenance signalling is defined for prompt release of traffic channels at the end of a conversation, or in case communication is lost during a call.  (See section 1.3.5 and section 9.)

As a precaution against fraudulent use of a system by an unauthorised radio unit, the TSC may at any time instruct a radio unit to transmit its unique serial number; comparison of the received serial number with the expected value will assist in the detection of fraudulent users.  (See section 15.)

1.2.4	Multi-site systems

The standard leaves scope for various multi-site wide-area coverage techniques to be used, for example:

-  synchronous/quasi-synchronous operation

-  a separate control channel at each site

-  a single control channel shared by time division.

The protocol includes a registration facility to assist the implementation of multi-site systems and networks of TSCs: a radio unit can inform the TSC of its location as it roams between sites or systems.  (The system identity code distinguishes the signalling from different sites and systems).  The standard defines signalling procedures for registration (section 8), but the criteria for registration will be system-dependent.

A TSC can broadcast information to assist radio units hunting for a control channel when they roam; for example, it can announce the channels which may be used for control by itself or by TSCs on adjacent sites.

1.3	Guide to Some Key Protocol Aspects

This section provides an introduction to the operation of the protocol which, because of its scope and flexibility, is necessarily complex.  The section outlines the control channel structure, the random access protocol and some message exchange procedures for call set-up.

This section is intended only as a guide: it should not be regarded as a protocol specification.  Readers should refer to the main body of the standard for the complete and precise definition.

1.3.1	Control channel signalling structure

The signalling for setting up calls is transmitted on a "control" channel.  Time on the control channel is divided into slots of duration 106.7 ms (128 bits), and one signalling message can be sent in each slot. The basic control channel signalling structure is illustrated in   Figure 1-1.

Signalling on the forward channel (base station transmit frequency) is nominally continuous, with each slot comprising two 64-bit codewords, usually:

i)	A Control Channel System Codeword (CCSC). 
The CCSC identifies the system to radio units and provides synchronisation for the following "address" codeword.

ii)	An "address" codeword.
An address codeword is the first codeword of any message, and defines the nature of the message.

Both the CCSC and address codewords are displaced when the Trunking System Controller (TSC) transmits longer messages, with "data" codewords appended to an address codeword.

A radio unit can receive a message from the TSC in one slot, transmit a response in the next slot and then retune to the forward channel in time to decode the following message from the TSC.  (In Figure 1-1, the response is shown aligned with the outbound message; however, there are tolerances on the timing.)

|  |  |  |  | 1 slot |  |
| --- | --- | --- | --- | --- | --- |
| TSC to radio units |  |  |  | ¬¾¾¾¾¾¾¾¾¾¾® |  |
| CCSC | address codeword | CCSC | address codeword | CCSC | address codeword |

| radio unit response |  |  | synch bits | address codeword |
| --- | --- | --- | --- | --- |

Fig. 1-1   Control channel signalling structure

1.3.2		Control channel signalling messages

The messages sent on a control channel may be classified as follows:

Aloha messages	-	Sent by the TSC to invite and control random access.

Requests 	-	Sent by radio units to request calls/transactions.

"Ahoy" messages	-	Sent by the TSC to demand a response from an addressed radio unit.

Acknowledgements	-	Sent by the TSC and by radio units.

Go To Channel messages 	-	Sent by the TSC to allocate traffic channels.

Single address messages	-	Currently sent only by radio units.

Short data messages 	-	Sent by the TSC and by radio units.

Miscellaneous messages 	-	Sent by the TSC for system control.

Some uses of these messages are illustrated in the following sections.

1.3.3		Random access protocol

1.3.3.1	Principle of operation

One of the problems of mobile radio signalling schemes is the clash of messages from different radio units transmitting at the same time.  In this standard, the problems of clashing are controlled by a random access protocol which is based on slotted Aloha, with a superimposed framing structure.  The access protocol can be used to minimise access delays, ensure stability and maintain peak throughput under heavy traffic loads.

The basic principle of the access protocol is described with reference to Figure 1-2, which illustrates signalling on a control channel.  The TSC transmits a synchronisation message (indicated by ALH in Figure 1-2) to invite radio units to send random access messages.  The ALH message contains a parameter (N) which indicates the number of following timeslots, constituting a frame, that are available for access.  If a frame is already in progress when a user initiates a call, the radio unit may send its random access message in the next slot.  Otherwise the unit waits for a frame to be started and then chooses a random slot from the frame for its message. A unit wishing to send a repeat transmission after an unsuccessful message (corrupted by fading or clashing) chooses again from a new frame.

|  |  |  |  |  |  |  |  |  |  |  | 1 slot ¬¾® |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSC to Radio Units |  |  |  |  | ALH (4) |  |  |  | ALH (3) |  |  |  |  |  |

| Radio Units to TSC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | \______________________/ frame |  |  |  | \________________/ frame |  |  |  |  |

Fig. 1-2   Two random access frames, each marked by an ALH message

1.3.3.2	Features of the random access protocol

The main features of the access protocol are as follows:

a)	The TSC can monitor activity on the control channel and can optimise the system performance by varying the framelength to prevent excessive clashing and to minimise access delays.  Figure 1-3 illustrates an example of random access control.

b)	The signalling overhead for random access control is kept small by allowing Acknowledgements and Go To Channel messages to contain the framelength parameter (N), so that frames can be marked without requiring an explicit Aloha message.  For example, see Figure 1-3.

c)	During a frame, the TSC may transmit messages that demand a response from a specified radio unit.  These outbound messages inhibit random access in the following slot, and so reserve the slot for the unit's reply.

d)	The TSC may reserve frames for:

-	specific types of call request, by means of specific Aloha messages (for instance, the Aloha message ALHE invites emergency calls only);

-	subsets of the radio unit population (subdivision by address).

| TSC to Radio Units |  |  |  |  |  |  |  |  | ALH (1) |  | ALH (1) |  | ALH (2) |  | ALH (0) |  | ACKQ (1) |  | ACKQ (1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Radio Units to TSC |  |  |  |  |  |  |  |  |  |  |  | RQS1 RQS2 |  |  |  | RQS1 |  | RQS2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | \____/ frame |  | \____/ frame |  | \__________/ frame |  |  |  | \____/ frame |  | \____/ frame |  |  |  |  |  |  |  |  |

The TSC detects the clashing of requests RQS1 and RQS2, and marks a longer frame (with message ALH(2)).  The radio units repeat their requests and, in this example, choose different slots.  Each request is acknowledged in the following slot.

ALH(0) does not mark a frame.

ACKQ(1) acknowledges a request and also marks a new frame.

In the absence of clashing, the framelength may be reduced.

Fig. 1-3   Example of random access control

1.3.4		Addressing

A unit address is a 20-bit number comprising two fields: a 7-bit prefix and a 13-bit ident.  (Normally, all members of a fleet will be allocated the same prefix.)  The division into prefix and ident allows most messages to accommodate two addresses, the calling and called party, by including the prefix only once.  For instance, call requests and Go To Channel messages contain two idents and only one prefix.

For a call to a unit with the same prefix, a request message contains all the information necessary to make the call.  However, for a call to a unit with a different prefix, the call details cannot be accommodated in a single address codeword; this type of call requires the use of "extended addressing" procedures (as do some PABX and most PSTN calls).

1.3.5		Examples of signalling sequences

The precise signalling required for a call depends on the type of call and on the design of the TSC; (the standard does not prescribe the TSC algorithms).  This section contains some examples of message exchange sequences.  Note that, although not shown in the examples, messages will be retransmitted in the case of corruption by propagation errors or collision.

Examples of message exchange sequences for call set-up are presented in sections 1.3.5.1 to 1.3.5.3.  These examples show control channel signalling, for:

-  call requests

-  instruction to send extended address information

-  checking availability of radio units

-  traffic channel allocation.

Signalling is also sent on an allocated traffic channel, for call maintenance and call clear-down.  For instance:

a)	To assist call maintenance, a radio unit sends a "Pressel Off" message at the end of each speech transmission.  The system may also require the unit to start each speech transmission with a "Pressel On" message and to send call maintenance messages periodically within the transmission.

b)	The calling unit in a group call, or both units in an individual call, send "Disconnect" messages to indicate end-of-channel-use when the user goes on-hook or equivalent.

c)	The TSC sends CLEAR messages to clear down a call (after receiving a valid Disconnect message or if a time-out has expired).

However, the examples do not cover traffic channel signalling.

The final example (section 1.3.5.4) illustrates the transmission of a short data message.  This type of transaction does not use a traffic channel: it requires control channel signalling only.

1.3.5.1	Example: radio unit calls a group

Figure 1-4 illustrates a message sequence on a control channel to set up a group call between radio units with the same prefix.

The sequence includes call request and channel allocation signalling. (For group calls, an availability check on the called units is not performed.)  In this example, all traffic channels are in use when the call is requested and so the call is queued.

|  |  |  |  |  |  |  |  |  |  |  |  | slot ¬--® |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TSC to RUs |  | 1 ALH (1) | ALH (1) | 3 ACK (1) |  |  |  |  |  |  | 4 GTC (1) | GTC (1) |  |  |  |  |  |  |  |  |  |
| RUs to TSC |  |  | 2 RQS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | \____/ frame | \____/ frame |  |  |  |  |  |  |  | \____/ frame | \____/ frame |  |  |  |  |  |  |  |  |

1.  ALH	:	General Aloha invitation (one-slot frame).

2.  RQS	:		The calling radio unit transmits its request, complying with the random access protocol.

3.  ACKQ	:	The TSC acknowledges the RQS message, informing the calling unit that the call has been queued.

4.  GTC	:	When a traffic channel is available, the TSC sends the Go To Channel command, addressed to the calling unit and called group; this message instructs the units to switch to the traffic channel for their conversation.  In this example the GTC is repeated, for added reliability.

Fig. 1-4   Common-prefix group call

Alternative acknowledgements from the TSC are available if, for instance, the call request is invalid or the system is overloaded.

If a traffic channel is available when a group call is requested then the TSC may omit the ACKQ and send the GTC command immediately.

In this example the GTC message is repeated immediately.  However, repeat messages may be delayed for other signalling.

1.3.5.2	Example: radio unit calls a unit with the same prefix

Figure 1-5 illustrates a message sequence on a control channel to set up a call between two radio units with the same prefix.  The sequence includes call request, availability check and channel allocation signalling.

| TSC to RUs |  |  |  |  | 1 ALH (3) | ALH (0) | 3 AHY | ALH (2) | 5 GTC (0) | GTC (1) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RUs to TSC |  |  |  |  |  | 2 RQS |  | 4 ACK |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | \________________/ frame |  |  | \__________/ frame |  | \____/ frame |  |  |  |  |  |  |

1.  ALH :	General Aloha invitation (three-slot frame).

2.  RQS :	Random access call request.

3.  AHY :	Availability check message

-	acknowledges the RQS message

-	demands a response from the called radio unit (thereby checking whether the called unit is in radio contact)

-	inhibits random access in the next slot.

4.  ACK :	Acknowledgement from the called radio unit, sent in the reserved slot.

5.  GTC :	Go To Channel message instructing both radio units to switch to the specified traffic channel for their call. In this example the GTC is repeated, for added reliability.

Fig. 1-5   Common-prefix individual call

In this example, the called unit is in radio contact and therefore responds to the AHY.  If the called unit cannot be contacted, the TSC may indicate the failure to the calling unit by sending acknowledgement ACKV.

In both this and the following example, the TSC checks only that the called unit is in radio contact before allocating a traffic channel. The TSC may also check whether the called user is ready; if he is not, the unit responds with acknowledgement ACKI and takes action to alert him. Then, when the user is ready to receive the call, the unit may send a status message (RQQ) to inform the TSC.

The ALH(0) message in these examples is used as a "dummy" message, in slots carrying no signalling relevant to the example.  In practice, these slots may be used for signalling for another call, or for broadcast messages (which contain information about system parameters).

1.3.5.3	Example: radio unit calls a unit with a different prefix

Figure 1-6 illustrates a message sequence on a control channel to set up a call between two radio units with different prefixes.

The sequence includes call request, availability check and channel allocation signalling (as in the previous example).  However, this sequence has an extra phase: after receiving the RQS message, the TSC sends AHYC to invite the calling unit to transmit the full called address.  Also, separate GTC messages instruct the two units, because GTC contains only one prefix.

| TSC to RUs |  |  |  |  |  |  | 1 ALH (4) | ALH (0) | 3 AHYC | ALH (0) | 5 AHY | ALH (2) | 7 GTC (0) | 8 GTC (1) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RUs to TSC |  |  |  |  |  |  |  | 2 RQS |  | 4.. SAMIS |  | 6 ACK |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | \_____________________/ frame |  |  |  |  | \________/ frame |  | \____/ frame |  |  |  |  |

1.	ALH	:	General Aloha invitation (four-slot frame).

2.	RQS	:	Random access request for an interprefix call.  (The request contains the calling unit's address (prefix/ident), but the called ident is set to a special "gateway" ident to indicate that extended addressing procedures are needed.)

3.	AHYC	:	Short data invitation message

-  acknowledges the RQS message

-  instructs the calling unit to send the called address

-  inhibits random access in the next slot.

4.	SAMIS	:	Single Address Message from the calling radio unit, containing the address (prefix/ident) of the called unit.

5.	AHY	:	Availability check message demanding a response from the called radio unit. 
In this example, the availability check is a single-codeword message i.e. the address of the calling unit is not supplied.

6.	ACK	:	Acknowledgement from the called radio unit.

7.	GTC	:	Go To Channel message instructing the called radio unit to switch to the specified traffic channel for the call.

8.	GTC	:	Go To Channel message instructing the calling radio unit to switch to the specified channel for the call.

Fig. 1-6   Interprefix individual call

1.3.5.4	Example: radio unit sends a short data message

Figure 1-7 illustrates a message sequence on a control channel for sending a short data message from one radio unit to another radio unit. In this example, the data message comprises an address codeword and two appended data codewords; (each of the data codewords contains 46 bits of free format data).

In the sequence, the radio unit sends its request; the TSC instructs the unit to send the data message, forwards the data message to the called unit and then indicates the success of the transaction to the calling unit.

| TSC to RUs |  |  | 1 ALH (1) | ALH (1) | 3 AHYQ | ALH (0) | ALH (2) | 5 HEAD | 5 data | ALH (1) | 7 ACK (1) | ACK (1) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RUs to TSC |  |  |  | 2 RQC |  | 4 HEAD | 4 data |  |  | 6 ACK |  |  |  |  |
|  |  |  |  | \____/ frame | \____/ frame |  |  | \__________/ frame |  |  | \____/ frame | \____/ frame |  |  |

1.	ALH  :	General Aloha invitation (one-slot frame).

2.	RQC  :	Random access request to transmit a short data message.  (The request indicates the number of timeslots required for the data message: in this case, two slots.)

3.	AHYC :	Short data invitation message

-	acknowledges the RQC message

-	instructs the calling unit to send the data message in the next two slots.

4.	HEAD + data	:	The calling radio unit sends its short data message to the TSC.  In this example the message comprises an address codeword (HEAD) and two appended data codewords.

5.	HEAD + data	:	The TSC forwards the short data message to the called radio unit.

6.	ACK  :	Acknowledgement from the called unit - message accepted.

7.	ACK  :	Acknowledgement sent to the calling unit to indicate that the called unit has accepted the data message.  In this example the TSC immediately repeats the  ACK message, for added reliability.

Fig. 1-7   Short data message