15.		DATA INTERROGATION PROCEDURES

This section defines data interrogation procedures, which allow the TSC to demand that an addressed radio unit transmits a data message of a prescribed type.  This demand is an interrogation by the TSC, not part of the signalling for a call requested by the radio unit.  It may be sent on either a control channel or an allocated traffic channel.

The TSC interrogates the radio unit by sending message AHYC, Mode 2 (see 5.5.3.2.8).  In this message, PFIX/IDENT1 is set to the radio unit's individual address and IDENT2 is the ident of the interrogator (a non-radio-unit ident).  The type of data to be transmitted by the radio unit is indicated by the descriptor field DESC and the non-radio-unit ident.

The TSC does not acknowledge receipt of the radio unit's data message (though it may take appropriate action as a result of the received data).

Currently, for data interrogation, only one value of the data message descriptor field DESC has been assigned.  This value is used for implementing serial number checks: the TSC may at any time, on a control channel or traffic channel, instruct a radio unit to send its 38-bit serial number.  Comparison of the received serial number with the expected value (held in store at the TSC) will assist in the detection of fraudulent users.

15.1		Data Interrogation Procedures for TSC

15.1.1	Data interrogation on a control channel

The TSC may demand that a radio unit on a control channel transmits a data message of a prescribed type, by sending the AHYC message with:

-	PFIX/IDENT1 set to the individual address of the radio unit

-	IDENT2 set to the ident of the interrogator

(for example, for a serial number check, IDENT2 = TSCI)

-	DESC set to indicate the type of data message required; see 5.5.3.2.8

(for example, for a serial number check, DESC = '000')

-	SLOTS set appropriately; see 5.5.3.2.8

(for example, for a serial number check, SLOTS = '01').

The AHYC message instructs the addressed radio unit to transmit a data message in the following SLOTS slot(s) (see 15.2.1).  If the TSC does not successfully decode a reply, it may repeat the AHYC message when convenient. (The TSC does not acknowledge receipt of the data message).

Note that AHYC bars random access only in the first following return slot.  When demanding a multi-codeword data message, the TSC shall take appropriate action to reserve the subsequent return slot(s) if they are within a frame (e.g. by sending the AHY message with both idents set to DUMMYI).

15.1.2  Data interrogation on a traffic channel

The TSC may demand that a radio unit on an allocated traffic channel transmits a data message of a prescribed type, by sending the AHYC message with:

-	PFIX/IDENT1 set to the individual address of the radio unit

-	IDENT2 set to the ident of the interrogator

-	DESC set to indicate the type of data message required; see 5.5.3.2.8

-	SLOTS set appropriately; see 5.5.3.2.8.

The AHYC message instructs the addressed radio unit to transmit a data message (see 15.2.2).  If the TSC does not successfully decode a reply, it may repeat the AHYC message.

15.2		Procedures for All Radio Units

The procedures in this section shall be obeyed by all radio units that are equipped to recognise a received Mode 2 AHYC message. (The requirement to recognise AHYC, Mode 2 will be system-dependent.)

15.2.1	Data interrogation message (AHYC, Mode 2) on a control channel

If a radio unit on a control channel receives an AHYC message with PFIX/IDENT1 matching its individual address then it shall either send a data message in the following SLOTS slot(s), or transmit ACKX(QUAL=0), as indicated below.  For timing, see section 6.2.1.3.

If

IDENT2 	is set to TSCI

and	DESC   	is set to '000'

and	SLOTS  	is set to '01'

and	the unit is equipped to transmit its serial number on interrogation

then it shall transmit its serial number, conforming to the codeword format defined in section 5.6.1.2.2 (SAMIS, Mode 2, DESC='000').  (The form of the serial number is system-dependent.)

Otherwise

The unit shall transmit ACKX(QUAL=0), with the same prefix and idents as the AHYC.

15.2.2	Data interrogation message (AHYC, Mode 2) on an allocated traffic channel

If a radio unit on a traffic channel receives an AHYC message with PFIX/IDENT1 matching its individual address then it shall either send  a data message or transmit ACKX(QUAL=0), as indicated below.  For timing, see section 6.2.2.2.

If

IDENT2 	is set to TSCI

and	DESC   	is set to '000'

and	SLOTS  	is set to '01'

and	the unit is equipped to transmit its serial number on interrogation

then it shall transmit its serial number, conforming to the codeword format defined in section 5.6.1.2.2 (SAMIS, Mode 2, DESC='000').

Otherwise

The unit shall transmit ACKX(QUAL=0), with the same prefix and idents as the AHYC.