8.		MAN-MACHINE INTERFACE

8.1		Confidence Indications

8.1.1		Introduction

The user is an important element in any communications network since his behaviour can contribute to the overall efficiency of network operation and the grade of service which can be offered to users.  Experience with telecommunications systems has shown that the provision of appropriate indications to the user of any call or transaction using the network to which he is a party can help to regulate user behaviour in ways which benefit system efficiency.  Indications of this type, often referred to as confidence indications, also benefit the user and should improve user satisfaction levels.

This section of the Specification seeks to promote an acceptable and appropriate regime of user confidence by ensuring that users will experience a similar set of confidence indications, especially the audible indications, no matter what manufacture of radio unit is used.

It is recognised that manufacturers of radio units will wish to exercise design independence in their products and, accordingly, the requirements of this section of the Specification have been kept reasonably flexible in that, though the cadences are defined for ease of recognition, the audible quality of the tones is not specified. Similarly the visual indications  have not been specified and these may be, for example, steady or flashing illuminated legends, coloured lamps or legends displayed on alpha-numeric displays, thus giving a large freedom of choice. Manufacturers are recommended, in the case of equipment which is designed to be mounted in vehicles, to pay due regard to the requirements of the Highway Code and Road Safety considerations in general.  In such equipment audible signals are often to be preferred to visual indications.  Manufacturers should also bear in mind that, where visual indications are employed that may occur during speech calls, they should be easily visible to the user at all times during normal call operations.

The designer of each radio unit is free to design radio units to either accept or reject incoming signalling for a new call, while the mobile is waiting for its own call to be set up.  However if the radio unit accepts signalling for the new incoming call, it shall suspend the confidence indications associated with its own call, replacing them with indications for the new call.

When the new call is complete the suspended confidence indications shall be resumed, providing that the call waiting timer for its own call has not expired.

8.1.2		Basic Requirements

Each individual confidence indication is described in the following terms:

-  Concerned Party (either CALLING or CALLED)

-  Conditions to Initiate

-  Conditions to Cancel

-  Provision Mandatory or a Standard Option

Figure 8.1 details each indication in summary form.  A more detailed description of the indications is given in the following section.

Designers of radio units should note that the use of additional indications is not prohibited.  However any such additional indications shall be implemented so as to be consistent with those detailed in this section.

The indications are not necessary if the call requires no human involvement, for example automatic data messages or covert emergency calls.

Note also that an indication need not be repeated if further messages which initiate the indication are received for the transaction.

8.1.3		Type of Confidence Indications

Mobiles shall provide the following confidence indications:

(a)	Call Set Up in Progress (Calling Party)

(b)	Call Set Up in Progress (Called Party)

(c)	Number Unobtainable

(d)	Call Fail

(e)	Alert

(f)	GTC sent on a control channel

(g)	Transaction Confirmed

(h)	Call Clear Indication

y provide the following indications, which are recommended:

(i)	No Service

(j)	Call Queued

(k)	Called Party Ringing

(l)	Manual Call Diversion

Mobiles may provide the following indications which are optional:

(m)	GTC sent on a traffic channel

The indications shall be unambiguous with the following permitted exceptions.  Indications (a) and (b) may use the same form of indication.  Indications (f) and (m) may use the same form of indication.

Audible indications shall be of the form indicated in sections 8.1.3.1 to 8.1.3.11.

Cadence Timings shall be as defined with a tolerance of ± 20% with the tone ON to tone OFF ratio controlled to within ± 20%.

The two audible tones noted in each section shall be approximately:

"Hi" - maximum frequency 1000Hz

"Lo" - minimum frequency  300Hz

The "Hi" frequency shall be twice that of the "Lo" frequency ±100 Hz.

8.1.3.1.	No Service

The provision of any No Service indication is optional.

8.1.3.2	Call Set Up in Progress (CSUIP)

8.1.3.2.1	CSUIP (Calling)

The provision of a CSUIP (Calling) indication is mandatory.  the indication tells the user that his call request is being processed.

CSUIP (Calling) shall commence when the calling user has completed entering the address and call details into the unit, or on the first transmission of the request message (RQS, RQE, RQT, RQQ (not STATUS = 0 or 31 when sent with IDENT1 = TSCI)).

CSUIP (Calling) is cancelled upon receipt of any of the following messages for the call:

-	ACKI (QUAL = '0'), if the unit is equipped to indicate Called Party Ringing.

-	ACKX

-	ACKV

-	ACK  (QUAL = '0')

-	ACKB (QUAL = '0')

-	ACKT (QUAL = '0')

-	ACKQ, if the unit is equipped to indicate Call Queued

-	GTC

-	ACK (QUAL = '1') as a result of "User Call Set Up Abort" (sections 8.3.3.)

-	Expiry of the timeouts TC, TW, TJ or TI

-	AHYX

The audible "Lo Tone" indication shall be:

cadence 0.8s on 1.2s off.

The indication shall be cancelled as a result of "User Call Set Up Abort" (section 8.3.3.) if no call request message has yet been sent by the radio unit.

8.1.3.2.2	CSUIP (Called)

The provision of a CSUIP (Called) indication is mandatory.

The indication shall be audible.

The called unit shall indicate CSUIP (Called) to the user, when the user signals "Called Party Answer" (section 8.3.3.) to the unit following the receipt of AHY (CHECK='1') for an incoming call.

The indication shall be cancelled upon the receipt of any of the following messages:

-	GTC

-	AHYX for the call

-	Expiry of TA

-	ACK (QUAL  = '1'), if attempting RQX for "Cancel Called Party Answer" (section 8.3.3.)

-	ACK (QUAL = '0') for "on-hook" RQQ for "Cancel Called Party Answer" (section 8.3.3.)

-	ACKX

The audible "Lo Tone" indication shall be:

cadence 0.8s on 1.2s off.

8.1.3.3.	Call Queued

The provision of a Call Queued indication is optional.

A suitably equipped calling unit shall indicate Call Queued upon receipt of an ACKQ message as a result of a call request.

The indication shall be cancelled upon receipt of the following messages:

-	ACKX

-	 ACKV

-	 ACK  (QUAL = '0')

-	 ACKB (QUAL = '0')

-	 ACKT (QUAL = '1')

-	 ACKI (QUAL = '0') if the unit is equipped to indicate "Called Party Ringing"

-	 GTC

-	 Expiry of the timeouts TW, TJ or TI

-	 ACK (QUAL = '1') after attempts to cancel or abort call

- AHYX

The audible "Lo Tone" indication shall be:

cadence 0.8s on 1.2s off.

8.1.3.4. 	Called Party Ringing

The provision of a Called Party Ringing indication is optional.

A suitably equipped calling unit shall generate a Called Party ringing indication upon receipt of ACKI (QUAL = '0') from the network.  The unit shall cancel this indication upon receipt of any of the following messages:

-	GTC

-	ACKV

-	AHYX for the call

-	Expiry of the timeouts TW or TI

-	ACK (QUAL = '1') as a result of "Call Set Up Abort" (section 8.3.3.)

-	ACKX

-

-	ACKT (QUAL = '0')

-	ACKB (QUAL = '0')

The audible "Lo Tone" indication shall be:

cadence 0.4s on  0.2s off, 0.4s on 2s off, i.e. 3s cycle repeating.

Where possible this tone shall represent a typical telephone "ring" tone.

8.1.3.5.	Call Fail

The provision of a Call Fail indication is mandatory.  It can be signalled to both the calling and the called party.

The audible indications are noted against each failure condition.

Message			Indication

ACKX (QUAL = '1')	System Busy

ACKV (QUAL = '0')	Unavailable

ACKV (QUAL = '1')	Called Party Busy

AHYX	Unavailable

ACKT	Unavailable, if the unit is not equipped for call diversion.

The unit shall also give an audible "Unavailable" indication:

-	If, when the user initiates a call, the radio unit is not receiving on an acquired channel, and has remained without an acquired control channel for a preceding continuous time TC.

-	If, when the user indicates a call, the radio unit is not receiving on an acquired control channel and the time since the unit last commenced the control channel acquisition procedures is less than TC, then the unit shall indicate "unavailable" if it has still not acquired a control channel at a time TC after it last commenced the control channel acquisition procedures.

-	Expiry of the timeouts TA, TI, TJ or TW.

-	Expiry of the timeout TC.

The audible tones shall be:

System Busy :		optional, if not implemented to be replaced by "Called Party Busy".

Audible "Lo Tone"; cadence 0.4s on, 0.35s off, 0.225s on, 0.525s off, (ie 1.5 second cycle) - for 4 cycles.

Unavailable:		Audible "Lo Tone"; cadence 1.5 sec duration.

Called Party Busy:	Audible "Lo Tone"; cadence 0.375s on 0.375s off for 6 cycles.

The indications shall be truncated by operation of the user "Cancel Indication" (section 8.3.3).

8.1.3.6	Number Unobtainable (NU)

The provision of a NU indication is mandatory.

The calling unit shall signal NU to the user upon receipt of ACKX (QUAL = '0') in response to a call request.

The calling unit shall also signal NU to the user when it rejects a dialled number string which it does not recognise or cannot action.

The audible "Lo Tone" shall be shall be a continuous tone of 5 seconds duration unless truncated by operation of the user "Cancel Indication" (section 8.3.3).

8.1.3.7	Call Diversion

The provision of a call diversion indication is optional.  Its purpose is to invite the calling user to initiate a new call to a different address.  The form of the indication may include the new number which the calling users is invited to dial, or alternatively this number may be stored in the mobile without being displayed to the user.

The Call Diversion indication shall be initiated by the receipt of ACKT (QUAL = '0').

The audible tone indication shall be "Hi-Lo-Hi":  cadence 0.1s on, 0.4s off, 0.1s on, 0.4s off, 0.1s on, 0.4s off.

8.1.3.8	Call Clear Indication

The provision of a call clear indication is mandatory.  Call clear shall be indicated whenever the radio unit leaves a traffic channel, except when moving to a different traffic channel as directed by a GTC message received on a traffic channel, or when moving to a different channel after the radio unit has received a "Call End Request" (section 8.3.3).

Audible "Hi-Lo", cadence 0.3s of each tone.

8.1.3.9	Alert

The provision of an Alert indication is mandatory.

The indication shall be audible.

Units shall indicate Alert indication if they receive AHY (POINT = 'O', CHECK='1') with IDENT2=Ident (1 to 8100), INCI, IPFIXI, PSTNGI, or PABXI and if they respond by transmitting ACKI (QUAL = '0').

The Alert Indication shall be cancelled by user "Called Party Answer", (section 8.3.3) or by the expiry of the Called Party Alert State timeout TA, or by receipt of an appropriate AHYX message.

Where the received AHY message has a zero value of the E bit, Tthe audible "Hi Tone" shall be cadence:

0.4s on, 0.2s off, 0.4s on, 2s off, ie. 3s cycle repeating.

Where possible this tone shall represent a typical telephone "alert" tone.

8.1.3.10	Transaction Confirmed

The provision of a Transaction Confirmed indication is mandatory.

Transaction Confirmed is used primarily in non-speech calls:

-	RQC

-	RQT

-	RQQ (not STATUS=0 or 31 when sent with IDENT1 = TSCI)

-	Include

The radio unit shall initiate the Transaction Confirmed Indication if ACK (QUAL = '0') is received as a result of any of these request messages.  The unit shall also initiate the Transaction Confirmed indication if ACKB (QUAL = '0') is received as a result of an RQS message.

The audible tone indication shall be "Lo-Hi":  cadence 0.3s each tone.

8.1.3.11	GTC on Control Channel "Blip" Indication

The provision of a control channel GTC indication is mandatory.

The indication shall be at least audible.

Called radio units shall commence the GTC indication upon receipt of GTC.  However if the radio unit has responded to the incoming AHY for that call with ACKI (QUAL = '0') (ie. AHY was CHECK = '1') then the radio unit may optionally omit the GTC indication upon receipt of the GTC.

Calling radio units shall commence the GTC indication upon receipt of GTC. However if the radio unit has previously received ACKI(QUAL ='0') for the call, then the calling radio may optionally omit the GTC indication upon receipt of the GTC.

The GTC indication shall be implemented as a transitory indication, and its duration shall in all circumstances be less than 500 ms.

Where the incoming message for the call had a zero value of the E bit, The audible "Hi Tone" indication shall be 2 short "blips".  Where the incoming AHY message for the call had the E bit set to '1', the radio unit may generate any appropriate audible tone for the GTC indication.

8.1.4		Note on use of Tone Sets

The radio unit may employ an optional tone set which differs from this specified tone set, if required for specific user needs.  If such a tone set is implemented it shall be as an additional,  alternative set to the one specified in this section, and its use shall be agreed by the network operator.

| No | INDICATION | CALL-ING | CALL-ED | TYPE | INITIATE | CANCEL |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NO SERVICE | / | / | 0 | LOSS OF CONTROL CH | ACQUIRE NEW CONTROL CH |
| 2 | CSUIP (CALLING) | / |  | M | ADDRESS TERMINATOR OR TRANSMISSION OF REQUEST | ACQUIRE NEW CONTROL CH ACKX, ACKV ACK(QUAL=0) ACKB(QUAL=0) ACKT(QUAL=0) ACK(QUAL=0) (IF CALLED PARTY RINGING EQUIPPED ACKQ(IF CALL QUEUED EQUIPPED) GTC, ACK(QUAL=1) EXPIRY OF TC,TW,TJ,TI AHYX CALL SET UP ABORT (NO REQUEST) |
| 3 | CSUIP (CALLED) |  | / | M | CALLED PARTY ANSWER IF ANY WAS (CHECK=1) | GTC, AHYX, ACK(QUAL=0) ACK(QUAL=1) EXPIRY OF TA ACKX |
| 4 | CALL QUEUED | / |  | O | ACKQ | AS No 2 LESS RECEIPT OF ACKQ |
| 5 | CALLED PARTY RINGING | / |  | O | ACKI (QUAL=0) | GTC,ACKV, ACKX,ACKT(QUAL=0) ACKB(QUAL=0) AHYX EXPIRY OF TW OR TI ACK(QUAL=1) |
| 6 | CALL FAIL | / | / | M | ACKX (QUAL=1) ACKV,AHYX ACKT(IF NO CALL DIVERSION)EXPIRY OF TW,TJ,TI,TA CALL REQUEST IF NO CONTROL CHAN | CANCEL INDICATION (OR INDICATION TIMEOUT) |
| 7 | NUMBER UNOBTAINABLE | / |  | M | ACKX(QUAL=0) OR INVALID NUMBER ENTRY | CANCEL INDICATION (OR INDICATION TIMEOUT) |
| 8 | CALL DIVERSION | / |  | O | ACKT(QUAL=0) | INDICATION TIMEOUT |
| 9 | CALL CLEAR | / | / | M | CLEAR | INDICATION TIMEOUT |
| 10 | ALERT |  | / | M | ACKI(QUAL=0) IN RESPONSE TO AHY(CHECK=1) | AHYX,EXPIRY OF TA, CALLED PARTY ANSWER |
| 11 | TRANSACTION CONFIRMED | / |  | M | ACKI(QUAL=0) AFTER RQC RQT, OR RQQ (NOT 0 OR 31) OR INCLUDE REQUEST AND ACK B (QUAL=0)AFTER RQS | INDICATION TIMEOUT |
| 12 | GTC | / | / | M | GTC UNLESS RESPONSE TO ALERTING AHY WAS ACKI (QUAL=0) OR FOR CALLING UNIT UNLESS ACKI(QUAL=0) RECEIVED | INDICATION TIMEOUT |
| 13 | GTC ON TRAFFIC CHANNEL | / | / | O | GTC | INDICATION TIMEOUT |
|  | M = MANDATORY O = OPTIONAL |  |  | UNIT CONFIDENCE INDICATION |  |  |

Figure 8.1

8.2		Call Number Convention

This section covers the provision of dialling facilities to users of radio units.  It addresses both the entry and display of call number information.  It assumes the provision of a 12 button numerical keypad, although radio units may employ other means of entering numerical information.  Throughout this section the terms "mandatory" and "optional" have the following meanings:

Mandatory	- 	Units which are equipped with a means of manually entering numbers for the purposes of calling other parties on the subscriber's network or other services shall implement the specified function.

Optional	-	Units which are to provide the facility described in the option shall at least implement the option in the manner described.

This convention applies to all radio units operating on Band III, sub-bands 1 and 2 commercial systems.  Units which are not equipped with numerical keypads or displays but use functionally specific control buttons need not implement algorithms for the numbering scheme within the unit.  However such units shall be identified by numbers which accord with the principles of the call number convention to enable calls to be directed to them from units which are fitted with dialling facilities.

Units which are equipped with a means of manually entering numbers for the purposes of calling parties on the subscriber's network or other services may, optionally and as an alternative, employ a numbering scheme which is not consistent with the requirements of this section provided that the alternative scheme shall only be enabled when selected by network personalisation. Failing such selection, the radio unit shall default to the use of the scheme described in this section, supporting at least the mandatory requirements.

The buttons on the keypad may be arranged in the standard C.C.I.T.T. recommended layout, thus:

| 1 | 2 | 3 |
| --- | --- | --- |
| 4 | 5 | 6 |
| 7 | 8 | 9 |
| * | 0 | # |

Figure 8.2 Keyboard Layout

Any other keyboard layout may be used if found more convenient.

Units which do not employ all 10 numeric keys need not implement the call number convention in the manner described.

All dialled strings, as written in this section, are read from left to right and are dialled in the sequence in which they are read.  Throughout this section all representations of dialled strings are underlined.  Where MPT 1327 addresses are referred to they are expressed in the decimal form of PFIX/IDENT thus: 34/3456.

The primary use for the keypad is to enable the user to originate speech calls from the radio unit.  Calls may be made to other units operating on the network, to extensions on user associated private automatic branch exchanges (PABX) and to subscribers on the public switched telephone network (PSTN).  Other services may be added as required.

Codes which commence with an asterisk (*) or a number sign (#) provide secondary uses for the keypad.  Functions such as the modification of call requests to originate status messages, and the implementation of other MPT 1327 facilities (data, diversion, etc), are controlled in this manner.

Control of the radio unit's internal functions which affect MPT 1327 related activities also employ similar codes.

Any facility requested by means covered in this section which is not a mandatory requirement of Section 11 need not be incorporated into such a unit.  For units which have a numerical keypad any facility which is incorporated into the unit shall be implemented in the manner described in this section.  As an option it may be possible to bar access to any call type other than in-fleet calls.

Other uses of the keypad are not prohibited as long as no conflict occurs with the specified use.  All dialled strings which the unit does not recognise shall result in the unit rejecting the dialled string and signalling the rejection with a number unobtainable indication (see section 8.1).

8.2.1		Network Numbering Structure

Each unit is allocated an individual network number (unique within the network to which the user subscribes) which bears a fixed relationship with an MPT 1327 address.  The network number is in three parts: a number prefix, a fleet number and a unit number.  Groups of units are allocated a network group number in a similar manner.  A unit may be allocated to more than one group and thus may respond to more than one group number.

A 20 bit MPT 1327 address is divided into a prefix and an identity.  The prefix is the first 7 bits of the 20 bit address.  The identity is the remaining 13 bits.  Fleet organisation shall ensure that most calls between users are made between those who share the same prefix.  The identity range is divided into blocks which are allocated to fleets by the network operator.  Each fleet user shall be allocated an adequate number of identities to allow for reasonable expansion requirements.

The objective of the scheme is to enable a short dialled string to be used for the more regularly used numbers instead of the long dialled string that the direct decimal representation of the binary prefix and identity would entail.  Once this primary objective has been attained the secondary objective of permitting access to other speech and data services completes the scheme.  The scheme is expandable to allow the introduction of additional services in the future.

In-fleet individual calls are implemented by allocating a starting number (20 or 200 depending on fleet size) to the lowest identity which is allocated to the fleet.  The lowest identity (base ID) used by the fleet for individual calls shall be retained within each radio unit as part of the unit's fleet personality.  All units in the fleet are allocated numbers which increment in step with their allocated identities.

In-fleet group calls are similarly implemented by allocating a starting number (in this case 90 or 900 depending on fleet size) to the lowest fleet identity.  The lowest identity (base ID) used by the fleet for group calls shall be retained within each radio unit as part of the unit's fleet personality.

In-fleet numbering schemes are thus related to allocated identities by means of an algorithm.

PABX calls also employ an offset to ensure that maximum use is made of the MPT 1327 addressing range of 0 to 8191 when sending extension numbers.  As the lowest value of extension number in a four digit numbering scheme will not be less than 1000, all dialled strings are reduced in value by 1000 before transmission.  This ensures that numbers up to 8999 may be transmitted in a single codeword call.

Calls require the use of the extended addressing facility when PABX extension numbers, or routing codes plus extension numbers, have five or more digits.  If 7, 8 or 0 are used as leading digits within the PABX network then the same network dialling strings may be employed with the radio unit.  If other leading digits are used within the PABX network then the most reasonable choice of leading digit from 7, 8 or 0 may be used for PABX access, with appropriate action being taken at the radio network/PABX interface point to ensure that the correct string is offered to the PABX to ensure proper call routing.

PSTN calls are made either by using abbreviated addressing with TSC translation or by entering the full national number.  The leading zero of the called number is redundant and is not transmitted to the TSC.  This allows all national numbers to be contained within one SAMIS.

8.2.2		Philosophy

8.2.2.1	Relationship with Address

The numerical digits which are dialled by the user are translated into an MPT 1327 specified address.  This is transmitted in an address codeword with, if necessary, additional date codewords.

Use is made of MPT 1327 flags and addresses to send relevant signalling information between the radio unit and the TSC as part of the call set-up procedure.  Dialled string length is an integral part of the scheme.  The length of the dialled string and the leading digit indicate the service which the calling party requires and are both used to set the relevant flags and addresses in the address codeword.

8.2.2.2	Partitioning of Identities

The scheme partitions the identity range used by TSCs into separate blocks, each block being associated with a fleet of users.  The numbering scheme for each fleet starts at the partition and uses an algorithm to generate the called identity from the dialled string and the fleet base ident.  The fleet base ident has a fixed relationship with the Fleet Individual Number (FIN) or the Fleet Group Number (FGN) respectively.  The unit shall have a record of the size of the allocated blocks to ensure that in-fleet calls are not accidentally made outside the block to a unit in some other fleet.

|  | IDENTS |  | FIN | Individual Number | Fleet Size |
| --- | --- | --- | --- | --- | --- |
| (Base ID) | 2269 . . 2201 2200 |  | 3100 | 89 . . 21 20 | 70 |
| (Base ID) | 2199 . . 2007 2006 2005 2004 |  | 3002 | 395 . . 203 202 201 200 | 196 |
| (Base ID) | 2003 . . 1982 |  | 2291 | 41 . . 20 | 22 |

Figure 8.3   Relationship of Unit Numbers to IDENTS showing blocks of numbers

|  | IDENTS |  | FGN | Group Number | Fleet Size |
| --- | --- | --- | --- | --- | --- |
| (Base ID) | 7099 . . 7006 7005 7004 |  | 5502 | 995 902 901 900 | 96 |
| (Base ID) | 7003 . . 6996 |  | 5498 | 97 . . 90 | 8 |

Figure 8.4   Relationship of Group Numbers to IDENTS showing blocks of numbers

8.2.2.3	Method of Dialling

Pre-origination dialling is used in the radio units.  The user indicates completion of entries by the use of the # key.  When it is necessary to modify the function of the call set-up, though not the destination, call modifiers may be dialled before the called party number.  Modifiers are separated from the called number by use of the * key.  See section 8.2.8.

8.2.2.4	User Familiarity

To make the scheme familiar to the user, the numbering scheme attempts to follow normal UK telephony practice in the allocation of numbers.  Thus:

-	PSTN access commences with 0 and used dialled strings of 8 or more digits; the called subscriber's full national number is dialled,

-	network operator services start with a leading 1 and are three digits in length,

-	PABX operators may be obtained with a single 0

-	Local PABX numbering schemes, four digits, with the leading digit in the range 1 to 8,

-	PABX network numbering schemes, dialled strings to 5 to 7 digits, with leading digits 0, 7 and 8,

-	PABX network numbering schemes, dialled strings of 8 and 9 digits, with leading digits 7 and 8.

To accommodate the above, dialled strings for calls to radio units have the following constraints:

-	the number prefix leading digit is 2 or 3 (dialled strings of 9 and 10 digits),

-	the fleet number leading digit is in the range 2 to 6 (dialled strings of 6 and 7 digits),

-	the unit number leading digit is in the range 2 to 8 (dialled strings of 2 and 3 digits).

8.2.2.5	Types of Access

The numbering scheme accommodates at least:

-	Members of the same fleet	-	Members of other fleets

-	Members of other fleets with	-	PABX extensions associated
a different prefix		with the user's fleet

-	Access to nominated PSTN	-	Full access to PSTN subscribers
subscribers

-	Network operator services	-	Data services (to be defined)

8.2.2.6	Use of Display

If the radio unit is fitted with a display and numbers are to be shown then they must be shown in the form in which the user would dial the number when making a call.

8.2.2.6.1	Display of Called Party

Called party numbers shall be entered on the display as the user keys in the called number.  If * and # cannot be displayed, either an alternative non-numeric symbol may be displayed (eg -) or the display may be cleared by the input of the non-numeric character.

8.2.2.6.2	Display of Calling Party

Calling party numbers that can be displayed are those from units in the same fleet as the displaying unit and other fleets for which the unit retains appropriate data.  Calls where the calling party number cannot be thus shown may have a suitable indicator to show the general type of call which has been received, eg external fleet (common or inter prefix), PABX, PSTN, etc.

8.2.2.6.3	Display of Diversion Number

Numbers that are displayed on receipt of ACKT (diversion numbers) (!!9.2.1.4!!) shall be decoded from the incoming IDENT1 using the methods in 8.2.2.6.2 above.  When the ACKT has appended data codewords, the gateway address in the ACKT and the style of the codewords determine the action taken.  (!!5.5.2.1!!)  Calls where the diversion number can be displayed are those from units in the same fleet as the displaying unit and other fleets for which the unit retains a data base.  PABX and PSTN numbers are displayed using the data in the appended data codewords.

PSTN numbers will be received without the leading '0' which shall be added and displayed by the unit.  If the diversion address cannot be translated into a displayable number the unit may retain the address and show a general indication of the state of the call, or it may terminate the call action.

8.2.2.7	Technician Access

A facility may be provided to allow technicians to dial full radio unit addresses for test purposes by using the decimal representation of the MPT 1327 address (see 8.2.4.8).  It is recommended that the facility is protected to avoid unauthorised access.

8.2.3		Terms, Relationships and Storage Requirements

8.2.3.1	Terms and Relationships

Identity prefix (PFIX):	Defined in MPT 1327 (!!2!!)

Base Ident (BI):	Any ident having an even value (lease significant bit = 0) between 0 and 8100 which is the lowest value in the block of identities assigned to the fleet of units.  Range is 2 to 8100.  0 is DUMMYI and cannot be used.

Individual Base	The Base Ident assigned to the block of identities used Ident

Ident (IBI):	for individual calls.

Group Base Ident (GBI):	The Base Ident assigned to the block of identities used for group calls.

Number Prefix (NP):	NP = PFIX + 200.  Range is 200 to 327.

Fleet Number (FN):	FN = BI/2 + 2000, where BI is the lowest ident in the block allocated to the fleet.  Range is 2001 to 6050.  The network operator may restrict the range for his own purposes.

Fleet Individual

Number (FIN):	An FN which identifies a block of Unit Numbers.

Unit Number (UN):	The Unit Number consists of either two or three digits depending on the fleet size and is unique to one unit in any single fleet.

2 digit:	UN = Unit individual identity - IBI + 20

Numbering range: 20 to 89 (fleet size up to and including 70).

3 digit:	UN = Unit individual identity - IBI + 200

Numbering range: 200 to 899 (fleet size up to and including 700).

Fleet Group Number

(FGN): 	An FN which identifies a block of Group Numbers.

Group Number (GN):	The Group Number consists of either two or three digits depending on the fleet size and is allocated to units which have common interests.

2 digit:	GN = Group identity - GBI + 90

Numbering range: 90 to 99

3 digit:	GN = Group identity - GBI + 900

Numbering range: 900 to 998 (999 reserved for emergency service).

Block Size:	The size of the block allocation to a fleet of users for each form of Base Ident used by the members of the fleet.  The unit shall store either block size or the maximum value of dialled number in some form to avoid placing calls outside the block.

Numbers shall be written in the form:

|  | NP |  | FIN |  | UN | for individual numbers |
| --- | --- | --- | --- | --- | --- | --- |
| e.g. | 245 201 |  | 3456 3578 |  | 23 467 |  |

|  | NP |  | FGN |  | GN | for group numbers |
| --- | --- | --- | --- | --- | --- | --- |
| e.g. | 245 201 |  | 5456 5578 |  | 93 967 |  |

Numbers are dialled commencing at the most significant block of numbers which differs from the unit's own number.

8.2.3.2	Storage Requirements

8.2.3.2.1	Mandatory Storage Requirements

In order to follow the procedures specified in this section, the radio unit shall provide the following storage requirements (in read-only memory) which are appropriate to the selected network if the unit is equipped with a means of accepting called party numbers from the user:

i.	The individual base identity for the fleet of which the unit is a member (own fleet).

ii.	The group base identity for the unit's own fleet.

iii.	A flag to indicate the choice of own fleet individual number string length, ie 2 or 3 digit strings.

iv.	A flag to indicate the choice of own fleet group number string length, ie 2 or 3 digit strings.

v.	The highest permitted own fleet individual identity or dialled number.

vi.	The highest permitted own fleet group identity or dialled number.

vii.	A flag to bar inter-fleet group calls.

8.2.3.2.2	Optional Storage Requirements in read-only memory

i.	Ten actual called party identities or numbers for translation.  Each is associated with an entry of one of the single digit dialled strings.

ii.	A multiplicity of i) to vi) in 8.2.3.2.1 above per fleet, each stored against the two or three leading digits of the five digit calls to units in other fleets.

iii.	An address or number to complete call details for the *0# string.

iv.	An address or number to complete call details for the *0nn# string.

v.	An address or number to complete call details for the *9# string.

8.2.4		Radio Unit and Line Unit Numbering

8.2.4.1	Single Digit Dialled Strings (Optional)

The translation between single digit dialled strings and corresponding identities is not specified.  It is recommended that 0 be used either for calls to a despatcher or to a PABX operator (see 8.2.5).

8.2.4.2	Two Digit Dialled Strings (Mandatory)

Two digit dialled strings in the range 20 to 89 inclusive shall be used to call units within the same fleet.  The unit shall refer to its individual base ident (IBI) and shall generate the called party identity by the following algorithm from the dialled Unit Number (UN):

Called individual ident = IBI + UN - 20

Two digit dialled strings in the range 90 to 99 inclusive shall be used to call groups of units within the same fleet.  The unit shall refer to its group base ident (GBI) and shall generate the called group identity by the following algorithm from the dialled Group Number (GN):

Called group ident = GBI + GN - 90

Two digit group numbers and three digit unit numbers may be used within the same fleet.

The unit shall verify that the dialled UN or GN is not greater than the highest value assigned during network personalisation.

Note:	Numbers 00 to 02, 07 to 09 and 10 to 19 are not specified.  For use of 03 to 06, see section 8.2.5.

8.2.4.3	Three Digit Dialled Strings (Mandatory)

Three digit dialled strings in the range 200 to 899 inclusive shall be used to call units within the same fleet.  The unit shall refer to its individual base ident (IBI) and shall generate the called party identity by the following algorithm from the dialled Unit Number (UN):

Called individual ident = IBI + UN - 200

Three digit dialled strings in the range 900 to 998 inclusive shall be used to call groups of units within the same fleet.  The unit shall refer to its group base ident (GBI) and shall generate the called group identity by the following algorithm from the dialled Group Number (GN):

Called group ident = GBI + GN - 900

Three digit group numbers and two digit unit numbers may be used within the same fleet.

The unit shall verify that the dialled UN or GN is not greater than the highest value assigned during network personalisation.

Note:	Numbers 000 to 099 are not specified.  Certain numbers in the range 100 to 199 and also 999 are used for calls to assistance operators (optional - see section 8.2.7).

8.2.4.4	Four Digit Dialled Strings (Optional)

Four digit dialled strings in the range 1000 to 8999 are used for PABX access, see 8.2.5.  Other four digit dialled strings are not specified.

8.2.4.5	Five Digit Dialled Strings.  Leading Digit 2 or 9 (Optional)

Five digit dialled strings with leading 2 or 9 may be used to make inter-fleet calls to individual units or groups of units.  If both types of call are to be made then the convention of using 2 for individual calls and 9 for group calls shall be adopted wherever possible.  Dialled string manipulation depends upon the length of the called party unit number.  The first part of the dialled string forms a "routing code" which identifies the fleet of which the called unit is a member.  If the called unit number is 2 digits in length then the code is 3 digits in length.  If the called unit number is 3 digits in length then the code is 2 digits in length.  The unit shall contain a look-up table (translation field) in which the "routing code" is converted to the FN (or its equivalent address) which is appropriate to the called unit.  The table shall also contain data on the partition point between the "routing code" and the unit number and whether the call is to a group address.  This latter data shall be used before accepting a command to set RQS or RQE, FLAG1 = '1' in a group call.

eg for a call to an individual unit (RC UN): 28 789#

28 is translated by the unit, for example, to fleet Base Address: 34/936

The prefix is used directly and the called IDENT, 1525, is calculated by using the algorithm:

ID	=	IBI - 200 + UN

1525	=	936 - 200 + 789

Either a common prefix call or an extended addressing call follows appropriately.

For a call to a group (RC GN): 984 98#

984 is translated, for example, to Fleet Group Base Address: 34/5936

The prefix is used directly and the called IDENT, 5944, is calculated by using the algorithm:

ID	=	GBI -  90 + GN

5944	=	5936 - 90 + 98

Either a common prefix call or an extended addressing call follows appropriately.

Note:	Dialled strings of this length commencing with 1 are not specified.  Dialled strings commencing with digits 3, 4, 5, 6, 7, 8 and 0 are used for PABX calls.

8.2.4.6	Six Digit Dialled Strings.  Leading Digit 2 to 6 (Mandatory)

A six digit dialled string with a leading digit in the range 2 to 6 shall be used for inter-fleet calls where the call is between units having a common prefix.  The first four digits are the called party fleet number (FN) from which the unit shall create a base identity.

BI = 2(FN - 2000)

Using the created base identity and the rest of the dialled string as a unit number (UN) the unit creates a common prefix call using the procedures specified in 8.2.4.2.

Called individual ident = BI + UN - 20

Group calls, which are identifiable by 9 in the fifth digit, are handled similarly.

Called group ident = BI + GN - 90

It shall be possible to suppress inter-fleet group calls by unit personalisation.

Note:	Dialled strings of this length commencing with 0, 7 and 8 are used for PABX  calls.  Those commencing with 1 are not specified.

8.2.4.7	Seven Digit Dialled Strings.  Leading Digit 2 to 6 	(Mandatory)

A seven digit dialled string with a leading digit in the range 2 to 6 shall be used for inter-fleet calls where the call is between units having a common prefix.  The first four digits are the called party fleet number (FN) from which the unit shall create a base identity.

BI = 2 (FN - 2000)

Using the created base identity and the rest of the dialled string as a unit number (UN) the unit creates a common prefix call using the procedures specified in 8.2.4.3.

Called individual ident = BI + UN - 200

Group calls, which are identifiable by 9 in the fifth digit, are handled similarly.

Called group ident = BI + GN - 900

It shall be possible to suppress inter-fleet group calls by unit personalisation.

Note:	Dialled strings of this length commencing with 0, 7 and 8 are used for PABX calls.  Those commencing with 1 are not specified.

8.2.4.8	Eight Digit Dialled Strings.  Leading Digit 1 (Technician facility - Optional)

When the technician facility is enabled then 8 digit dialled strings with a leading 1, digits 2 to 4 in the range 000 to 127 and digits 5 to 8 in the range 0001 to 8100, shall set up a call to an address of which the last seven digits in the dialled string are the decimal representation.

The dialled string shall be of the form:

1 PFIX IDENT

e.g.	1 012 0246

The unit shall translate the dialled decimal representations of the prefix and identity into their binary equivalents and use these to transmit a single address codeword or extended addressing call request, whichever is appropriate to the radio unit's own address.

Note that although addresses are written in the form 12/246, the leading zeros shall be dialled thus: 012 0246.  Idents above 8100 shall be blocked to prevent calls to gateway addresses which may cause potential fault conditions.

Note:	Dialled strings of this length commencing with 0 are used for PSTN calls.  Dialled strings with other leading digits are not specified.

8.2.4.9	Nine Digit Dialled Strings.  Leading Digit 2 or 3 (Mandatory)

A nine digit dialled string with leading digits 2 or 3 shall be used for inter-fleet calls where the call is between units having different prefixes.  the first three digits are the called party's Number Prefix (NP), from which the unit shall create a prefix (PFIX).

PFIX = NP - 200

The rest of the dialled string shall be manipulated as specified in 8.2.4.6.  An extended addressing call shall be made to IPFIXI and the unit shall respond to an AHYC with a SAMIS containing the called party's address (!!9.2.2.1!!).  The unit shall trap the Number Prefix if it corresponds to the unit's own Number Prefix and shall resort to a common prefix, single address codeword call.

Note:	Dialled strings of this length commencing with 0 are used for PSTN calls.  Dialled strings with other leading digits are not specified.

8.2.4.10	Ten Digit Dialled Strings.  Leading Digit 2 or 3 (Mandatory)

A ten digit dialled string with leading digits 2 or 3 shall be used for inter-fleet calls where the call is between units having different prefixes.  The first three digits are the called party's Number Prefix (NP), from which the unit shall create a prefix (PFIX).

PFIX = NP - 200

The rest of the dialled string shall be manipulated as specified in 8.2.4.7.  An extended addressing call shall be made to IPFIXI and the unit shall respond to an AHYC with a SAMIS containing the called party's address (!!9.2.2.1!!).  The unit shall trap the Number Prefix if it corresponds to the unit's own Number Prefix and shall resort to a common prefix, single address codeword call.

Note:	Dialled strings of this length commencing with 0 are used for PSTN calls.  Dialled strings with other leading digits are not specified.

8.2.5		PABX Numbering (Optional)

MPT 1327 allows the radio unit to transmit numbers between 0 and 8191 with EXT = '1' in an RQS or RQE single address codeword call, in order to indicate that the called identity is an extension number of a PABX which is associated with the radio unit caller (!!5.5.3.1.1!!).  FLAG1 and FLAG2 are used to select one from a maximum of four PABXs.  The TSC forwards the dialled number to the selected PABX.  Extended addressing procedures are required via the PABXI gateway for calls to extensions on exchanges employing five or more digits, or to larger PABX networks.

Most PABXs use fixed length dialled strings whose leading digit is equal to or greater than 1.  Thus the largest range of numbers which may be encountered and which can be accommodated in a single address codeword call is 1000 to 8999.  However this range may only be accommodated by interposing an off-set in the transmitted request from the calling unit.  This off-set is removed by the TSC.  The off-set is 1000; thus for dialled extension numbers (EXTN) in the range 1000 to 8999 the unit sends 0 to 7999 in IDENT1.

IDENT1 = EXTN - 1000

Calls to PABXs with dialled strings shorter than four digits shall have dummy digits inserted in the leading digit positions (to be described as a dialling code in the user's directory) to make up the length of the dialled string to four digits.  Recommended values are 7, 77, etc.  Different codes may be used to route calls to different branch exchanges by arrangement between the network operator and the PABX user.  Shorter dialled strings on PABXs with mixed length numbering shall be dealt with in a similar manner.

If access is required to a small group of branch exchanges (a maximum of four), then FLAG1 and FLAG2 in the RQS or RQE address codeword shall be set to indicate to which exchange the required extension is connected.  The flags are set in the radio unit by the use of a leading digit before the extension number, which thus requires the user to dial a five digit string.

Access to larger PABX networks uses extended addressing routines.  These are set up by dialled string lengths of five to eight digits and with particular leading digits.

8.2.5.1	Single digit Dialled Strings

To call a PABX operator the unit may be set up to call identity 8000 with EXT = '1', FLAG='0' and FLAG2 = '0' when 0 is dialled.

8.2.5.2	Two Digit Dialled Strings

A two digit dialled string in the range 03 to 06 shall be used to select one of four different branch exchange operators.  The radio unit shall be set up to call identity 8000 with EXT = '1', and FLAG1 and FLAG2 set according to the following table:

FLAG1	FLAG2

03	'0'	 '0'

04	'0'	 '1'

05	'1'	 '0'

06	'1'	 '1'

8.2.5.3	Three Digit Dialled Strings

Three digit dialled strings are not used for PABX calls.

8.2.5.4	Four Digit Dialled Strings

The radio unit shall accept any four digit dialled string as a call to a PABX and shall set the EXT flag to '1'.  FLAG1 and FLAG2 shall be set to '0'.  The dialled extension number (EXTN) shall be decremented by 1000 and the resultant shall be transmitted as the called identity (see 8.2.5).

IDENT = EXTN - 1000

8.2.5.5	Five Digit Dialled Strings.  Leading Digit 3 to 6

Five digit dialled strings with a leading digit from 3 to 6 function similarly to four digit dialled strings, but the radio unit shall set FLAG1 and FLAG2 in RQS or RQE, or EXCHANGE in SAMIS, according to the following table:

Leading Digit	FLAG1 FLAG2	EXCHANGE (SAMIS)

!!5.5.3.1.1!!	!!5.6.1.2.2!!

3	'0'	 '0'	'00'

4	'0'	 '1'	'01'

5	'1'	 '0'	'10'

6	'1'	 '1'	'11'

Having used the leading digit in the five digit dialled string to set FLAG1 and FLAG2, the radio unit shall process the remaining four digits as described in 8.2.5.4.

8.2.5.6	Five to Seven Digit Dialled Strings.  Leading Digits 0, 7 or 8

Dialled strings of five to seven digits with leading digits 0, 7 or 8 shall be used to send a call to the PABXI gateway.  On receipt of a Mode 1 AHYC with IDENT1 = PABXI and DESC = '010', the radio unit shall respond with a SAMIS containing DESC = '010', and with the full dialled string occupying BCD blocks commencing with BCD1.  Unused blocks shall be filled with NULL.

Note:	Five digit dialled strings with leading 2 or 9 are used for inter-fleet calls and leading digit 1 is not specified.,

8.2.5.7	Eight or Nine Digit Dialled Strings.  Leading Digits 7 or 8

Dialled strings of eight or nine digits with leading digits 7 or 8 shall cause the radio unit to send a call to the PABXI gateway.  On receipt of a Mode 1 AHYC with IDENT1 = PABXI and DESC = '010', the radio unit shall respond with a SAMIS containing DESC = '010' and with the full dialled string occupying BCD blocks commencing with BCD1.  When the dialled string length is 8 digits the unused block shall be filled with NULL.

8.2.6		PSTN Numbering (Optional)

8.2.6.1	PSTN Abbreviated Dialling

Single address codeword calls and diversions may be made to 15 pre-arranged numbers by using the special idents PSTNSIj.  The procedures are defined in section 8.2.8.1.

8.2.6.2	Calls to PSTN Subscribers

Dialled strings of eight or more digits with leading digit 0 shall cause the radio unit to send an RQS or RQE call to the PSTNGI gateway with FLAG1 set to '0' to indicate a dialled string length of 10 or less digits, or set to '1' to indicate 11 to 32 digits.  On receipt of a Mode 1 AHYC with IDENT1 = PSTNGI, DESC = '001' and SLOTS = '01' or '10' as appropriate, the radio unit shall respond with SAMIS, DESC = '001' with the full dialled string, less the leading 0, occupying BCD blocks commencing with BCD1.  If SLOTS = '10' and the dialled string has a length of 11 to 21 digits inclusively then the accompanying SAMIS shall have L set to '01' and the radio unit shall append one data codeword which shall contain the digits that cannot be accomodated in the SAMIS. If SLOTS = '10' and the dialled string length is greater than 21 digits then the accompanying SAMIS shall have L set to '10' and the radio unit shall append two data codewords which shall contain the digits that cannot be contained in the SAMIS. Unused BCD groups shall be filled with NULL.

8.2.7		Network Operator Services (3 digit calls) (Optional)

Three digit dialled strings in the table below shall be used to call network services.  The table lists dialling strings and the special identities into which they shall be translated by the unit.

Dialled		Special 	Dialled 	Special

String		Ident 		String 		Ident

100		8170 		151 		8175

111		8171 		161 		8176

112		8180 		171 		8177

121		8172 		181 		8178

131		8173 		191 		8179

141		8174		999		8180

Figure 8.5   Table of Special Idents for Operator Services

112 and 999# shall initiate an RQE call request message.

8.2.8		Call Modifiers and Radio Unit Control

Calls are modified from speech calls to other types of call, and the radio unit functions are controlled by, dialled strings commencing with * or #.  This section defines codes which directly affect MPT 1327 address codewords.  Other functions which the unit is required to perform may use codes of a similar type but the unit manufacturer has choice of code and implementation.

Codes which modify a call precede the dialled number and are separated from the number by use of the * key.  Generally, codes which change a function between two states initiate the state by commencing the code with *, and re-set the state by the same code commencing with #.

The # key is always used to signal the end of dialling activity, with the exception of when the unit is alerting for an incoming call.

e.g.	-	To make a priority call to a unit dial *8 *234#

-	To make a non-prescribed data call to an abbreviated PSTN destination dial *31 **12#

-	To make a conference group call dial *1 *923#

As well as the keypad functions, any of the function codes may also be assigned to special function keys which carry out the same action as a code but in a single keystroke.

8.2.8.1	Call Initiation

8.2.8.1.1	Abbreviated Dialling (Optional)

Codes for TSC translation of special idents shall be of the form:

**nn#

nn shall be in the range 1 to an upper limit specified by the network operator, but not greater than 49.  nn shall be translated by the radio unit into a special ident by the following algorithm:

Ident = 8120 + nn

8.2.8.1.2	Call ALLI: Codes *1981# to *1987# (Optional)

The entry of any of the codes *1981# to *1987# may be used to cause the radio unit to send RQS, RQE or RQC (as specified below) with IDENT1 set to the special ident 8191 (the system-wide ident - refer to !!4!!). The type of request message to be sent and the values of certain fields within that message shall depend upon the value of the code entered, as specified below:

Code	Type of System-	Request Message	Specified Field

wide Call		Type	Value

*1981#	Priority voice	RQS	DT='0'  LEVEL='0'

*1982#	Emergency voice	RQE	DT='0'

*1893#	Priority np data	RQS	DT='1'  LEVEL='0'

*1984#	Emergency np data	RQE	DT='1'

*1985#	Short data	RQC

*1987#	Standard voice	RQS	DT='0'  LEVEL='1'

The above codes may not be used in conjunction with any additional digits (eg other call modifiers or destination digit strings).

8.2.8.2	Call Modifiers (Optional)

8.2.8.2.1	Despatcher and Status calls

Enter Despatcher Queue: Code *0

The radio unit shall set up an RQQ (STATUS = 0) call to the dialled number.  The radio unit may default to a pre-arranged number if a number is not dialled, ie the unit may set up an RQQ call to an address which is programmed within the unit if the # key is depressed immediately after the code (!!5.5.3.1.7!!).

Leave Despatcher Queue: Code #0

The radio unit shall set up an RQQ (STATUS = '31') call to the dialled number.  The radio unit may default to the last despatcher queue which had been entered if no number is dialled (!!5.5.3.1.7!!).

Send Status: Code *0nn

The radio unit shall set up an RQQ (STATUS = 'nn') call to the dialled number.  The radio unit may default to a pre-arranged number if a number is not dialled, ie the unit may set up an RQQ call to an address which is programmed within the unit if the # key is depressed immediately after the code.  nn is a single or two digit code without a leading zero and shall be in the range 1 to 30 (!!5.5.3.1.7!!).

8.2.8.2.2	Conference/Broadcast Group Facility

Units which receive group calls may allow the user to reply to the calling party (conference) or the unit may inhibit replies (broadcast) on receipt of a maintenance message on the traffic channel.  The unit originating the call sends RQS (FLAG1 = '1') or RQE (FLAG1 = '1') to disable the called units' ability to reply.  The calling unit shall only set FLAG1 to '1' if the called address is a group address (!!5.5.3.1.1!!).

Conference and broadcast calls may be specifically requested using the call modifiers described in 8.2.8.2.2.1 and 8.2.8.2.2.2. No default is prescribed for cases where neither call modifier is used; however it is recommended that the default is conference call.

8.2.8.2.2.1	Conference Call Code *1

*1 sets RQS or RQE, FLAG1 to '0'

8.2.8.2.2.2	Broadcast Call Code *11

*11 sets RQS or RQE, FLAG1 to '1'

8.2.8.2.3	Data Facilities

*2 initiates a section 14 Control Channel Short Data Message.Paragraph 8.2.8.2.3.2 specifies the use of the code. Codes of the format "*2n" where "n" is any length string commencing with 1 or 2 are reserved for future definition. Codes where the string commences with any digit from 3 to 9 or 0 are available for customisation.

*31 is specified in this section. Codes of the format "*3n" where "n" is any length string commencing with 2 are reserved for future definition. Codes where the string commences with any digit from 3 to 9 or 0  are available for customisation.

8.2.8.2.3.1	Non-prescribed Data.  Code *31

*31 sets RQS (DT = '1') or RQE (D = '1') as appropriate (!!5.5.3.1.1!!).  This code requests that a transparent end to end audio path be set up by the network.  The user supplies the data communications equipment (DCE) to terminate the audio path.  The radio unit's acoustic transducers are inhibited and the audio connection to the DCE is enabled on receipt of GTC.  Call maintenance messages are inhibited unless required by the network operator.

8.2.8.2.3.2	Short Data on the Control Channel.  Code *2

*2 causes the radio unit to initiate an RQC message. The dialled format is *2*<data>*<called party number>#. The radio unit shall respond to the associated AHYC from the TSC with the data field of the dialled string in accordance with section 14. The STF field, and MESS field if applicable, may be set in any appropriate manner for the input data.

*2# shall cause the radio unit to send data from an external device to a pre-arranged address.

*2*<data># shall cause the radio unit to send data from the keyboard to a pre-arranged address.

*2**<called party number># shall cause the radio unit to send data from an external device to the called party number.

8.2.8.2.3.2.1	Use of the data field

If the data field contains no input the radio unit shall substitute data from an external data input device. Any format prescribed in section 14 may be implemented and the GFI field shall be set appropriately. The external device may also originate short data messages without the need for the user to make an entry on the radio unit keyboard. The protocol between the radio unit and the external device is not defined.

8.2.8.2.3.2.2	Use of the number field

Any number defined in section 8.2.4 to 8.2.7 may be dialled.

8.2.8.2.4	Priority Call Code *8

The radio unit shall set up an RQS (LEVEL = '0') call to the dialled number (!!5.5.3.1.5!!).  The user may dial, and the unit shall accept, any further suitable modifier which sets other bits in the RQS address codeword.  The further modifier shall be dialled with its leading * character, eg *1, *11, *32.  The code shall also function with abbreviated dialling codes commencing with **.

8.2.8.2.5	Emergency Call Code *9 (!!10.2!!)

The radio unit shall set up an RQE call to the dialled number (!!5.5.3.1.5!!).  The radio unit may default to a pre-arranged number if a number is not dialled, ie the unit may set up an RQE call to an address which is programmed within the unit if the # key is depressed immediately after the code.  The user may dial, and the unit shall accept, any further modifier which sets other bits in the RQE address codeword.  The further modifier shall be dialled with its leading * character, eg *1, *11, *32.  The code shall also function with abbreviated dialling codes commencing with **.

The special Emergency mode (FLAG2 = '1') (!!10.2!!) is entered by dialling an additional string after *9.  If the first digit in the additional string is in the range 1 to 4 then FLAG1 and D are set according to the following table:

Digit	FLAG1		D

1	'0'	'0'

2	'0'	'1'

3	'1'	'0'

4	'1'	'1'

The remaining digits are the decimal representation of the 13 bit number which is sent in IDENT1.  Leading zeros which result from the binary to decimal conversion are not dialled.

If the first and only digit in the additional string is 9 then the unit expects to receive an input from an external unit to set FLAG1, D and IDENT1.

e.g.	*91#  sets all bits to '0'

*923# sets D to '1', FLAG1 to '0', and IDENT1 to '0000000000011'

The TSC handles the received RQE by agreement with the user or service provider.  No other call modifiers or called party number shall be accepted by the unit in conjunction with this facility.

112# and 999# shall set up an RQE call without the need to dial *9.

8.2.8.3	Radio Unit or Network Control (Optional)

The radio unit may incorporate one or more routines to allow calls to be controlled fully and to modify unit action on receipt of incoming calls etc.  MPT 1327 related functions include call diversion, rejection of incoming call interrogations etc.

8.2.8.3.1	Divert Own Calls: Code *41n (!!12.2!!)

*41 shall cause the radio unit to send RQT (DIV = '0', FLAG1 = '0', FLAG2 = '0') (unless FLAG1 is set to '1' by PSTN number string length) to the TSC (!!5.5.3.1.4!!).  The RQT instructs the TSC to offer the number which has been dialled after the code to any caller who is attempting to make a call to the originating radio unit as an alternative destination for the call.  The number to which calls are to be diverted, and which follows the code, shall be any number which the user is able to dial for a normal speech call (RQS).  If no number is dialled after the code then the unit shall not transmit a diversion request.

If n is not dialled, all call types are diverted.

RQT, SD shall be set to '00'.

If n = 1, only speech calls are diverted.

RQT, SD shall be set to '01'.

If n = 2, only data calls are diverted.

RQT, SD shall be set to '10'

e.g.	*41 *234# diverts all types of call to unit 234 in the same fleet as the originator of the diversion.

*411 *3456# diverts speech calls to extension 3456 in the PABX which is associated with the diversion originator's fleet.

When unit numbers which would normally result in a single address codeword call are entered as a call diversion request, they shall be sent as IDENT1 in an RQT.

When unit numbers which would normally result in an extended addressing call are entered as a call diversion request, they shall be sent in an RQT with IPFIXI in IDENT1.  The dialled number shall be sent in a SAMIS (DESC = '000') in PARAMETERS1 in response to an AHYC with the same address information as contained in the originating RQT.

When calls to PABX extension numbers which would normally result in a single address codeword call are entered as a call diversion request, they shall be sent as an RQT with PABXI in IDENT1 and the dialled number shall be sent in a SAMIS (DESC = '010', SP = '1').  EXCHANGE shall be set to '00' for four digit calls and shall be set as listed in the table in 8.2.5.5 for five digit calls  The dialled number is manipulated by the algorithm used for creating IDENT1 in an RQS or RQE and is transmitted in Number.  The SAMIS is sent in response to an AHYC with the same address information as contained in the originating RQT.

When called extension numbers which would normally result in an extended addressing call are entered as a call diversion request, they shall be sent as an RQT with PABXI in IDENT1 and the dialled number shall be sent in a SAMIS (DESC = '010', SP = '0'), with the BCD blocks filled in the same manner as for a normal call (8.2.5.7).  The SAMIS is sent in response to an AHYC with the same address information as contained in the originating RQT.

When abbreviated calls to PSTN numbers which would normally result in a single address codeword call are entered as a call diversion request, they shall be sent as an RQT with the correct PSTNSIj in IDENT1.

When calls to PSTN numbers which would normally result in an extended addressing call are entered as a call diversion request, they shall be sent as an RQT with PSTNGI in IDENT1.  If the dialled string (including the leading 0) is equal to or greater than 11 digits, FLAG1 = '1'.  The dialled number shall be sent in a SAMIS (DESC = '001') with the BCD blocks filled in the same manner as for a normal call (See 8.2.6.2).  The appended codeword shall also be sent when necessary.  The SAMIS is sent in response to an AHYC with the same address information as contained in the originating RQT.

8.2.8.3.2	Cancel Own Diversion:  Code #41n#

Upon being dialled the radio unit shall send RQT to the TSC to instruct it to cancel the divert state.

If n is not dialled, all call types shall cease to be diverted.

RQT (SD = '00')

If n = 1, only speech calls shall cease to be diverted.

RQT (SD = '01')

If n = 2, only data calls shall cease to be diverted.

RQT (SD = '10')

Cancellation of divert condition causes the radio unit to transmit RQT (DIV = '1', FLAG1 = FLAG2 = '0') with IDENT1 = IDENT2 (the originating unit's own address).

8.2.8.3.3	Divert Third Party Calls:  Code *44n

*44n is followed by the number of the third party whose calls are to be diverted, and then by the diversion destination.

*44 shall cause the radio unit to send RQT (DIV = '0', FLAG1 = '0', FLAG2 = '1') (unless FLAG1 is set to '1' by PSTN number string length) to the TSC.  The RQT instructs the TSC to offer the call destination to any caller who is attempting to make a call to the third party number as an alternative destination for the call.  The number to which calls are to be diverted shall be any number which the user is able to dial for a normal speech call (RQS).  If less than two numbers are dialled after the code then the unit shall not transmit a call diversion request.

If n is not dialled, all call types are diverted.

RQT, SD shall be set to '00'

If n = 1, only speech calls shall be diverted.

RQT, SD shall be set to '01'

If n = 2, only data calls shall be diverted.

RQT, SD shall be set to '10'

e.g.	*442 *234 *432#  Data calls directed to unit 234 in the same fleet as the diversion originator shall be re-directed to unit 432 in the same fleet.

*441 *234 *3456#  Speech calls to unit 234 in the same fleet as the diversion originator shall be re-directed to extension 3456 in the PABX which is associated with the diversion originator's fleet.

Methods of sending the diversion data are specified in 8.2.8.3.1.  In addition, the blocked third party address is sent in response to an AHYC with IDENT1 = DIVERTI, PFIX/IDENT2 = requesting unit's address, DESC = '000', SLOTS = '01'.  The order in which the two AHYCs are sent is not defined.

8.2.8.3.4	Cancel Third Party Diversions: Code #44n

This code is followed by the number of the third party whose calls are presently diverted.

The radio unit shall send RQT to the TSC to instruct it to cancel the divert state for the nominated third party.  If a number is not dialled after the code then the unit shall not transmit a diversion cancellation request.

If n is not dialled, all call types cease to be diverted.

RQT, SD shall be set to '00'

If n = 1, only speech calls cease to be diverted.

RQT, SD shall be set to '01'

If n = 2, only data calls cease to be diverted.

RQT, SD shall be set to '10'

e.g.	#441 *234# allows unit 234 in the same fleet as the call request originator to receive speech calls directed to the unit.

Cancellation of the divert condition causes the radio unit to transmit RQT (DIV = '1', FLAG1 = FLAG2 = '0') with IDENT1 = third party identity, or IPFIXI and IDENT2 = the requesting unit's own address.  If IDENT1 = IPFIXI, the full address of the third party unit is transmitted in a SAMIS (DESC = '000') in PARAMETERS1, in response to an AHYC with IDENT1 = IPFIXI.

8.2.8.3.5	Cancel Incoming diversions: Code #45n#

This code causes the radio unit to send RQT to the TSC to instruct it to cancel any divert state which may be directing calls to the originating radio unit.  Cancellation of incoming diversions causes the radio unit to transmit RQT (DIV = '1', FLAG1 = FLAG2 = '0') with IDENT1 = DIVERTI.

If n is not dialled, all call types cease to be diverted.

RQT, SD shall be set to '00'

If n = 1, only speech calls cease to be diverted.

RQT, SD shall be set to '01'

If n = 2, only data calls cease to be diverted.

RQT, SD shall be set to '10'

8.2.8.3.6	Queue Incoming Calls: Code *48#

This code causes the radio unit to respond to AHY (D='0') messages with ACKB (QUAL = '0') (!!9.1.1.5!!) and place the call details in a call queue.  Selection of this state shall cancel any previously entered "Don't Disturb" (8.2.8.3.8) in respect of AHY (D='0') message.

8.2.8.3.7	Cancel Queueing of Incoming Calls: Code #48#

This code causes the radio unit to cease queueing calls and resort to acknowledging all AHY (D='0') messages with any acknowledgement which is appropriate for the radio unit state (!!9.1.1.5!!).

8.2.8.3.8	Don't Disturb:  Code *49n#

This code causes the radio unit to acknowledge all appropriate AHY messages with ACKV (QUAL = '1'), thus blocking the call (!!9.1.1.5!!).

If n is not dialled, the unit shall respond ACKV (QUAL = '1') to all appropriate AHYs to the unit.  D may be either '0' or '1'.

If n = 1, the unit shall respond ACKV (QUAL = '1') to appropriate AHYs to the unit when D is set to '0' (speech calls).

If n = 2, the unit shall respond ACKV (QUAL = '1') to appropriate AHYs to the unit when D is set to '1' (data calls).

8.2.8.3.9	Cancel Don't Disturb:  Code #49n#

This code causes the radio unit to acknowledge all appropriate AHY messages with any acknowledgement which is appropriate for the radio unit state (!!9.1.1.5!!).

If n is not dialled, the unit shall cease to respond ACKV (QUAL = '1') to any appropriate AHYs to the unit.  D may be either '0' or '1'.

If n = 1, the unit shall cease to respond ACKV (QUAL = '1') to appropriate AHYs to the unit when D is set to '0' (speech calls).

If n = 2, the unit shall cease to respond ACKV (QUAL = '1') to appropriate AHYs to the unit when D is set to '1' (data calls).

8.2.8.4	Radio Unit Control (Mandatory)

The unit shall incorporate keyboard routines to allow the unit to abandon call attempts, clear calls and accept incoming calls (see section 8.3.3.).

8.2.8.4.1	Call Set-Up Abandon:  Call Complete:  Code *#

*# may be dialled after digits and a terminator have been entered on the keyboard.  If the radio unit has not transmitted a call request, it shall abandon the call and resume an idle state on the control channel.

If the radio unit has started to set up a call, it shall transmit a call cancel request (RQX) (!!9.1.1.8!!).

If *# is dialled whilst the unit is on a traffic channel, the radio unit shall terminate the call by transmitting MAINT (OPER = '011') where appropriate to its call condition (!!9.2.3.5!!).

If *# is dialled whilst the radio unit is indicating call failure or termination by a continuing confidence indication, then the radio unit shall cancel that indication.

Notes:

1.	*# may also be used to abandon erroneous keyboard entries.

2.	If *# is dialled after an "off-hook" indication has been sent to the TSC to accept an incoming call, but before the radio unit receives GTC, the radio unit shall transmit an "on-hook" indication RQQ (STATUS = 31) to TSCI (!!5.5.3.1.7!!).

8.2.8.4.2	Accept Incoming Call:  Code # (Alerting State only)

If # is dialled when the radio unit is alerting the user for an incoming call, the radio unit shall send RQQ (STATUS = 0) to TSCI.  In the alerting state the # key is not used as a dialled string terminator.  An incoming call shall cause abandonment of a dialling attempt which has not been terminated by the string terminator (!!5.5.3.1.7!!).

8.2.9		Summary of Numbering Convention  	(Notes)

Single digit dialled strings (Z).  Not defined.  Proposed use:

Z	0		Associated PABX Operator 	(Ident 8000)

Z	1 - 9		Despatcher	(Ident looked up in unit)

Two digit dialled strings (YZ)

Y	0		PABX operator in exchange group, EXT = '1'

Z	3 - 6		PABX access number for exchange as defined in five digit numbers below (Ident 8000 with FLAGs 1 and 2 set appropriately)

YZ	20 - 89		Individual number

YZ	90 - 99		Group number

Three digit dialled strings (XYZ)

X	1		Network operator services

YZ	00			(Ident 8170)

11			(Ident 8171)

21			(Ident 8172)

31			(Ident 8173)

41			(Ident 8174)

51			(Ident 8175)

61			(Ident 8176)

71			(Ident 8177)

81			(Ident 8178)

91			(Ident 8179)

XYZ	200 - 899		Individual calls in fleet

999		Emergency Operator 	(Ident 8180)

X	9		Group call

YZ	00 - 98		Group number

Four digit dialled strings (WXYZ)

WXYZ	1000 - 8999		Single addressword calls to single, associated PABX.

EXT = '1'. Flags 1 and 2 default to '0'.

Five digit dialled strings (VWXYZ)

V	2, 9		Large fleet short form access

W	0 - 9		Routing code

XYZ	200 - 998		Individual or group number

WX	00 - 99		Routing code

YZ	20 - 99		Individual or group number

V	3 - 6		PABX access number for single addressword

calls (value sets FLAGs 1 and 2), EXT = '1'

WXYZ	1000 - 8999		PABX extension number

V	0, 7 or 8		PABX network access number (extended

addressing protocol) 	(PABXI)

WXYZ	NNNN		Exchange routing digits and extension number

Six digit dialled strings (UVWXYZ)

UVWX	2001 - 6050		Fleet number

YZ	20 - 99		Individual or group number

U	0, 7 or 8		PABX network access number	(PABXI)

VWXYZ	NNNNN		PABX routing digits and extension number

Seven digit dialled strings (TUVWXYZ)

TUVW	2001 - 6050		Fleet number

XYZ	200 - 998		Individual or group number

T	0, 7 or 8		PABX network access number	(PABXI)

UVWXYZ	NNNNNN		PABX routing digits and extension number

Eight digit dialled strings (STUVWXYZ)

S		1	Service technician's access only

TUV	000 - 127	Transpose keyboard entry to binary and

send in PFIX as part of inter-prefix call

WXYZ	0001 - 8100		Transpose keyboard entry to binary and

send in IDENT1 as part of inter-prefix call

S		0	PSTN access number 	(PSTNGI)

TUVWXYZ 	NNNNNNN		PSTN routing digits and subscriber number

S		7 or 8	PABX network access number	(PABXI)

TUVWXYZ	 NNNNNNN		PABX routing digits and extension number

Nine digit dialled strings (RSTUVWXYZ)

RST	200 - 327		Number prefix

UVWXYZ	-		For details see six digit numbers, inter-fleet numbers

R		0	PSTN access number	(PSTNGI)

STUVWXYZ	 NNNNNNNN		PSTN routing digits and subscriber number

R	7 or 8		PABX network access number 	(PABXI)

STUVWXYZ	 NNNNNNNN		PABX routing digits and extension number

Ten digit dialled strings (PRSTUVWXYZ)

PRS	200 - 327		Number prefix

TUVWXYZ		-	For details see seven digit numbers, fleet numbers

P	0		PSTN access number (PSTNGI)

RSTUVWXYZ	 NNNNNNNN		PSTN routing digits and subscriber number

Eleven or more digit dialled strings (NPRSTUVWXYZ)

N	0		PSTN access number

PRSTUVWXYZ	 NNNNNNNNNN		Routing digits and subscriber number

8.2.10		Summary of Control Codes

**nn	Abbreviated dialling codes

nn = 1  to 15.  PSTN abbreviated numbers prescribed method

nn = 16 to 49.  Network operator specified abbreviated dialling

*#  	Call set-up abandoned, call complete

*0	Despatcher queue

*0nn	Status 'nn' to despatcher

*1	Conference call

*11	Broadcast call

*1981#	Priority voice system-wide call

*1982#	Emergency voice system-wide call

*1983#	Priority np data system-wide call

*1984#	Emergency np data system-wide call

*1985#	Short data system-wide call

*1987#	Standard voice system-wide call

*2	Short Data on the Control Channel.

*3	*31 is prescribed. other codes are either reserved for future definition or are available for customisation.

*31	Non-prescribed data call

If "n" is dialled in the following codes, the function is restricted to speech calls if n = 1, or data calls if n = 2.

*41n	Divert own calls, ie calls directed to originator of diversion

*44n	Divert third party calls, ie calls directed to a nominated party

*48	Queue incoming calls (speech calls only)

*49n	Don't disturb

*8	Priority call

*9	Emergency call, RQE

*9xxx#	Special emergency facility by arrangement with network operator

#	End dialled string, accept call if radio unit is alerting

#0	Leave despatcher queue

#41n	Cancel own diversions, ie allow calls to a unit for which previously a call diversion was in force

#44n	Cancel third party diversions, ie allow calls to a nominated third party for which previously a call diversion was in force

#45n	Cancel incoming diversions

#48	Cancel queueing of incoming calls (speech calls only)

#49n	Cancel Don't Disturb.

8.2.11		Glossary of Terms Specificed to Section 8.2

Base Indent:	The lowest value of ident allocated to a "fleet" of units

Block:	An allocation of contiguous idents to a user "fleet"

Dialled String:	The numbers dialled by a caller on the keypad of his radio unit to call any other party.  Pre-origination dialling is employed and the end of the dialled string is indicated by the user dialling #

Fleet:	All or a sub-set of a user's vehicles which are equipped with radio units, and which require direct communication between users by means of two or three digit dialling

Fleet Number:	The dialled digits which identify the identity "block" for the fleet containing the called unit (digits 4 to 7 of the "Network Number").  Directly related to the "Fleet Base Ident"

Fleet Individual	A "Fleet Number" via which calls to individual units may be made

Number:

Fleet Group	A "Fleet Number" via which calls to groups of units may be made

Number:

Group Base Ident:	The lowest value of ident allocated to a "fleet" of units for group addresses

Group Ident:	An ident used to call a group of users

Group Number:	The last group of digits in the "Network Group Number" which identifies the group of unit within the "fleet".  Related to the "Group Ident" via the "Group Base Ident".

Individual Ident:	An ident used to call an individual user

Network Number:	The number by which a unit or group may be identified in a directory, all or part of which is dialled by a caller wishing to communicate with the identified party or parties.  It consists of the "Number Prefix", the "Fleet Number" and the "Unit Number"

Network Individual	The number which identifies an individual unit in a "fleet"

Number:

Network Group 	The number which identifies a group of units in a "fleet"

Number:

Number Prefix:	The three leading digits of the "Network Number".  Directly related to the prefix Unit Number:	The last group of digits in the "Network Individual Number" which identifies the unit within the "fleet".  Related to the "Individual Ident" via the "Fleet Base Ident"

8.3		Ready for Communication Control

8.3.1		Readiness for Communication

In the signalling standard MPT 1327 the terms "off-hook" and "on-hook" are used to designate the state of user readiness for communication.  These terms have been used to reflect a parallel with the fixed telephone network where removal of the telephone handset from the switch cradle indicates a readiness for communication (off-hook).  When the handset is resting in the switch cradle the user is assumed to be not ready for communication.

An indication of the state of readiness of a party to a call is necessary to enable the following functions, applicable to commercial networks in Band III:

(a)	To enable the user to instruct the radio unit to attempt to transmit a call request for a called party address indicated by pre-origination dialling or other appropriate method (calling party off-hook).

(b)	In systems which employ full off-air call set up, to enable the user to indicate to the radio unit readiness to accept an incoming call (called party off-hook).  This function is not applicable to group calls.

(c)	To enable the user to instruct the radio unit to attempt to cancel a previously made request during the call set up phase or to request clear-down of a call in progress (calling party on-hook).

(d)	To enable the user to instruct the radio unit to attempt to cancel a called party off-hook instruction during the call set up phase or to request clear-down of a call in progress (called party on-hook).

(e)	To enable the user to cancel any confidence indication generated by the radio unit for a failed call attempt (calling or called party on-hook).

For convenience these functions are regarded as being fulfilled by a single control that shall be able to be activated on all radio units, although it is permissible to use more than one control for these purposes.  This control, or group of controls, is designated the "Ready for Communication Control" (RFCC).

The RFCC must be capable of providing a real indication of the state of user readiness to the radio unit in order to support these functions.  Accordingly, deliberate arrangements which allow the RFCC to be set by the user in an off-hook mode, and which do not reflect the true state of user readiness, shall not be permitted.

8.3.2		The Form of the Ready for Communications Control

It is not the intention of this specification to restrict unduly the scope for individual design of radio units.  Consequently no requirements for the physical form that the RFCC shall take are prescribed.  However, the following general comments are offered to assist the designer of radio units.

Failure to request traffic channel clear-down at the end of communication is a potentially serious cause of wasted air-time.  RFCC devices which do not solely rely on user discipline in this matter are to be preferred to those, such as an ordinary switch-hook, which do.

RFCC devices which are designed to allow a means of acceptance of incoming full off-air call set up calls even when the user did not take the correct action to clear down the previous calls are to be preferred (eg, a unit left off-hook from a previous call shall be capable of signalling an on-hook to off-hook transition for a new incoming call).

A switch-hook type design is generally not applicable to hand portables and may not be the most suitable design in many other applications.

The design of the RFCC control shall take due account of the effects of normal operation upon road safety.

8.3.3		Control Requirements

The RFCC control, or group of controls, shall permit the user to signal the following to the radio unit, which shall then initiate the appropriate action prescribed elsewhere in the specification:

-	an instruction for a call request to a called party address indicated by pre-origination dialling, or other appropriate method; "call request initiate",

-	a called party answer instruction following receipt of AHY(CHECK='1'); "called party answer" (see MPT 1327 section 13.1.2.1),

-	a call clear-down instruction for an individual call to which the user is a party; "call end request" (see MPT 1327 section 9.2.3.5),

-	a calling party call cancellation instruction at any time prior to the receipt of a GTC message for the requested call; "call set-up abort"

(see MPT 1327 section 9.2.1.7),

-	an instruction to a radio unit, which is indicating call failure or termination by a continuing confidence indication, to cancel that indication; "cancel indication" (see 8.1).

The RFCC controls, or group of controls, may in addition permit the user to signal the following to the radio unit:

-	an instruction to reject an incoming call; "incoming call reject"

(see MPT 1327 section 13.1.2.1),

-	an instruction to cancel a previously signalled called party off-hook state; "cancel called party answer" (see MPT 1327 section 13.1.2.6).