7.		SECURITY

The radio unit shall meet the following requirements:

1.	Each radio unit shall have a unique 38 bit security number (which may be stored encoded) which will be programmed only by the radio unit manufacturer.  The radio unit manufacturer shall take reasonable steps to ensure that:

(a)	the method of programming the security number is known only to that manufacturer,

(b)	any modification to the security number other than by that manufacturer shall disable the radio unit.

2.	The radio unit shall be designed such that removal of the device (or any of the devices) containing the security number is highly likely to cause irreparable damage to the radio unit and/or the device(s) (eg encasing a PROM containing the security number and the immediate surrounding area in epoxy resin).

3.	Removal of any device containing the security number or part of the security number shall disable the radio unit.

4.	The radio unit shall not contain the method by which the 8 check bits defined below are calculated.

5.	The security number (excluding the check bits, see below) of the radio unit shall be marked visibly and permanently on a part of the radio unit which is inseparable without damage from the device containing the security number.  It is preferable that this number is visible with a minimum of dismantling of the radio unit, eg by removing 2 screws or equivalent.

6.	It is recommended that a check is made on the validity of the security number within the radio unit, using a different set of check bits to the 8 check bits defined below, and that the radio unit shall be disabled if the check bits differ.

7.	The format of the security number is as follows:

| Manufacturer's Code | Model | Serial number for this model | Check bits |
| --- | --- | --- | --- |
| 8 bits | 4 bits | 18 bits | 8 bits |

Note:	The security number need not be stored in this format in the radio unit.

8.	When requested by the TSC (see section 11.15.2 and also section 15.2 of MPT 1327), the radio unit shall send the security number data to the TSC using the SAMIS message (see MPT 1327 sections 15 and 5.6.1.2.2).  The parameter fields of the SAMIS message are constructed as follows:

| PARAMETERS 1 (20 bits) |  |  | PARAMETERS 2 (18 bits) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Manufacturer's Code | Model | Check bits |  |  | Serial number for this model |  |
| 8 bits | 4 bits | 8 bits |  |  | 18 bits |  |

| Manufacturer's code: | An 8 bit number (0 to 255), one or more of which is issued to each radio unit manufacturer by the Radiocommunications Agency(RA). |
| --- | --- |
| Model | A 4 bit number (0 to 15) which is unique to a radio unit type for a given manufacturer's code. The model number is allocated by the manufacturer as and when new radio unit models are to be type approved. In the event of a radio unit manufacturer producing more than 16 type approved models, the manufacturer may apply for an additional manufacturer's code. |
| Serial number (for this model): | The serial number of the radio unit of a given model. This number is allocated by the manufacturer, and would normally run from 1 up towards 262143. Where this capacity is exceeded, the radio unit manufacturer may allocate an additional model number to radio units of the same type approval type. |
| Check bits: | The algorithm for calculating the check bits is based on the data contained in the other fields above. The algorithm used in the UK is available from the Radiocommunications Agency to manufacturers and network operators. If the check bits are incorrect in a radio unit, a network may refuse access to that radio unit. The algorithm shall not be present in the radio unit. |

9.	The security number shall be marked on the radio unit in the following form:

xxx/yy/zzzzzz	where 	xxx	is the manufacturer's code,

yy 	is the model, and

zzzzzz		is the serial number.

eg	019/09/000129

³	³	³

³	³	ÀÄÄÄÄ	Serial number 129

³	ÀÄÄÄÄÄÄÄÄÄ	Model type 9

ÀÄÄÄÄÄÄÄÄÄÄÄ	Manufacturer number 19

10.	Information about the check bits shall not be visible.

11.	Manufacturers codes and the security algorithm may can be obtained by writing to the following address:

Mobile Technology Section

Radiocommunications Agency

South Quay Three

189 Marsh Wall

London   E14 9SX