5.		RECEIVER PARAMETERS

The requirements of this section shall be met under normal test conditions, as defined in MPT 1323 section 2.3, unless otherwise specified.

5.1		Frequency Parameters

This section contains the frequency parameters for radio units operating in Band III sub-bands 1 and 2.

All radio units shall operate on any of the channels in the frequency range to which they are directed by network messages and shall examine and if necessary, operate on that channel, in that range, in accordance with the procedures specified in Section 9. If directed by network messages to any channel outside this range, radio units shall not comply with the messages and shall remain  on the channel from which the message was sent.

5.1.1		Channel Spacing and Designations for Sub-band 1

The channel spacing shall be 12.5 kHz. The channel designations shall be as follows:

| Channel Number | Binary representation of channel number * | Radio Unit Receive Frequency |
| --- | --- | --- |
| 1 |  | 176.5000 MHz |
| 58 | 1000000001 | 177.2125 MHz |
| 560 | 1111110111 | 183.4875 MHz |

* Applicable to CHAN, CONT and CHAN4 fields

The radio equipment shall receive over channels 58 to 560 (177.2125 MHz to 183.4875 MHz) inclusively.

5.1.2		Channel spacing and designations for Sub-band 2

The channel spacing shall be 12.5 kHz. The channel designations shall be as follows:

|  | Binary representation of channel number* | Radio Unit Receive Frequency |
| --- | --- | --- |
| 1 |  | 200.5000 MHz |
| 58 | 0000000001 | 201.2125 MHz |
| 560 | 0111110111 | 207.4875 MHz |

* Applicable to CHAN, CONT and CHAN4 fields

The radio equipment shall receive over channels 58 to 560 (201.2125 MHz to 207.4875 MHz) inclusively.

5.1.3		Frequency Tolerance

The centre frequency of the response of the radio receiver shall be within 1.5 kHz of the nominal receive frequency except during channel switching.

This tolerance includes crystal ageing and shall be maintained under both normal and extreme temperature conditions, as defined in MPT 1323 sections 2.3.1 and 32.4.1, for a period of at least one year, without re-adjustment.

5.1.4		Channel Switching

The radio equipment shall meet the channel switching time requirements of MPT 1327.  (See MPT 1327 section 6).

5.2		Demodulation Characteristics

5.2.1		Speech Signals

The speech demodulator shall have a phase demodulation response characteristic.  The permitted departure from the phase demodulation response amplitude characteristic over the audio frequency range 300 Hz to 2.55 kHz shall be ±3 dB, ie the total audio output power shall fit totally within the mask of figure 5.1.  The radio unit shall be deemed to have passed this requirement if it meets the permitted departure either at the electrical connections to the loudspeaker (with a suitable resistive load if the loudspeaker is disconnected), or acoustically.

µ §

Figure 5.1

5.2.2		Data Signals

Received standard data signals have a fast frequency shift keying (FFSK) characteristic.  Standard data modulation follows the requirements of MPT 1323, that is phase continuous 1200 baud FFSK, with 1800 Hz representing binary '0' and 1200 Hz representing binary '1'.  Units shall meet the error performance requirements of Appendix A of this specification.

Note:	The received peak deviation of standard data does not exceed ±2.0 kHz and is not less than ±1.0 kHz.  For non-prescribed data, the peak frequency deviation does not exceed ±2.5 kHz.

5.3		Muting

The audio (other than confidence indications) normally heard by the user shall be muted when one of the following conditions is met:

(a)	If the radio unit is not active on a traffic channel.

(b)	If a call is set up as a result of the receipt of a GTC message (see MPT 1327 section 5.4) with the 'D' bit set to '1' (ie the call is a non-prescribed data call).

(c)	If a call is set up as a result of the receipt of a GTT message (ie the call is a standard data call, see MPT 1327 section 5.7.3).

5.4		Performance Parameters

Radio units shall meet the requirements of MPT 1323.