3.		GENERAL

3.1		Definitions

Section numbers appearing between !!  !! delimiters in this specification refer to section numbers within MPT 1327.

Words appearing between asterisks in this section are defined terms.

Acquisition: The condition obtained by the successful completion of the *control channel acquisition procedure* for a control channel which allows a radio unit to transmit on that channel.

Acquisition Authorisation Data:  *Network personalisation* data which enables the *radio unit* to determine by examination of the relevant sub-fields within a *system identity code* received on a *control channel* whether the *radio unit* is authorised to access on that *control channel*.

Active on a Channel:  A *radio unit* is *active on a channel* when, on that channel, it is enabled to respond to *messages* addressed to it, or is transmitting, or is in transition between the two states.

Note:	a *radio unit* becomes active on an assigned *traffic channel* as soon as it can receive on that channel, whereas, on a *control channel* it shall not become active until it has received a codeword containing an appropriate *system identity code*.

Address: A 20-bit number by which a unit or group of units is known within a *system*.  The *address* comprises two *fields*; a 7-bit *prefix* and a 13-bit *ident*.

Address Codeword: A 64-bit codeword, conforming to the requirements of MPT 1327, where the first bit is set to '1'.  An *address codeword* is always the first codeword in any *message*, and defines the nature of the *message*.

Available for Customisation:  If the *trunking system controller* implements such a customised function, then if the *radio unit* implements the function it shall respond in the manner specified by the network operator of that *trunking system controller*.  Such functions will not modify existing standardised functions.  If the radio unit does not understand the customised function in the context of a *system* it is currently using, then it shall ingnore that function.  The *radio unit* shall not infringe any of the requirements of section 5 of MPT 1327.

Base Station: The entirety of transmitters and receivers operated by a *trunking system controller* at any one site.

Call: A complete information exchange between two or more *parties* which includes one or more *transactions* and may include direct user-to-user communication on a *traffic channel*.

Called Unit (or Group): The unit, or group of units, which a *calling unit* identifies as the desired recipient(s) of a *call*.  The *called unit (or group)* retains this designation for the duration of the *call* and this convention is used in messages relating to that particular *call* irrespective of the origin of such *messages*.

Calling Unit: A *radio unit* or *line unit* which requests a *call*.  The *calling unit* retains this designation for the duration of a *call* and this convention is used in *messages* relating to that particular *call* irrespective of the origins of such *messages*.

Common Prefix Call:  A *call* where the values of the *prefixes* in the calling and called *addresses* are the same.  *Common prefix calls* use *short addressing* procedures.

Confirmation:  A procedure employed by the *radio unit* to facilitate the selection of an appropriate *control channel* to allow a *session* with a *system* to be initiated or continued.  The *confirmation* process consists of applying prescribed tests to the signal content of the received *forward channel*.

Control Category:  A designation given to the *radio unit* during *network personalisation* which governs that unit's right of access to *control channels* radiated by that *network*.  A *radio unit* shall only access a *control channel* when the value of the LAB field in the *system identity code* indicates that the unit's *control category* is permitted to use that channel.

Control Channel:  A *forward channel* and *return channel* being used for the transmission of *messages* conforming to MPT 1327 with the primary purpose of enabling the *trunking system controller* to control *radio units*.

Control Channel Acquisition Procedure:  A procedure employed by the *radio unit* to select an appropriate *control channel* to allow a *session* with a *system* to be initiated or continued.  The procedure consists of *hunting* for candidate *control channels* which are submitted to *confirmation* tests to determine if *acquisition* is permitted.  *Hunting* continues until such time as a successful *acquisition* is confirmed.

Data Codeword: A 64-bit codeword, conforming to the requirements of MPT 1327, where the first bit is set to '0'.  *Data codewords* are concatenated to an *address codeword* and supplement the information in the *address codeword*.

Decodable: A transmitted codeword shall be considered *decodable* if, after receipt, and after error correction (see Section 11.3.2.3) has been applied, a valid codeword from the (64,48) code defined in section 3.2.3 of MPT 1327 is formed.

Diversion: A procedure whereby a *party* may request that future *calls* to a particular called *address* be directed to an alternative destination.

Explicit Registration: A *registration* achieved by means of an RQR message.

Extended Addressing: A method which allows called *party* details to be conveyed to the *trunking system controller* when the *call* details cannot be accommodated in a single *address codeword*.  These called-party details may be an *address* or addressing information in a different form (eg PSTN dialling digits).

Fall-back Mode: A customised form of operation which may be used by a *network* suffering equipment malfunction.

Field: A number of contiguous bits in a codeword which is specified in terms of the position within the codeword and the number of bits.

Fixed Radio Unit: A *radio unit* used with its antenna at a fixed location.

Forward Channel: A radio bearer where the direction of transmission is from the *trunking system controller* to the *radio units*.

Free Format Data: Data within a codeword which, in this specification, is constrained only by its position and length.

Gateway: A *special ident* which is used to identify a *message* relating to a *call or transaction* to or from a communications service outside the *system* (eg the PSTN).  For the purpose of this specification the interprefix *ident*, IPFIXI, is also regarded as a *gateway*.

Group Address: An *address* which is common to more than one unit and which, when nominated as the called *address*, signifies a *group call*.  Units may be assigned any practicable number of *group addresses*.

Group Call: A *call* in which a *group address* is specified as the called *party* and, accordingly, provides a means of communication between more than two units.  The calling *party* in a *group call* may opt for a conversational mode, where all *parties* are able to speak, or for an announcement mode where only the caller may speak.

Head Message: Single transmission of an MPT1327 HEAD *address codeword* and its appended *data codewords*.

Hunting: A procedure employed by the *radio unit* to facilitate the selection of an appropriate *control channel* to allow a *session* with a *system* to be initiated or continued.  The *hunting* process consists of systematic sampling of the *forward channel* frequencies by the *radio unit* until it selects and *confirms* one of the *forward channels*, which together with its paired *return channel* is an appropriate *control channel*.

Hunting Sequence: A procedure employed by the *radio unit* during *hunting* which consists of one or more *hunting stages*.  There are five described *hunting sequences*:

(a)  "Resuming a control channel sequence"

(b)  "Single channel hunt sequence"

(c)  "Preferential hunt sequence"

(d)  "Normal hunt sequence"

(e)  "Comprehensive hunt sequence"

A *hunting sequence* is completed when all *hunting stages* appropriate to that *hunting sequence* have been completed or, prior to this, if a successful *control channel* *confirmation* is achieved.

Hunting Stage: A procedure employed by the *radio unit* during *hunting* which consists of sampling all channel numbers applicable to the type of *hunting sequence* in progress at a particular acquisition threshold level.  A *hunting stage* is completed when all channel numbers applicable have been sampled and *confirmation* attempted on any candidate *control channels* located or may be completed prior to this if a successful *control channel* *confirmation* is achieved.

Ident: A 13-bit number used for identification purposes.  Values of *ident* between 1 and 8100 inclusive are assigned to individual units or groups (see section 8.2), in which case they are associated with a *prefix* to form a 20-bit address.  Values of *ident* above 8100 are designated *special idents* and these are not associated with any particular *prefix*, nor is the *ident* value 0 (DUMMYI).

Idle State: A *radio unit* is in the *idle state* on a *system* when it is *active on a channel*, is receiving on an appropriate *control channel* from the *system*, is not currently within a *message* exchange and has no current *message* transfer requirement.

Implicit Registration: A *registration* achieved by means other than an RQR message and associated Acknowledgement message from the *network*.

Include: A procedure whereby *parties* may be introduced into a *call* in progress at the request of an existing *party* to the *call*.

Individual Address: An *address* by which a single unit is known within a *system*, allowing that unit to be uniquely addressed by that *system*.  Units may be assigned any practicable number of *individual addresses* provided that at least one is assigned to each unit.

Individual Call: A *call* between a calling *party* and a single called *party*.

Interprefix Call: A *call* where the values of the *prefixes* in the calling and called *addresses* are different.  *Interprefix calls* require *extended addressing* procedures.

Item: A complete user transmission on a traffic channel by one *party* within a *call* at the conclusion of which that *party* rests from transmission.  It is possible for a *call* to contain only one *item*.

Line Unit (LU): A user station which is allocated an *individual address* and is directly connected to a *trunking system controller* via a medium other than that part of the radio spectrum to which this specification applies.

Mandatory: The *radio unit* shall implement the function or facility.

Message: A single contiguous data transmission which consists of a codeword synchronisation sequence, an *address codeword* and (optionally) one or more *data codewords* conforming to MPT 1327.

Network: Interconnected *systems*.

Network Personalisation: A procedure undertaken when a subscription to *network* is taken out by a user whereby that user's *radio units* are implanted with data to enable it to function correctly on that *network*.  *Radio units* which are required to access more than one *network* shall be personalised for each *network* to be accessed.

Non-dedicated: A procedure which may be adopted by a *trunking system controller* whereby channels used by the *system* may be flexibly allocated as *control channels* or *traffic channels* depending on the traffic load on the *system*.  When this procedure is employed, the current *control channel* would normally be allocated as a *traffic channel* when all other channels are in use for traffic and the *control channel* facility restored when possible, eg by allocating the next *traffic channel* to become free as a new *control channel*.

Non-prescribed data: Any data traffic which does not conform to the data protocols defined in MPT 1327.

Normal Operation Mode: The operation mode of a *radio unit* or *control channel* which is not in the *fall-back mode*.

Normal Registration Mode: A *control channel* mode in which the *network* will accept and maintain *registration* requests.

Normal Registration Record: A *registration* record which has an indication that the *network* is maintaining the *registration* record.

Null Registration Record: A *registration* record which includes a NULL indicator together with the AREA code.

Party: A source and/or recipient of information within a *call*.  The term includes the totality of equipment at the user station and, where the context permits, the equipment user.  A *party* may be an individual or a group.

Personality: The totality of *network personalisation* data entered into the *radio unit*.

Portable radio unit: A portable radio unit is distinguished from any other radio unit by the following criteria:-

-	the equipment covered is intended to be used only with an integral antenna, which may be detachable. When this equipment is used with a non-integral antenna it ceases to be defined as a portable radio unit for the purpose of this specification;

-	the maximum effective radiated power shall not exceed 5 watts;

-	the equipment shall be powered by a self contained battery unit.

Prefix: The 7 most significant bits of an *address*.  Normally units within a fleet will be allocated the same *prefix* since *calls* between units and groups with the same *prefix* can be made without the use of *extended addressing* procedures.  A *prefix* is only relevant to *individual addresses* and *group addresses*.

Prime Registration Record: The most recently created *registration* record held by the *radio unit*.  Note that the *prime registration record* does not include timing information.

Radio Unit (RU): A mobile or other user station contacting a *system* by normal land mobile radio in accordance with the specification.

Random Access Attempt: The method by which a *radio unit* transmits an unsolicited *message* to the *trunking system controller* on a *control channel*.  The method requires that a *radio unit* repeats a random access *message* if a response *message* is not received within a designated waiting time.  Further repeats are required, in the absence of an appropriate acknowledgement, until a designated number of repeats is reached.  In this specification a *random access attempt* covers the period from initiation of the *transaction* to the receipt of an appropriate acknowledgement or the expiry of a timeout.

Read-only Memory: A storage facility within the *radio unit* which contains *network personalisation* data.  The contents of the *read-only memory* cannot be modified or added to by the action of the *radio unit* or its user.  It is intended that the contents of *read-only memory* are placed there only during *network personalisation* by a suitably authorised agent (normally a service provider).

Read/Write Memory: a storage facility within the *radio unit* the contents of which may be modified by the *radio unit*.

Ready for Communication Control (RFCC): A device or system to inform a unit of the user's readiness to communicate (see section 8.3).

Registration: A procedure which confirms that a *radio unit* is in a *session* on a *system*.  The *registration* procedures may be initiated by a demand from the *trunking system controller*,  or at the initiative of the *radio unit*, depending on the circumstances of the registration.

Requested Unit (or Group): A unit, or group of units, which takes part in a *transaction* initiated by the *trunking system controller* or another *party*.

Requesting Unit: A *radio unit* or *line unit* which initiates a *transaction* with the *trunking system controller* or another *party*, via the *trunking system controller*.

Reserved: Codewords and *fields* which are designated as *reserved* in MPT 1327 are intended for future phases of standardisation and shall not be used in the interim for the conveyance of information.  *Reserved fields* must be set to the default value specified in MPT 1327.

Return Channel: A radio bearer where the direction of transmission is from *radio units* to the *trunking system controller*.

Segment: For a known *transaction*, the portion of the *T-message* contained in a *HEAD message*.

Selected Network:  The *network* which the user of the *radio unit* has selected for the time being and which is the only *network* that the *radio unit* may attempt to access until such time as the user designates a new *selected network*.  Where the *radio unit* does not allow the user to select from a choice of *networks* then only one *network* shall be available to that *radio unit* for access and this *network* shall be, per se, the *selected network*.

Session: A *session* is a period of operation associated with one *system*.  A *session* on a *system* starts when a *radio unit* becomes *active on a control channel* of that *system*, either after switch on or after being *active on a control channel* of a different *system*.  A *session* ends either when the *radio unit* is switched off or when it starts its next *session*.

Short Addressing: The method used when the *parties* to a *call* can be completely specified by a single *prefix* and two *idents*.  This form of addressing minimises the signalling required.

Short Data: A procedure which allows a data *message* to be exchanged between *parties* or between *parties* and the *trunking system controller*.  This procedure does not support *messages* which include more than four *data codewords*.

Short-Form PSTN Destination: A called PSTN *party* previously agreed between the system operator and the user of the *calling unit* which can be specified by a *special ident* rather than the full stream of dialling digits representing the directory number.

Single Segment Transaction (SST): Short data *transaction* requiring only one *segment*.

Spare: Codewords and *fields* which are designated *spare* are available for free use by *systems* (ie *system* customisation) provided that the conditions of MPT 1327 are not infringed.  The use of *spare* codewords and *fields* may vary from *system* to *system*.

Special Ident: An *ident* with a value greater than 8100.  These *idents* are used for a variety of special purposes.  Some of these are specified in MPT 1327, others may be nominated by system operators.  *Special idents* are not associated with a *prefix* to form an *address*.

Speech Call: A *call* as a result of the receipt of a GTC *message* with the 'D' *field* set to '0'.  The audio is not muted.

Standard Data: The procedure by which information exchange takes place using the data protocol defined in section 17 of MPT 1327.

Standard Option: Any optional feature for which the implementation has been standardised in MPT 1327.  Any unit implementing such an option shall implement it at least in the specified standardised manner.  Units and *systems* shall not infringe any of the standardised formats in MPT 1327 when implementing customised features.

System: The totality of equipment required to provide the communication facilities at one location, associated with one or more *system identity codes*.  *Systems* may be combined to form larger communications facilities.

System Identity Code: A 15-bit number which identifies a *system*.  This code is radiated on each *forward control channel* within the *system* (in the SYS *field*).

T-message: totality of *free-format data* which is intended to be conveyed in a *short data* *transaction*.

Temporary Registration Mode: A *control channel* mode in which the *network* will accept but may not maintain *registration* requests.  The *radio unit* may be required to re-register when the *normal registration mode* is resumed.

Temporary Registration Record: A *registration* record which has an indication that the *network* may not be maintaining the *registration* record.

Timed Registration Record: A *registration* record held by the *radio unit*, that has been displaced from the *prime registration record*.  The record has a timer TD associated with it.

Traffic Channel: A *forward channel* and *return channel* being used primarily for user communications.

Transaction: A complete information exchange consisting of one or more *messages* between a *party* and the *trunking system controller*, or another *party*, via the *trunking system controller*.

Trunking System Controller (TSC): The central control intelligence necessary to enable the trunking system to function according to MPT 1327.  The *trunking system controller* may control one or more *base stations*.

Undefined Registration Record: A *registration* record which has no indication that the *network* may or may not be maintaining the *registration* record.

Verified AREA code: The AREA subfield of the received *system identity code* (SYS) used to verify a *control channel*.

Verified SIL code: The SIL field of the *system identity code* (SYS) received and verified by a *radio unit* from the *control channel* on which that *radio unit* was most recently *active*.

3.2		Test Conditions

Unless otherwise specified, the *radio unit* shall meet the requirements of this specification under extreme test conditions, as defined in MPT 1323 section 2.4

3.3		General Requirements

The *radio unit* shall not operate duplex.

The facility to switch from one *network* to another is a *standard option*.

If implemented, it is *mandatory* that *network* switching shall be under control of the user.  Autonomous switching between *networks* by the *radio unit* shall not be permitted.

It shall be possible to transfer, between *network(s)* in a *radio unit* by changing the *personality* (see section 6).