6.		STORAGE REQUIREMENTS

6.1		Introduction

This section tabulates data and parameters which radio units are required to store, either temporarily or long-term, in order to carry out functions defined in this specification.

The stored data and parameters may be gleaned by the radio unit from a number of sources:

-	by network personalisation; examples of such data are the radio unit's individual address and operating parameters which define the required mode of operation;

-	data implanted into the unit by the manufacturer; this method is specific to the radio unit security number (see section 7);

-	data gleaned from operation on the selected network; examples of such data are the records of registrations following a registration attempt by the radio unit on an acquired control channel, or data obtained from BCAST messages received from the selected network.

This section is a compilation of the stipulations expressed in other areas of this specification or in MPT 1327 and reference to these should be made for a precise definition of storage requirements.

Radio units which provide the facility for the user to switch between selected networks shall maintain sufficient storage for the requirements herein to be met for each network, save that stored data which is not required to be preserved after a change of selected network may be accommodated in common storage areas which may be sufficient for only one network.

The requirement on radio units to store data depends upon the facility for which the data is required.  For facilities which are mandatory in this specification, the storage of necessary data is also mandatory and all radio units shall provide storage for data identified as mandatory.  For facilities which are optional but must be carried out in a prescribed manner, the storage of applicable data is mandatory only for radio units which employ the option.  Storage of such data is regarded as a standard option.  Some facilities are optional and are not fully prescribed.  Radio units which employ such options, may choose to follow the procedure identified in this section for the storage of applicable data.  Such storage is regarded as optional.

Where parameters relate to a requirement for the radio unit to measure elapsed time (the parameters T-), the radio unit shall be capable of implementing such measurements to a tolerance of ±10%.

6.2		Types of Memory

The storage requirements summarised in this section include a stipulation of the type of memory applicable to each item of data.  In some cases variation is allowed, and this has been indicated.  Four categories of memory are designated:

Type A - Read only memory which may be set by an external agency but not by the action of the radio unit.  The medium of this storage must be such that the data may be reset by an external agency as required to accommodate changes in network subscription details (including changes in network allegiances).

Type B - Read/Write memory which shall be protected from the effects of switching off the radio unit for a period of at least 120 hours or disconnecting the external source of supply to the radio unit for a maximum period of 5 seconds. It is permissible for data to be held in unprotected memory whilst operational and transferred to protected memory on power down or equivalent. The radio unit shall discard any data held in protected memory unless its validity is reasonably assured.

Type C - Read/Write memory the contents of which may be discarded at some time between switching off the radio unit and being made ready for service after being switched on subsequently.

Type D - Read only memory, protected by the security measures specified in Section 7.

6.3		Summary of Storage Requirements

The radio unit storage requirements are summarised in Table 6.1.  The columns of this table contains the following information:

Column 1 - a serial item number which has no relevance other than allowing ease of reference to items in the tabulation.

Column 2 - a brief description of the data required to be stored.  For a more precise definition of the data, reference should be made to the area of the specification indicated in Column 8.

Column 3 - the storage category to be used.  Type A, B, C or D.

Column 4 - the data item size in bits where this is specified.  If not specified, the range of values to be accommodated and granulation, where appropriate, are given.  FLAG indicates that one of two possible states must be recorded.  Where a range of values and, where appropriate, a granulation are stated these shall be accommodated by the radio unit in its storage allocation.  Where the size of the parameter is stated (eg 13 bits), this is provided for information only and not intended as a limitation on the form in which the radio unit stores the date.

Column 5 - the number of individual records which shall be accommodated.  In some cases the number given is indicated as a maximum or minimum value.

Column 6 - the source of the data.  NP indicates network personalisation data.  Manufacturer indicates data implanted into the unit by the manufacturer.  Operation indicates data gleaned from operation on the selected network.

Column 7 - whether the storage requirement is mandatory (M), a standard option (SO), optional (O), mandatory for units implementing the keypad scheme in section 8.2 (MK) or an option for units implementing the keypad scheme (OK).

Column 8 - comments for clarification.

Column 9 - the area in MPT 1343 or !!MPT 1327!! where the requirement is defined.

| 1 | Own Prefix | A | 7 bits | 1 | NP | M |  | !!4!! |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | Own Individual Ident | A | 13 bits | 1 | NP | M |  | !!4!! |
| 3 | Own group address | A | 20 bits | 4 Min | NP | M | Incoming Group Calls Responded to | !!4!! |
| 4 | Individual Base Ident | A | 13 bits | 1 | NP | MK |  | 8.2.3 |
| 5 | Group Base Ident | A | 13 bits | 1 | NP | MK |  | 8.2.3 |
| 6 | Two or three digit individual calls appropriate to own fleet | A | Flag | 1 | NP | MK | Could be deduced from form of Highest Permitted Own Fleet Individual Ident (item 8), if stored in dialled digit form | 8.2.1 |
| 7 | Two or three digit group calls appropriate to own fleet | A | Flag | 1 | NP | MK | Could be deduced from form of Highest Permitted Own Fleet Group Ident (item 9), if stored in dialled digit form | 8.2.1 |
| 8 | Highest Permitted Own Fleet Individual Ident | A | 13 bits | 1 | NP | MK | Could be stored as dialled number rather than IDENT | 8.2.4 |
| 9 | Highest Permitted Own Fleet Group Ident | A | 13 bits | 1 | NP | MK | Could be stored as dialled number rather than IDENT | 8.2.4 |
| 10 | Single Digit Number Destination | A | Unspecified | 10 Max | NP | OK | Destination applicable to Digit 0-9 | 8.2.4.1 |
| 11 | Look-up table for 5-digit Inter-Fleet Calls | A | Unspecified | Unspecified | NP | OK |  | 8.2.4.5 |
| 12 | Inter-Fleet Group Calls Barred | A | Flag | 1 | NP | MK |  | 8.2.4.8 |
| 13 | *0# and *0nn# Immediate Access | A | 13 bits | 1 or 2 | NP | OK | Prime Dispatcher Address | 8.2.8.2.1 |
| 14 | ** Abbreviated Dialling Maximum Limit | A | 8 bits | 1 | NP | OK |  | 8.2.8.1.1 |
| 15 | *9# Immediate Address | A | Undefined | 1 | NP | OK | Prime Emergency Number | 8.2.8.2.5 |
| 16 | Acquisition Authorisation Data | A | Max 9 bits plus Unspecified Classifier | 8 Min | NP | M |  | 9.3.4.2.3 |
| 17 | Zone sub-field length (LZ) | A | Range 0-9 bits | 1 | NP | M |  | 9.3.4.2.3 |
| 18 | Area sub-field length (LA) | A | Range 0-9 bits | 1 | NP | M |  | 9.3.4.2.3 |
| 19 | Identity Code of Selected Network | A | NET-2 bits | 1 | NP | M | Need to differentiate 2 bit and 7 bit values | 9.3.4.2.3 |
| 20 | Radio Unit Control Category | A | 4 possible Values | 1 | NP | M | LAB field choice | 9.5.3 |
| 21 | Normal Hunt Channel Numbers | A | 10 bits + TS Flag | 32 | NP | M |  | 9.2.1 |
| 22 | Size of Normal Hunt | A | Range 1 to 32 | 1 | NP | M | Could be achieved by Null Values of Channel Numbers | 9.2.1 |
| 23 | Lowest Channel No. in Network | A | 10 bits | 1 | NP | M |  | 9.2.1 |
| 24 | Highest Channel No. in Network | A | 10 bits | 1 | NP | M |  | 9.2.1 |
| 25 | Non-Applicable Channel Numbers | A | 10 bits | Unspecified | NP | O | Channels which may be omitted in Comprehensive Hunt | 9.3.3.5 |
| 26 | Last Active Control Channel - Channel Number | C | 10 bits | 1 | Operation | M |  | 9.2.1 |
| 27 | Last Active Control Channel - SYS Code used for Confirmation | C | 12 bits | 1 | Operation | M |  | 9.4.1 |
| 28 | Add Channel Nos. to Normal Hunt | C | 10 bits + TS Flag | Unspecified | Operation | SO | Compiled from BCAST SYSDEF = '00000' | 9.2.2 |
| 29 | Subtract Channel Nos. from Normal Hunt | C | 10 bits | Unspecified | Operation | SO | Compiled from BCAST SYSDEF = '00001' | 9.2.2 |
| 30 | Adjacent Site Information | C | 10 bits + Unspecified | 15 | Operation | O | Use of BCAST SYSDEF '00100' and '00101' Unspecified | 9.2.2 |
| 31 | Suppress Comprehensive Hunt | A | Flag | 1 | NP | M |  | 9.3.3.5 |
| 32 | Timed Registration Record - AREA Code | C | Range 0-9 bits plus NULL Flag | 2 | Operation | SO |  | 10.3.1 |
| 33 | Timed Registration Record - Channel Number | C | 10 bits | 2 | Operation | SO |  | 10.3.1 |
| 34 | Timed Registration Record - Registration Indication | C | 2 bits | 2 | Operation | SO |  | 10.3.1 |
| 35 | Prime Registration Record - AREA Code | B | Range 0-9 bits plus NULL Flag | 1 | Operation | M |  | 10.2.1 and 10.3.1 |
| 36 | Prime Registration Record - Channel Number | B | 10 bits | 1 | Operation | SO |  | 10.2.1 and 10.3.1 |
| 37 | Prime Registration Record - Registration Indicator | B | 2 bits | 2 | Operation | M |  | 10.2.1 and 10.3.1 |
| 38 | Denied Registration - AREA Code | C | Range 0-9 bits | 8 Min | Operation | M | FIFO Store | 10.2.1 and 10.3.1 |
| 39 | REG - Temporary or Normal Registration Indicator | B | 1 bit (plus "undefined" state indicator) | 1 | Operation | M | Received in BCAST SYSDEF '00011' | 10.2.1 and 10.3.1 |
| 40 | Value of INFO to be used in RQR | A | 15 bits | 1 | NP | M | Default to all zeros | !!8.2.2.2!! |
| 41 | Channel Number for Fall-Back Service | A | 10 bits | 1 | NP | SO | If set to zero disables fall-back | 13.2 |
| 42 | Value of NDD in SYS Code for Fall-Back Service | A | 4 bits or 9 bits | 1 | NP | SO |  | 13.2 |
| 43 | Security Serial Number | D | 38 bits | 1 | Manufacturer | M |  | 7 |
| 44a | LM1 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 44b | LM2 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 44c | LM3 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 44d | LM4 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 45 | LM2 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 46 | LM3 - Level margin | A | 0-40 dB in steps of 6 dB | 1 | NP | SO | Tolerance ±6 dB | 9.3.3.7.6 |
| 47 | NA - Number of Multiple Registrations | B | Range 1 to 3 in steps of 1 | 1 | Operation | SO | Received in BCAST SYSDEF '00011' | 10.3.1 |
| 48 | NC1 - Size of Error Check Sample prior to confirmation | A | 0-255 Codewords in steps of 1 | 2 | NP | M | Different Values for Time-shared and continuous channels. Maximum value may be reduced on review. | 9.3.4.3 |
| 49 | NC2 - Size of Error Check Sample after confirmation | A | 0-255 Codewords in steps of 1 | 2 | NP | M | Different Value for Time-shared and continuous channels. Maximum value may be reduced on review. | 9.3.4.3 |
| 50 | NT - Maximum TSC response delay to unsolicited traffic channel message | A | 103-1236 bit periods in steps of 103 bit periods | 1 | NP | SO |  | !!Appendix 1!! |
| 51 | NV - Number of consecutive CCSCs to select a value of SYS for verification | A | 1-16 in steps of 1 | 2 | NP | M | Different Value for Time-shared and continuous channels | 9.3.4.2.1 |
| 52 | NX1 - Error Codewords Limit prior to confirmation | A | 0-255 Codewords in steps of 1 | 2 | NP | M | Different Value for Time-shared and continuous channels. Maximum value may be reduced on review. | 9.3.4.3 |
| 53 | NX2 - Error Codewords Limit after confirmation | A | 0-255 Codewords in steps of 1 | 2 | NP | M | Different Value for Time-shared and continuous channels. Maximum value may be reduced on review. | 9.3.4.3 |
| 54 | NZ1 - Number of contiguous error check samples containing no error events | A | 1-255 Samples | 1 | NP | M | Maximum value may be reduced on review | 9.3.4.4 |
| 55 | NZ2 - Number of contiguous error check samples each generating a codeword error event following an initial error event | A | 1-255 Samples | 1 | NP | M | Maximum value may be reduced on review | 9.4 |
| 56 | NPON - Number of Pressel On messages on traffic channel | A | 1-5 | 1 | NP | SO |  | !!9.2.3.1!! |
| 57 | NPOFF - Number of Pressel Off messages on traffic channel | A | 1-5 | 1 | NP | SO |  | !!9.2.3.1!! |
| 58 | NS - Number of samples | A | 1-10 in steps of 1 | 1 | NP | SO |  | 9.3.3.7.6 |
| 59 | NDD preference data | A | Maximum 9 bits | 4 min. | NP | SO |  | 9.3.4.2.3 |
| 60 | PREFERRED NDD sub-field length | A | Range 0-9 bits | 4 min. | NP | SO |  | 9.3.4.2.3 |
| 61 | PREFERRED NDD priority indicator | A | 1-10 in steps of 1 | 4 min. | NP | SO |  | 9.3.4.2.3 |
| 62 | TA - Timeout for called radio unit after receiving AHY | A | 10-120 Sec. steps OR fixed at 60 Secs. | 1 | NP | M | Standard option that TA is variable by network personalisation | Appendix B !!Appendix 1!! |
| 63 | TH - Sampling activity duration | A | 0-100 slots in 1 slot steps | 1 | NP | SO |  | 9.3.3.7.3 |
| 64 | TL - Sampling activity interval | A | 0-3000 slots in 100 slot steps or 0-320 seconds in 10 second steps | 1 | NP | SO |  | 9.3.3.7.1 |
| 65 | SIL System Indicator Locator sub-field length | A | Range 3-9 bits in Steps of 1 | 1 | NP | SO |  | 9.3.4.2.2 |
| 66 | TC - Random Access Timeout | A | 0-120 Secs. in 10 Sec. Steps | 1 | NP | M |  | Appendix B !!Appendix 1!! |
| 67 | TD - Registration Record Timeout | A | 0-70 Minutes in 5 Minute Steps | 1 | NP | SO |  | 10.3.1 |
| 68 | TGI - Individual Incoming Short Data Timer | A | 1-15 Secs. in 1 Sec. Steps | 1 | NP | SO |  | 14.4.2.1.3 |
| 69 | TGG - Group Incoming Short Data Timer | A | 1-30 Secs. in 1 Sec. Steps | 1 | NP | SO |  | 14.4.2.2.2 |
| 70 | TJ - Further Signalling Timeout | A | 0-60 Secs. in 10 Sec. Steps | 1 | NP | M |  | Appendix B !!Appendix 1!! |
| 71 | TN - Traffic Channel Timeout | A | 0-10 Secs. in 1 Sec. Steps | 1 | NP | M |  | Appendix B !!Appendix 1!! |
| 72 | TS - Delay before leaving a Control Channel | A | 0-10 Secs. in 1 Sec. Steps | 1 | NP | M |  | Appendix B !!Appendix 1!! |
| 73 | TT - Maximum Item Duration | A | 0-60 Secs. in 10 Sec. Steps | 1 | NP | M |  | Appendix B !!Appendix 1!! |
| 74 | TU - Data Call Duration Timer | A | 30 Secs.-3 Minutes in 30 Sec. Steps | 1 | NP | SO |  | 12.4 |
| 75 | Suppress Data Call Duration Timer | A | Flag | 1 | NP | SO |  | 12.4 |
| 76 | TW - Timeout for Radio Unit waiting for Call | A | 10-120 Secs. in 10 Sec. Steps OR fixed at 60 Secs. | 1 | NP | M | Standard option that TW is variable by network personalisation | Appendix B !!Appendix 1!! |
| 77 | WT - Wait Parameter in Operation | C | 3 bits | 1 | Operation | M | Storage of latest advised WAIT Value | !!7.3.7!! |
| 78 | Latest Value of PER | C | 1 bit | 1 | Operation | M | Received in BCAST SYSDEF '00010' | !!9.2.2.6!! |
| 79 | Latest Value of IVAL | C | 5 bits | 1 | Operation | M | Received in BCAST SYSDEF '00010' | !!9.2.2.6!! |
| 80 | Latest Value of PON | C | 1 bit | 1 | Operation | M | Received in BCAST SYSDEF '00010' | !!9.2.2.6!! |
| 81 | Latest Value of ID | C | 1 bit | 1 | Operation | M | Received in BCAST SYSDEF '00010' | !!9.2.2.6!! |
| 82 | "Home" ZONE | A | Range 0-9 bits | 1 | NP | M |  | 10.2.1 and 10.3.1 |
| 83 | Disable FOASCU | A | Flag | 1 | NP | SO | Default State is Enabled | 11.9.2.2.2 |
| 84 | CLIM - Network personalisation data; maximum call duration for a non-emergency call. | A | 10 seconds to 4 mins. 14 seconds, in 1 sec. steps, or 5 mins. to 13 mins. in 1 min. steps, or inhibited. | 1 | NP | O |  | 11.9.2.3.6 |
| 85 | CLIME - Network personalisation data; maximum call duration for an emergency call. | A | 10 seconds to 4 mins. 14 seconds, in 1 sec. steps, or 5 mins. to 13 mins. in 1 min. steps, or inhibited. | 1 | NP | O |  | 11.9.2.3.6 |
| 86 | TSCLIM - Current traffic channel call duration. | C | 8 bits | 1 | Operation | O | Received in BCAST SYSDEF '00010' | 11.9.2.3.6 |
| 87 | *2 Immediate Access | A | Undefined | 1 | NP | O | Prime Short Data Number | 8.2.8.2.3.2 |
| 88 | TM - Timeout for called radio unit after receiving AHY, IDENT2=SDMI or an MST segment | A | 1-60 Sec. 1 second steps OR fixed at 60 Secs. | 1 | NP | O | Standard option that TM is variable by network personalisation | 14.4.2.1.2.3 |
| 89 | MST Flag | A | 1 bit | 1 | NP | O | Determines whether unit uses original or revised MST procedures | 14.4.2.1 |
| 90 | MST call abandonment (E=1) | A | 1 bit | 1 | NP | O | Causes radio unit to abandon an MST for a non--emergency AHY in mid-call. | 14.4.2.1.2.b) |
| 91 | MST call abandonment (E=0) | A | 1 bit | 1 | NP | O | Causes radio unit to abandon an MST for an emergency AHY in mid-call. | 14.4.2.1.2.b) |
| 92 | TT timeout Disconnect Inhibit | A | Flag | 1 | NP | O |  | 9.2.3.6 B |
| 93 | Dynamic Regroup Prefix/Ident | B | 20 bits | 16 | Operation | O |  | Appendix AN6 |
| 94 | Electronic Security Number Flag | A | Flag | 1 | Operation | SO | Allows ESN check before expecting ACK or ACKX to RQR | 10.2.3 |

The next 7 pages are page footers for pages 4 to 10 (done in landscape) containing table 6.1  Run sheets through printer a second time to print table.