12.		NON STANDARD DATA INTERFACE PROVISION

The provision of a non standard data facility on a radio unit is a standard option.

Within the procedures RQS and RQE with DT=1, provision may be made for connection of external data equipment to radio units for transmission over transparent signalling paths.  The quality of the paths will be determined by individual networks.

12.1		Muting

The equipment shall be constructed so that the data path shall never be disturbed by the squelch for speech reception.  Receiver audio shall be muted during data reception.

12.2		Maladjustment

Those controls which if maladjusted might increase the interference potential of the transceiver shall not be easily accessible to the user.

12.3		Standard Signalling

Whilst in a data call and receiving signals from the TSC the Radio unit shall monitor the channel continuously for messages from the TSC and shall take appropriate action.

The radio unit shall not send call maintenance messages except disconnect messages but may send other call maintenance messages only by pre-arrangement with the network operator.

12.4		Data Call Handling

The radio unit shall incorporate a Data Call Duration Timer TU, and an associated suppression flag as part of its network personalisation.

12.4.1		Call Establishment

Calls are established by the unit receiving a GTC (D=1).  Timer TU shall be started upon first tuning to the traffic channel.

12.4.2		Call Clearance

The radio unit shall send disconnect messages as specified in MPT 1327 section 9.2.3.5,

either:

on expiry of the Data Call Duration Timer,

or:

at end of the data transaction when detected by the radio unit (whereupon TU is de-activated), whichever is earlier.

12.5		Facilities

Equipment that does not integrate the keyboard and display or other means of data entry or retrieval into the transceiver shall provide a suitable interface covering at least:

RX Audio	-	shall not be affected by the radio unit volume control setting

TX Audio	-	at levels that shall not affect the overall deviation requirements.

Keyline	-	form unspecified; the keyline shall be inhibited until at least TR has elapsed from the receipt of GTC (D=1) or on completion of a standard data transaction on the traffic channel which starts within TR and over-runs TR.

In addition the following facilities may be made available:

Data Channel

Ready		-	This signal is generated by the RU shall become active after a period of at least TR has elapsed from the receipt of GTC (D=1) or on completion of a standard data transaction on the traffic channel which starts within TR and over-runs TR.  This signal does not guarantee that an end to end communication path has been established.

Data Equipment

Available	-	This signal is generated by the external data equipment and shall only be active when the data equipment is ready to receive or transmit data.  This signal enables the RU to provide the appropriate control channel signalling for call set-up and rejection for RQS (DT=1), data calls.  AHY (D=1) or GTC (D=1).  If during a data call the Data Equipment Available signal is de-activated, this shall initiate a call clear down.

NOTE

It is recommended that manufacturers of external data equipment should list those Radio Units for which the equipment combination complies with MPT 1323.