11.		CALL PROCESSING

In this section, the section numbers have been formed by prefixing the section numbers of MPT 1327 by "11.".  Thus sub-sections here refer directly to, and should be read in parallel with, sections of MPT 1327.  The requirements of MPT 1327 are also mandatory requirements of this specification.  Each sub-section below defines the requirements for the facilities and functions relevant to the radio unit for call processing.  These requirements are categorised as follows:

Mandatory	The radio unit shall implement the function or facility.

Standard Option	If the radio unit implements the function, then it shall be implemented at least in the specified manner.

Available for	If the TSC implements such a customised function, then if Customisation	the radio unit implements the function it shall operate in the manner  specified by the network operator of that TSC. Such functions will not modify existing standardised functions.  If the radio unit does not understand the customised function in the context of system it is currently using, then it shall ignore that function.  The radio unit shall not infringe any of the requirements of section 5 of MPT 1327.

Optional	The radio unit may use the information or implement the facility at the discretion of the manufacturer.

Informative	The corresponding section within MPT 1327 is primarily informative or related only to TSC specification, with no requirements on the radio unit arising directly from the contents.  Related requirements may be included in other sections, however.

The radio unit shall ignore those fields of received messages which it does not understand.

11.1		Introduction

Informative.

11.1.1		User Facilities

Informative.

11.1.1.1	Types of Call

Informative.

11.1.1.2	Making Calls

It shall be mandatory for the radio unit to be able to make calls to individual radio units and line connected units.  The ability to make calls to the other destinations listed is a standard option.

It shall be mandatory for the radio unit to be able to make interprefix speech calls, although this does not necessarily require prefix number entry by the user (see section 8.2).

The requirements on the radio unit for the use of call progress information sent by the TSC for confidence indication is defined in section 8.1.

11.1.1.3	Receiving Calls

The requirement to respond to correctly addressed incoming calls that are received is mandatory.  This requirement includes interprefix calls.  The response may, however, be a rejection if, for example, a radio unit does not have a particular non-mandatory facility, or if the user has indicated that certain calls are to be rejected.

The implementation of a busy control is a standard option.

The implementation of a call back control is a standard option.

The implementation of a "ready for communication control" (RFCC) is mandatory.

Confidence indication requirements are specified in section 8.1.

11.1.1.4	Diverting Calls

Diversion of calls is a standard option.  Automatic re-dial to the diversion IDENT is optional.

11.1.2		System Features and Facilities

11.1.2.1	System Dimensions

It shall be mandatory for the radio unit to be able to store in its personality any one of the addresses in the full addressing range as its individual address.  In addition it shall be mandatory for the radio unit to be able to store in its personality up to 4 of the addresses in the full addressing range as its group addresses (see section 6).

11.1.2.2	System Control

Informative.  See section 13 of this specification for details of fall-back operation, which is a standard option.

11.1.2.3	Call Handling

Informative.  The requirements for security are defined in section 11.15.

11.1.2.4	Multi-site Systems

Informative.  The registration procedures are defined in Section 10 of this specification.

11.1.3		Guide to Some Key Protocol Aspects

Informative.

11.1.3.1	Control Channel Signalling Structure

Informative.

11.1.3.2	Control Channel Signalling Messages

Informative.

11.1.3.3	Random Access Protocol

11.1.3.3.1	Principle of Operation

Informative.

11.1.3.3.2	Features of the Random Access Protocol

Informative.

11.1.3.4	Addressing

Informative.  The use of extended addressing is mandatory for a radio unit making a simple call to a radio unit or line unit with a different prefix.

11.1.3.5	Examples of Signalling Sequences

Informative.

11.1.3.5.1	Example: Radio Unit Calls a Group

The ability to make group calls is a standard option.

11.1.3.5.2	Example: Radio Unit Calls a Unit with the Same Prefix

The ability for the radio unit to make such calls is mandatory.

11.1.3.5.3	Example: Radio Unit Calls a Unit with a Different Prefix

The ability for the radio unit to make such calls is mandatory.

11.1.3.5.4	Example: Radio Unit Sends a Short Data Message

The ability for the radio unit to make such calls is a standard option.

11.2		Definitions

The definitions listed in section 2 of MPT 1327 are consistent with those listed in section 3 of this specification.

11.3		Signalling Formats

Informative.  The requirements listed in this section apply only to prescribed signalling.  Non-prescribed signalling is not covered by this specification.

11.3.1		Basic Format

Mandatory as specified.

11.3.1.1	LET

Mandatory as specified.

11.3.1.2	Preamble

Mandatory as specified.

11.3.1.3	Message

Mandatory as specified.

11.3.1.4	Hang-over Bit, H

Mandatory as specified.

11.3.2		Message Format

Mandatory as specified.

11.3.2.1	Codeword Synchronisation Sequence

Mandatory as specified.

11.3.2.1.1	Control Channel Codeword Synchronisation Sequence

Mandatory as specified.

11.3.2.1.2	Traffic Channel Codeword Synchronisation Sequence

Mandatory as specified.

11.3.2.2	Codewords

Mandatory as specified.

11.3.2.3	Encoding and Error Checking

The encoding is mandatory as specified.

The radio unit shall not accept any codeword from which the derived syndrome and parity bit indicate that 3 or more hard errors have occurred in an error burst of length 6 bits or greater.

The radio unit shall reject any codeword if it does not correct potential bit errors indicated by the coset leader for the syndrome.

The radio unit need not perform error correction, although error correction may simplify receiver and modem design requirements in meeting the error performance requirements of the receiver specified in Appendix A.

11.3.3		Signalling Transmission Variants

The radio unit shall be designed to cope with the variants specified.

11.3.3.1	Single Message Format

Mandatory as specified.

11.3.3.2	Multiple Message Format on the Traffic Channel

Mandatory as specified.

11.3.3.3	Forward Control Channel Format

11.3.3.3.1	Basic Control Channel Format

Mandatory as specified.

11.3.3.3.2	Data Codeword Displacement

Mandatory as specified.

11.4		Addressing

The radio unit shall understand those special IDENTS and DUMMYI that are required by the mandatory call procedures, and also those required by any standard options that are implemented.

11.5		Codeword Structures

The standardised messages which the radio unit shall understand are defined in the following sections.

Standardised Fields

Mandatory as specified.

Reserved Fields

Mandatory as specified.

Spare Fields and Codewords

Available for Customisation in the manner specified.

11.5.1		System Codewords

11.5.1.1	Control Channel System Codeword (CCSC)

Mandatory to make use of the CCSC for slot synchronisation.

The use of the DCSCU is a standard option.

11.5.1.2	Data Channel System Codeword (DCSC)

The use of the DCSC is a standard option.

11.5.2		General Address Codeword Structures

Informative.

11.5.3		List of Address Codewords

Messages received by the radio unit which it shall understand and take any mandatory action required:

GTC

ALH, ALHS, ALHX, ALHR, ALHF

ACK, ACKI, ACKQ, ACKX, ACKV, ACKT, ACKB

ACKE (if configured to send RQE messages)

AHY, AHYX, AHYQ, AHYC

MAINT, MOVE, CLEAR, BCAST

Messages received by the radio unit which it shall understand and take any mandatory action required if the unit is individually addressed, and for which it is a standard option for the radio unit to use if not individually addressed:

ALHD, ALHE

Messages received by the radio unit which it is a standard option for the radio unit to understand:

AHYD

DACK +DAL, DACK+DALG, DACK+DALN, DACK+GO, DACKD, DACKZ, DAHY, DAHYX, DAHYZ

GTT, MARK, HEAD

RLA, SACK, SITH

Messages for which the functions are not yet defined (informative):

SAMO

Messages which the radio unit shall be required to send by the protocol (mandatory):

ACKI (QUAL = '0')

ACK, ACKX

RQS, RQX, RQR, RQQ (indicating hook status)

MAINT (see section 11.5.5.4.2 for applicable messages)

SAMIS

Messages which the radio unit may send in the standardised format if permitted by the protocol (standard option):

ACKV, ACKB

RQT, RQE, RQC, RQQ (other than for hook status)

DACK+GO, DACKD, DACKZ

DRQG, DRQX, DRQZ, DRUGI

RLA, RQD, SACK, SITH

HEAD

Messages for which the functions are not yet defined (informative):

SAMIU

Reserved and spare message fields shall be as specified.

11.5.4		Go To Traffic Channel Message, GTC

Mandatory as specified.

11.5.5		Category '000' Messages

11.5.5.1	Aloha Messages (Type '00')

It is mandatory that the radio unit shall use the following received messages in making random access attempts:

ALH, ALHS, ALHX, ALHR

It is a standard option that the radio unit shall use the following received messages in random access attempts:

ALHD, ALHE

It is mandatory for the radio unit to understand the ALHF message; it is a standard option for the radio unit to implement fall-back mode (see section 13).

Use of the WT, M, CHAN4 and N fields shall be mandatory when making use of any Aloha message.  The CHAN4 bits correspond to the 4 least significant four bits of the 10 bit binary representation of the channel number of the control channel on which the message was receivedCHAN (see section 5.1).

It is mandatory that the radio unit shall respond to all individually addressed Aloha messages (ie those addressed to that specific unit, see !!7.4.1!!).

11.5.5.2	Acknowledgement Messages (Type '01')

Messages received by the radio unit which it shall understand and take any mandatory action required:

ACK, ACKI, ACKQ, ACKX, ACKV, ACKT, ACKB

Messages received by the radio unit which it shall understand and take any mandatory action required if the radio unit is configured to send RQE messages:

ACKE

Messages which the radio unit shall send where required by the protocol (mandatory):

ACK, ACKX, ACKI (QUAL = '0')

Messages which the radio unit may send in the standardised format if permitted by the protocol (standard option):

ACKV, ACKB

11.5.5.2.1	Acknowledgement Messages Sent by the TSC

Messages received by the radio unit which it shall understand and take any mandatory action required:

ACK, ACKI, ACKQ, ACKX, ACKV, ACKT, ACKB

ACKE (if configured to send RQE messages)

Use of the QUAL field of the received message is a standard option except where:

i.	mandatory confidence indications are required in section 8.1;

ii.	the protocol of MPT 1327 requires a different mandatory action based on the QUAL field.

Use of the diversion address or number of data codewords appended to ACKT (QUAL = '0') is optional. If IDENT1=PSTNGI in ACKT (QUAL='0') then the BCD digits in the appended data codeword (s) shall contain the full dialled string, less the leading 0.

ACKT (QUAL = '1') is reserved.

11.5.5.2.2	Acknowledgements Sent by Radio Units

Messages which the radio unit shall send where required by the protocol (mandatory):

ACK (QUAL = '0' and QUAL = '1'), ACKX (QUAL = '0'), ACKI (QUAL = '0')

Messages which the radio unit may send in the standardised format if permitted by the protocol (standard option):

ACKV (QUAL = '1'), ACKX (QUAL = '1'),

ACKB (QUAL = '0' and QUAL = '1')

11.5.5.3	Type '10' Messages (Requests and Ahoys)

Informative.

11.5.5.3.1	Request Messages (Type '10')

Messages which the radio unit shall send where required by the protocol when making calls (mandatory):

RQS, RQX, RQR

RQQ (for hook status)

Messages which the radio unit may send in the standardised format if permitted by the protocol (standard option):

RQT, RQE, RQC

RQQ (other than hook status)

11.5.5.3.1.1	Request "Simple" Call Message, RQS

Mandatory for RQS containing:

(IDENT1 = Ident or IPFIXI) and

DT = '0' and

LEVEL = '1' and

EXT = '0' and

FLAG1 = '0' and

FLAG2 = '0'

Standard option for other combinations.  For calls other than group calls or a shot addressing call to a PABX extension, FLAG2 is se to zero, unless permittd by the network.

11.5.5.3.1.2	Request Codeword Free for Customisation

Section deleted in MPT 1327.

11.5.5.3.1.3	Call Cancel/Abort Transaction Request Message, RQX

Mandatory as specified.

11.5.5.3.1.4	Request Call Diversion Message, RQT

Standard option.

11.5.5.3.1.5	Request Emergency Call Message, RQE

Standard option.

11.5.5.3.1.6	Request to Register Message, RQR

Mandatory as specified.

The "Info" field shall be set to all zeros, unless permitted otherwise by the network.

11.5.5.3.1.7	Request Status Transaction, RQQ

Mandatory for indication of hook status (see MPT 1327 section 13).  Standard option otherwise.

11.5.5.3.1.8	Request to Transmit Short Data Message, RQC

Standard option.

11.5.5.3.2	Ahoy Messages (Type '10')

Messages received by the radio unit which it shall understand and take any mandatory action required:

AHY, AHYX, AHYQ, AHYC

11.5.5.3.2.1	General Availability Check Message, AHY

It is mandatory that the radio unit responds to an individually addressed AHY message.  Use of the address in any data codeword appended to the AHY is a standard option.

11.5.5.3.2.2	Reserved Section

Informative.

11.5.5.3.2.3	Cancel Alert State Message, AHYX

Mandatory as specified.

11.5.5.3.2.4	Reserved Section

Informative.

11.5.5.3.2.5	Reserved Section

Informative.

11.5.5.3.2.6	Reserved Section

Informative.

11.5.5.3.2.7	Status Ahoy Message, AHYQ

Mandatory as specified.

11.5.5.3.2.8	Short Data Invitation Message, AHYC

Mandatory as specified.

11.5.5.4	Miscellaneous Control Messages (Type '11')

Messages received by the radio unit which it shall understand and take any mandatory action required:

MAINT, MOVE, CLEAR, BCAST

Messages received by the radio unit which it is a standard option for the mobile to understand:

MARK

Messages which the radio unit shall send where required by the protocol (mandatory):

MAINT (see section 11.5.5.4.2 for applicable messages)

11.5.5.4.1	Control Channel Marker, MARK

It is a standard option whether the radio unit makes use of the MARK message.

The CHAN4 bits correspond to the 4 least significant four bits of the 10 bit binary channel representation of the channel number of the control channel on which the message was receivedCHAN (see section 5.1).

11.5.5.4.2	Call Maintenance Message, MAINT

Messages received by the radio unit which it shall understand and take any mandatory action required:

MAINT (OPER = '110', '111')

Messages which the radio unit shall send where required by the protocol (mandatory):

MAINT (OPER = '000', '001', '010', '011')

For OPER = '110' or OPER = '111' the message format is as follows:

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 11 | FUNC 001 | CHAN | OPER | RSVD | STI | SIL3 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 10 | 3 | 1 | 1 | 3 | 16 |

Field definitions are as for MPT 1327 with the addition of:

STI	-	Site Indicator Flag. If non-zero then SIL3 holds the three least significant bits of the SIL sub-field of the system identity code (SYS) currently being propagated by the system originating the message.

SIL3	-	The three least significant bits of the SIL sub-field of the system identity code (SYS) currently being propagated by the system originating the message.

For STI = '0' then SIL3 = RSVD   (RSVD = '000')

It is mandatory for the radio unit to understand and act on the messages:

MAINT (OPER = '110')   (See also 11.9.2.3.7)

MAINT (OPER = '111')   (See also 11.9.2.3.3)

It is mandatory for the radio unit to send these messages where required by the protocol:

MAINT (OPER = '000', '001', '010', '011')

For OPER = '110' or OPER = '111', the STI and SIL3 fields are reserved and shall be set to zero for MAINT messages transmitted by radio units.

11.5.5.4.3	Clear-Down Message, CLEAR

Mandatory as specified.

If the radio unit receives a CLEAR message with the CONT field set to '0000000000' then the radio unit shall either return to the last active control channel, or remain on the nominated fall-back channel if in fall-back mode (see Sections 9 and 13). The message format is as follows:

| 1 | CHAN | CONT | 1 | CAT 000 | TYPE 11 | FUNC 010 | STI | SIL3 | TSI | SPARE | REVS 101010101010 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 10 | 10 | 1 | 3 | 2 | 3 | 1 | 3 | 1 | 1 | 12 | 16 |

Field definitions are as for MPT 1327 with the addition of:

STI	-	Site Indication Flag. If non-zero then SIL3 holds the three least significant bits of the SIL sub-field of the system identity code (SYS) currently being propagated by the system originating the message.

SIL3	-	The three least significant bits of the SIL sub-field of the system identity code appropriate to the system originating the message.

For STI = '0' then SIL3 = RSVD    (RSVD = '000')

TSI	-	Time-shared control channel indicator.  See section 9.

'0'	- 	Time-shared control channels are not expected on channel number CONT.

'1'	- 	Time-shared control channels may be encountered on channel number CONT.

11.5.5.4.4	Move to Control Channel, MOVE

Mandatory as specified.

The radio unit shall enter a "preferential hunt" if it receives a MOVE message with the CONT field set to '0000000000' (see section 9).

The message format is as follows:

| 1 | PFIX | IDENT1 | 1 | CAT 000 | TYPE 11 | FUNC 011 | CONT | (M) | RSVD | TSI | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 7 | 13 | 1 | 3 | 2 | 3 | 10 | 5 | 2 | 1 | 16 |

Field definitions are as for MPT 1327 with the addition of:

TSI	-	Time-shared control channel indicator.  See section 9.

'0'	-	Time-shared control channels are not expected on channel number CONT.

'1'	-	Time-shared control channels may be encountered on channel number CONT.

11.5.5.4.5	Broadcast Message, BCAST

Mandatory for the following SYSDEF value:

'00010'	Specify call maintenance parameters

'00011'	Specify registration parameters

Standard option for the following SYSDEF values:

'00000'	Announce control channel

'00001'	Withdraw control channel

'00100'	Broadcast adjacent site control channel number

'00101'	Vote now advice

11.5.5.4.5(a)		Announce Control Channel (SYSDEF = '00000')

Standard option.

The message format is as follows:

| 1 | SYSDEF 00000 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | TSI | SPARE | RSVD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 1 | 1 | 6 | 16 |

Field definitions are as for MPT 1327 with the addition of:

TSI	-	Time-shared control channel indicator.  See section 9.

'0'	-	Time-shared control channels are not expected on channel number CHAN.

'1'	-	Time-shared control channels may be encountered on channel number CHAN.

11.5.5.4.5(b)		Withdraw Control Channel (SYSDEF = '00001')

Standard option.

11.5.5.4.5(c)		Specify Call Maintenance Parameters (SYSDEF = '00010')

The message format is as follows:

| 1 | SYSDEF 00010 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | PER | IVAL | PON | ID | RSVD | TSCLIM | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 1 | 5 | 1 | 1 | 2 | 8 | 16 |

All facilities listed in MPT1327 are mandatory. Field definitions are as for MPT1327 with the addition of:

TSCLIM	-	Specifies the maximum traffic channel call duration for non-emergency calls.

The binary values used are as follows:

'00000000': Duration to be CLIM

'00000001 to '00001001': Duration to be the decimal equivalent of the binary field value in minutes plus 4 minutes.  '00001010' to '11111110': Duration to be the decimal equivalent of the binary field value in seconds.

'11111111': Call timer infinite.

11.5.5.4.5(d)		Specify Registration Parameters (SYSDEF = '00011')

It is mandatory for the radio unit to understand and act on this message.

The message format is as follows:

| 1 | SYSDEF 00011 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | RSVD | NA | REG | SPARE | RFFD | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 4 | 2 | 1 | 5 | 6 | 16 |

Field definitions are as for MPT 1327 with the addition of:

NA	-	Specifies the maximum number of registration records which a radio unit shall store (see section 10):

'00'	reserved for future definition in MPT 1343

'01'	one registration record

'10'	two registration records

'11'	three registration records

REG	-	Specifies registration mode (see section 10):

'0' normal

'1' temporary

RFFD	-	Reserved for future definition in MPT 1343.

Default value = '000000'.

11.5.5.4.5(e)	Broadcast Adjacent Site Control Channel Number (SYSDEF = '00100')

Standard option.

The message format is as follows:

| 1 | SYSDEF 00100 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | TSI | SPARE | RSVD | ADJSITE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 1 | 1 | 2 | 4 | 16 |

Field definitions are as for MPT 1327 with the addition of:

TSI	-	Time-shared control channel indicator.  See section 9.

'0'	-	Time-shared control channels are not expected on channel number CHAN.

'1'	-	Time-shared control channels may be encountered on channel number CHAN.

11.5.5.4.5(f)		Vote Now Advice (SYSDEF = '00101')

Standard option.

| 1 | SYSDEF 00101 | SYS | 1 | CAT 000 | TYPE 11 | FUNC 100 | CHAN | TSI | SPARE | RSVD | ADJSITE | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 15 | 1 | 3 | 2 | 3 | 10 | 1 | 1 | 2 | 4 | 16 |

Field definitions are as for MPT 1327 with the addition of:

TSI	-	Time-shared control channel indicator.  See section 9.

'0'	-	Time-shared control channels are not expected on channel number CHAN.

'1'	-	Time-shared control channels may be encountered on channel number CHAN.

11.5.6		Category '001' Messages

11.5.6.1	Single Address Messages (Type '0')

11.5.6.1.1	Outbound Single Address Messages, SAMO

The basic word format is informative.

11.5.6.1.2	Inbound Single Address Messages

11.5.6.1.2.1	Inbound Unsolicited Single Address Message, SAMIU

The basic word format is informative.

11.5.6.1.2.2	Inbound Solicited Single Address Message, SAMIS

Mandatory for Mode 1, DESC = '000' (Interprefix calls).

Mandatory for Mode 2, DESC = '000' (Serial number transfer).

Standard option otherwise.

11.5.6.2	Short Data Message Header, HEAD (Type '1')

Standard option.

11.5.7		Codewords applicable to Standard Data

11.5.7.1	Request Standard Data Communication RQD

Standard option.

11.5.7.2	Availability Check for Standard Data AHYD

Standard option.

11.5.7.3	Go to Transaction GTT

Standard option.

11.5.7.4	Standard Data Random Access Radio Unit General Information DRUGI

Standard option.

11.5.8		Codewords applicable to Standard Data Transactions

11.5.8.1	Standard Data General Purpose Acknowledgement DACKD

Standard option.

11.5.8.2	Standard Data Codeword DACK containing Submessages DAL, DALG, DALN or GO

Standard option.

11.5.8.3	Standard Data Acknowledgement for Expedited Data DACKZ

Standard option.

11.5.8.4	Standard data General Ahoy DAHY

Standard option.

11.5.8.5	Standard Data Ahoy containing expedited Data DAHYZ

Standard option.

11.5.8.6	Standard Data Ahoy for closing a Trans DAHYX

Standard option.

11.5.8.7	Repeat last ACK - RLA

Standard option.

11.5.8.8	Repeat Group Message DRQZ

Standard option.

11.5.8.9	Request containing expedited Data DRQZ

Standard option.

11.5.8.10	Request to close a Transaction DRQX

Standard option.

11.5.8.11	Standard Data Selective Acknowledgement Header SACK

Standard option.

11.5.8.12	Standard Data Address codeword for a Dataitem SITH

Standard option.

11.6		Channel Discipline

Informative.

11.6.1		Channel Discipline for TSC

11.6.1.1	Control Channel Discipline for TSC

Informative.

11.6.1.2	Traffic Channel Discipline for TSC

11.6.1.2.1	Monitoring

Informative.

11.6.1.2.2	Signal Timing

Informative.

11.6.1.3	Data Channel Discipline for TSC

Informative.

11.6.1.3.1	Monitoring

Informative.

11.6.1.3.1	Signal Timing

Informative.

11.6.2		Channel Discipline for Radio Units

11.6.2.1	Control Channel Discipline for Radio Units

11.6.2.1.1	Control Channel Acquisition

Mandatory where specified.  The radio unit shall also meet the requirements of section 9 of this specification.

11.6.2.1.2	Retaining a Control Channel

Mandatory as specified, where:

-	an applicable system identity code is any system identity code where bits 1-12 differ from the verified value or where bits 13-15 (LAB) are a value which does not permit access to the control category assigned to the radio unit;

-	appropriate codewords are CCSC's and, if the radio unit is equipped to receive them, MARK messages;

-	a correct value of the system identity code is one in which bits 1-12 of the SYS field from a received appropriate codeword match bits 1-12 of the verified value value and bits 13-15 (LAB) are a value which permits access to the control category assigned to the radio unit.

The radio unit shall also meet the requirements of section 9 of this specification.

11.6.2.1.3	Signal Timing

Mandatory as specified.

The reception-to-reception retuning time limits stipulated in MPT 1327, [6.2.1.3], [6.2.2.1], [7.4.2], [9.2.2.5], [9.2.3.4] and [9.2.3.8] all refer to the time between the end of the relevant invoking codeword and the start of the last 16 bits of the preamble to the standard data message on the new channel.  The only requirement is to be able to decode such a message.

Time limits for satisfactory reception of speech or non-standard data are not specified.

11.6.2.2	Traffic Channel Discipline for Radio Units

11.6.2.2.1	Monitoring

Mandatory as specified.

For definition of the re-tuning time limits, see 11.6.2.1.3.

11.6.2.2.2	Signal Timing

Mandatory as specified.

11.6.2.2.2.1	Radio Unit Response

Mandatory as specified.

11.6.2.2.2.2	Unsolicited Transmission that Requires a Response

Standard option.

11.6.2.3	Data Channel Discipline for Radio Units

Standard Option

11.6.2.3.1	Monitoring

Standard Option

11.6.2.3.2	Signal Timing

Standard Option

11.7		Random Access Protocol

Informative.

11.7.1		The Principle

Informative.

11.7.2		TSC Random Access Facilities

11.7.2.1	Marking Random Access Frames

Informative.

11.7.2.2	Subdividing the Radio Unit Population

Informative.

11.7.2.3	Inviting Specific Types of Random Access MessageFrame

Informative.

11.7.2.4	TSC Responses

Informative.

11.7.2.5	Withdrawing Slots from Frames

Informative.

11.7.3		Radio Unit Random Access Protocol

Mandatory as specified.

11.7.3.1	Checking Subsets of the Radio Unit Population

Mandatory as specified.

11.7.3.2	Checking the Aloha Function

Mandatory as specified.  The recognition of ALHF is mandatory; the implementation of the fall back mode is a standard option (see section 13).

11.7.3.3	Frames Defined by Aloha Numbers

Mandatory as specified.

11.7.3.4	First Try Option

Standard Option as stated.

11.7.3.5	Choosing a Slot from a New Frame

Mandatory as specified.

11.7.3.6	Check for Withdrawn Slot

Mandatory as specified.  It is mandatory for the radio unit not to make a random access when a codeword is not decodable (or no signal is received) (item d. of MPT 1327 section 7.3.6).

11.7.3.7	Noting the Response Delay

Mandatory as specified.  The required value of parameter NW is defined in Appendix B of this specification.

11.7.3.8	Retry Decision and Time-outs

Mandatory as specified.  Requirements for confidence indications are given in section 8.1.  The required values of timeout and default parameters listed are defined in section 6 and Appendix B of this specification.

11.7.4		Related Procedures for all Radio Units on a Control Channel

11.7.4.1	Individually  Addressed Aloha Message

Mandatory as specified.

11.7.4.2	MOVE Message

Mandatory as specified (see also section 11.5.5.4.4).  For definition of the retuning time limits, see 11.6.2.1.3.

11.8		Registration Procedures

Informative.  Mandatory requirements for registration are specified in section 10 of this specification.

11.8.1		Registration Facilities

Informative.  The function of ALHF is specified in sections 9 and 13.

11.8.2		Procedures for Registration by Random Access

11.8.2.1	TSC Procedures

Informative.

11.8.2.1.1	Responses to a Random Access RQR Message

Informative.

11.8.2.1.2	Acknowledgements Sent to Indicate Progress of Registration

Informative.

11.8.2.1.3	TSC Time-out

Informative.

11.8.2.2	Radio Unit Procedures for Registration by Random AccessRequest

11.8.2.2.1	Criteria for Registration

Mandatory where specified.  The requirements placed on the radio unit as regards registration are given in section 10.  The radio unit shall not attempt random access to a system unless that system is identified in its personality (see section 6).  Where a system requires a radio unit to register, the radio unit shall be required to register successfully before attempting to make any calls.

11.8.2.2.2	Registration Request and Valid Responses

Mandatory where specified.  The required values of timeout and default parameters listed are defined in section 6 and Appendix B of this specification.  The radio unit shall set the INFO field to all zeros, unless permitted otherwise by the network.

11.8.2.2.3	Acknowledgement Received

Mandatory where specified.  The action that the radio unit shall take when registration is denied is defined in section 10 of this specification.

11.8.2.2.4	Time-out after Waiting

Mandatory where specified.  The action that the radio unit shall take when registration is denied is defined in section 10 of this specification.

11.8.3		Procedures for Registration on Demand

11.8.3.1	TSC Procedures for Demanding Registration

Informative.

11.8.3.2	Radio Unit Procedures for Registration on Demand

11.8.3.2.1	Individually Addressed ALHR Message

Items a1, a2: mandatory as specified.  Item b: all radio units shall have the capability to register, and hence the radio unit shall respond with RQR in this case.  Item c.  is not applicable.

11.8.3.2.2	Response to RQR sent on Demand

Mandatory as specified.  Additional mandatory requirements are placed on the radio unit in section 10 of this specification.

11.9		Basic Call Procedures

Informative.

It shall be mandatory for the radio unit to be able to make "simple" calls to the following destinations:

Radio units and line units with the same prefix.

Radio units and line units with a different prefix.

It is a standard option for the radio unit to make "simple" calls to destinations other than those listed above.

It is a standard option for a radio unit to make the following calls:

Calls to a group.

Calls to all units on a system.

11.9.1		Basic Call Procedures for TSC

Informative.

11.9.1.1	Basic TSC Procedures for Setting Up Calls

11.9.1.1.1	Responses to a Short Addressing RQS Message

Informative.

11.9.1.1.2	Responses to an Extended Addressing RQS Message

Informative.

11.9.1.1.3	Instructions to Send Extended Address Information

Informative.

11.9.1.1.4	Acknowledgements Sent to a Calling Unit to Indicate Progress of a Simple Call

Informative.

11.9.1.1.5	Availability Check on Called Radio Unit

Informative.

11.9.1.1.6	Availability Check for Calls to PABX Extensions and PSTN Destinations

Informative.

11.9.1.1.7	Availability Check on Requesting Radio Unit

Informative.

11.9.1.1.8	Call Cancellation

Informative.

11.9.1.1.9	Call Amalgamation

Informative.

11.9.1.1.10	Queue Management and Queue Time-out

Informative.

11.9.1.1.11	Resolving Call Conflicts

Informative.

11.9.1.1.12	Traffic Channel Allocation

Informative.

11.9.1.2	Basic TSC Procedures for Maintenance and Clear-Down of Calls

Informative.

11.9.1.2.1	Call Maintenance Options

Informative.

11.9.1.2.2	Availability Check on Traffic Channel

Informative.

11.9.1.2.3	Disabling User Transmission

Informative.

11.9.1.2.4	Allocating Replacement Traffic Channel

Informative.

11.9.1.2.5	Clearing Down Unwanted Radio Units During a Call

Informative.

11.9.1.2.6	Call Cleardown

Informative.

11.9.2		Basic Call Procedures for Radio Units

Mandatory where specified.

It is a mandatory for the radio unit to be fitted with a "ready for communication control" (RFCC).

11.9.2.1	Procedures for Radio Units Making Simple Calls

Mandatory where specified.

It is a standard option for a radio unit to make calls to a PABX or PSTN destination.

11.9.2.1.1	Request for a Simple Call

Mandatory as specified.

It shall be mandatory for the radio unit to be able to make simple calls to common prefix and interprefix destinations.

It is a standard option for a radio unit to make data calls.

11.9.2.1.2	Valid Responses to Short Addressing RQS

Mandatory as specified.

11.9.2.1.3	Valid Responses to Extended Addressing RQS

Mandatory as specified.

11.9.2.1.4	Acknowledgement Received

Mandatory as specified.

Requirements for confidence indications are given in section 8.1.  The radio unit shall make an indication to the user following receipt of ACKB (QUAL='0').

The facility to cancel a call accepted for call back by use of an RQQ message (STATUS = '11111') is a standard option.

Diversion requests (RQT) and use of diversion information are a standard option.  Automatic re-dial to the diversion IDENT is optional.

11.9.2.1.5	Availability Check and Channel Allocation for Own Call

Mandatory as specified.

11.9.2.1.6	Time-out after Waiting

Mandatory as specified.  The requirements for confidence indications are given in section 8.1.

11.9.2.1.7	Call Cancellation

Mandatory.  It shall be possible for the user to cancel the call.  The requirements for confidence indications are given in section 8.1.

11.9.2.2	Basic Procedures for All Radio Units on a Control Channel

Informative.

11.9.2.2.1	Instruction to Send Address Information or Data Message

Mandatory for the following function:

Interprefix calls.

Standard option for the other transaction types listed.

11.9.2.2.2	Availability Check on Called Radio Unit

Mandatory where specified.  It is optional whether the radio unit indicates the IDENT of the caller to the user.  It is optional whether the radio unit makes a distinct indication for an emergency call.

If, while waiting for an incoming traffic channel call, a radio unit receives a repeat AHY, and the user has already activated the RFCC, the unit shall not re-alert the user.

In the event of an incoming traffic channel call (IDENT2 = Ident(1-8100), INCI, IPFIXI, PSTNGI or PABXI) the unit shall respond with one of ACK(QUAL='0'), ACKI(QUAL='0'), ACKB(QUAL='0'), ACKB(QUAL='1'), ACKV(QUAL='1') or ACKX(QUAL='0') in accordance with MPT 1327.  In addition, the radio unit shall only respond with ACKX(QUAL='0') if one of the following conditions is met:

i.	Bit D of the received AHY message is '0' and the unit is not accepting speech calls.

ii.	Bit D of the received AHY message is '1' and the unit is not accepting or not ready for data calls (see section 12).

The radio unit shall not accept a call for call back (using ACKB(QUAL='0')) unless it has facilities for making the return call.

It is mandatory for the radio unit to send RQQ off-hook/on-hook signalling.

As a standard option, the radio unit may respond to AHY, bit CHECK='1', with ACK (QUAL=-'0'). This option shall only be "enabled" by network personalisation and the default state shall be "disabled".

11.9.2.2.3	Availability Check on Requesting Radio Unit

Mandatory as specified.

11.9.2.2.4	Cancelling Alert/Waiting State of Called Unit

Mandatory where specified.

The requirements for confidence indications are given in section 8.1.

11.9.2.2.5	Traffic Channel Allocation

Mandatory as specified.  User indication of calling IDENT is optional.  For definition of the retuning time limits, see 11.6.2.1.3.

11.9.2.2.6	Storing Call Maintenance Parameters

Mandatory as specified.

It is an option for radio units to store the value of TSCLIM from the last decodable BCAST, SYSDEF='00010' message received in read/write memory (see 11.5.5.4.5 c).

11.9.2.3	Procedures for all Radio Units on an Allocated Traffic Channel

Mandatory as specified.

11.9.2.3.1	Call Maintenance Messages

For values of NPON=1 and NPOFF=1 : mandatory as specified.

For values of NPON>1 or NPOFF>1 : standard option.

11.9.2.3.2	Availability Check on Traffic Channel

Mandatory as specified.

Section A)	Mandatory as specified

Section B)	Standard Option

11.9.2.3.3	Disabling User Transmission

See also 11.5.5.4.2.

If the radio unit on a traffic channel receives a MAINT (OPER = '111') message with the STI flag equal to zero then:

i.	the SIL3 field = RSVD and has no meaning;

ii.	the radio unit shall understand and take any mandatory action required.

If the radio unit on a traffic channel receives a MAINT (OPER ='111') message with the STI flag NOT equal to zero, then:

i.	the user transmission shall only be inhibited if the SIL3 field matches the three least significant bits of the verified SIL code.

11.9.2.3.4	Replacement of Traffic Channel

Mandatory where specified.

For definition of the retuning time limits, see 11.6.2.1.3.

The radio unit shall meet the requirements of any prearrangement made as regards the sending of periodic messages during data calls (refer to section 6).

11.9.2.3.5	Going "on-hook" on Traffic Channel

Mandatory as specified.

11.9.2.3.6	Time-outs on Traffic Channel

For values of NPON=1 and NPOFF=1: mandatory where specified.

For values of NPON>1 or NPOFF>1: standard option.

The requirements for confidence indications are given in section 8.1.

The definition of inactivity that shall be used in this section of MPT 1327 is that a radio unit is considered inactive on a traffic channel when it is not transmitting and the received audio is muted because the receiver quieting is insufficient.

It is an option for radio units to incorporate a maximum call duration timer. This timer shall be initialised immediately that the radio unit has tuned to a designated forward traffic channel following receipt of a GTC message in accordance with !!9.2.2.5!! (unless inhibited by the latest received value of TSCLIM, see below). The timer shall remain in operation for as long as the radio unit remains tuned to either the forward or return channel of the designated traffic channel or any other traffic channel to which the radio unit may be directed by subsequent GTC messages in accordance with !!9.2.3.4!! but shall be cancelled when the radio unit tunes to a control channel in accordance with !!9.2.3.5!!, !!9.2.3.6!!, !!9.2.3.7!! or !!9.2.3.8!!.

Upon expiry of the maximum call duration timer the radio unit shall:

i)	Mute the audio.

ii)	If the radio unit is transmitting it shall send one or more Pressel Off messages to indicate the end of the item in accordance with !!9.2.3.1!!.

iii)	Send ND1 or ND2 Disconnect messages if its individual address is PFIX/IDENT1 or PFIX/IDENT2 from the GTC (as in section !!9.2.3.5!!).

iv)	Cease transmission on the traffic channel, indicate the end of the call to the user in accordance with 8.1.3.8 and enter the control channel acquisition  procedures (see section 9).

The radio unit may also use the call duration timer to indicate to the user that the above action is imminent, at some time prior to the action being carried out.

The call duration timer shall expire after a period as determined below:

-	Upon acquiring a new control channel and until a BCAST (SYSDEF='00010') message has been received, the period shall be CLIM.

-	If at least one decodable BCAST (SYSDEF='00010') message has been received by the radio unit since the start of the session or since acquiring a new control channel, the period shall be as indicated by the value of TSCLIM received in the last decodable BCAST (SYSDEF='00010') message (see 11.5.5.4.5c) unless that value of TSCLIM is '00000000' when the period shall be CLIM or is '11111111' when the maximum call duration timer shall be inhibited.

-	If the radio unit has tuned to the traffic channel as a result of receiving a GTC message whilst waiting for signalling for an emergency call following the receipt of an AHY message with bit E set to '1' or following the transmission of RQE, the period shall be CLIME.

If the radio unit transmits an item that reaches the maximum permitted duration TT then it shall mute the audio and shall

as a default satisfy the procedures defined in !!9.2.3.6.A!!

As an alternative !!9.2.3.6.B!! may be activated during personalisation of the radio unit using the TT flag defined in Table 6.1 (Item nn)

11.9.2.3.7	"Selective" Clear-Down Message: MAINT with OPER='110'

See also 11.5.5.4.2.

The requirements for confidence indications are given in section 8.1.

If a radio unit on a traffic channel receives a MAINT (OPER ='110') with the STI flag equal to zero then:

i.	the SIL3 field = RSVD and has no meaning;

ii.	the radio unit shall understand and take any mandatory action required.

If the radio unit on a traffic channel receives a MAINT (OPER ='1101') message with the STI flag NOT equal to zero, then:

i.	the radio unit shall clear down only if the SIL3 field matches the three least significant bits of the verified SIL code.

11.9.2.3.8	CLEAR Message

If a radio unit on a traffic channel receives a clear-down message CLEAR with the STI flag equal to zero

and

i.	channel number (CHAN) equal to the number of the traffic channel

and

ii.	field REVS equal to '101010101010',

then it shall immediately mute the audio and move to the forward control channel indicated by the field CONT in the CLEAR message (to be capable of receiving within 35 ms after the end of the CLEAR address codeword) and may indicate to the user that the call has ended.

If a radio unit on a traffic channel receives a clear-down message CLEAR with the STI flag NOT equal to zero

and

i.	channel number (CHAN) equal to the number of the traffic channel

and

ii.	field REVS equal to '101010101010'

and

iii.	the SIL3 field matches the three least significant bits of the verified SIL sub-field,

then it shall immediately mute the audio and move to the forward control channel indicated by the field CONT in the CLEAR message (to be capable of receiving within 35 ms after the end of the CLEAR address codeword) and may indicate to the user that the call has ended.

If the field CONT is set to '0000000000' then the radio unit shall either return to the last active control channel , or remain on the nominated fall-back channel if in fall-back mode.

The requirements for confidence indications are given in section 8.1.

11.10		Emergency Call Procedures

Standard option.

Other modes of customised emergency service are not precluded.

11.10.1	Standard Emergency Call Procedures for TSC

Entire subsection: informative.

11.10.2	Standard Emergency Call Procedures for Radio Units

Entire subsection: standard option.

Standard emergency call procedures on a traffic channel are defined in MPT 1327 section 9.2.3.

11.11		Include Call Procedures

Standard option.

11.11.1	TSC Procedures for Include Calls

Entire subsection: informative.

11.11.2	Procedures for Radio Units Requesting Include

Entire subsection: standard option.

11.11.3	Procedures for All Radio Units on an Allocated Traffic Channel

11.11.3.1	Instruction to Send Extended Address Information

Standard option.

11.12		Call Diversion Procedures

Standard option.

11.12.1	TSC Procedures for Call Diversion Requests

Entire subsection: informative.

11.12.2	Procedures for Radio Units Requesting Call Diversion

Entire subsection: standard option.

11.13		Status Message Procedures

Entire section: procedures involving RFCC signalling are mandatory.

It is mandatory that the radio unit is able to recognise and respond to the AHYQ message (refer to MPT 1327 section 13.2.3).

Otherwise: standard option.

11.14		Short Data Message Procedures

Entire section: standard option.

11.15		Data Interrogation Procedures

Informative.

11.15.1	Data Interrogation Procedures for TSC

11.15.1.1	Data Interrogation on a Control Channel

Informative.

11.15.1.2	Data Interrogation on a Traffic Channel

Informative.

11.15.2	Procedures for All Radio Units

It is a mandatory requirement that radio units shall recognise Mode 2 AHYC messages and respond with the serial number transmission as specified below.

11.15.2.1	Data Interrogation Message (AHYC, Mode 2) on a Control Channel

The radio unit shall be equipped to transmit its serial number on interrogation using the SAMIS message.  The form of the serial number transmitted is defined in section 7.

11.15.2.2	Data Interrogation Message (AHYC, Mode 2) on an Allocated Traffic Channel

The radio unit shall be equipped to transmit its serial number on interrogation using the SAMIS message.  The form of the serial number transmitted is defined in section 7.

11.16

This paragraph is not used.

11.17		Standard Data Procedures

Entire section: standard option.