4.		TRANSMITTER PARAMETERS

4.1		Frequency Parameters

This section contains the frequency parameters for radio units operating in Band III sub-bands 1 and 2.

All radio units shall operate on any of the channels in the frequency range to which they are directed by network messages and shall examine and if necessary,operate on that channel, in that range, in accordance with the procedures specified in Section 9. If directed by network messages to any channel outside this range, radio units shall not comply with the messages and shall remain  on the channel from which the message was sent.

4.1.1		Channel Spacing and Designation for Sub-band 1

The channel spacing shall be 12.5 kHz. The channel designations shall be as follows:

| Channel Number | Binary representation of channel number * | Radio Unit Transmit Frequency |
| --- | --- | --- |
|  | width 1.5" |  |
| 1 |  | 184.5000 MHz |
| 58 | 1000000001 | 185.2125 MHz |
| 560 | 1111110111 | 191.4875 MHz |

* Applicable to CHAN, CONT and CHAN4 fields

The radio unit equipment shall operate over channels 58 to 560 (185.2125 MHz to 191.4875 MHz) inclusively.  Transmission outside this frequency range is not permitted.

4.1.2		Channel Spacings and Designation for Sub-band 2

The channel spacing shall be 12.5 kHz. The channel designations shall be as follows:

| Channel Number | Binary representation of channel number * | Radio Unit Transmit Frequency |
| --- | --- | --- |
| 1 |  | 192.5000 MHz |
| 58 | 0000000001 | 193.2125 MHz |
| 560 | 0111110111 | 199.4875 MHz |

* Applicable to CHAN, CONT and CHAN4 fields

The radio unit equipment shall operate over channels 58 to 560 (193.2125 MHz to 199.4875 MHz) inclusively. Transmission outside this frequency range is not permitted.

4.1.3		Frequency Tolerance

The carrier frequency of a signal transmitted by a radio unit shall be within 1.5 kHz of the nominal transmit frequency.

This tolerance includes crystal ageing and shall be maintained under both normal and extreme temperature conditions, as defined in MPT 1323 sections 2.3.1 and 32.4.1, for a period of at least one year, without re-adjustment.

4.2		Modulation Characteristics

4.2.1		General

The peak permissible frequency deviation shall not exceed +2.5 kHz.

4.2.2		Speech Modulation

Speech transmissions shall employ a phase modulation characteristic.

4.2.3		Data Modulation

4.2.3.1	Data Modulation Method

Standard data modulation shall be by subcarrier modulation employing phase continuous fast frequency shift keying (FFSK) to a frequency modulation characteristic.  Speech shall be muted by at least 35 dB (measured at the transmitter output) during transmission of standard data.  The parameters of the modulation shall be as follows:

Bit rate	1200 bit/s

Modulation rate	1200 baud

Binary '0'	1800Hz

Binary '1'	1200Hz

Amplitude difference	<1.5dB

During the transmission of call maintenance messages (pressel-on, periodic and pressel-off), speech shall be muted as follows:

(a)	During all maintenance messages, speech shall be muted by at least 35dB from the start of the preamble to the end of the hang-over bit.

(b)	For the periodic and pressel-off messages, the muting of speech shall not commence more than 20ms before the start of the preamble.

(c)	For the periodic and pressel-on messages, the muting shall be removed no later than 10ms after the end of the hang-over bit.

The method for modulating non-standard data is not prescribed in this specification.

4.2.3.2	Data Peak Deviation

For standard data modulation.

Angle modulation:	Nominal	1.5 kHz ±250 Hz

Extreme	1.5 kHz ±500 Hz

4.3		Performance Parameters

Radio units shall meet the requirements of MPT 1323.