# 14	SHORT DATA ON THE CONTROL CHANNEL 		USING RQC

## 14.1	Introduction

This section describes the air interface requirements necessary to support signalling between radio units and TSCs during the transfer of short data messages on the control channel.  The implementation of the procedures defined in this section is a standard option.

The transfer of short data messages conforms with the basic procedure defined in MPT1327 section 14.  This allows transmission of HEAD messages containing free format data on the control channel.  Implementation of the short data standard option defined in this document requires some of these bits to carry control information.  The protocol allows the use of only MPT1327 procedures, or the procedures as described in this section.

A calling radio unit requests to transmit a short data message by sending an RQC random access request message addressed to the called unit or service.  For extended addressing PSTN and PABX calls (and optionally for interprefix calls), the TSC will solicit the full called party address information using the MPT1327 extended addressing procedures at an appropriate point in call set-up.  The TSC may check the availability of a called radio unit using the General Availability Check Message AHY, before requesting the caller to send a HEAD message by means of the Short Data Invitation Message AHYC (refer to sections !!5.5.3.2.1!! and !!5.5.3.2.8!!).  The calling party sends a Short Data Message Header HEAD and up to four appended data codewords to the TSC.  The TSC then forwards the data by re-transmitting the HEAD message to the called party which is required to respond with an acknowledgement in accordance with the procedures outlined in section 14 of MPT1327.  The TSC sends an acknowledgement to the calling party to advise the receipt of the HEAD message (or otherwise) by the called party.  Where the called party is a group and not an individual address, no acknowledgement by radio units in that group to a HEAD message is permitted (see section !!14.3.1.2!!) and, in this case, the TSC sends an acknowledgement to the calling party to advise whether the HEAD message has been received by the TSC and transmitted to the group.

Note:  The term "HEAD message", where used in this section, shall be taken to mean "HEAD address codeword and appended data codewords" collectively.

The procedures defined in !!14!! support the transmission of a single segment of free-format data. (A "segment" is that amount of free-format data which can be accommodated in a single HEAD message; see section 3.1).  This specification extends the scope of the above referenced procedures to allow up to four segments to be associated with a single request (RQC).  For convenience a short data transaction for which the T-message (see section 3.1) is confined to a single segment is referred to as a Single Segment Transaction (SST) in this specification.  A short data transaction for which the T-message comprises more than one segment is referred to as a Multiple Segment Transaction (MST).

### 14.1.1	General Description

A radio unit requests to transmit short data HEAD messages in accordance with the procedures of !!14!! by sending an RQC message on a control channel.  The TSC then solicits the  transmission of a HEAD message using the address codeword AHYC.  In the case of a Multiple Segment Transaction, each HEAD message of the transaction is individually solicited by the TSC using an AHYC message.  The TSC need not support Multiple Segment Transactions, in which case this will be indicated in the AHYC message.  In these circumstances a radio unit wishing to send a T-message comprising more than one segment is required instead to generate an RQC for each segment treating each as an SST.

In the case of an MST, the TSC is responsible for requesting each segment from the radio unit and forwarding it to the called party.  The TSC either will assemble the complete T-message before forwarding it or will forward each segment by means of a HEAD message as soon as it has been received correctly.

SSTs and MSTs may be addressed to individual units, to groups or to a TSC gateway.

A simple message repeat error correction protocol is incorporated into this specification.  If an error is detected by the TSC in a return channel data codeword (calling radio unit to TSC) a repeat may be demanded until a satisfactory error-free segment can be assembled.  If an error is detected by an individually called radio unit in a forward channel data codeword (TSC to called party), repeat transmissions may be requested.  The TSC may make repeat transmissions, subject to timing rules and network limits, when no acknowledgement of receipt is obtained from the called party.

### 14.1.2	Facilities

T-messages may be sent in one of the 8 formats listed below.  Changing between these formats within a transaction is not permitted.

-	binary

-	BCD (in accordance with MPT1327, Appendix 5)

-	ITU-T Alphabet No 2 (Telex), Recommendation S1

-	ITU-T Alphabet No 5 (ASCII), Recommendations V3 and V4

-	two formats which are reserved for future definition

-	A mechanism to transport control information to a radio unit.  (See Appendix AN6)

-	one format which is spare for customisation

A short data HEAD address codeword and appended data codewords may occupy up to three control channel timeslots.  The data formats specified in this section allow an SST to convey 44 BCD characters, 35 ITU-T Alphabet No 2 (Telex) characters or 25 ITU-T Alphabet No 5 (ASCII) characters.  An MST is capable of carrying 176 BCD characters, 140 ITU-T Alphabet No 2 (Telex) characters or 100 ITU-T Alphabet No 5 (ASCII) characters.

## 14.2	Message Formats

The procedures for short data message transmission described in this section utilise MPT1327 address codewords and data codewords.  The formats of the address codewords are as prescribed in MPT1327 with, in the case of the AHYC codeword, some additional meanings ascribed to the values of one field within the codeword.  The format of data codewords is not specified in MPT1327, but a data codeword structure is specified for the procedures in this section to allow control parameters to be incorporated and defined data character formats to be utilised.

These particular applications of address and data codewords are  described below.

### 14.2.1	Short Data Invitation Message, AHYC

The format of this message is as specified in !!5.5.3.2.8!! with the following addition (note that IDENT1 shall be set to SDMI for inviting short data HEAD messages):

DESC:	'000'	TSC supports SSTs only. Instruction to calling party to send a HEAD message.

'1xx' TSC supports MSTs.  Instruction to calling party to send a HEAD message containing the appropriate segment of the MST -

xx = value assigned to the short data segment.

xx = '00'	-	First segment of MST or only segment of SST.

xx = '01'	-	Second segment.

xx = '10'	-	Third segment.

xx = '11'	-	Fourth segment.

### 14.2.2	Data Codewords

#### 14.2.2.1

Up to four data codewords may be appended to a HEAD address codeword.  Each data codeword shall conform to one of the following two general structures, depending on its position relative to the HEAD codeword:

i)	First and third data codewords following the HEAD codeword:

MPT1327 Short Data Message Format (STF = '0')

| 0 | STF | DATA | P |
| --- | --- | --- | --- |
| 1 | 1 | 46 | 16 |

STF	-	Segment Transaction Flag.

'0' - MPT1327 short data message format (46 bits of free format data in each of up to 4 data codewords).

DATA	-	Free format binary digits.

P	-	Parity check bits.

MPT1343 Short Data Message Format (STF = '1')

| 0 | STF | MESS | DATA | P |
| --- | --- | --- | --- | --- |
| 1 | 1 | 4 | 42 | 16 |

STF	-	Segment Transaction Flag. '1'- MPT1343 short data message format as defined in this specification).

MESS	-	Message Control Field (refer to 14.2.2.2).

DATA	-	Free format binary digits or binary-encoded free format text (refer to 14.2.2.3).

P	-	Parity check bits.

ii)	Second and fourth data codewords following the HEAD codeword:

| 0 | RSA | DATA | P |
| --- | --- | --- | --- |
| 1 | 1 | 46 | 16 |

RSA	-	Return Slot Access Flag.  When transmitted by the TSC on a control channel:

'0'	-	Radio units shall not attempt random access in the following slot on the return control channel.

'1'	-	Radio units may attempt random access in the following slot on the return control channel.

In all other cases of transmission, the meaning of the RSA flag is reserved, default = '0'.

DATA	-	Free format binary digits or binary-encoded free format text (refer to 14.2.2.3).

P	 -	Parity check bits.

#### 14.2.2.2	Structure of MESS Field

The format of the MESS field shall conform to one of the following two structures, as determined by the position of the data codeword containing the MESS field relative to the HEAD codeword:

i)	First data codeword following the HEAD codeword:

| GFI | I |
| --- | --- |
| 3 | 1 |

GFI	-	General Format Information, states the format in which the T-message is presented in the DATA fields of this and subsequent data codewords (see also 14.2.2.3).

- '000'		Binary.

- '001'		BCD (Appendix 5, MPT1327).

- '010'		ITU-T Alphabet No 2 (Telex).

- '011'		ITU-T Alphabet No 5(ASCII).

- '100'		Reserved.

- '101'		Spare

- '110'		Command Message (see Appendix AN6)

- '111'		Spare. (Note that MAP27 uses this format

I	-	Initial Segment flag.

- '1'	First segment. (For an SST, I shall always be set to '1').

- '0'	Subsequent segment.

ii)	Third data codeword following the HEAD codeword:

| NSEG | CSEG | RSVD |
| --- | --- | --- |
| 2 | 1 | 1 |

NSEG	-	Indicates the number of segments to follow in an MST.

-	'00' Last segment.

-	'01' One segment to follow.

-	'10' Two segments to follow.

-	'11' Three segments to follow.

(For an SST, NSEG shall always be set to '00' where this codeword is used.)

CSEG	 -	Indicates whether the message containing the next segment of an MST requires 2 or 3 control channel timeslots.

-	'0' Two slots required.

-	'1' Three slots required.

(Where NSEG = '00', CSEG shall be set to '0'.  Where NSEG = '10' or '11', CSEG shall be set to '1'.)

RSVD	 -	Reserved for future definition.

#### 14.2.2.3	Structure of DATA Field

The format of the DATA field shall be determined by the value of the GFI field and shall be as specified below.

Note:  For Telex type characters the start and stop units or elements are omitted (as is also the case for the start and stop bits when transmitting ASCII characters).  For the purposes of describing the order in which ITA2 code units or elements are sent within the DATA field Code Element No. 1 may be considered to be the most significant and Code Element No. 5 the least significant.  The elements are always sent in the order 1, 2, 3, 4, 5.  Thus in the formats given below, “n*” represents the most significant bits of an encoded character whose remaining bits form the start of the DATA field of the next codeword.  “*m” represents the m least significant bits of an encoded character whose preceding bits concluded the DATA field of the previous codeword.

i)	Binary

(GFI = '000')

First and third codeword:

DATA

BINARY

42

Second and fourth codeword:

DATA

BINARY

46

BINARY	-	Free format binary data.

ii)	BCD (as in Appendix 5 of MPT1327)

(GFI = '001')

First codeword:

| DATA |  |
| --- | --- |
| ten CHARs | 2* |
| 40 | 2 |

Second codeword:

| DATA |  |
| --- | --- |
| *2 | eleven CHARs |
| 2 | 44 |

Third codeword:

| DATA |  |
| --- | --- |
| ten CHARs | 2* |
| 40 | 2 |

Fourth codeword:

| DATA |  |
| --- | --- |
| *2 | eleven CHARs |
| 2 | 44 |

CHAR	-	Binary value, as prescribed in MPT1327 Appendix 5, of unspecified BCD character.

Note:  The maximum number of BCD characters which can be included in a segment is 44.

iii)	ITU-T Alphabet No 2 (Telex Type Characters)

(GFI = '010')

First codeword:

| DATA |  |  |
| --- | --- | --- |
| SPARE | eight CHARs | 1* |
| 1 | 40 | 1 |

Second codeword:

| DATA |  |  |
| --- | --- | --- |
| *4 | eight CHARs | 2* |
| 4 | 40 | 2 |

Third codeword:

| DATA |  |  |
| --- | --- | --- |
| *3 | seven CHARs | 4* |
| 3 | 35 | 4 |

Fourth codeword:

| DATA |  |
| --- | --- |
| *1 | nine CHARs |
| 1 | 45 |

SPARE	-	Available for customisation.

CHAR	-	Binary value, as prescribed in ITU-T Recommendation S1 (Alphabet No 2), of an unspecified character.

Note:  The maximum number of ITU-T Alphabet No 2 characters which can be included in a segment is 35.

iv)	ITU-T Alphabet No 5 (7 bit ASCII)

(GFI = '011')

First codeword:

| DATA |  |  |
| --- | --- | --- |
| SPARE | five CHARs | 6* |
| 1 | 35 | 6 |

Second codeword:

| DATA |  |  |
| --- | --- | --- |
| *1 | six CHARs | 3* |
| 1 | 42 | 3 |

Third codeword:

| DATA |  |  |
| --- | --- | --- |
| *4 | five CHARs | 3* |
| 4 | 35 | 3 |

Fourth codeword:

| DATA |  |
| --- | --- |
| *4 | six CHARs |
| 4 | 42 |

SPARE -	Available for customisation.

CHAR	-	Binary value, as prescribed in ITU-T Recommendation s V3, V4 (Alphabet No 5), of an unspecified character.

Note: The maximum number of ITU-T Alphabet No 5 (7 bit ASCII) characters which can be included in a segment is 25.

v)	Other formats

Formats for GFI values of '100' and '101' are is reserved for future definition and GFI='101 is spare.  GFI = '110' indicates a control message directed to the recipient radio unit (see Appendix AN6), GFI = '111' is spare for customisation. (Note that MAP27 uses the GFI='111'  format)

## 14.3	The Use of Control Fields for STF = '1'

A calling radio unit divides its T-message into a maximum of four segments, where all segments except the last utilise exactly four data codewords.  The last segment may utilise up to four data codewords as required to accommodate the T-message.  The segments of a Multiple Segment Transaction are logically linked by the 'NSEG', 'CSEG' and 'I' fields.

A HEAD message containing the first segment shall have the I bit set to '1', and those containing subsequent segments shall have I set to '0'.

The NSEG field represents a decrementing counter which shall indicate the number of segments to follow to complete the transaction such that the value of NSEG is '00' for the last segment of an MST.  If the last segment uses less than three data codewords, then the NSEG field will not be transmitted and the recipient shall behave as though its value had been '00'.

The CSEG field informs the TSC of the number of slots required for data codewords containing the next segment.  The value shall always be '1' when transmitted with leading segments (NSEG = '10' or '11') and may be either '1' or '0' when transmitted with the penultimate segment (NSEG = '01').  With the final segment CSEG defaults to '0' and the TSC will ignore the value.

HEAD messages transmitted by the TSC to a called radio unit or group shall contain the same values of I, NSEG and CSEG as those of messages containing the corresponding segments received by the TSC from a calling radio unit.  The called radio unit may perform a check to ensure that the values of these fields are logically consistent.

An example, illustrating the use of the Control fields, is given in 14.5.

The called radio unit uses the values of STF and NSEG in the HEAD messages to determine which value to use in the individual incoming short data timer.  See section 14.4.2.1.

## 14.4	Procedures for Radio Units

Radio units which implement the option to transmit and receive short data messages (either SSTs or MSTs) shall comply with the requirements of !!14!!.  In addition radio units shall meet the requirements of this specification which are given below.

### 14.4.1	Radio Unit Actions when Sending Short Data Messages

#### 14.4.1.1	Composition of HEAD Messages

A radio unit complying with the procedures of this section shall  set STF = '1' in all appropriate data codewords transmitted.

For an SST, the calling radio unit shall transmit its T-message by sending one segment.  For an MST, all segments except possibly the last shall utilise exactly four data codewords.

A HEAD message containing a segment shall only be transmitted following an invitation from the TSC.  For every message transmitted to, or received from, the TSC the radio unit shall operate the appropriate timer (see !!14.2.6!!).  After sending the last segment the radio unit shall wait for the appropriate acknowledgement (see !!14.2.4!!).

The control information for the first segment (of an SST or MST) shall be composed as follows:

i)	The GFI bits in the MESS field in the data codeword directly following the HEAD address codeword shall be set as appropriate to indicate the format of the T-message.

ii)	The I bit of the MESS field in the DATA codeword directly following the HEAD address codeword shall be set to '1'.

In the case of a HEAD address codeword with three or more appended data codewords, the control fields in the third data codeword shall be set as follows:

i)	NSEG shall be set to indicate the number of segments to follow (NSEG shall be set to '00' for an SST).

ii)	CSEG shall be set to indicate the number of timeslots required for the next MST segment (CSEG shall be set to '0' when NSEG has the value '00').

The control information for subsequent segments of an MST shall be composed as prescribed above except that the I bit of the MESS field shall be set to '0'.

#### 14.4.1.2	Message Transmission Procedures

When a radio unit is ready to send a short data message it shall send RQC (with the value of SLOTS set either to '10' for an SST requiring a HEAD message with one or two data codewords, or to '11' for either an SST requiring a HEAD message with more than two data codewords or an MST).  It shall then await responses as specified in !!14.2.1!! to !!14.2.3!!.

Upon receipt of a Mode 1 AHYC (DESC = '000'), with IDENT1 set to SDMI and PFIX/IDENT2 matching its individual address, a radio unit shall send a HEAD message with NSEG = '00', CSEG = '0' and I = '1' as a response.  The same HEAD message shall be retransmitted by the radio unit if a further identical AHYC is received.  Note that a radio unit wishing to transmit a T-message with more than one segment which receives AHYC (DESC = '000') shall respond with a HEAD message containing the first segment.  After the completion of this transaction, it may then attempt to initiate transmission of the remainder of the T-message by sending one or more subsequent RQC random access request messages.

Upon receipt of a Mode 1 AHYC (DESC = '100'), with IDENT1 set to SDMI and PFIX/IDENT2 matching its individual address, a radio unit shall send a HEAD message containing either the complete T-message of an SST, or the first segment of the T-message of an MST, as a response.  The same HEAD message shall be retransmitted by the radio unit if a further identical AHYC is received.

Upon receipt of a Mode 1 AHYC, with IDENT1 set to SDMI and PFIX/IDENT2 matching its individual address, and where DESC has incremented by one binary count from that in the previous AHYC  message addressed to it, a radio unit shall send a HEAD message containing the next segment of the MST (note: DESC values of '101', '110' and '111' correspond to 2nd, 3rd and 4th MST segments respectively).  The same HEAD message shall be retransmitted by the radio unit if a further identical AHYC is received.  If a Mode 1 AHYC message is received whose value of DESC is not consistent with the correct segment order for the T message then the radio unit shall transmit ACKX (QUAL = '0').

### 14.4.2	Procedures for All Radio Units on a Control Channel

The procedures in this section shall be obeyed by all radio units that are equipped to accept short data messages (either SSTs or MSTs).  The ability to accept short data messages is a standard option.

If flag STF in a received HEAD message is set to '0' (see 14.2.2) then the radio unit behaviour shall be in accordance with !!14.3!! but any additional procedures shall be system dependent.

If flag STF in a received HEAD message is set to '1' then the procedures below shall apply.

#### 14.4.2.1	Receiving Individually Addressed Short Data

Alternative Procedures for Radio Units receiving individually addressed MSTs and SSTs are specified.  The first, in Section 14.4.2.1.1, conforms to the requirements specified in this document as revised and printed in September 1991.  The second, in Section 14.4.2.1.2, is a revised procedure which offers a receiving radio unit the opportunity to accept MSTs and SSTs at an increased rate, limited by the data signalling capacity of the control channel more than the expiry of timers in the radio unit.  All radio units are required, when implementing this second option, to at least implement the first procedure to ensure compatibility with existing networks.  The second procedure may be implemented and used when allowed by network personalisation.

Radio Units that are able to function using either procedure shall implement a Network Personalisation MST Flag which when set to '0' shall cause the unit to carry out the procedure specified in Section 14.4.2.1.1 and when set to '1' shall cause the unit to carry out the procedure specified in Section 14.4.2.1.2.

##### 14.4.2.1.1	Original Procedure

###### 14.4.2.1.1.1	Called Unit Response to AHY Message

If a radio unit on a control channel receives an AHY message with:

- POINT set to '0',

- PFIX/IDENT1 as its individual address, and

- IDENT2 set to SDMI,

then it shall respond by sending a suitable acknowledgement; see !!9.2.2.2B!!.

If the radio unit's response is ACK (QUAL = 0) then the unit shall start timer TA and shall wait for further signalling; see also sections 14.4.2.1.1.2, 14.4.2.1.1.3 14.4.2.1.3 and 14.4.2.1.4.

###### 14.4.2.1.1.2	Receiving Individually Addressed HEAD Message

If a radio unit on a control channel receives a HEAD message with PFIX1/IDENT1 in the HEAD address codeword matching its individual address then it shall behave as described below.

a.	The radio unit shall accept, reject or solicit a repeat of that message by responding with a suitable acknowledgement (respectively ACK (QUAL = 0), ACKX or ACKV (QUAL = 1), or ACKB (QUAL = 1)).  See !!14.3.1.1!! and also points b. to f. below.

b.	If the radio unit is in a state of waiting for further HEAD messages (see section 14.4.2.1.1.3) then it shall reject the received segment by responding with ACKX (QUAL = 1) if PFIX2/IDENT2 in the HEAD address codeword does not match PFIX2/IDENT2 from previously received HEAD messages for that transaction to which it responded with ACK (QUAL = 0) or ACKB (QUAL = 1).

c.	The radio unit shall note the value of the I flag in the MESS field of the first data codeword.

If the radio unit:

-	is not in a state of waiting for further HEAD messages (see section 14.4.2.1.1.3), or

-	is awaiting retransmission of a HEAD message with I set to '1' which it solicited using ACKB (QUAL = 1), or

-	is in a state of waiting for further HEAD messages, having received a HEAD message with I  set to '1' and NSEG set to '00',

then it shall reject the received HEAD message if the I flag is set to '0' by responding ACKX (QUAL = 1).

If the radio unit has sent ACK (QUAL = 0) or ACKB (QUAL = 1) in response to a previously received HEAD message for that transaction containing I set to '0', then it shall reject the received HEAD message if the I flag is set to '1' by responding ACKX (QUAL = 1).

d.	The radio unit shall note the value of LEN in the HEAD address codeword.  For LEN = '00' or '01' the radio unit shall not expect to receive NSEG in the appended data codewords but shall behave as though its value had been '00' (see !!15.6.2!!).

e.	The radio unit shall record the value of the NSEG control field in the third data codeword if the HEAD message contains three or four data codewords.

If the radio unit is in a state of waiting for further HEAD messages (see section 14.4.2.1.1.3) then:

-	the HEAD message shall be deemed to be a repeat transmission of the last received HEAD message if both have identical values of NSEG;

-	the segment contained in the HEAD message shall be deemed to be the first transmission thereof to be received by the radio unit if the value of NSEG has been decremented by one binary count from that of the last received HEAD message.

-	If a head message is received whose value of NSEG is not consistent with the correct segment order for the T message then the radio unit shall transmit ACKX(QUAL=1).

The radio unit shall reject any received HEAD message by sending ACKX (QUAL = 1) where the value of NSEG:

-	has been incremented since the last received HEAD message, or

-	has been decremented by more than one binary count since the last received HEAD message, or

-	has been decremented and the previous segment has not been decoded successfully.

If the radio unit is not in a state of waiting for further HEAD messages then any value of NSEG is valid.

f.	If the HEAD message contains one or more corrupted data codewords, the radio unit may extract uncorrupted data codewords from this and subsequent retransmissions thereof, until it is able to assemble a complete and uncorrupted segment.  If a retransmission of the HEAD message is required, then the radio unit shall respond with ACKB (QUAL = 1).

When the segment has been assembled, the radio unit shall respond with either ACK (QUAL = 0) or ACKX (QUAL = 1) as appropriate.

g.	If the radio unit responds to the HEAD message with ACK (QUAL = 0) or ACKB (QUAL = 1), then the unit shall start timer TM (if NSEG¹0) or TGI (if NSEG=0) and shall wait for further signalling; see also section 14.4.2.1.1.3.

For other responses, the radio unit shall leave its individual incoming short data timer in its existing state (either running, or not set, as appropriate).

The radio unit shall offer the complete T-message to the user when a HEAD message with NSEG = '00' has first been successfully decoded (or its associated segment  has been assembled as in point f. above).  The T-message shall be formatted in accordance with the GFI bits in the MESS field.  The radio unit shall then continue to wait as defined in section 14.4.2.1.1.3.

###### 14.4.2.1.1.3	Individual Time-out TGI/TA

Radio units shall operate an individual incoming short data timer which shall be set to either of two different values as described below.

The timer shall be set to a value TA and started or restarted when the radio unit responds with ACK (QUAL = 0) to an AHY message with IDENT2=SDMI.  The timer shall be set to a value TGI and started or restarted when the radio unit responds with ACK (QUAL = 0) or ACKB (QUAL = 1) to a HEAD message.

A radio unit is in a state of waiting for further signalling for an incoming individually addressed short data transaction if its individual incoming short data timer is running.  A radio unit is in a state of waiting for further HEAD messages if this timer is running and the radio unit has received and responded to an earlier HEAD message with ACK (QUAL = 0) or ACKB (QUAL = 1) during that transaction.

A radio unit waiting for further signalling for an incoming individually addressed short data transaction (SST or MST) shall assume that no further signalling will be received for that transaction if its individual incoming short data timer expires.

Thereupon, if the T-message has not been fully decoded, the short data transaction shall be deemed to have failed and the radio unit may generate an appropriate indication to the user.  The radio unit may offer an incomplete T-message to the user only if a suitable warning of incompleteness is included with the data presented to the user.

Note: if the T-message has been fully decoded, then it already should have been offered to the user at that time (see 14.4.2.1.1.2).

After expiry of the individual incoming short data timer, the radio unit shall discard the recorded values of the control fields and shall assume that future HEAD and AHY (IDENT2 = SDMI) messages are for another transaction.

The value of TGI is network dependent and shall be programmable as part of a radio unit's personality with a range of 1 to 15 seconds in 1 second steps.  The recommended value for TGI is 5 seconds.  For the permissible value for TA, refer to section 6.

##### 14.4.2.1.2	Revised Protocol

The procedures specified in this section differs from the procedures defined in 14.4.2.1 in that:

a)	The TSC retains in its subscriber database, or by other means, a flag to indicate that the individually addressed radio unit complies with this revised protocol i.e. operates timer TM (see below) and is able to clear timer TGI on receipt of either AHY, IDENT2=SDMI or AHY, D=0 etc.

b)	Once the Radio Unit has responded to an AHY, IDENT2 = SDMI with ACK, QUAL = '0' and has then received an applicable HEAD message, any new AHY, IDENT2 = SDMI indicates the commencement of a new MST.  If the new AHY is received before the completion of the current MST in progress the unit may respond to the new AHY with an appropriate response rejecting it or may abandon the call in progress and discard the message so far received.  The chosen course is forced by network personalisation flags.

c) 	Once the Radio Unit has responded to an AHY, IDENT2 = SDMI with ACK, QUAL = '0' and has then received an applicable HEAD message, a new AHY, IDENT2 = Individual Address indicates the commencement of a speech call.  If the AHY is received before the completion of the current MST in progress the unit may respond to the AHY, IDENT2 = Individual Address with an appropriate response rejecting it or may abandon the call in progress and discard the message so far received.  The choice of action may be made dependent on the state of the 'E' flag in the new AHY message.  The chosen course is forced by network personalisation flags.

b)	Once the Radio Unit has responded to an AHY, IDENT2 = SDMI with ACKI, QUAL = '1' and has then received an applicable HEAD message, any new AHY, IDENT2 = SDMI indicates the commencement of a new MST.  The unit may respond to the AHY with an appropriate response rejecting the ahoy or may abandon the call in progress and discard the message so far received dependent on the state of the 'E' flag in the AHY message.  The chosen course is forced by network personalisation flags.

dc:	For individually addressed MSTs, once all segments of an MST have been received and the radio unit has sent a final acknowledgement, the receipt of an AHY, with any value of IDENT2, from the TSC indicates that the acknowledgement has been received by the TSC, which is commencing a new transaction with the unit.  The Radio Unit therefore terminates all timers associated with the previous message and responds to the TSC with an acknowledgement to indicate that it is accepting the ahoy as the commencement of a new transaction.

ed)	A new timer, TM, has been introduced to avoid the use of TA.

fe)	The radio unit may accept a new MST for each address which it contains whilst in the process of assembling a message in progress.  i.e. the unit's individual address and each of its group addresses.  The radio unit may limit the number of group addresses for which it accepts MSTs simultaneously to any number.  Note that the unit risks the chance of missing a segment if the TSC sends an applicable group HEAD message in the slot immediately after an applicable individual HEAD message because the radio unit will be acknowledging the individually addressed message.

###### 14.4.2.1.2.1	Called Unit Response to AHY Message

If a radio unit on a control channel receives an AHY message with:

- POINT set to '0',

- PFIX/IDENT1 as its individual address,

- D='1', and

- IDENT2 set to SDMI,

then it shall respond by sending a suitable acknowledgement; see !!9.2.2.2B!!.

The unit shall start timer TM and shall wait for further signalling; see also sections 14.4.2.1.2.2, 14.4.2.1.2.3, 14.4.2.1.3 and 14.4.2.1.4.

If the radio unit is waiting in timer TM for further signalling in an MST and receives an AHY, POINT = '0' it shall may respond with either:

-	ACKV (QUAL = 1) to indicate that it is busy or

-	with an acknowledgement appropriate to the AHY and abandon any MST which is in progress.  See 14.2.1.2 b) above.

###### 14.4.2.1.2.2	Receiving Individually Addressed HEAD Message

If a radio unit on a control channel receives a HEAD message with PFIX1/IDENT1 in the HEAD address codeword matching its individual address then it shall behave as described below.

a.	The radio unit shall accept, reject or solicit a repeat of that message by responding with a suitable acknowledgement (respectively ACK (QUAL = 0), ACKX or ACKV (QUAL = 1), or ACKB (QUAL = 1)).  See !!14.3.1.1!! and also points b. to f. below.

b.	If the radio unit is in a state of waiting for further HEAD messages (see section 14.4.2.1.2.3) then it shall reject the received segment by responding with ACKX (QUAL = 1) if PFIX2/IDENT2 in the HEAD address codeword does not match PFIX2/IDENT2 from previously received HEAD messages for that transaction to which it responded with ACK (QUAL = 0) or ACKB (QUAL = 1).

c.	The radio unit shall note the value of the I flag in the MESS field of the first data codeword.

If the radio unit:

-	is not in a state of waiting for further HEAD messages (see section 14.4.2.1.2.3), or

-	is awaiting retransmission of a HEAD message with I set to '1' which it solicited using ACKB (QUAL = 1), or

-	is in a state of waiting for further HEAD messages, having received a HEAD message with I  set to '1' and NSEG set to '00',

then it shall reject the received HEAD message if the I flag is set to '0' by responding ACKX (QUAL = 1).

If the radio unit has sent ACK (QUAL = 0) or ACKB (QUAL = 1) in response to a previously received HEAD message for that transaction containing I set to '0', then it shall reject the received HEAD message if the I flag is set to '1' by responding ACKX (QUAL = 1).

d.	The radio unit shall note the value of LEN in the HEAD address codeword.  For LEN = '00' or '01' the radio unit shall not expect to receive NSEG in the appended data codewords but shall behave as though its value had been '00' (see !!15.6.2!!).

e.	The radio unit shall record the value of the NSEG control field in the third data codeword if the HEAD message contains three or four data codewords.

If the radio unit is in a state of waiting for further HEAD messages (see section 14.4.2.1.2.3) then:

-	the HEAD message shall be deemed to be a repeat transmission of the last received HEAD message if both have identical values of NSEG;

-	the segment contained in the HEAD message shall be deemed to be the first transmission thereof to be received by the radio unit if the value of NSEG has been decremented by one binary count from that of the last received HEAD message.

-	If a head message is received whose value of NSEG is not consistent with the correct segment order for the T message then the radio unit shall transmit ACKX(QUAL=1).

The radio unit shall reject any received HEAD message by sending ACKX (QUAL = 1) where the value of NSEG:

-	has been incremented since the last received HEAD message, or

-	has been decremented by more than one binary count since the last received HEAD message, or

-	has been decremented and the previous segment has not been decoded successfully.

If the radio unit is not in a state of waiting for further HEAD messages then any value of NSEG is valid.

f.	If the HEAD message contains one or more corrupted data codewords, the radio unit may extract uncorrupted data codewords from this and subsequent retransmissions thereof, until it is able to assemble a complete and uncorrupted segment.  If a retransmission of the HEAD message is required, then the radio unit shall respond with ACKB (QUAL = 1).

When the segment has been assembled, the radio unit shall respond with either ACK (QUAL = 0) or ACKX (QUAL = 1) as appropriate.

g.	If the radio unit responds to the HEAD message with ACK (QUAL = 0) or ACKB (QUAL = 1), then the unit shall start timer TM and shall wait for further signalling unless, when responding ACK (QUAL = 0), the NSEG field is set to '00' or there is no third data codeword in the segment twhen the unit shall start timer TGI; see also section 14.4.2.1.2.3.

For other responses, the radio unit shall leave its individual incoming short data timer in its existing state (either running, or not set, as appropriate).

The radio unit shall offer the complete T-message to the user when a HEAD message with NSEG = '00' has first been successfully decoded (or its associated segment  has been assembled as in point f. above).  The T-message shall be formatted in accordance with the GFI bits in the MESS field.  The radio unit shall then continue to wait as defined in section 14.4.2.1.2.3.

h.	Once a radio unit has received a HEAD message with the NSEG field set to any value other than '00' it may reject any AHY message containing IDENT2 = SDMI with ACKX (QUAL = 1) and continue to wait for the next HEAD message or abandon the transaction in progress, abandon the message so far received and commence a new transaction dependent on the state of the 'E' flag on the AHY message.  (See 14.4.2.1.2, b)).

###### 14.4.2.1.2.3	Individual Time-out TGI/TM

If the Network Personalisation Flag is set to '1' (See 14.4.2.1.) Rradio units shall operate an individual incoming short data timer which shall be set to either of two different values as described below.

The timer shall be set to a value TM and started or restarted when the radio unit responds with ACKI (QUAL = 01) to an AHY message with IDENT2=SDMI.  The timer shall also be set to a value TM and started or restarted when the radio unit responds with ACK (QUAL = 0) to a HEAD message which contains a value other than '00' in the NSEG field or ACKB (QUAL = 1) to any HEAD message.  The timer shall be set to a value TGI and started or restarted when the radio unit responds with ACK (QUAL = 0) to a HEAD message which contains a value of '00' in the NSEG field or where the segment does not contain a third data codeword.  (i.e. if the message is an SST or for the last segment of an MST.)

A radio unit is in a state of waiting for further signalling for an incoming individually addressed short data transaction if its individual incoming short data timer is running.  A radio unit is in a state of waiting for further HEAD messages if this timer is running and the radio unit has received and responded to an earlier HEAD message with ACK (QUAL = 0) or ACKB (QUAL = 1) during that transaction.

A radio unit waiting for further signalling for an incoming individually addressed short data transaction (SST or MST) shall assume that no further signalling will be received for that transaction if its individual incoming short data timer expires.

Thereupon, if the T-message has not been fully decoded, the short data transaction shall be deemed to have failed and the radio unit may generate an appropriate indication to the user.  The radio unit may offer an incomplete T-message to the user only if a suitable warning of incompleteness is included with the data presented to the user.

Note: if the T-message has been fully decoded, then it already should have been offered to the user at that time (see 14.4.2.1.2.2).

After expiry of the individual incoming short data timer, the radio unit shall discard the recorded values of the control fields and shall assume that future HEAD and AHY (IDENT2 = SDMI) messages are for another transaction.

The value of TM is network dependent and shall be programmable as part of a radio unit's personality with a range of 1 to 60 seconds in 1 second steps.  The recommended value for TM is 15 seconds.  See Section 6.

The value of TGI is network dependent and shall be programmable as part of a radio unit's personality with a range of 1 to 60 seconds in 1 second steps.  The recommended value for TGI is 5 seconds.  See Section 6.

If a final or single segment is sent which contains only two data codewords following the HEAD codeword then NSEG is not transmitted.  In this case the radio unit shall behave as if NSEG is set to '00'.

When a radio unit has responded with ACK (QUAL=0) to a HEAD message containing NSEG='00' and it receives AHY, with IDENT1 set to the unit's individual address it shall cease to wait for further signalling for the short data message it has just received.  If IDENT2=SDMI the radio unit shall start the individual incoming short data timer with a value of TM for a new message.  (During the time TGI after the final HEAD message an AHY message sent by the TSC indicates that it has received the ACK as well as announcing a new message).

##### 14.4.2.1.3	Receiving AHYX Message

If, whilst waiting for further signalling for an incoming individually addressed short data transaction, a radio unit receives an AHYX message with:

- PFIX/IDENT1 as its individual address, and

- IDENT2 set to SDMI

then it shall respond with ACK (QUAL = 1) as defined in !!9.2.2.4!! and shall cease to wait for further signalling.  It shall terminate the individual incoming short data timer (TGI, TM and TA), discard any segments decoded in connection with that transaction, discard the recorded values of the control fields and assume that future AHY (IDENT2 = SDMI) and HEAD messages are for another transaction.

This use of AHYX is in addition to the MPT1327 prescribed functions.

##### 14.4.2.1.4	Ignoring Group Call GTC Messages

A radio unit waiting for further signalling for an incoming individually addressed short data transaction shall ignore any received GTC message unless the unit is individually addressed by that GTC message (either by PFIX/IDENT1 or by PFIX/IDENT2) or IDENT1 is set to ALLI.

This requirement is in addition to the procedure in !!9.2.2.5!!.

##### 14.4.2.1.5	Maintaining Timers

A radio unit waiting for further signalling for an incoming short data transaction shall maintain any relevant timers, including its individual incoming short data timer, as prescribed in section 9.4.2, when leaving the control channel on which it is currently active.

#### 14.4.2.2	Receiving Short Data Addressed to a Group

The procedures for receiving group call short data messages remains unchanged in this revision of the specification.

##### 14.4.2.2.1	Receiving HEAD Message Addressed to a Group

If a radio unit on a control channel receives a HEAD message with PFIX2/IDENT2 not matching its individual address and:

- PFIX/IDENT1 matching one of its group addresses for that system,   or

- IDENT1 set to the system-wide all-call ident ALLI

then it may follow the procedures outlined in this section but shall transmit no response.  For the purposes of this section, the term "group-addressed HEAD message" shall mean any HEAD message so addressed.

If a radio unit receives a group-addressed HEAD message containing the I flag set to '1' and NSEG is set to '00' or not included, then this message constitutes an SST.  If a radio unit receives a group-addressed HEAD message containing NSEG set to a value other than '00' or I is set to '0' then this message constitutes part of an MST.

If a radio unit receives a group-addressed HEAD message whilst in the state of waiting for further signalling for a short data transaction addressed to that group, it shall ignore that message if the calling address PFIX2/IDENT2 does not match PFIX2/IDENT2 from any previously accepted HEAD message for that transaction.

If a received group-addressed HEAD message contains one or more corrupted data codewords, the radio unit may extract uncorrupted data codewords from this and subsequent retransmissions thereof, until it is able to assemble a complete and uncorrupted segment.  The radio unit shall use the fields PFIX1/IDENT1, PFIX2/IDENT2, LEN, I and NSEG to identify repeat transmissions and to determine whether all segments of a T-message have been decoded as follows:

a.	If a received HEAD message contains the I flag set to '0' and the radio unit is not in the state of waiting for further signalling for a short data transaction addressed to this group, then the unit shall deem that insufficient segments will be received for that transaction to assemble a complete T-message. b.	The radio unit shall note the value of LEN in the HEAD address codeword of a received group-addressed HEAD message.  For LEN = '00' or '01' the radio unit shall not expect to receive NSEG in the appended data codewords but shall behave as though its value had been '00' (see !!5.6.2!!).

b.	The radio unit shall note the value of LEN in the HEAD address codeword of a received group-addressed HEAD message.  For LEN = '00' or '01' the radio unit shall not expect to receive NSEG in the appended data codewords but shall behave as though its value had been '00' (see !!5.6.2!!).

c.	The radio unit shall record the value of the NSEG control field in the third data codeword of a received group-addressed HEAD message if it contains three or four data codewords.

If the radio unit is in a state of waiting for further signalling for a short data transaction for that group (see section 14.4.2.2.2) then:

- 	the HEAD message shall be deemed to be a repeat transmission of the last received HEAD message for that group if both have identical values of NSEG.

-	the segment contained in the HEAD message shall be deemed to be the first transmission thereof to be received by the radio unit if the value of NSEG has been decremented from that of the last received HEAD message for that group.

-	If a head message is received whose value of NSEG is not consistent with the correct segment order for the T message then the radio unit shall transmit ACKX(QUAL=1).

The radio unit shall deem that insufficient segments will be received to assemble a complete T-message where the value of NSEG:

-	has been decremented by more than one binary count since the last received HEAD message, or

-	has been decremented and the previous segment has not been decoded successfully.

After accepting a group-addressed HEAD message the radio unit shall then wait for further HEAD messages for that transaction (see 14.4.2.2.2).

When a HEAD message with NSEG set to '00' has first been successfully decoded (or its associated segment has been assembled) then, if the T-message is complete, the unit shall offer the T-message to the user.  Otherwise, if the T-message is incomplete, the radio unit may offer the received data to the user only if a suitable warning of incompleteness is included with the data presented to the user.  The T-message or segments thereof shall be formatted in accordance with the GFI bits in the MESS field.  The radio unit shall then continue to wait, as defined in section 14.4.2.2.2.

##### 14.4.2.2.2	Group Time-out TGG

A called radio unit waiting for further signalling for a short data transaction addressed to a group shall assume that no further signalling will be received for that transaction (SST or MST) if a time TGG has elapsed since the last HEAD message it received for that transaction.  The radio unit shall discard the recorded values of the control fields and shall assume that any future HEAD messages are for another transaction.

Thereupon, if the T-message has not been fully decoded, the short data transaction shall be deemed to have failed.  The radio unit may offer an incomplete T-message to the user only if a suitable warning of incompleteness is included with the data presented to the user.

The value of TGG is network dependent and shall be programmable as part of a radio unit's personality with a range of 1 to 30 seconds in 1 second steps.  The recommended value for TGG is 10 seconds.

##### 14.4.2.2.3	Maintaining Timers

A radio unit waiting for further signalling for an incoming short data transaction shall maintain any relevant timers, including its group incoming short data timer, as prescribed in section 9.4.2, when leaving the control channel on which it is currently active.

## 14.5	An Example of The Procedure for Extended Data Messages

In order to illustrate the use of the procedure in section 14, a typical message interchange is shown in Figure 14.1.  The example shows a successful Multiple Segment Transaction linking three segments.  An SST would be similar to the transmission of the first segment of the MST, but with the appropriate field values changed in the HEAD message.  This example is illustrative only and does not form part of the specification.

In the example, an availability check is carried out on the called radio unit and some segment repeats are shown.

Figure 14.1 - Example of Successful MST Transaction

| REQUEST I NG PART Y | ¾¾¦ 1 ¦¾® ¬¾¦ 4 ¦¾¾ ¾¾¦ 5 ¦¾® ¬¾¦ 6 ¦¾¾ ¾¾¦ 7 ¦¾® ¬¾¦ 8 ¦¾¾ ¾¾¦ 9 ¦¾® ¬¾¦ 10 ¦¾¾ ¾¾¦ 11 ¦¾® ¬¾¦ 21 ¦¾¾ | T S C | ¾¾¦ 2 ¦¾® ¬¾¦ 3 ¦¾¾ ¾¾¦ 12 ¦¾® ¬¾¦ 13 ¦¾¾ ¾¾¦ 14 ¦¾® ¬¾¦ 15 ¦¾¾ ¾¾¦ 16 ¦¾¾ ¾¾¦ 17 ¦¾® ¬¾¦ 18 ¦¾¾ ¾¾¦ 19 ¦¾® ¬¾¦ 20 ¦¾¾ | C A L L E D P A R T Y |
| --- | --- | --- | --- | --- |

Descriptions of message interchanges

|1|:	An RQC message requests a short data transaction   (Assuming extended addressing is not required).  The value of SLOTS in the RQC is '11', indicating that three slots are required for the HEAD message containing the first segment of the MST.

|2|:	An AHY (POINT=0) message checks the availability of the called radio unit (optional).

|3|:	An ACK (QUAL=0) message indicates general acknowledgement of the AHY.

|4|:	Any AHYC message is sent to the requesting radio unit with DESC set to '100'. The first bit of DESC indicates that MSTs are supported.  The second and third bits indicate that the first segment is required.  SLOTS is set to '11' to indicate that three slots have been reserved for the HEAD message containing the first segment of the T-message.

|5|:	A HEAD message containing the first segment of the MST is sent.  The GFI field indicates the format of the T-message data.  NSEG = '10' indicates that two more segments are yet to be sent.  CSEG = '1' indicates that three slots are required for the HEAD message containing the next segment.

|6|:	An AHYC message with DESC '101' solicits a HEAD message containing the second segment of the MST. SLOTS is set to '11' to indicate that three slots have been reserved for the HEAD message.

|7|:	A HEAD message containing the second segment of the ST is sent.  NSEG = '01' indicates that one more data segment is to be sent.  CSEG = '0' indicates that two slots are required for the HEAD message containing the next segment.

|8|:	An AHYC message with DESC = '101' solicits a repeat of the second segment of the MST.  SLOTS is set as in |6|.

|9|:	A HEAD message containing the second segment is retransmitted as in |7|.

|10|:	An AHYC message with DESC = '110' solicits a HEAD message containing the third segment of the MST. SLOTS is set to '10' to indicate that two slots have been reserved for the HEAD message.

|11|:	A HEAD message containing the third segment of the MST is sent.  Since the message only contains two data codewords, NSEG (which is transmitted in the third codeword when present) is not sent.  The TSC behaves as though its value had been '00'.

|12|:	The TSC sends a HEAD message containing the first segment of the MST to called party.  LEN is set to '11' to indicate that four data codewords are appended to the HEAD codeword and NSEG in the third appended data codeword is set to '10' to indicate that two segments of the MST are to follow.

|13|:	An ACKB (QUAL=1) message indicates that the called party required the first segment to be transmitted.

|14|:	The TSC sends a HEAD message containing a repeat of the first segment as in |12|.

|15|:	An ACK (QUAL=0) message indicates successful receipt of the HEAD message by the called radio unit.

|16|:	The TSC sends a HEAD message containing the second segment of the MST.  LEN is set to '11' to indicate that four data codewords are appended to the HEAD codeword and NSEG in the third appended data codeword is set to '01' to indicate that one segment of the MST is to follow.

|17|:	No acknowledgement is received by the TSC in the subsequent slot so a HEAD message containing a repeat of the second segment is transmitted as in |16|.

|18|:	An ACK (QUAL=0) message indicates successful receipt of the HEAD message by the called radio unit.

|19|:	The TSC sends a HEAD message containing the final segment of the MST. LEN is set to '01' to indicate that two data codewords are appended to the HEAD codeword.  Since the message only contains two data codewords NSEG (which is transmitted in the third codeword when present) is not sent.  The radio unit behaves as though its value had been '00'.

|20|:	An ACK (QUAL=0) message indicates successful receipt of the HEAD message by the called radio unit.

|21|:	An ACK (QUAL=0) message is sent by the TSC to the calling party to indicate that the transaction has been successfully completed.  This acknowledgement may be  repeated for reliability.