9.	RADIO UNIT PERFORMANCE TESTS

The following tests are to be performed by the manufacturer of the radio unit. No limits have been specified as the purpose of the tests is to provide information to a network operator for the purposes of service area classification.

9.1	Traffic Channel Sensitivity

9.1.1	Test Numbers

90101C

90102C

90103C

9.1.2	Purpose of Test

To measure the minimum signal level of which a SINAD measurement of 12dB (unweighted) is obtained from the audio output of the radio unit receiver if the mute is open at the required signal level of the level at which the mute opens if higher.

9.1.3	Initialisation

The radio unit shall use the C personalisation.  The tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 286.  The radio unit shall be idle on this control channel immediately prior to the test.

9.1.4	Test Method

At the appropriate time in each test the tester shall cause the General Test Assembly to radiate the following traffic channels at a level of -100dBm, modulated by 1000 Hz tone at a level to give a peak deviation of 60% of maximum permitted frequency deviation, in each named test:

TEST 	CHAN FIELD	CHAN FIELD	CHANNEL

for sub-band 1	for sub-band 2	NUMBER

90101C	'1011000010'	'0011000010'	251

90102C	'1000000001'	'0000000001'	 58

90103C	'1111110111'	'0111110111'	560

The tester shall cause the General Test Assembly to generate codeword 208 with CHAN set to the value appropriate to each test as listed above.  When the tester hears tone in the radio unit loudspeaker he shall set the audio power output to 50% of the manufacturer's declared maximum output.  He shall then reduce the signal until the receiver mute closes and then immediately increase the signal strength to re-open the mute.  The tester shall then measure SINAD and, if necessary, increase the signal strength until SINAD is 12 dB.  The r.f. signal level at this point shall be recorded.  At the end of each test the tester shall clear the radio unit from the traffic channel by means of the front panel controls.

9.1.5	Limits

None.

9.2	Control Channel Acquisition Speed

9.2.1	Test Numbers

90201C

9.2.2	Purpose of Test

To obtain a measure of the time required to confirm the control channel successfully.

9.2.3	Initialisation

The radio unit shall utilise the C personalisation.  The tester shall connect the radio unit to the General Test Assembly which shall be radiating one active (Active 1) and two dummy forward control channels.  A further active control channel (Active 2) shall be radiated during periods of the test.  On each of the control channels, the system codeword shall be codeword 102 with SYS and CCS set as indicated:

Channel	SYS			CCS	Signal

No					Level

Active 1	286	'101010110100010'	'0010111011010010'		-	109dBm

Active 2	286	'101010110100100'	'1011111001010000'		-	109dBm

Dummy 1 	107	'100010100111001'	'0011001011100110'		-	85dBm

Dummy 2	377	'100101011000001'	'1110101101001001'		-	85dBm

The radio unit shall be in the idle state on the active channel immediately prior to the commencement of the test.

9.2.4	Test Method

Cease radiating the Active 1 forward control channel for a period of 15 seconds.  Radiate the Active 2 forward control channel.  Note the time between the commencement of the radiation of the Active 2 forward control channel and the first transmission by the radio unit of codeword 501.  The General Test Assembly shall respond to each transmission of codeword 501 with codeword 401 in accordance with 7.2.4. and 8.2.1. of MPT 1327. Repeat the test a further 9 times noting the results.

Record the mean of the noted times.

9.3	Control Channel Data Decode Sensitivity

9.3.1	Test Numbers

90301D

90302D

90303D

9.3.2	Purpose of the Test

To measure the minimum increasing signal level at which the radio unit can successfully confirm a control channel.

9.3.3	Initialisation

At the appropriate time in each test the tester shall cause the General Test Assembly to radiate the following forward control channels in each named test:

Test No:	90301D	Channel number 286

Test No:	90302D	Channel number  58

Test No:	90303D	Channel number 560

The radio unit shall utilise the D personalisation.  The tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel at a level of -100dBm on the designated channel number.  The radio unit shall be in the idle state on this control channel immediately prior to this test.

9.3.4	Test Method

The tester shall cause the General Test Assembly to generate codeword number 606 to obtain a response from the radio unit.  When the radio unit responds to codeword 606 the tester will then reduce signal strength of the radiated forward control channel in 5dB steps.  At each step the tester shall send up to 10 off codeword 606 until the radio unit responds.  When the radio unit fails to respond to all 10 codewords the tester shall reduce the signal further 10dB and repeat the test whilst increasing the signal strength in 1 dB steps.  For each step the tester shall dwell for at least the period given in the result of test 90301C for this radio unit, prior to causing the General Test Assembly to generate codeword 606.  The test continues until the unit responds to at least 5 off codeword 606 out of the 10 which are sent at which point the tester shall record the strength of the signal being sent into the radio unit from the General Test Assembly.

9.3.5	Radio Unit Response

The radio unit shall respond to codeword 606 with codeword 407 during the time that it is confirmed on the forward control channel.

9.3.6	Limits

None.