MPT 1352

MPT 1352

Test schedule for the approval of

radio units to be used with

commercial trunked networks

operating in

Band III sub-bands 1 and 2

September 1991

Reissued and reprinted xxxxxxxxx1996

FOREWORD

This specification describes the test procedures to be performed on a radio unit model which is required to conform to MPT1343, "System interface specifification for radio units to be used with commercial trunked networks operating in Band III, sub-bands 1 and 2". In addition, Appendix A outlines the procedures to be carried out to gain type approval of a radio unit.

Appendix B is a sample application form.

A number of functions and facilities are not formally tested. However the manufacturer is expected to ensure as far as is practicable that radio units conform in all respected to MPT1343. A list of the untested items is given in Appendix C.

Intellectual Property Rights

Firms intending to manufacture equipment which complies with the specification should be aware that certain features of the specification are subject to IPR claims.

All firms are therefore advised that they should make appropriate enquiries through their Patent Agents before proceeding.

CONTENTS

SCOPE

1.	INTRODUCTION

1.1	Purpose of these Tests

1.2	Other Relevant Specifications and Documents

2.	STANDARD TERMS AND DEFINITIONS

2.1	Defined Terms used in the Specification

2.2	Test Numbering System

2.3	Default Test Conditions

2.4	Radio Unit Personality Change

3.	ITEMS TO BE SUPPLIED PRIOR TO TEST

3.1	Network Parameters

3.2	Radio Unit Personalisation

3.3	Additional Radio Unit Preparation

3.4	Documentation

4.	CONDITIONS OF MEASUREMENT

4.1	Standard Test Conditions

4.2	General Test Assembly

5.	PARAMETRIC TESTS

5.1	Relationships to MPT 1323

5.2	Transmitter Tests

5.3	Receiver Tests

6.	BASIC PROTOCOL TESTS

6.1	Introduction

6.2	Passive Tests

6.3	Active Tests

7.	OPERATIONAL SEQUENCE MESSAGE TESTS

7.1	Type of Aloha Frame

7.2	Acquiring and relinquishing a control channel

7.3	Not Used

7.4	Registration

7.5	Security Number Checking

7.6	Call Set-up

7.7	Traffic Channel Procedures

7.8	System Parameter Calls - (not included in this issue)

7.9	Emergency Calls - (not included in this issue)

7.10	Call Cancellation

7.11	Call Back (ACKB)

7.12	Call Include - (not included in this issue)

7.13	Call Diversion - (not included in this issue)

7.14	Short Data  Message Procedures - (not included in this issue)

7.15	Optional Status Calls - (not included in this issue)

7.16	Time-shared Tests

8.	CHECK FOR PERSONALITY CHANGE - (not included in this issue)

9.	RADIO UNIT PERFORMANCE TESTS

9.1	Traffic Channel Sensitivity

9.2	Control Channel Acquisition Speed

9.3	Control Channel Data Decode Sensitivity

10.	CLASSIFICATION

APPENDIX A		ARRANGEMENTS FOR APPROVAL

APPENDIX B		DECLARATION

APPENDIX C		TESTS NOT COVERED IN THIS ISSUE

SCOPE

This specification contains the type approval requirements for radio units which are required to conform to MPT 1343.

1.	INTRODUCTION

1.1	Purpose of these Tests

The tests contained in this specification have been designed with the intention of establishing that the radio units will operate correctly with the infrastructure equipments conforming to MPT1347.

For sub-band 1 the radio units operate over the radio frequency band 185.2125 MHz to 191.4875 MHz (radio unit transmit band) and 177.2125 MHz to 183.4875 MHz (radio unit receive band).

For sub-band 2 the radio units operate over the radio frequency band 193.2125 MHz to 199.4875 MHz (radio unit transmit band) and 201.2125 MHz to 207.4875 MHz (radio unit receive band).

The nominal channel separation between adjacent carrier frequencies shall be 12.5 kHz.

Further, as described in MPT1343, all radio units include certain basic facilities and properties in a standard manner, whilst certain other optional facilities and properties are permitted only if provided in a standard manner to ensure ready transfer from network to network. Any given radio unit need only meet those requirements that are mandatory and  those appropriate to the standard options included in the unit.

1.2	Other Relevant Specifications and Documents

This section describes the specifications and documents that are associated with the trunking scheme adopted for use in sub-bands 1 and 2 of Band III in the United Kingdom. The following specifications and documents must be taken together to describe the overall trunking systems.

| MPT1317 (1981) | Code of practice for the transmission of digital information over land mobile radio systems |
| --- | --- |
| MPT1318 (1986) | Engineering memorandum: Trunked systems in the land mobile service |
| MPT1323 (1986) | Angle modulated radio equipment for use at base and mobile stations in the private mobile radio service operating in the frequency band 174-225 MHz |
| MPT1327 (1988) | A signalling standard for trunked private land mobile radio systems |
| MPT1331 (1987) | Code of Practice for radio site engineering |
| MPT1343 (1988) | System interface specification for radio units to be used with commercial trunked networks operating in Band III sub-bands 1 and 2 |
| MPT1347 (1988) | Radio interface specification for commercial trunked networks operating in Band III sub-bands 1 and 2 |