3.	ITEMS TO BE SUPPLIED PRIOR TO TEST

This section outlines the arrangements and previously prepared information needed for the type approval tests.

Radio Units shall be presented for type approval with at least the five sets of personality information noted in this specification (section 3.1 and 3.2.) prepared.  This shall either be already included in the unit or arrangements shall be made to ensure a speedy change at the appropriate stage in the tests.

The tests listed in this specification shall normally be conducted with the radio until set to operate with personality "A".  Then, when it is required to establish the ability of the radio unit to migrate to another network, the operating personality shall be changed to "B".  The majority of the tests are then repeated using this second personality.

Network operator classification tests are conducted with the radio unit set to operate with personality "C" or "D".  In order to conduct tests for time-shared operation, the radio unit shall be set to operate with personality "E".

For some tests it is necessary that this "default" information is changed to conduct the test. This is descried in section 3.3. more fully.

There is no limit to the number of personalities that a radio unit may have.

3.1.	Network Parameters

All radio units operating in a MPT 1327 trunked environment must have pre-programmed information on the "network" on which they are to make calls.  In addition to the values assigned to the MPT 1327 system variables in Appendix B of MPT 1343 the following parameters must have values assigned or a statement made to the effect that the relevant facility is not implemented and that appropriate measures have been taken (see 3.5.2.).

3.1.1.	Acquisition Authorization Data

A,C,D,E 		B

1.	'001'	Zone	1.	'000'	Area

2.	'0100010'	Area	2.	'001'	Area

3.	'0100100'	Area	3.	'010'	Area

4.	'0100110'	Area	4.	'011'	Area

5.	'0101000'	Area	5.	'100'	Area

6.	'0101010'	Area	6.	'101'	Area

7.	'010110000'	Full Code	7.	'1100'	Full Code

8.	'011001010'	Full Code	8.	No Entry Listed

3.1.2.	Zone Sub-Field Length (LZ)

A,C,D,E = 3 bits    B = 0 bits

3.1.3.	Area Sub-Field Length (LA)

A,C,D,E = 7 bits    B = 3 bits

3.1.4.	Network Identity Code

A,C,D,E (Net) = '01'  B (OPID)  =  '0010100'

3.1.5.	Lowest Channel Number in the Network

A,C,D,E = 58   B = 157

3.1.6.	Highest Channel Number in the Network

A,C,D,E = 560 B = 366

3.1.7.	Size of the Normal Hunt

A,C,D,E = 32 channels  B = 20 channels  D = 3 channels

3.1.8.	Normal Hunt Channel Numbers

The CHAN/CONT values correspond to Band III Sub Band 2. For Band III Sub Band 1, replace the leading binary digit '0' by '1'.

A,E	CHAN/CONT VALUE	B	CHAN/CONT VALUE

*1	 58	'0000000001'	157	'0001100100'

2	 62	'0000000101'	160	'0001100111'

3	 64	'0000000111'	163	'0001101010'

4	 77	'0000010100'	166	'0001101101'

5	 79	'0000010110'	169	'0001110000'

6	 85	'0000011100'	172	'0001110011'

7	 87	'0000011110'	175	'0001110110'

8	107	'0000110010'	178	'0001111001'

9	117	'0000111100'	181	'0001111100'

10	120	'0000111111'	247	'0010111110'

11	123	'0001000010'	254	'0011000101'

12	126	'0001000101'	281	'0011100000'

13	129	'0001001000'	284	'0011100011'

14	132	'0001001011'	287	'0011100110'

15	135	'0001001110'	290	'0011101001'

16	138	'0001010001'	293	'0011101100'

17	177	'0001111000'	357	'0100101100'

18	180	'0001111011'	360	'0100101111'

19	186	'0010000001'	363	'0100110010'

20	189	'0010000100'	366	'0100110101'

21	217	'0010100000'

22	219	'0010100010'

23	258	'0011001001'

24	278	'0011011101'

25	282	'0011100001'

26	286	'0011100101'

27	358	'0100101101'

28	361	'0100110000'

29	364	'0100110011'

30	367	'0100110110'

31	372	'0100111011'

32	377	'0101000000'

* Table location 1 (channel Number 58) shall have its time shared bit set for personality 'E'.

3.1.8a	Normal Hunt Channel Numbers

C	CHAN/CONT VALUE	 D	CHAN/CONT VALUE

1	 58	'0000000001'	 58	'0000000001'

2	 62	'0000000101'	286	'0011100101'

3	 64	'0000000111'	560	'0111110111'

4	 77	'0000010100'

5	 79	'0000010110'

6	 85	'0000011100'

7	 87	'0000011110'

8	107	'0000110010'

9	117	'0000111110'

10	120	'0000111111'

11	123	'0001000010'

12	126	'0001000101'

13	129	'0001001000'

14	132	'0001001011'

15	135	'0001001110'

16	138	'0001010001'

17	177	'0001111000'

18	180	'0001111011'

19	186	'0010000001'

20	189	'0010000100'

21	217	'0010100000'

22	219	'0010100010'

23	258	'0011001001'

24	278	'0011011101'

25	282	'0011100001'

26	286	'0011100101'

27	258	'0100101101'

28	361	'0100110000'

29	364	'0100110011'

30	367	'0100110110'

31	372	'0100111011'

32	560	'0111110111'

3.1.9.	Non-Applicable Channel Numbers in Comprehensive Hunt

There are no non-applicable channel numbers.

3.1.10	Suppression of the Comprehensive Hunt

A = No	B,C,D,E = Yes

3.1.11.	Value of the Home Zone

A,C,D,E = '010'    B = Not Applicable

3.1.12.	Value of INFO in RQR

A,B,C,D,E = 0

3.1.13.	Fall Back Channel

Not tested in this issue.

3.1.14.	Value of NDD in SYS Code if Fall Back is Fitted

Not tested in this issue.

3.1.15.	Value of NC1

Channels with TSI = 0

A,C = 30 codewords  B = 2 codewords  D = 20 codewords

Channels with TSI = 1

E= 67 codewords

3.1.16.	Value of NC2

Channels with TSI = 0

A,C = 32 codewords B = 17 codewords   D = 10 codewords

Channels with TSI = 1

E = 67 codewords

3.1.17.	Value of NT (include Calls Only)

A,C,D,E = 206 bit periods   B = 309 bit periods

3.1.18.	Value of NV

Channels with TSI = 0

A,C,D = 1 CCSC   B = 1 CCSC

Channels with TSI = 1

E = 1 CCSC

3.1.19.	Value of NX1

Channels with TSI = 0

A,C = 2 codewords   B = 1 codeword  D = 18 codewords

Channels with TSI = 1

E = 52 codewords

3.1.20.	Value of NX2

Channels with TSI = 0

A,C = 3 codewords   B = 2 codewords  D = 9 codewords

Channels with TSI = 1

E = 52 codewords

3.1.21.	Value of NZ1

A,B,C,D,E = 1 sample

3.1.22.	Value of NZ2

A,C,E = 4 samples   B,D = 2 samples

3.1.23.	Value of TC

A,C,D,E = 60 sec B = 30 sec

3.1.24.	Value of TD

A,C,D,E = 5 min    B = 0

3.1.25.	Value of TJ

A,C,D,E = 20 sec   B = 30 sec

3.1.26.	Value of TN

A,C,D,E = 7 sec    B = 3 sec

3.1.27.	Value of TS

A,C,E = 5 sec   B = 10 sec   D = 2 sec

3.1.28.	Value of TT

A,C,D,E = 60 sec    B = 10 sec

3.1.29.	Pressel Messages

If the radio unit manufacturer declares that the unit implements the option the send multiple messages then the following parameters shall be set. Otherwise the unit shall conform to the mandatory requirement.

3.1.29.1.	Value of NPON

A,C,D,E = 2   B = 1

3.1.29.2.	Value of NPOFF

A,C,D,E = 3   B = 1

3.1.30	Optional Hunting Routines

If the radio unit manufacturer declares that the radio unit implements the "preferential NDD hunt stage", the "preferential sampled hunt stage" or the "background search sequence" then the following parameters shall be set, where applicable, as indicated.  The unit is set to inhibit all such hunts to ensure that all tests operate as indicated in the Test Schedule; NA indicates that the setting is not applicable:

3.1.30.1.	Value of NDD preference data

A,B,C,D,E  = NA

3.1.30.2.	Preferred NDD sub-field lengths

A,B,C,D,E = All sub-field lengths set to 0

3.1.30.3.	Preferred NDD priority indicator

A,B,C,D,E = NA

3.1.30.4.	LM1 Level margin

A,B,C,D,E = NA

3.1.30.5.	LM2 Level margin

A,B,C,D,E, = NA

3.1.30.6.	LM3 Level margin

A,B,C,D,E = NA

3.1.30.7.	NS - No. of samples in background sampling activity

A,B,C,D,E = NA

3.1.30.8.	TH - Sampling activity duration

A,B,C,D,E = 0

3.1.30.9.	TL - Sampling activity interval

A,B,C,D,E = 0

3.2.	Radio Unit Personalisation

All radio units attempting type approval shall have the following settings.

3.2.1.	Own PFIX

A,C,D,E = 39   B = 100

3.2.2.	Own Individual IDENT

A,B,C,D,E = 2046

3.2.3.	Two or Three Digit Individual Calls

A,C,D,E = 3 digits  B = 2 digits

3.2.4.	Individual Base IDENT (keypad radios)

A,C,D,E = 2004   B = 2034

3.2.5.	Two or Three Digit Group Calls

A,C,D,E = 2 digit   B = 3 digit

3.2.6.	Highest Own Fleet Individual IDENT (keypad radios)

A,C,D,E = 2199   B = 2099

3.2.7.	Group Addresses

The radio unit shall be a member of at least the following group:

A,C,D,E = 39/0605  B = 100/0605

The remaining three groups are not defined.

3.2.8.	Inter-Fleet Group Calls Barred Flag

Networks may require that certain radio units are not allowed to make inter-fleet group calls (see 8.2.3.2.1.).

A,C,D,E = Barred   B = Not Barred

3.2.9.	Group Base IDENT

A,B,C,D,E = 0600

3.2.10.	Highest Own Group IDENT

A,B,C,D,E = 0609

3.2.11.	Control Category

A,C,D,E = CCAT A   B = CCAT B

3.3.	Additional Radio Unit Preparation

In addition to the five sets of personalisation data (A,B,C,D and E) specified in 3.1 and 3.2 some tests in this specification may require additional programming or preparation of radio units.

The tests in section 5 relating to frequency deviation for standard data, receiver frequency tolerance and demodulation characteristic require that the radio unit is set to the same state that was used when the unit was attempting type approval to MPT 1323 (i.e the unit shall not operate in the trunked mode).  This state is referred to in this specification as "manual mode".

Some tests in section 7 require that the radio unit shall be equipped to be able to generate codeword 503 (RQS) as a result of a key operation (or equivalent) by the Tester.  For radio units equipped when keypad dialling facilities as specified in section 8.2. of MPT 1343 the generation of codeword 503 may be achieved by the key sequence 275#.  Radio units not equipped with such keypad dialling facilities shall be programmed with PFIX = 39 IDENT = 2079 as one of the radio unit's calling options, however this is achieved (refer to the manufacturer's data supplied under the provisions of section 3.4.).

Some tests in section 7 require that, if the radio unit provides the appropriate standard options, it shall be equipped to be able to generate codeword 504 (RQS) as a result of a key operation (or equivalent) by the Tester.  For radio units equipped with keypad dialling facilities as specified in section 8.2. of MPT1343 the generation of codeword 504 may be achieved by the key sequence *11*95#.  Radio units not equipped with such keypad dialling facilities shall be programmes with PFIX=39 IDENT 0605 (configured as a broadcast group call request) as one of the radio unit's calling options, however this is achieved (refer to the manufacturer's data supplied under the provision of section 3.4.).

Radio units which are eligible for testing of the facility to accept an incoming call for call back will require to be configured to accept calls in this way for the purposes of one test in section 7.11.

One test in section 9 requires that the radio unit's loudspeaker connections shall be accessible.  Information and any special tools shall be supplied in order to obtain such access.

3.4.	Documentation

The manufacturer shall supply a copy of the radio unit User's Guide for the radio and the following information (which shall conform to MPT1343 where applicable).

Optional Facilities Supplied

Operating Instructions - These shall incorporate a description of the confidence indications used on the radio unit.

Installation Instructions

Instructions to dismantle the radio unit to view the security number mark.

Instruction to dismantle the radio unit to provide access to the loudspeaker connections.

Instructions to Change Network

Contact reference for Technical Questions

Instructions for configuring the radio unit for a required personality.

The manufacturers rated audio output power of the radio unit.

In addition to the information in the user's guide the manufacturer shall supply the values of the fields carrying security number information in the SAMIS response to a security check validation message (AHYC) which are correct for the radio unit supplied for test (refer to codeword 702 in 4.1.3.7.).  The manufacturer shall also supply information concerning the precise location and identity of the component within the radio unit which stores the security number of the type of unit utilised for this storage facility.