4.	CONDITIONS OF MEASUREMENT

4.1	Standard Test Conditions

4.1.1	Test conditions, power supply sources and ambient temperatures

The tests in this specification shall be made under normal test conditions as specified in 2.3 of MPT1323 and also, where stated, under extreme test conditions as specified in 2.4 of MPT1323. Tests under extreme temperature conditions shall be carried out in accordance with the requirements of 2.5 of MPT1323. During the tests the power source of the equipment shall be replaced by a test power source meeting the requirements of 2.2 of MPT1323.

4.1.2	Electrical test conditions and measurement accuracy

Where appropriate to the test specified the electrical test conditions in MPT1323 shall apply and the accuracy of measurements shall conform to the requirements of section 7 of MPT1323.

4.1.3	Standard messages

This section defines the structure of codewords and multi-codeword message sequences to be used in the tests in this specification and assigns values to the fields within codewords. Each codeword and message sequence is allocated a unique three digit number to facilitate reference in the test descriptions.

The following symbolism is used in this section:

*	indicates that the value of a field is specified within the test description and may vary between tests.

'01' 	indicates a value in binary form

10 	indicates a decimal value of a binary field

X 	indicates a decimal digit which is not specified

Where CHAN fields are specified in the codewords, they refer to channels in sub-band 2. For corresponding channels in sub-band 1 replace the leading binary digit '0' by '1'.

4.1.3.1	Control Channel System Codewords (CCSC)

Codeword 101

(CCSC)

| '0' | SYS '101010110000001' | CCS '1100100111001110' | PREAMBLE '1010101010101010' | P '1100010011010111' |
| --- | --- | --- | --- | --- |

Codeword 102

(CCSC)

| '0' | SYS * | CCS * | PREAMBLE '1010101010101010' | P '1100010011010111' |
| --- | --- | --- | --- | --- |

Codeword 103

(CCSC)

| '0' | SYS '101011001010001' | CCS '0111011110110010' | PREAMBLE '1010101010101010' | P '1100010011010111' |
| --- | --- | --- | --- | --- |

4.1.3.2	GTC codewords

| Codeword 201 (GTC) | '1' | PFIX 39 | IDENT1 2046 | '0' | D '0' | CHAN * | IDENT2 2079 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 202 (GTC) | '1' | PFIX 39 | IDENT1 2046 | '0' | D '1' | CHAN * | IDENT2 2079 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 203 (GTC) | '1' | PFIX 39 | IDENT1 2048 | '0' | D '0' | CHAN * | IDENT2 2079 | (N) * | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 204 (GTC) | '1' | PFIX 100 | IDENT1 2046 | '0' | D '0' | CHAN * | IDENT2 2079 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 205 (GTC) | '1' | PFIX 39 | IDENT1 2079 | '0' | D '0' | CHAN '0011111100' | IDENT2 2046 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 206 (GTC) | '1' | PFIX 39 | IDENT1 605 | '0' | D '0' | CHAN '0011111100' | IDENT2 2079 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 207 (GTC) | '1' | PFIX 39 | IDENT1 605 | '0' | D '0' | CHAN '0011111100' | IDENT2 2046 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 208 (GTC) | '1' | PFIX 39 | IDENT1 605 | '0' | D * | CHAN * | IDENT2 2079 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 209 (GTC) | '1' | PFIX 39 | IDENT1 2079 | '0' | D '* | CHAN * | IDENT2 2046 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 210 (GTC) | '1' | PFIX 39 | IDENT1 605 | '0' | D * | CHAN * | IDENT2 2046 | (N) X | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.3	Aloha Codewords

| Codeword 301 (ALH) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT X | RSVD '00' | (M) 0 | (N) 0 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 302 (ALHR) | '1' | PFIX 39 | IDENT1 2946 | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT X | RSVD '00' | (M) 20 | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 303 (AL -N variable) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT '0' | RSVD '00' | (M) 0 | (N) * | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 304 (AL -M variable) | '1' | PFIX 39 | IDENT1 1982 | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT '1' | RSVD '00' | (M) * | (N) 2 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 305 (AL -WT variable) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT * | RSVD '00' | (M) 0 | (N) 1 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 306 (ALH0) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '010' | CHAN4 * | WT * | RSVD '00' | (M) * | (N) 1 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 307 (ALHE) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '011' | CHAN4 * | WT X | RSVD '00' | (M) 0 | (N) 1 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 308 (ALHR) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '100' | CHAN4 * | WT * | RSVD '00' | (M) 0 | (N) 1 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 309 (ALHX) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '101' | CHAN4 * | WT X | RSVD '00' | (M) 0 | (N) 1 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 310 (ALH) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT X | RSVD '00' | (M) 0 | (N) 8 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 311 (ALH) | '1' | PFIX XXX | IDENT1 XXXX | '1' | CAT 000 | TYPE '00' | FUNC '000' | CHAN4 * | WT X | RSVD '00' | (M) 0 | (N) 8 | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.4	Acknowledgement codewords

| Codeword 401 (ACK) | '1' | PFIX 39 | IDENT1 8185 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 402 (ACK) | '1' | PFIX 39 | IDENT1 8185 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2048 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 403 (ACK) | '1' | PFIX 100 | IDENT1 8185 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 404 (ACKB) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '111' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 405 (ACKX) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '011' | IDENT2 2046 | QUAL '1' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 406 (ACKX) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '011' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 407 (ACK) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 0 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 408 (ACKI) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '001' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 409 (ACK) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '001' | IDENT2 2046 | QUAL '1' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 410 (ACK) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '1' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 411 (ACK) | '1' | PFIX 39 | IDENT1 8190 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 412 (ACKB) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '01' | FUNC '111' | IDENT2 2079 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 413 (ACK) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '100' | IDENT2 2046 | QUAL * | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 414 (ACK) | '1' | PFIX 39 | IDENT1 605 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 415 (ACKI) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '01' | FUNC '001' | IDENT2 2079 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 416 (ACK) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2079 | QUAL * | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 417 (ACKX) | '1' | PFIX 39 | IDENT1 8185 | '1' | CAT 000 | TYPE '01' | FUNC '011' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 418 (ACK) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '01' | FUNC '000' | IDENT2 2046 | QUAL '0' | (N) XX | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.5	Request codewords

| Codeword 501 (RQR) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '101' | IDENT2 '000000000000000' | RSVD '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 502 (RQR) | '1' | PFIX 100 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '101' | IDENT2 '000000000000000' | RSVD '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 503 (RQS) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '0' | FLAG1 '0' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 504 (RQS) | '1' | PFIX 39 | IDENT1 605 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '0' | FLAG1 '1' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 505 (RQX) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '10' | FUNC '010' | IDENT2 2046 | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 506 (RQQ) | '1' | PFIX 39 | IDENT1 8190 | '1' | CAT 000 | TYPE '10' | FUNC '110' | IDENT2 2046 | STATUS * | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 507 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '0' | FLAG1 '0' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 508 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '1' | FLAG1 '0' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 509 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '1' | FLAG1 '1' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 510 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT '1`' | FLAG1 '0' | FLAG2 '1' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 511 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT * | LEVEL '1' | EXT '0' | FLAG1 '0' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 512 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '1' | EXT * | FLAG1 * | FLAG2 * | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 513 (RQS) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | DT '0' | LEVEL '0' | EXT '0' | FLAG1 '0' | FLAG2 '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.6	Ahoy codewords

| Codeword 601 (AHYC) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '111' | IDENT2 8190 | SLOTS '01' | DESC '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 602 (AHY) | '1' | PFIX 39 | IDENT1 2047 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 XXXX | D '0' | POINT '01' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 603 (AHYC) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '111' | IDENT2 8190 | SLOTS '01' | DESC '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 604 (AHY) | '1' | PFIX 39 | IDENT1 2048 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 8190 | D '0' | POINT '1' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 605 (AHY) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | D '0' | POINT '0' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 606 (AHY) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 0 | D '0' | POINT '0' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 607 (AHY) | '1' | PFIX 39 | IDENT1 0 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 0 | D '0' | POINT '0' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 608 (AHY) | '1' | PFIX * | IDENT1 * | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 0 | D '0' | POINT '0' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 609 (AHY) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2079 | D '0' | POINT '0' | CHECK '1' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 610 (AHY) | '1' | PFIX 39 | IDENT1 2079 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2046 | D '0' | POINT '1' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 611 (AHYC) | '1' | PFIX 39 | IDENT1 8189 | '1' | CAT 000 | TYPE '10' | FUNC '111' | IDENT2 2046 | SLOTS '01' | DESC '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 612 (AHY) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '000' | IDENT2 2079 | D '0' | POINT '0' | CHECK '0' | E '0' | AD '0' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 613 (AHYX) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '10' | FUNC '010' | IDENT2 2079 | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.7	Miscellaneous control codewords

| Codeword 701 (MOVE) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '011' | CONT * | (M) 20 | RSVD '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 702 (CLEAR) | '1' | CHAN * | CONT * | '1' | CAT 000 | TYPE '11' | FUNC '010' | RSVD/SPARE '000000' | REVS '101010101010' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 703 (MOVE) | '1' | PFIX 100 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '011' | CONT * | (M) 20 | RSVD '000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 704 (BCAST) | '1' | SYSDEF '00011' | SYS * | '1' | CAT 000 | TYPE '11' | FUNC '100' | RSVD '0000' | NA * | REG '0' | SPARE '00000' | RSVD '000000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 705 (BCAST) | '1' | SYSDEF '00011' | SYS * | '1' | CAT 000 | TYPE '11' | FUNC '100' | RSVD '0000' | NA * | REG '1' | SPARE '00000' | RSVD '000000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 706 (MAINT) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '000' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 707 (MAINT) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '010' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 708 (MAINT) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '001' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 709 (BCAST) | '1' | SYSDEF '00010' | SYS '101010110000001' | '1' | CAT 000 | TYPE '11' | FUNC '100' | PER '1' | IVAL '00000' | PON '0' | ID '1' | RSVD '00' | SP '00000000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 710 (BCAST) | '1' | SYSDEF '00010' | SYS '101010110000001' | '1' | CAT 000 | TYPE '11' | FUNC '100' | PER '0' | IVAL '00010' | PON '1' | ID '1' | RSVD '00' | SP '00000000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 711 (MARK) | '1' | CHAN4 '0101' | SYS '101010001001001' | '1' | CAT 000 | TYPE '11' | FUNC '000' | B '101011011010011111' | P '1100010011010111' |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 712 (MAINT) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '011' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 713 (MAINT) | '1' | PFIX 39 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '111' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 714 (MAINT) | '1' | PFIX 39 | IDENT1 605 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '111' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 715 (MAINT) | '1' | PFIX 39 | IDENT1 * | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '110' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 716 (MAINT) | '1' | PFIX 100 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '001' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 717 (MAINT) | '1' | PFIX 100 | IDENT1 2046 | '1' | CAT 000 | TYPE '11' | FUNC '001' | CHAN '0011111100' | OPER '011' | RSVD '00000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

4.1.3.8	Other codewords

| Codeword 801 (SAMIS) | '1' | MERCODE (NOTE1) | MODEL (NOTE1) | CHECK BITS (NOTE2) | '1' | CAT '001' | TYPE '0' | SOL '0' | DESC '000' | SERIAL NO (NOTE1) | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Codeword 802 (DATA) | '0' | '11110000111000110010000011110001110011011111000' | P |
| --- | --- | --- | --- |

| Codeword 803 (SAMIS) | PRIX 120 | '1' | IDENT1 1739 | CAT '001' | TYPE '0' | SOL '0' | DESC '000' | PARAMETERS2 '000000000000000000' | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

NOTE 1

The information in these fields shall comply with the security number of the radio unit under test as advised by the manufacturer (see 3.4).

NOTE 2

The information in this field shall be the result of applying the security algorithm to the fields covered by Note 1.

4.1.3.9	Multi-codeword messages

| Message 901 | LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 302 | NON-SPECIFIED CODEWORDS |
| --- | --- | --- | --- | --- | --- | --- | --- |

Message 902

¬¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾¾

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 302 | NON-SPECIFIED CODEWORDS (NOTE 1) | CODEWORD 101 | CODEWORD 203 | NON-SPECIFIED CODEWORDS (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

¾¾¾¾REPEAT PATTERN¾¾¾¾®

| CODEWORD 101 | CODEWORD 402 | NON-SPECIFIED CODEWORDS (NOTE 1) |
| --- | --- | --- |

Message 903

¬¾¾¾¾REPEAT PATTERN¾¾¾¾®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 304 | NON-SPECIFIED CODEWORDS (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- |

Message 904

¬¾¾¾¾¾¾¾¾¾¾¾¾¾¾REPEAT PATTERN¾¾¾¾¾¾¾¾¾¾¾¾¾¾®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 305 | CODEWORD 101 | CODEWORD 402 | CODEWORD 101 | CODEWORD 402 | CODEWORD 301 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Message 905

¬¾¾¾¾¾REPEAT PATTERN¾¾¾®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 303 | NON-SPECIFIED CODEWORDS (NOTE 1) |
| --- | --- | --- | --- | --- | --- | --- |

Message 906

¬REPEAT PATTERN®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 306 |
| --- | --- | --- | --- | --- | --- |

Message 907

¬REPEAT PATTERN®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 307 |
| --- | --- | --- | --- | --- | --- |

Message 908

¬REPEAT PATTERN®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 308 |
| --- | --- | --- | --- | --- | --- |

Message 909

¬REPEAT PATTERN®

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 103 | CODEWORD 309 |
| --- | --- | --- | --- | --- | --- |

Message 910

¬¾¾¾¾¾¾REPEAT PATTERN¾¾¾¾¾¾

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 711 | CODEWORD 101 | CODEWORD 711 | CODEWORD 101 | CODEWORD 606 | CODEWORD 101 | CODEWORD 607 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

¾¾¾¾¾¾¾¾¾¾®

| CODEWORD 101 | CODEWORD 607 |
| --- | --- |

Message 911

¬¾¾¾¾¾¾¾REPEAT PATTERN¾¾¾¾¾

100 TIMES

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 711 | CODEWORD 101 | CODEWORD 711 | CODEWORD 101 | CODEWORD 608 | CODEWORD 101 | CODEWORD 607 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

¾¾¾¾¾¾¾¾¾¾®

| CODEWORD 101 | CODEWORD 607 |
| --- | --- |

Message 912

¬¾¾¾¾¾¾¾¾REPEAT PATTERN 8064 TIMES¾¾¾¾¾¾¾¾®

| CODEWORD 101 | CODEWORD * | CODEWORD 101 | CODEWORD 607 | CODEWORD 101 | CODEWORD 607 |
| --- | --- | --- | --- | --- | --- |

Message 913

¬REPEAT PATTERN®

7 TIMES

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 711 | CODEWORD 310 | CODEWORD 101 | CODEWORD 301 | CODEWORD 607 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Message 914

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 601 | CODEWORD 101 | CODEWORD 311 |
| --- | --- | --- | --- | --- | --- | --- | --- |

¬REPEAT PATTERN®

6 TIMES

| CODEWORD 101 | CODEWORD 301 |
| --- | --- |

Message 915

| LET | PREAMBLE (16 BITS) | SYNC | CODEWORD 301 | CODEWORD 101 | CODEWORD 201 | NON-SPECIFIED CODEWORDS |
| --- | --- | --- | --- | --- | --- | --- |

NOTE 1

The number of non-specified codewords shall be equal to 2(SLTS-1), where SLTS is equal to the number of slots in the Aloha frame specified by the value of the N field in the preceeding address codeword.

4.1.4		CHAN and CONT Fields

With the exception of sections 5.2.1.4 and 5.2.1.5, the CHAN and CONT fields referred to in the tests of sections 5, 6 and 7 are for Band III sub-band 2. For tests made on radio units operating in Band III sub-band 1, replace  the leading binary digit '0' by '1' (except for CHAN='0000000000' which is valid for both sub bands).

4.2	General Test Assembly

4.2.1	Properties of the General Test Assembly

The tests in this specification all require the use of a General Test Assembly which simulates a TSC and base station(s) with transmissions complying to MPT1347 except where specified.  Other test equipment required for specific tests is specified as necessary.

The General Test Assembly which is illustrated conceptually in figure 4.1, shall have the following properties not usual in a practical system conforming to MPT1347:

a)	Transmitting level normally set at -85 dBm, but variable for specific tests as specified.

b)	Ability to transmit planned strings of messages dependant only on tests being conducted and also to fill non-specific parts of any control channel with codewords (except CCSC and MARK) conveying no information for the radio unit being tested.

c)	The capability to reduce the S/N ratio perceived by the radio unit to worse than 0 dB, as measured over a channel bandwidth.  The reduction shall not start before bit 3 nor finish later than bit 62 of any selected codeword, and shall last at full value for 30 ± 10 ms.

d)	Only one received channel shall be examined at any instant, depending on the test being conducted.  The channel receiver shall incorporate a data decoder which at least properly recovers bits at any signal level equal to or greater than 90% of the maximum received signal power.

e)	For each received data transmission the instant of beginning of each SYNC or SYNT sequence shall be determined.

Indication that at least 16 bits of preamble have been received may be helpful.

f)	A radio signal strength indicator (RSSI) to indicate the instants at which the received power rises above and falls below -60dB (termed "L-60") relative to maximum signal power.  L is the signal level at the input to the RSSI when the radio unit under test is transmitting at full power.

g)	A wide-band power-indicating receiver covering at least 174 to 225 MHz, and with sensitivity, as measured over any 8 kHz band, not worse than -50 dB, nor better than -60dB, relative to maximum radio unit power.

h)	A non-error-correcting data receiver with foreknowledge of the message expected from the radio unit, and indication of any unexpected or faulty transmission from the radio unit under test.

j)	A classifying counter of all random access events, capable of counting the numbers of such events for each possible length of random access frame according to the time-slot positions within that length frame.

k)	SINAD test Set.

l)	An expected response window function with a counter for responses inside the window and a counter for responses outside the window.

4.2.2		Non-specific codewords in any forward control channel

For tests in this specification some particular messages are transmitted on the forward control channel. The General Test Assembly shall fill all other parts of a forward control channel with non-specific message codewords having the approximate relative frequency of occurrence given below. All the types of codeword listed shall at least be transmitted from time to time.

Except for MARK, miscellaneous system control messages shall not be transmitted unless specified for particular tests.

So far as possible, all the messages listed below shall occur both inside and outside random access frames. All messages shall comply with MPT1327. Unless otherwise specified, approximately half of the data codewords in the second half of a slot shall have the RSA flag set to '0'.

Message Type	Appropriate Percentage Fill

of Forward Control Channel

CCSC as required for each test	35

String of 2 data codewords displacing one CCSC	20

String of 4 data codewords displacing two CCSCs 	10

Go-to-channel messages not relevant to RU	10

(For these GTC messages, N=0 is suggested but, in any case, values of N shall be consistent with the correct application of the random access protocol in section 7 of MPT1327)

Aloha messages with N=0, and WT and M = last previous values	5

Acknowledgements not relevant to RU	5

(For these Acknowledgements, N=0 is suggested but, in any case, values of N shall be consistent with the correct application of the random access protocol in Section 7 of MPT1327)

Ahoy type messages not relevant to RU	5

MARK	5

SAMO message not relevant to RU	2.5

HEAD address not relevant to RU	2.5

Conceptual Block Diagram of General Test Assembly

µ §

Figure 4.1