2.	STANDARD TERMS AND DEFINITIONS

2.1	Defined terms used in this specification

This specification uses the nomenclature used in the related specifications MPT1327 and MPT1343 in the standard way. In addition there is a small number of terms that are used in this specification only. These are defined as follows:

Mute the Received Audio is the action of preventing the audio signal being amplified and used to drive an audio transducer such as would othewise allow the user to hear the recovered audio signal.

Manual Mode is the operational state that the radio unit can be set into to support the standard tests for radio peformance.  When the radio unit is in this state the signalling requirements of MPT1327 and MPT1343 are suspended. This mode can also be used to perform the tests required to obtain type approval to MPT1323.

The Expected Response Window is the region in the time domain in which a radio unit message transmission should be received in its entirity to comply with the requirements of MPT1327, Section 6.2.1.3.

2.2	Test numbering system

Throughout this specification the tests are numbered in accordance with the section in which they fall and also give guidance on the required state of the radio unit under test.  This is achieved within the structure of the test number itself.  All tests are numbered as follows:

Sssnnc

Where

S	is the MPT1352 major section number in which the test is defined.

ss	is the MPT1352 minor section number in which the test is defined.

nn	is a number which forms a simple incremental count of tests within the section referred by Sss.

c	is one of the following alphanumeric characters defined as follows:

A	requires personality A to be operational (see section 3).  In some cases the personality may be left undefined.

B	requires personality B to be operational (see section 3).

X	requires that the radio be set into the manual mode.

Y	notes that the test is continuous.

2.3	Default test conditions

This section defines certain test conditions that are to be taken as the default conditions unless otherwise specified in the body of the document.

2.3.1	Receiver input power level

Unless otherwise directed in the test the standard input signal power level measured at the input terminal of the radio unit receiver shall be nominally set to -85 dBm.

2.3.2	Idle state

A radio unit is in the idle state if it has acquired a control channel and has successfully registered if required to do so.  In the idle state there are no signalling transactions currently in progress.

2.3.3	Values of SYS and CCS

Unless otherwise directed in the tests the standard values of SYS and CCS in control channel system codewords transmitted by the General Test Assembly (GTA) shall be as noted in the table below:

| Personality | SYS | CCS |
| --- | --- | --- |
| A B | '101010110000001' '000101000100001' | '1100100111001110' '0011001111000001' |

2.4	Radio unit personality change

1.	In this specification the radio unit under test is required to operate with two different personalities. It must also operate in the manual mode. Since it is mandatory that radio units can migrate between networks conforming to MPT1347 by personality change (see 3.3 of MPT1343) all tests must be carried out on a single radio unit.

2.	The organisation seeking type approval shall furnish the Tester with all the equipment and information necessary to perform the personality changes. Such changes may entail hardware and/or software modification to the radio unit. Any hardware modification shall be minor in nature. e.g., replacement of one or two integrated circuits.