7	OPERATIONAL SEQUENCE MESSAGE TESTS

7.1	Type of Aloha Frame

7.1.1	Introduction

The tests in this section are designed to check the correct operation of the radio unit under test when connected to a control channel radiating various ALH messages.

7.1.2	ALHD function

7.1.2.1	Test number

70101A

7.1.2.2	Purpose of test

To ensure that, when the control channel is radiating ALHD, a radio unit will not attempt to access the control channel when a simple speech call is requested by the user in accordance with 7.3.2 of MPT1327.

7.1.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.1.2.4	Test method

The Tester shall cause the General Test Assembly to send message 906 continuously on the forward control channel on channel number 282.  In codewords 301 and 306 the value of CHAN4 shall be set to '0001'.

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3).  After 40 seconds ±1 second the Tester shall cause the General Test Assembly to replace a single codeword 306 in message 906 by codeword 303.  In codeword 303 the value of CHAN4 shall be set to '0001' and the value of N shall be set to 1.  Any attempt by the radio unit to transmit shall be recorded.

7.1.2.5	Limits

The radio unit shall not transmit prior to codeword 303 (ALH) being sent and shall then attempt access in the slot immediately following codeword 303, with codeword 503 (RQS).

7.1.3	ALHE function

7.1.3.1	Test number

70102A

7.1.3.2	Purpose of test

To ensure that, when the control channel is radiating ALHE, a radio unit will not attempt to access the control channel when a simple speech call is requested by the user in accordance with 7.3.2 of MPT1327.

7.1.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.1.3.4	Test method

The Tester shall cause the General Test Assembly to send message 907 continuously on the forward control channel on the channel number 282.  In codewords 301 and 307 the value of CHAN4 shall be set to '0001'.

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3).  After 40 seconds ±1 second the Tester shall cause the General Test Assembly to replace a single codeword 307 in message 907 by codeword 303.  In codeword 303 the value of CHAN4 shall be set to '0001' and the value of N shall be set to 1.  Any attempt by the radio unit to transmit shall be recorded.

7.1.3.5	Limits

The radio unit shall not transmit prior to codeword 303 (ALH) being sent, and shall then attempt access in the slot immediately following codeword 303, with codeword 503 (RQS).

7.1.4	ALHR function

7.1.4.1	Test number

70103A

7.1.4.2	Purpose of test

To ensure that, when the control channel is radiating ALHR, a radio unit will not attempt to access the control channel when a simple speech call is  requested by the user in accordance with 7.3.2 of MPT1327.

7.1.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio Unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.1.4.4	Test method

The Tester shall cause the General Test Assembly to send message 908 continuously on the forward control channel on channel number 282.  In codewords 301 and 308 the value of CHAN4 shall be set to '0001'.

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3).  After 40 seconds ±1 second the Tester shall cause the General Test Assembly to replace a single codeword 308 in message 908 by codeword 303.  In codeword 303 the value of CHAN4 shall be '0001' and the value of N shall be set to 1.  Any attempts by the radio unit to transmit shall be recorded.

7.1.4.5	Limits

The radio unit shall not transmit prior to codeword 303 (ALH) being sent, and shall then attempt access in the slot immediately following codeword 303, with codeword 503 (RQS).

7.1.5	ALHX function

7.1.5.1	Test number

70104A

7.1.5.2	Purpose of test

To ensure that when the control channel is radiating ALHX, a radio unit will not attempt to register in accordance with 7.3.2 of MPT1327.

7.1.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 278 and 282.

The forward control channel on channel number 282 shall carry codeword 101 as the CCSC and unspecified codewords in the second half of the slots.

The radio unit shall be in the idle state on control channel number 282 immediately prior to this test.

7.1.5.4	Test method

The Tester shall cause the General Test Assembly to send message 909 continuously on the forward control channel 278.  In codewords 301 and 309 the value of CHAN4 shall be set to '1101'.

The Tester shall send codeword 701 on forward control channel number 282 with CONT set to a value of '0011011101'.  At 15 seconds ±1 second after the transmission of codeword 701 the Tester shall cause the General Test Assembly to change a single codeword 309 in message 909 on channel number 278 to codeword 303.  In codeword 303 the value of CHAN4 shall be set to '1101' and the value of N shall be set to 1.  The General Test Assembly shall monitor the return control channel number 278 for any transmissions by the radio unit.

7.1.5.5	Limits

The radio unit shall not transmit prior to codeword 303 being sent, and shall then transmit codeword 501 in the slot immediately following codeword  303.

7.2	Acquiring and Leaving A Control Channel

7.2.1	General

The tests in this section are designed to check the correct operation of the radio unit procedures for acquiring and leaving a control channel.  The necessary radio unit behaviour is described in Section 6.2 of  MPT1327 and Section 9 of MPT1343.  The tests in this section cover the following:

a)	Storage of Personalisation and Acquisition Authorisation Data. The 120 hour memory protection for key data.

b)	The acquisition threshold of L2.

c)	Error rate monitoring in the mobile.

d)	The order in which channels are sampled is randomised in accordance with 9.3.3.1 of MPT1343.

e)	Single Channel Hunt, Resuming a Channel, Preferential Hunt, Normal Hunt, and the Comprehensive Hunt.

f)	Entry into Control Channel Acquisition from a traffic channel.

g)	Entry into Control Channel Acquisition from a Control Channel.

h)	Correct utilisation of Acquisition Authorisation Data.

The General Test Assembly is required for these series of tests to generate two simulated control channels on various RF frequencies, and generate a simulated traffic channel on one fixed frequency.

The tests cover operation of the radio unit with both the A personalisation and the B personalisation.  Sections 7.2.2 to 7.2.5 require the radio unit to utilise the A personalisation and 7.2.6 to 7.2.8 repeat the majority of these tests with the radio unit utilising the B personalisation.  Corresponding tests in 7.2.2 to 7.2.5 and 7.2.6 to 7.2.8 are allocated the same test number with the corresponding letter suffix A or B (e.g., 70201A and 70201B).

The initialisation requirement for each test is generally the state achieved upon satisfactory completion of the preceding test.  It may therefore be found convenient to carry out the tests in the order in which they appear in this section.

7.2.2	Leaving a control channel and hunting for a new control channel

The following series of tests simulates conditions under which a radio unit shall leave its current control channel and seek a new control channel.  Two control channels, selected from the set of Normal Hunt channel numbers, are employed.

Initially the SYS fields used for both channels differ only in FREE bits, and the radio unit is authorised to acquire both (by virtue of the AREA field).

The radio unit is forced to relinquish a control channel by conditions or events on that channel that shall cause the radio unit to enter the control channel acquisition procedures. The security number check procedure is used to verify the identity of the control channel acquired subsequently.

7.2.2.1	Leaving upon receipt of MOVE

7.2.2.1.1	Test number

70201A

7.2.2.1.2	Purpose of test

To ensure that the radio unit leaves the current control channel, enters the control channel acquisition procedures and acquires the nominated control channel after receiving an applicable MOVE codeword on its current control channel in accordance with 9.3.3.2.2, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 70.  The radio unit shall be in the idle state on control channel number 58 immediately prior to the test.  (This may be achieved more easily if the radiation of the forward control channel on channel number 70 is delayed until after the radio unit has acquired channel number 58).

The forward control channels on channel numbers 58 and 70 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

58	'101010001000001'	'1100000001101010'

70	'101010001001001'	'0011100110101101'

7.2.2.1.4 	Test method

The Tester shall use the General Test Assembly to send MOVE codeword number 701 with the value of the CONT field set to '0000001101' within the forward message stream on channel number 58.

At some appropriate time the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 7062.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.2.2.1.5	Radio unit response

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.2.2	Leaving upon change of SYS field value

7.2.2.2.1	Test number

70202A

7.2.2.2.2	Purpose of test

To ensure that the radio unit leaves the current control channel, enters the control channel acquisition procedures and acquires another control channel after a change of value of bits 1-12 in the SYS field in the CCSC has existed continuously for a period in excess of TS in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 70.  The radio unit shall be in the idle state on control channel number 70 immediately prior to the test. (This may be conveniently achieved by the successful completion of test 70201A immediately prior to test 70202A).

The forward control channel shall carry the values of fields in the CCSC as specified for test 70201A.

7.2.2.2.4	Test method

The Tester shall cause the General Test Assembly to change the SYS field value radiated in all CCSCs on the forward control channel number 70 to the  value '101001000000001' (with a corresponding change in CCS to '0010001001010100').

At some appropriate time after the value of SYS field has been changed, but prior to a period TS has been exceeded, the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 58. The General Test Assembly shall be reset to transmit, on channel 70, the values of SYS field specified in test 70201A, 112 codewords after the change of value of the SYS field required by this test. At some appropriate time after the period TS has been exceeded the Tester shall again cause the General Test Assembly to send codeword 601 on forward control channel number 58.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to both transmissions of codeword 601 in the slot immediately following transmission of the codewords.

7.2.2.2.5	Radio unit response

The radio unit shall not respond to the first transmission of codeword 601 (AHYC) and shall respond to the second transmission with codeword 801 (SAMIS) in the correct response slot.

7.2.2.3	Relinquishing upon break in transmission of CCSC

7.2.2.3.1	Test number

70203A

7.2.2.3.2	Purpose of test

To ensure that the radio unit relinquishes the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has failed to receive a decodable CCSC over a continuous period in excess of TS in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62.  The radio unit shall be in the idle state on control channel number 58 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70202A immediately prior to test 70203A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

7.2.2.3.4	Test method

The Tester shall cause the General Test Assembly to transmit data codeword 802 continuously on forward control channel number 58 in the form of a contiguous stream. Accordingly no CCSC codewords shall be transmitted following the first transmission of data codeword 802.

At some appropriate time after the first transmission of codeword 802, but prior to a period TS has been exceeded, the Tester shall cause the General Test Assembly to send codeword601 on the forward control channel on channel number 62.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to the transmission of codeword 601 in the slot immediately following transmission of the codeword.

The General Test Assembly shall then be reset to transmit a forward control channel conforming to 3.3.3.1 of MPT1327 and with the SYS field value in the CCSC set to the value specified for test 70201A.

7.2.2.3.5	Radio unit response

The radio unit shall respond to the transmission of codeword 601 (AHYC) with codeword 801 (SAMIS) in the control response slot.

7.2.2.4	Relinquishing upon change of LAB field value

7.2.2.4.1	Test number

70204A

7.2.2.4.2	Purpose of test

To ensure that the radio unit relinquises the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has received, in two consecutive decodable CCSCs, a value of LAB field which indicates that the control category of the radio unit is no longer permitted access on the current control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62.  The radio unit shall be in the idle state on control channel number 62 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70203A immediately prior to test 70204A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

7.2.2.4.4	Test method

The Tester shall cause the General Test Assembly to change the SYS field value radiated in one of the CCSCs on the forward control channel number 62 to '101010001001011' (with a corresponding change in CCS to '0100100111010011') thus modifying the value of the LAB field to '011'.

At some appropriate time after the transmission of the CCSC with a different value of SYS field the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 58.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot  immediately following.

Subsequently the Tester shall cause the General Test Assembly to change the SYS field value radiated by all CCSCs on the forward control channel number 62 to '101010001001011'. At some appropriate time after this change the Tester shall again cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 58.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to this further transmission of codeword 601 in the slot immediately following.

The General Test Assembly shall then be reset to transmit the values of SYS field specified in test 70201A.

7.2.2.4.5	Radio unit response

The radio unit shall not respond to the first transmission of codeword 601 (AHYC) and shall respond to the second transmission with codeword 801 (SAMIS) in the correct response slot.

7.2.2.5	Relinquishing due to errors

7.2.2.5.1	Test number

70205A

7.2.2.5.2	Purpose of test

To ensure that the radio unit reliquishes the current control channel, and enters the control channel acquisition procedures and acquires another control channel after it has detected errors exceeding the prescribed rate on the received forward control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62, set to give a nominal signal level at the receiver of -103 dBm.

The radio unit shall be in the idle state on control channel number 58 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70204A immediately prior to test 70205A).  The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

7.2.2.5.4	Test method

The Tester shall cause the General Test Assembly to produce "errored" codewords in the radio unit by reducing the signal to noise ratio perceived  by the radio unit to 0 dB measured over a channel bandwidth.  This reduction shall not start prior to bit 3 nor finish later than bit 62 of selected codewords and shall last at full value for 30 ±10 ms (refer to section 4.2.1).

The Tester shall cause the General Test Assembly to send a repeating pattern of 4 "errored" codewords in consecutive groups of 20 codewords on control channel number 58 whilst maintaining an "unerrored" control channel on channel number 62.

At some appropriate time after "errored" codewords have begun to be transmitted, the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 62.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.   The General Test Assembly shall then be reset to transmit "unerrored" codewords continuously.

7.2.2.5.5	Radio unit response

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.2.6	Relinquishing due to receipt of CCSC with different SYS value

7.2.2.6.1	Test number

70206A

7.2.2.6.2	Purpose of test

To ensure that the radio unit relinquishes the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has detected CCSCs with a value of SYS field different to the verified value and that, by treating these as errored codewords, the error codeword rate exceeds the prescribed level on the received forward control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.2.6.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62, set to give a nominal signal level at the receiver of -103 dBm.

The radio unit shall be in the idle state on control channel number 62 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70205A immediately prior to test 70206A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

7.2.2.6.4	Test method

The Tester shall cause the General Test Assembly to insert codeword 102 with the SYS field value '101001000000001' and CCS value '0010001001010100' in place of some of the CCSCs with values as prescribed in 7.2.2.6.3.

The General Test Assembly shall insert codeword 102 in this way as the first codeword in two consecutive slots on the forward control channel number 62 followed by eight consecutive slots with the CCSC specified in 7.2.2.6.3.  Data codewords shall not displace any inserted codeword 102 or the CCSC following any inserted codeword 102.

At some appropriate time after the commencement of the insertion of codewords 102, the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 58.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

The General Test Assembly shall then be reset to the CCSC field values specified in test 70201A.

7.2.2.6.5	Radio unit response

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.3	Leaving a traffic channel and hunting for a new control channel

The following series of tests are designed to verify that the radio unit follows the correct procedures to acquire a control channel when leaving a traffic channel.  Two control channels, selected from the set of Normal Hunt channel numbers, and a traffic channel are employed.

Initially the SYS fields used for both control channels differ only in FREE bits, and the radio unit is authorised to acquire both by virtue of the (AREA field).

The radio unit is forced to relinquish a traffic channel by conditions or events on that channel that shall cause the radio unit to enter the control channel acquisition procedures.  Either the security number check procedure  or the receipt of a random access registration request is used to verify the identity of the control channel acquired subsequently, whichever is the most appropriate.

7.2.3.1	Resuming a control channel

7.2.3.1.1	Test number

70207A

7.2.3.1.2	Purpose of test

To ensure that the radio unit relinquishes the current traffic channel, enters the control channel acquisition procedures and re-acquires the control channel on which it was confirmed prior to being active on the traffic channel after receiving an applicable CLEAR codeword on its current traffic channel in accordance with 9.3.3.2.1 and 9.3.4.2 of MPT1343.

7.2.3.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62 and a traffic channel on channel number 309.  The radio unit shall be in the idle state on control channel number 58 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70206A immediately prior to test 70207A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 58.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to interrupt the modulating tone on traffic channel number 309 and  transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 58.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.2.3.1.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.3.2	Control channel acquisition by single channel hunt

7.2.3.2.1	Test number

70208A

7.2.3.2.2	Purpose of test

To ensure that the radio unit leaves the current traffic channel, enters the control channel acquisition procedures and acquires a nominated control channel after receiving an applicable CLEAR codeword on its current traffic channel in accordance with 9.3.3.2 and 9.3.4 of MPT1343.

7.2.3.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 58 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70207A immediately prior to test 70208A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 58.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT  field set to '0000000101' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 62.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.2.3.2.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.3.3	Preferential hunt

7.2.3.3.1	Test numbers

70209A to 70213A

7.2.3.3.2	Purpose of tests

To ensure that the radio unit relinquishes the current traffic channel, enters the control channel acquisition procedures and acquires a control channel as a result of a preferential hunt after receiving an applicable CLEAR codeword on its current traffic channel but being unable to reaquire the control channel on which it was confirmed prior to being active on the traffic channel in accordance with 9.3.3.2.1, 9.3.3 and 9.3.4 of MPT1343.

7.2.3.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 58 and 62 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 62 immediately prior to the test. (This may be conveniently achieved by the successful completion of test 70208A immediately prior to test 70209A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.3.4	Test method

Test 70209A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 62.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 58 and 62 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

64	'101010010000001'	'0110101111100101'

77	'101010001001001'	'0011100110101101'

The level of channel number 77 shall be set to give a nominal value at the receiver of -103 dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 77.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

Tests 70210A to 70213A

Test 70210A shall be performed following the successful completion of test 70209A since the initialisation required for 70210A is the final state for 70209A.

Likewise 70211A shall follow 70210A, 70212A shall follow 70211A and 70213A shall follow 70212A.

The test method shall follow the method given for test 70209A with the following modifications:

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70210A		77

70211A		85

70212A		107

70213A		120

The two new control channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS		CCS

70210A		79	'101010010000001' 	'0110101111100101'

85	'101010001001001'		'0011100110101101'

70211A		87	'101010010000001'		'0110101111100101'

107	'101010001001001'		'0011100110101101'

70212A		117	'101010010000001'		'0110101111100101'

120	'101010001001001'		'0011100110101101'

70213A		123	'101010010000001'		'0110101111100101'

126	'101010001001001'		'0011100110101101'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

Codeword 601 shall be transmitted on the second channel number in each pair.

7.2.3.3.5	Radio unit response

Tests 70209A to 70213A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.3.4	Normal hunt at level L2

7.2.3.4.1	Test numbers

70214A to 70218A

7.2.3.4.2	Purpose of tests

To ensure that the radio unit relinquishes the current traffic channel, enters the control channel acquisition procedures and acquires a control channel as a result of a normal hunt at the higher level after receiving an  applicable CLEAR codeword on its current traffic channel, but being unable to reacquire the control channel on which it was confirmed prior to being active on the traffic channel, or an alternative control channel with the same AREA code in accordance with 9.3.3.2.1, 9.3.3.3, 9.3.3.4 and 9.3.4 of MPT1343.

7.2.3.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 123 and 126 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 126 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70213A immediately prior to test 70214A).

The forward control channels on channel numbers 123 and 126 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

123	'101010010000001'	'0110101111100101'

126	'101010001001001'	'0011100110101101'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.4.4	Test method

Test 70214A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 126.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 123 and 126 and to replace these by two new control channels with channel numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

129	'101010010000001'	'0110101111100101'

132	'101010010001001'	'1001001000100010'

The level of channel number 132 shall be set to give a nominal value at the receiver of -103 dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 129 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70215A to 70218A

Test 70215A shall be performed following the successful completion of test 70214A since the initialisation required for 70215A is the final state for 70214A.

Likewise 70216A shall follow 70215A, 70217A shall follow 70216A and 70218A shall follow 70217A.

The test method shall follow the method given for test 70214A with the following modifications:

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70215A		129

70216A		135

70217A		177

70218A		186

The two new control channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS		CCS

70215A		135	'101010011000001'	'0000110101100000'

138	'101010011001001'	'1111010010100111'

70216A		177	'101010100000001'	'0000010011000100'

180	'101010100001001'	'1111110100000011'

70217A		186	'101010101000001'	'0110001001000001'

189	'101010101001001'	'1001101110000110'

70218A		217	'101010001000001'	'1100000001101010'

219	'101010001001001'	'0011100110101101'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the first channel number in the pair for codeword 501.

7.2.3.4.5	Radio unit response

Tests 70214A to 70218A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the channel number specified in 7.2.3.4.4.

7.2.3.5	Error checking prior to confirmation

7.2.3.5.1	Test numbers

70219A to 70223A

7.2.3.5.2	Purpose of tests

To ensure that the radio unit does not confirm a control channel with an excessive codeword error rate during the hunting procedures in accordance with 9.3.4.3 and 9.3.4.4 of MPT1343.

7.2.3.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 217 and 219 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 217 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70218A immediately prior to test 70219A).

The forward control channels on channel numbers 217 and 219 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

217	'101010001000001'	'1100000001101010'

219	'101010001001001'	'0011100110101101'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.5.4	Test method

Test 70219A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 217.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate control channels on channel numbers 217 and 219 and to replace these by two new control channels with channel numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

258	'101010010000001'	'0110101111100101'

278	'101010010001001'	'1001001000100010'

The level of both control channels shall be set to give a nominal value at the receiver of -103dBm.

The Tester shall cause the General Test Assembly to produce "errored" codewords in the radio unit by reducing the signal to noise ratio perceived  by the radio unit to 0 dB measured over a channel bandwidth.  This reduction shall not start prior to bit 3 nor finish later than bit 62 of selected codewords and shall last at full value for 30 ±10 ms (Refer to Section 4.2.1).

The Tester shall cause the General Test Assembly to send a repeating pattern of one "errored" codeword in consecutive groups of 10 codewords on control channel number 258 whilst maintaining an "unerrored" control channel on channel number 278. The errored codeword must be in the same ordinal position in each group of 10 codewords.

At some appropriate time after both new control channels are radiating and "errored" codewords have begun to be transmitted, the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 278 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70220A to 70223A

Test 70220A shall be performed following the successful completion of test 70219A since the initialisation required for 70220A is the final state for 70219A.

Likewise 70221A shall follow 70220A, 70222A shall follow 70221A and 70223A shall follow 70222A.

The test method shall follow the method given for test 70219A with the following modifications.

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70220A		278

70221A		286

70222A		361

70223A		367

The two new control channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS		CCS

70220A		282	'101010011000001'	'0000110101100000'

286	'101010011001001'	'1111010010100111'

70221A		358	'101010100000001'	'0000010011000100'

361	'101010100001001'	'1111110100000011'

70222A		364	'101010101000001'	'0110001001000001'

367	'101010101001001'	'1001101110000110'

70223A		372	'101010001000001'	'1100000001101010'

377	'101010001001001'	'0011100110101101'

In each case the first channel number in the pair shall have "errored" codewords introduced and the second channel number shall be transmitted "unerrored".

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 501.

7.2.3.5.5	Radio unit response

Tests 70219A to 70223A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the channel number specified in 7.2.3.5.4.

7.2.3.6	Normal hunt precedence over comprehensive hunt

7.2.3.6.1	Test number

70224A

7.2.3.6.2	Purpose of test

To ensure that the radio unit confirms a control channel on a channel number included in the radio unit personalisation of normal hunt channel, numbers in preference to a control channel on a channel number not included  in the normal hunt channel numbers in accordance with 9.3.3.1, 9.3.3.4 and 9.3.4.5 of MPT1343.

7.2.3.6.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 372 and 377 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 377 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70223A immediately prior to test 70224A).

The forward control channels on channel numbers 372 and 377, shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS			CCS

372	'101010001000001'	'1100000001101010'

377	'101010001001001'	'0011100110101101'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

The level of channel number 372 shall be set to give a nominal value at the receiver of -103dBm.

7.2.3.6.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward  message stream on channel number 377.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a new control channel on channel number 208 with codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001001001' and the value of the CCS field set to '0011100110101101'.  The Tester shall also change the values of the SYS and CCS fields in codeword 102 on control channel number 372 to '101010010000001' and '0110101111100101' respectively.

After a suitable time the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 372 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

7.2.3.6.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the return control channel number 372.

7.2.3.7	Comprehensive hunt facility

7.2.3.7.1	Test number

70225A

7.2.3.7.2	Purpose of test

To verify the correct operation of the comprehensive hunt facility in accordance with 9.3.3.5 of MPT1343.

7.2.3.7.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 208 and 372 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 372 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70224A immediately prior to test 70225A).

The forward control channels on channel numbers 208 and 372 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

208	'101010010000001'	'0110101111100101'

372	'101010010000001'	'0110101111100101'

The forward traffic channel shall be modulated wih a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.3.7.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 372.

Once the Tester has confirmed that the radio unit is active on the traffic   channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate the control channel on channel number 372.

After a suitable time the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 208.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.2.3.7.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.4	Use of acquisition authorisation data

The following series of tests are designed to verify that the radio unit makes correct use of the Acquisition Authorisation Data programmed as part of the radio unit personalisation.

7.2.4.1	Application of NET field personalisation

7.2.4.1.1	Test numbers

70226A to 70230A

7.2.4.1.2	Purpose of tests

To ensure that a radio unit acquires a control channel bearing the NET (Network Identity) sub-field value of the selected network in preference to one that bears an unknown NET sub-field in accordance with 9.3.4.2.4 of MPT1343.

7.2.4.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 208 and 372 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 208 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70208A immediately prior to test 70209A).

The forward control channels on channel numbers 208 and 372 shall carry the values of fields in the CCSC as specified for test 70225A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.4.1.4	Test method

Test 70226A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 208.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate control channels on channel numbers 208 and 372 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

58	'100010011000001'	'1010011001101001'

62	'101010011001001'	'1111010010100111'

The level of channel number 62 shall be set to give a nominal value at the receiver of -103 dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified  in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 62 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70227A to 70230A

Test 70227A shall be performed following the successful completion of test 70226A since the initialisation required for 70227A is the final state for 70226A. Likewise 70228A shall follow 70227A, 70229A shall follow 70228A and 70230A shall follow 70229A.

The test method shall follow the method given for test 70226A with the following modifications:

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70227A		62

70228A		77

70229A		85

70230A		107

The two new control channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS		CCS

70227A		64	'110010100000001'	'1100000111100000'

77	'101010100001001'	'1111110100000011'

70228A		79	'000010101000001'	'0001010100010010'

85	'101010101001001'	'1001101110000110'

70229A		87	'111010001000001'	'1010111001000111'

107	'101010001001001'	'0011100110101101'

70230A		117	'010010010000001'	'0111001010011011'

120	'101010010001001'	'1001001000100010'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 501.

7.2.4.1.5	Radio unit response

Tests 70226A to 70230A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the channel number specified in 7.2.4.1.4.

7.2.4.2	Application of ZONE field personalisation

7.2.4.2.1	Test numbers

70231A to 70235A

7.2.4.2.2	Purpose of tests

To ensure that the radio unit acquires a control channel bearing a ZONE sub-field value for which acquisition is authorised in its network personalisation data in preference to one that bears an unknown ZONE sub-field in accordance with 9.3.4.2.4 of MPT1343.

7.2.4.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 117 and 120 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 120 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70230A immediately prior to test 70231A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70230A.

The forward traffic channel shall be modulated with a suitable audio test (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.4.2.4	Test method

Test 70231A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 120.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate control channels on channel numbers 117 and 120 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

123	'101101000100001'	'0100010010010010'

126	'101001000100001'	'1000110100001001'

The level of channel number 126 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 126 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70232A to 70235A

Test 70232A shall be performed following the successful completion of test 70231A since the initialisation required for 70232A is the final state for 70231A.

Likewise 70233A shall follow 70232A, 70234A shall follow 70233A and 70235A shall follow 70234A.

The test method shall follow the method given for test 70231A with the following modifications:

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70232A		126

70233A		132

70234A		138

70235A		180

The two new trafficcontrol channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS		CCS

70232A		129	'101100001000001'	'1111000100100011'

132	'101001001000001'	'0100010011010001'

70233A		135	'101101001100001'	'0010001000010111'

138	'101001001100001'	'1110101110001100'

70234A		177	'101110010000001'	'1010001001111110'

180	'101001010000001'	'1110111101011110'

70235A		186	'101000011000001'	'1111010110110010'

189	'101001010100001'	'0100000000000011'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 501.

7.2.4.2.5	Radio unit response

Tests 70231A to 70235A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the channel number specified in 7.2.4.2.4.

7.2.4.3	Application of AREA field personalisation

7.2.4.3.1	Test numbers

70236A to 70240A

7.2.4.3.2	Purpose of tests

To ensure that the radio unit acquires a control channel bearing an AREA sub-field value for which acquisition is authorised in its network personalisation data in preference to one that bears an unknown AREA sub-field in accordance with 9.3.4.2.4 of MPT1343.

7.2.4.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 186 and 189 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 189 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70235A immediately prior to test 70236A).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70235A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.4.3.4	Test method

Test 70236A

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 189.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 186 and 189 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

217	'101010000100001'	'0000100110110010'

219	'101010001000001'	'1100000001101010'

The level of channel number 219 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 219 for codeword 501 from the radio unit.  Upon receipt of codeword 501 from the radio unit the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70237A to 70240A

Test 70237A shall be performed following the successful completion of test 70236A since the initialisation required for 70237A is the final state for 70236A.

Likewise 70238A shall follow 70237A, 70239A shall follow 70238A and 70240A shall follow 70239A.

The test method shall follow the method given for test 70236A with the following modifications:

The channel number on which codeword 201 is transmitted shall be as follows:

Test Number	Channel Number

70237A		219

70238A		278

70239A		286

70240A		361

The two new control channels to be radiated for each test shall be as follows:

Test Number	Channel Number		SYS			CCS

70237A		258	'101010001100001'	'0110111100110111'

278	'101010010000001'	'0110101111100101'

70238A		282	'101010010100001'	'1100010010111000'

286	'101010011000001'	'0000110101100000'

70239A		358	'101010011100001'	'1010001000111101'

361	'101010100000001'	'0000010011000100'

70240A		364	'101010100100001'	'1010101110011001'

367	'101010101000001'	'0110001001000001'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 501.

7.2.4.3.5	Radio unit response

Tests 70236A to 70240A

The correct acquisition of the traffic channel in response to codeword 201 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 501 (RQR) on the channel number specified in 7.2.4.3.4.

7.2.5	120 hour memory protection for prime registration data

7.2.5.1	Test number

70241A

7.2.5.2	Purpose of test

To ensure that the radio unit is capable of maintaining an AREA field value relating to its current prime registration without corruption for a minimum period of 120 hours with all external power supplies removed in accordance with 6.2, 9.2.1 and 10.2.1 of MPT1343.

7.2.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 364 and 367 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 367 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70240A immediately prior to test 70241A).

The forward trafficcontrol channels shall carry the values of field in the CCSC as specified for test 70240A.

The forward traffic channel shall be modulated with a suitable audio test tone (for example, a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.5.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 367.

The Tester shall then turn off the radio unit, removing all external sources of power.

At a suitable time the Tester shall cause the General Test Assembly to cease to radiate control channels on channel numbers 364 and 367 and to replace these by two new control channels with channel numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

372	'101010010000001'	'0110101111100101'

377	'101010101000001'	'0110001001000001'

The level of channel number 377 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating and after a period of 120 hours has elapsed since the radio unit was switched off the Tester shall restore power to the radio unit and switch it on.

At some appropriate time the Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 377.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.2.5.5	Radio unit response

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.6	Leaving a control channel and hunting for a new control channel (B personalisation)

The following is a repeat of the tests specified in 7.2.2 but with values specified appropriate to the B personalisation.

7.2.6.1	Leaving upon receipt of MOVE

7.2.6.1.1	Test number

70201B

7.2.6.1.2	Purpose of test

To ensure that the radio unit leaves the current control channel, enters the control channel acquisition procedures and acquires the nominated control channel after receiving an applicable MOVE codeword on its current control channel in accordance with 9.3.3.2.2, 9.3.4 and 9.4.1 of MPT1343.

7.2.6.1.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160.  The radio unit shall be in the idle state on control channel number 157 immediately prior to the test.  (This may be achieved more easily if the radiation of the forward control channel on channel number 160 is delayed until after the radio unit has acquired channel number 157).

The forward control channels on channel numbers 157 and 160 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

157	'000101000010001'	'0101011100101101'

160	'000101000011001'	'1010111011101010'

7.2.6.1.4	Test method

The Tester shall use the General Test Assembly to send MOVE codeword number 703 with the value of the CONT field set to '0001100111' within the forward message stream on channel number 157.

At some appropriate time the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 160.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.2.6.1.5	Radio unit response

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.6.2	This section is reserved

7.2.6.3	Relinquishing upon break in transmission of CCSC

7.2.6.3.1	Test number

70203B

7.2.6.3.2	Purpose of test

To ensure that the radio unit relinquishes the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has failed to receive a decodable CCSC over a continuous period in excess of TS in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.6.3.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160.  The  radio unit shall be in the idle state on control channel number 157 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70202B immediately prior to test 70203B).  The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

7.2.6.3.4	Test method

The Tester shall cause the General Test Assembly to transmit data codeword 802 continuously on forward control channel number 157 in the form of a contiguous stream. Accordingly no CCSC codewords shall be transmitted following the first transmission of data codeword 802.

At some appropriate time after the first transmission of codeword 802, but prior to a period TS has been exceeded, the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 160.  At some appropriate time after the period TS has been exceeded the Tester shall again cause the General Test Assembly to send codeword 603 on forward control channel number 160.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to both transmissions of codeword 603 in the slot immediately following transmission of the codewords.

At some appropriate time after the first transmission of codeword 802, but prior to a period TS has been exceeded, the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 160.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to the transmission of codeword 603 in the slot immediately following transmission of the codeword.

7.2.6.3.5	Radio unit response

The radio unit shall respond to the transmission of codeword 603 (AHYC) with codeword 801 (SAMIS) in the control response slot.

7.2.6.4	Leaving upon change of LAB field value

7.2.6.4.1	Test number

70204B

7.2.6.4.2	Purpose of test

To ensure that the radio unit leaves the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has received, in two consecutive decodable CCSCs, a value of LAB field which indicates that the control category of the radio  unit is no longer permitted access on the current control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.6.4.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160.  The  radio unit shall be in the idle state on control channel number 160 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70203B immediately prior to test 70204B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

7.2.6.4.4	Test method

The Tester shall cause the General Test Assembly to change the SYS field value radiated in one of the CCSCs on the forward control channel number 160 to '000101000011100' (with a corresponding change in CCS to '0111011000101001') thus modifying the value of the LAB field to '100'.

At some appropriate time after the transmission of the CCSC with a different value of SYS field the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 157.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following:

Subsequently the Tester shall cause the General Test Assembly to change the SYS field value radiated by all CCSCs on the forward control channel number 160 to '000101000011100'. At some appropriate time after this change the Tester shall again cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 157.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to this further transmission of codeword 603 in the slot immediately following.

The General Test Assembly shall then be reset to transmit the values of SYS field specified in test 70201B.

7.2.6.4.5	Radio unit response

The radio unit shall not respond to the first transmission of codeword 603 (AHYC) and shall respond to the second transmission with codeword 801 (SAMIS) in the correct response slot.

7.2.6.5	Leaving due to errors

7.2.6.5.1	Test number

70205B

7.2.6.5.2	Purpose of test

To ensure that the radio unit leaves the current control channel, and enters the control channel acquisition procedures and acquires another control channel after it has detected errors exceeding the prescribed rate on the received forward control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.6.5.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160, set to give a nominal signal level at the receiver of -103 dBm.

The radio unit shall be in the idle state on control channel number 157 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70204B immediately prior to test 70205B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

7.2.6.5.4	Test method

The Tester shall cause the General Test Assembly to produce "errored" codewords in the radio unit by reducing the signal to noise ratio perceived by the radio unit to 0 dB measured over a channel bandwidth.  This reduction shall not start prior to bit 3 nor finish later than bit 62 of selected codewords and shall last at full value for 30 ±10 ms (Refer to Section 4.2.1).

The Tester shall cause the General Test Assembly to send a repeating pattern of 2 "errored" codewords in consecutive groups of 4 codewords on control channel number 157 whilst maintaining an "unerrored" control channel on channel number 160.

At some appropriate time after "errored" codewords have begun to be transmitted, the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 160.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

The General Test Assembly shall then be reset to transmit "unerrored" codewords continuously.

7.2.6.5.5	Radio unit response

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.6.6	Leaving due to receipt of CCSC with different SYS value

7.2.6.6.1	Test number

70206B

7.2.6.6.2	Purpose of test

To ensure that the radio unit leaves the current control channel, enters the control channel acquisition procedures and acquires another control channel after it has detected CCSCs with a value of SYS field different to the verified value and that, by treating these as errored codewords, the error codeword rate exceeds the prescribed level on the received forward control channel in accordance with 9.3.3.3, 9.3.4 and 9.4.1 of MPT1343.

7.2.6.6.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160, set to give a nominal signal level at the receiver of -103 dBm.

The radio unit shall be in the idle state on control channel number 160 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70205B immediately prior to test 70206B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

7.2.6.6.4	Test method

The Tester shall cause the General Test Assembly to insert codeword 102 with the SYS field value '000101000100001' and CCS value '0011001111000001' in place of some of the CCSCs with values as prescribed in 7.2.6.6.3.  The General Test Assembly shall insert codeword 102 in this way as the first codeword in one slot on the forward control channel number 160 followed by one slot with the CCSC specified in 7.2.6.6.3.  Data codewords shall not displace any inserted codeword 102 or the CCSC following any inserted codeword 102.

At some appropriate time after the commencement of the insertion of codewords 102, the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 157.

The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

The General Test Assembly shall then be reset to the CCSC field values specified in test 70201B.

7.2.6.6.5	Radio unit response

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.7	Leaving a traffic channel and hunting for a new control channel

The following series a repeat of the tests specified in 7.2.3 but with values specified appropriate to the B personalisation.

7.2.7.1	Resuming a control channel

7.2.7.1.1	Test number

70207B

7.2.7.1.2	Purpose of test

To ensure that the radio unit leaves the current traffic channel, enters the control channel acquisition procedures and re-acquires the control channel on which it was confirmed prior to being active on the traffic channel after receiving an applicable CLEAR codeword on its current traffic channel in accordance with 9.3.3.2.1 and 9.3.4.2 of MPT1343.

7.2.7.1.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160 and a traffic channel on channel number 309.  The radio unit shall be in the idle state on control channel number 157 immediately prior to the test. (This may be conveniently achieved by the successful completion of test 70206B immediately prior to test 70207B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 157.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 157.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.2.7.1.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.7.2	Control channel acquisition by single channel hunt

7.2.7.2.1	Test number

70208B

7.2.7.2.2	Purpose of test

To ensure that the radio unit leaves the current traffic channel, enters the control channel acquisition procedures and acquires a nominated control channel after receiving an applicable CLEAR codeword on its current  traffic channel in accordance with 9.3.3.2 and 9.3.4 of MPT1343.

7.2.7.2.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 157 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70207B immediately prior to test 70208B).  The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 157.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0001100111' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 160.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.2.7.2.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.7.3	Preferential hunt

7.2.7.3.1	Test numbers

70209B to 70213B

7.2.7.3.2	Purpose of tests

To ensure that the radio unit leaves the current traffic channel, enters the control channel acquisition procedures and acquires a control channel as a result of a preferential hunt after receiving an applicable CLEAR codeword on its current traffic channel but being unable to reaquire the control channel on which it was confirmed prior to being active on the traffic channel in accordance with 9.3.3.2.1, 9.3.3 and 9.3.4 of MPT1343.

7.2.7.3.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160 and a  traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 160 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70208B immediately prior to test 70209B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70201B.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.3.4	Test method

Test 70209B

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 160.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 157 and 160 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

163	'000101000110001'	'1111100001110000'

166	'000101000011001'	'1010111011101010'

The level of channel number 166 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 166.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

Tests 70210B to 70213B

Test 70210B shall be performed following the successful completion of test 70209B since the initialisation required for 70210B is the final state for 70209B.

Likewise 70211B shall follow 70210B, 70212B shall follow 70211B and 70213B shall follow 70212B.

The test method shall follow the method given for test 70209B with the following modifications:

The channel number on which codeword 204 is transmitted shall be as follows:

Test number	Channel Number

70210B		166

70211B		172

70212B		178

70213B		247

The two new control channels to be radiated for each test shall be as follows:

Test number	Channel Number		SYS		CCS

70210B		169	'000101000110001'	'1111100001110000'

172	'000101000011001'	'1010111011101010'

70211B		175	'000101000110001'	'1111100001110000'

178	'000101000011001'	'1010111011101010'

70212B		181	'000101000110001'	'1111100001110000'

247	'000101000011001'	'1010111011101010'

70213B		254	'000101000110001'	'1111100001110000'

281	'000101000011001'	'1010111011101010'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

Codeword 603 shall be transmitted on the second channel number in each pair.

7.2.7.3.5	Radio unit response

Tests 70209B to 70213B

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.2.7.4	Normal hunt at level L2

7.2.7.4.1	Test numbers

70214B to 70218B

7.2.7.4.2	Purpose of tests

To ensure that the radio unit leaves the current traffic channel, enters the control channel acquisition procedures and acquires a control channel as a result of a normal hunt at the higher level after receiving an applicable CLEAR codeword on its current traffic channel, but being unable to reacquire the control channel on which it was confirmed prior to being active on the traffic channel, or an alternative control channel with the same AREA code in accordance with 9.3.3.2.1, 9.3.3.3, 9.3.3.4 and 9.3.4 of MPT1343.

7.2.7.4.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 254 and 281 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 281 immediately prior to the test. (This may be conveniently achieved by the successful completion of test 70213B immediately prior to test 70214B).

The forward control channels on channel numbers 254 and 281 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

254	'000101000110001'	'1111100001110000'

281	'000101000011001'	'1010111011101010'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.4.4	Test method

Test 70214B

The Tester shall use the General Test Assembly to send codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 281.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 254 and 281 and to replace these by two new control channels with channel numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS			CCS

284	'000101000110001'	'1111100001110000'

287	'000101000111001'	'0000000110110111'

The level of channel number 287 shall be set to give a nominal value at the receiver of -103 dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 284 for codeword 502 from the radio unit.  Upon receipt of codeword 502 from the radio unit the General Test Assembly shall respond with codeword 403 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70215B to 70218B

Test 70215B shall be performed following the successful completion of test 70214B since the initialisation required for 70215B is the final state for 70214B.

Likewise 70216B shall follow 70215B, 70217B shall follow 70216B and 70218B shall follow 70217B.

The test method shall follow the method given for test 70214B with the following modifications:

The channel number on which codeword 204 is transmitted shall be as follows:

Test number	Channel Number

70215B		284

70216B		290

70217B		357

70218B		363

The two new control channels to be radiated for each test shall be as follows:

Test number	Channel Number		SYS		CCS

70215B		290	'000101001010001'	'0011000110101000'

293	'000101001011001'	'1100100001101111'

70216B		357	'000101000100001'	'0011001111000001'

360	'000101000101001'	'1100101000000110'

70217B		363	'000101000010001'	'0101011100101101'

366	'000101000011001'	'1010111011101010'

70218B		157	'000101001000001'	'1111101000011001'

160	'000101001001001'	'0000001111011110'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the first channel number in the pair for codeword 502.

7.2.7.4.5	Radio unit response

Tests 70214B to 70218B

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 502 (RQR) on the channel number specified in 7.2.7.4.4.

7.2.7.5	Error checking prior to confirmation

7.2.7.5.1	Test numbers

70219B to 70223B

7.2.7.5.2	Purpose of tests

To ensure that the radio unit does not confirm a control channel with an excessive codeword error rate during the hunting procedures in accordance with 9.3.4.3 and 9.3.4.4 of MPT1343.

7.2.7.5.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 157 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70218B immediately prior to test 70219B).

The forward control channels on channel numbers 157 and 160 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

157	'000101001000001'	'1111101000011001'

160	'000101001001001'	'0000001111011110'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.5.4	Test method

Test 70219B

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 157.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate control channels on channel numbers 157 and 160 and to replace these by two new control channels with channel numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

163	'000101001010001'	'0011000110101000'

166	'000101001011001'	'1100100001101111'

The level of both control channels shall be set to give a nominal value at the receiver of -103dBm.

The Tester shall cause the General Test Assembly to produce "errored" codewords in the radio unit by reducing the signal to noise ratio perceived by the radio unit to 0 dB measured over a channel bandwidth.  This reduction shall not start prior to bit 3 nor finish later than bit 62 of selected codewords and shall last at full value for 30 ±10 ms (Refer to Section 4.2.1).

The Tester shall cause the General Test Assembly to send a repeating pattern of one "errored" codeword in consecutive groups of 15 codewords on control channel number 163 whilst maintaining an "unerrored" control channel on channel number 166.

At some appropriate time after both new control channels are radiating and "errored" codewords have begun to be transmitted, the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 166 for codeword 502 from the radio unit.  Upon receipt of codeword 502 from the radio unit the General Test Assembly shall respond with codeword 403 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70220B to 70223B

Test 70220B shall be performed following the successful completion of test 70219B since the initialisation required for 70220B is the final state for 70219B.

Likewise 70221B shall follow 70220B, 70222B shall follow 70221B and 70223B shall follow 70222B.

The test method shall follow the method given for test 70219B with the following modifications:

The channel number on which codeword 204 is transmitted shall be as follows:

Test number	Channel Number

70220B		166

70221B		172

70222B		178

70223B		247

The two new control channels to be radiated for each test shall be as follows:

Test number	Channel Number		SYS		CCS

70220B		169	'000101000100001'	'0011001111000001'

172	'000101000101001'	'1100101000000110'

70221B		175	'000101000010001'	'0101011100101101'

178	'000101000011001'	'1010111011101010'

70222B		181	'000101000110001'	'1111100001110000'

247	'000101000111001'	'0000000110110111'

70223B		254	'000101001000001'	'1111101000011001'

281	'000101001001001'	'0000001111011110'

In each case the first channel number in the pair shall have "errored" codewords introduced and the second channel number shall be transmitted "unerrored".

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 502.

7.2.7.5.5	Radio unit response

Tests 70219B to 70223B

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 502 (RQR) on the channel number specified in 7.2.7.5.4.

7.2.7.6	(Section not used).

7.2.7.7	Comprehensive hunt suppression facility

7.2.7.7.1	Test number

70225B

7.2.7.7.2	Purpose of test

To verify the correct operation of the comprehensive hunt suppression facility by network personalisation in accordance with 9.3.3.5 of MPT1343.

7.2.7.7.3  Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 208 and 281 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 281 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70223B immediately prior to test 70225B).

The forward control channels on channel numbers 208 and 281 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

208	'000101001001001'	'0000001111011110'

281	'000101001001001'	'0000001111011110'

The forward traffic channel shall be modulated wih a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.7.7.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 281.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate the control channel on channel number 281.

After a suitable time the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate  to a traffic channel).

At some appropriate time after the transmission of codeword 702 the Tester shall cause the General Test Assembly to send codeword 603 on the forward control channel on channel number 208.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.2.7.7.5	Radio unit response

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall not respond to codeword 603 (AHYC).

7.2.8	Use of acquisition authorisation data (B personalisation)

The following is a repeat of the tests specified in 7.2.4 but with values specified appropriate to the B personalisation.

7.2.8.1	Application of OPID field personalisation

7.2.8.1.1	Test numbers

70226B to 70230B

7.2.8.1.2	Purpose of tests

To ensure that a radio unit acquires a control channel bearing the OPID  (Network Identity) sub-field value of the selected network in preference to one that bears an unknown OPID sub-field in accordance with 9.3.4.2.4 of MPT1343.

7.2.8.1.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 254 and 281 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 281 immediately prior to the test.  (This may be achieved more easily if the radiation of the forward control channel on channel number 254 is delayed until after the radio unit has acquired channel number 281).

The forward control channels on channel numbers 254 and 281 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with field values set as follows:

Channel Number		SYS		CCS

254	'000101001000001'	'1111101000011001'

281	'000101001001001'	'0000001111011110'

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.8.1.4	Test method

Test 70226B

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 281.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test  Assembly to cease to radiate control channels on channel numbers 254 and 281 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

284	'000101011010001'	'1111110010100010'

287	'000101001011001'	'1100100001101111'

The level of channel number 287 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 287 for codeword 502 from the radio unit.  Upon receipt of codeword 502 from the radio unit the General Test Assembly shall respond with codeword 403 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70227B to 70230B

Test 70227B shall be performed following the successful completion of test 70226B since the initialisation required for 70227B is the final state for 70226B.

Likewise 70228B shall follow 70227B, 70229B shall follow 70228B and 70230B shall follow 70229B.

The test method shall follow the method given for test 70226B with the following modifications:

The channel number on which codeword 204 is transmitted shall be as follows:

Test number	Channel Number

70227B		287

70228B		293

70229B		360

70230B		366

The two new control channels to be radiated for each test shall be as follows:

Test number	Channel Number		SYS		CCS

70227B		290	'000111000100001'	'0000111000100001'

293	'000101000101001'	'1100101100010011'

70228B		357	'010101000110001'	'0101111110000101'

360	'000101000111001'	'1100100001101111'

70229B		363	'010001001000001'	'1001011000011110'

366	'000101001001001'	'1100100001101111'

70230B		157	'000110001010001'	'1011010100010011'

160	'000101001011001'	'1100100001101111'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 502.

7.2.8.1.5	Radio unit response

Tests 70226B to 70230B

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio  unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 502 (RQR) on the channel number specified in 7.2.8.1.4.

7.2.8.2	Application of ZONE field personalisation

No tests specified for B personalisation.

7.2.8.3	Application of AREA field personalisation

7.2.8.3.1	Test numbers

70236B to 70240B

7.2.8.3.2	Purpose of tests

To ensure that the radio unit acquires a control channel bearing an AREA sub-field value for which acquisition is authorised in its network personalisation data in preference to one that bears an unknown AREA sub-field in accordance with 9.3.4.2.4 of MPT1343.

7.2.8.3.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating two forward control channels on channel numbers 157 and 160 and a traffic channel on channel number 309.

The radio unit shall be in the idle state on control channel number 160 immediately prior to the test.  (This may be conveniently achieved by the successful completion of test 70230B immediately prior to test 70236B).

The forward control channels shall carry the values of fields in the CCSC as specified for test 70230B.

The forward traffic channel shall be modulated with a suitable audio test tone (for example a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation).

7.2.8.3.4	Test method

Test 70236B

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 160.

Once the Tester has confirmed that the radio unit is active on the traffic channel by aural detection of the test tone he shall cause the General Test Assembly to cease to radiate control channels on channel numbers 157 and 160 and to replace these by two new control channels with channels numbers and values of fields in the CCSC (codeword 102) as follows:

Channel Number		SYS		CCS

163	'000101001110001'	'1001111011110101'

166	'000101000011001'	'1010111011101010'

The level of channel number 166 shall be set to give a nominal value at the receiver of -103dBm.

When both new control channels are radiating the Tester shall cause the General Test Assembly to interrupt the modulating tone on traffic channel number 309 and transmit codeword 702 with the CHAN field set to '0011111100' and the CONT field set to '0000000000' in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Following the transmission of codeword 702 the General Test Assembly shall monitor the return control channel on channel number 166 for codeword 502 from the radio unit.  Upon receipt of codeword 502 from the radio unit the  General Test Assembly shall respond with codeword 403 in accordance with 7.2.4 and 8.2.1 of MPT1327.

Tests 70237B to 70240B

Test 70237B shall be performed following the successful completion of test 70236B since the initialisation required for 70237B is the final state for 70236B.

Likewise 70238B shall follow 70237B, 70239B shall follow 70238B and 70240B shall follow 70239B.

The test method shall follow the method given for test 70236B with the following modifications:

The channel number on which codeword 204 is transmitted shall be as follows:

Test number		Channel Number

70237B			166

70238B			172

70239B			178

70240B			247

The two new control channels to be radiated for each test shall be as follows:

Test number	Channel Number		SYS		CCS

70237B		169	'000101001110001'	'1001111011110101'

172	'000101000100001'	'0011001111000001'

70238B		175	'000101001110001'	'1001111011110101'

178	'000101000110001'	'1111100001110000'

70239B		181	'000101001110001'	'1001111011110101'

247	'000101001000001'	'1111101000011001'

70240B		254	'000101001110001'	'1001111011110101'

281	'000101001010001'	'0011000110101000'

In each case the second channel number in the pair shall be set to give a nominal value at the receiver of -103 dBm.

The General Test Assembly shall, in each case, monitor the second channel number in the pair for codeword 502.

7.2.8.3.5	Radio unit response

Tests 70236B to 70240B

The correct acquisition of the traffic channel in response to codeword 204 (GTC) shall be confirmed by aural detection of the test tone from the radio unit transducer.

After transmission of codeword 702 (CLEAR) the tone shall no longer be heard from the radio unit.

The radio unit shall transmit codeword 502 (RQR) on the channel number specified in 7.2.8.3.4.

7.3	(Section not used).

7.4	Registration

7.4.1	Introduction

The tests in the following sections are to be applied using the A personalisation only:  see sections 3.1 and 3.2 of this specification.

The Tester shall carry out the test procedures specified in the following sections which are designed to test that the radio unit is able to send registration requests and maintain registration records in accordance with MPT1343.

7.4.2	General test procedures

In the following tests the radio unit shall be connected to the General Test Assembly which shall be radiating one control channel on one of three specified channel numbers.  For each channel number the value of AREA sub-field in the SYS field specified is chosen from three candidate values as follows:

Channel	Candidate values	Registration record

number	of AREA sub-field	code

58	'0010001'		A

'0010100'		D

'0010111'		G

282	'0010010'		B

'0010101'		E

'0011000'		H

377	'0010011'		C

'0010110'		F

'0100010'		J

The starting conditions of most tests are dependant on the final conditions at the end of the previous test.  Therefore the tests described below shall  follow only in the prescribed order. Except as otherwise specified, the tests shall be carried out within the registration timeout periods present in the radio unit and without switch-off or network changing of the radio unit.  No general rules can be given for resumption of testing if the tests  suffer a significant interruption.  Testers should be aware that incorrect resumption of testing may result in drawing incorrect conclusions from the radio unit response.

A diagram is appended at the end of each test, which represents the registration records held within the radio unit during the test which is described.  These diagrams are provided for illustrative purposes only since the test procedures do not allow these records to be examined directly.

In the representations of registration records, the codes A to J are used to denote registration records containing the AREA sub-field values as detailed in the tabulation above.  An additional character appended to the letter is used to denote the type of registration record as follows:

undefined (-), normal (0), or temporary (1).

7.4.3	Test procedures

7.4.3.1	Registration and registration records

7.4.3.1.1	Test number

70401A

7.4.3.1.2	Purpose of test

To initialise the radio unit for the test sequence.

To establish that the radio unit is capable of sending registration requests and to establish a known primary registration record and value of NA for subsequent tests in accordance with 10.4.1 and 10.7 of MPT1343.

7.4.3.1.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.  The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001000001' and the value of the CCS field set to '1100000001101010'.

The radio unit shall be switched off prior to connection to the General Test Assembly and remain switched off until the commencement of the test.

7.4.3.1.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored, and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The radio unit shall be switched on.  After a suitable period (for channel hunting the Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '01'.

The Tester shall then cause the General Test Assembly to change the value of SYS field in all codewords 102 radiated on the forward control channel on channel number 58 to '101001000100001' with a corresponding change in CCS to '1000110100001001' and shall monitor the return control channel for codeword 501 from the radio unit.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001000100001' and the value of NA set to '11'.

7.4.3.1.5	Radio unit response

The radio unit may transmit codeword 501 (RQR) as a random access request after being switched on depending upon the registration record value stored in the radio unit prior to the test taking place.  After the value of the SYS field is changed the radio unit shall transmit codeword 501 (RQR) as a random access request.

7.4.3.1.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| ? | J0 | A0- | A0 | Prime |
|  |  |  | NULL | Second |
|  |  |  | NULL | Third |

1	-	At commencement of test

2	-	Upon first successful registration and following receipt of first BCAST message

3	-	Upon second successful registration

4	-	On receipt of second BCAST message

7.4.3.2	Retention of primary registration at switch-off

7.4.3.2.1	Test number

70402A

7.4.3.2.2	Purpose of test

To establish that the radio unit retains the primary registration when switched off in accordance with 10.2.1 a(ii), 10.4.1 and 10.3 of MPT1343.

7.4.3.2.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70401A.

7.4.3.2.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

Without changing the content of the General Test Assembly simulated control channel transmissions, the Tester shall switch off the radio unit in accordance with the manufacturer's instructions and, after a short period of time, shall switch the unit back on.

7.4.3.2.5	Radio unit response

When the radio unit is switched on, it shall not generate a random access registration attempt.

7.4.3.2.6	States of radio unit registration records during test

(For information only).

| 1 | 2 |  |
| --- | --- | --- |
| A0 | A0 | Prime |
| NULL | NULL | Second |
| NULL | NULL | Third |

1	- 	At commencement of test.

2 	- 	At end of test.

7.4.3.3	Acceptance and retention of secondary/tertiary registration records

7.4.3.3.1	Test number

70403A

7.4.3.3.2	Purpose of test

To establish that the radio unit is capable of accepting secondary and tertiary registration records and that these records are retained by the radio unit in acordance with 10.4.1, 10.2.1 and 10.3 (iv) of MPT1343.

7.4.3.3.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70402A.

7.4.3.3.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall cause the General Test Assembly to cease to radiate the control channel on channel number 58 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set   to '101001001001001' and the value of the CCS field set to'1011110100010110'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to'101001001100001' and the value of the CCS field set to '1110101110001100'.

Finally after the General Test Assembly has again received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 58 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCSC field set to'1000110100001001'.

7.4.3.3.5	Radio unit response

Each time that the control channel number is changed it shall be followed by a transmission of codeword 501 from the radio unit with the exception of the final change in channel number to 58 which shall not result in a transmission by the radio unit.

7.4.3.3.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 |  |
| --- | --- | --- | --- |
| A0 | B- | C- | Prime |
| NULL | A0 | B- | Second |
| NULL | NULL | NULL | Third |

1	- 	At commencement of test.

2	- 	After First ACK(QUAL='0').

3	- 	After Second ACK(QUAL='0').

7.4.3.4	Reduction of number of registration records

7.4.3.4.1	Test number

70404A

7.4.3.4.2	Purpose of test

To establish that the radio unit responds correctly when instructed to reduce the total number of registration records to two in acordance with 10.4.1 and 10.7 of MPT1343.

7.4.3.4.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70403A.

7.4.3.4.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001000100001' and the value of NA set to '10'.

7.4.3.4.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) following the transmission by the General Test Assembly of codeword 704 (BCAST).

7.4.3.4.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 |  |
| --- | --- | --- | --- |
| C | C- | A0 | Prime |
| B- | B- | C- | Second |
| A0 |  |  | Third |

1	- 	At commencement of test.

2	- 	On receipt of BCAST SYSDEF '00011'.

3	- 	After General Test Assembly sends ACK(QUAL='0').

7.4.3.5	Displacement of Primary record on implicit response

7.4.3.5.1	Test number

70405A

7.4.3.5.2	Purpose of test

To establish that an implicit registration response results in the displacement of the primary registration record in accordance with 10.4.1 and 10.5 of MPT1343.  (The presence of the correct primary registration record after an implicit registration response is verified by displacing it and checking its presence as a secondary registration record).

7.4.3.5.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70404A.

7.4.3.5.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.  Further the General Test Assembly shall respond to any transmission of codeword 503 (RQS) from the radio unit with codeword 404 (ACKB(QUAL='0')), in accordance with 7.2.4 and 9.1.1.1 of MPT1327.

The Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 58 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

After a suitable time has elapsed the Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3).

After the General Test Assembly has transmitted codeword 404 following receipt of codeword 503 from the radio unit the Tester shall then cause the  General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001001001' and the value of the CCS field set to '1011110100010110'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

7.4.3.5.5	Radio unit response

The radio unit shall not make any transmission subsequent to the change from channel number 58 to channel number 377 until it transmits codeword 503 (RQS) following initiation of the simple speech call request.

The radio unit shall transmit codeword 501 (RQR) subsequent to the change from channel number 377 to channel number 282 but shall make no transmission subsequent to the change from channel number 282 to channel number 377.

7.4.3.5.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 |  |
| --- | --- | --- | --- |
| A0 | C- | B- | Prime |
| C- | A0 | C- | Second |

1	- 	At commencement of test.

2	- 	After General Test Assembly sends ACKB(QUAL='0').

3	- 	After General Test Assembly sends ACK(QUAL='0').

7.4.3.6	Reduction of number of registration records to one

7.4.3.6.1	Test number

70406A

7.4.3.6.2	Purpose of test

To establish that the radio unit responds correctly when instructed to reduce the total number of registration records to one in accordance with 10.4.1 and 10.7 of MPT1343.

7.4.3.6.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 377.

The forward control channel on channel number 377 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

The radio unit shall be in the state prevailing at the end of test 70405A.

7.4.3.6.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')).

The Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001001100001' and the value of NA set to '01'.

The Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001001001' and the value of the CCS field set to '1011110100010110'.

7.4.3.6.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) subsequent to the transmission by the General Test Assembly of codeword 704 (BCAST).  The radio unit shall again transmit codeword 501 subsequent to the change from channel number 377 to channel number 282.

7.4.3.6.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| B- | B- | C0 | B- | Prime |
| C- |  |  |  | Second |

1	-	At commencement of test.

2	- 	On receipt of BCAST SYSDEF '00011'.

3	-	After General Test Assembly sends first ACK(QUAL='0').

4	- 	After General Test Assembly sends second ACK(QUAL='0').

7.4.3.7	Correct response to registration failure ACKX(QUAL=1)

7.4.3.7.1	Test number

70407A

7.4.3.7.2	Purpose of test

To establish the correct response to registration failure ACKX(QUAL='1') in accordance with 8.2.2 of MPT1327 and 10.4.1 of MPT1343.

7.4.3.7.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The forward control channel on channel number 282 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001001001' and the value of the CCS field set to '1011110100010110'.

The radio unit shall be in the state prevailing at the end of test 70406A.

7.4.3.7.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), except as otherwise specified.

The Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001001001001' and the value of NA set to '10'.

The Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 58 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The General Test Assembly shall respond to the first tranmission of codeword 501 by the radio unit subsequent to the change of channel from channel number 282 to channel number 58 with codeword 405 (ACKX(QUAL='1')), in accordance with 7.2.4 and 8.2.1 of MPT1327.  Any subsequent transmission of codeword 501 by the radio unit shall be responded to by codeword 401 as specified above.

7.4.3.7.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) subsequent to the change of channel number.  It shall again transmit codeword 501 (RQR) within a suitable period (to allow for channel hunting) of receiving codeword 405 from the General Test Assembly.

7.4.3.7.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 |  |
| --- | --- | --- | --- |
| B- | B0 | A- | Prime |
|  | NULL | B0 | Second |

1	-  	At commencement of test.

2	-  	After BCAST NA='10'.

3	-  	After successful registration.

7.4.3.8	Registration denial and inhibition of random access

7.4.3.8.1	Test number

70408A

7.4.3.8.2	Purpose of test

To establish that registration denied ACKX(QUAL='0') results in inhibition of random access call requests, and that at least eight registration denied records are held by the radio unit in accordance with 7.2.4 and 8.2.1 of MPT1327 and 10.2.1, 10.4 and 10.5 of MPT1343.

7.4.3.8.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70407A.

7.4.3.8.4	Method

For the duration of the first part of the test any transmissions of the radio unit on the return control channel shall be monitored and the General  Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 417 (ACKX(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall initiate a sequence of channel changes by causing the General Test Assembly to cease to radiate the control channel on channel number 58 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001000001' and the value of the CCS field set to '1100000001101010'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 417 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001100001001' and the value of the CCS field set to '0111100110111000'.

This procedure shall be repeated for the following new combinations of channel number, SYS field value and CCS value.

Channel Number		SYS		CCS

58	'101001011100001'	'0010011010000110'

377	'101001011000001'	'1000100111011011'

282	'101001010101001'	'1011100111000100'

58	'101001010000001'	'1110111101011110'

377	'101001001100001'	'1110101110001100'

282	'101001001001001'	'1011110100010110'

After this first sequence has been completed and the General Test Assembly has received codeword 501 from the radio unit on channel number 282 and has acknowledged this request with codeword 417 the Tester shall, after a suitable time has elapsed, cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3), noting any audible of visual confidence indicator states which occur as a result of the call request.

The Tester shall then initiate a second sequence of channel changes by causing the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 377 which shall carry codeword 102 at the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CSC field set to '1110101110001100'.

After a suitable time has elapsed the Tester shall again cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079  (see 7.4.2), noting any audible or visual confidence indicator states which occur as a result of the call request.

This procedure of channel change and initiation of a simple speech call request shall be repeated for the following new combinations of channel number, SYS field value and CCS value:

Channel Number		SYS		CCS

58	'101001010000001'	'1110111101011110'

282	'101001010101001'	'1011100111000100'

58	'101001011100001'	'0010011010000110'

377	'101010001000001'	'1100000001101010'

58	'101001000100001'	'1000110100001001'

After the last change of channel number (i.e., 377 to 58) the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit on control channel number 58 with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

Further the General Test Assembly shall respond to any transmission of codeword 503 (RQS) from the radio unit with codeword 605 (AHY) followed by codeword 404 (ACKB(QUAL='0')), in accordance with 7.2.4 and 9.1.1.1 of MPT1327.

7.4.3.8.5	Radio unit response

During the first sequence of changes in control channel number the radio unit shall transmit codeword 501 (RQR) once on each new control channel.

The call request initiation at the completion of the first sequence of channel changes shall result in a "call fail" condition being indicated by the radio unit and no transmission shall eminate from the radio unit.

During the second sequence of control channel changes no transmissions shall eminate from the radio unit and all call request initiations shall result in a "call fail" condition being indicated by the radio unit except   following the last change from channel number 377 to 58.  Subsequent to this change the radio unit shall transmit codeword 501 (RQR).  The call request initiated following receipt of this codeword shall result in the radio unit transmitting codeword 503 (RQS).

7.4.3.8.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| A- | NULL | NULL | A- | Prime |
| B0 | A- | NULL | NULL | Second |

1	- 	At commencement of test.

2	- 	After first registration attempt.

3	-	After second registration attempt (No change for subsequent attempts).

4	-	After final registration attempt.

7.4.3.8.7	Denied List

(For information only).

Denied

Registration

Record

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| J | J | J | J | J | J | J | J | First |
|  | H | H | H | H | H | H | H | Second |
|  |  | G | G | G | G | G | G | Third |
|  |  |  | F | F | F | F | F | Fourth |
|  |  |  |  | E | E | E | E | Fifth |
|  |  |  |  |  | D | D | D | Sixth |
|  |  |  |  |  |  | C | C | Seventh |
|  |  |  |  |  |  |  | B | Eighth |

1	-	After first transmission of codeword 406

2	-	After second transmission of codeword 406

3	-	After third transmsission of codeword 406

4	-	After fourth transmission of codeword 406

5	-	After fifth transmission of codeword 406

6	-	After sixth transmission of codeword 406

7	-	After seventh tranmsission of codeword 406

8	-	After eighth transmission of codeword 406

7.4.3.9	Deletion of 'registration denied' records

7.4.3.9.1	Test number

70409A

7.4.3.9.2	Purpose of test

To establish that registration denied records are deleted by the radio unit at switch off or switch on in accordance with 10.2.2 of MPT1343.

7.4.3.9.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70408A.

7.4.3.9.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 58 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

After a suitable time has elapsed the Tester shall switch the radio unit off in accordance with the manufacturer's instructions.

After a short time the Tester shall switch the radio unit on in accordance with the manufacturer's instructions.

7.4.3.9.5	Radio unit response

The radio unit shall make no transmission prior to being switch off.  Within a suitable period (to allow for channel hunting) of being switched on it shall transmit codeword 501 (RQR).

7.4.3.9.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 |  |
| --- | --- | --- | --- |
| A- | A0 | AC- | Prime |
| NULL | NULL | A0 | Second |

1	-	At commencement of test.

2	 - 	On unit being switched on.

3 	- 	At end of test.

7.4.3.10	Deletion of registration records after TD timeout

7.4.3.10.1	Test number

70410A

7.4.3.10.2	Purpose of test

To establish that registration records (other than the prime record) are deleted after timeout TD in accordance with 10.2.1 and 10.6 of MPT1343.

7.4.3.10.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 377.

The forward control channel on channel number 377 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

The radio unit shall be in the state prevailing at the end of test 70409A.

7.4.3.10.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')).

The Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001001001' and the value of the CCS field set to '1011110100010110'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

The Tester shall initiate a timing device at the instant that codeword 501 is received from the radio unit.

The radio unit shall remain connected to the General Test Assembly in the idle state on channel 377 and the General Test Assembly shall monitor the return control channel for any transmissions by the radio unit.  No further tests shall be performed whilst the radio unit is being monitored in this way.  The timing device shall be stopped upon receipt of a further codeword  501 from the radio unit and the elapsed time recorded.

7.4.3.10.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) subsequent to the change from channel number 377 to channel number 282 but shall make no transmission subsequent to the change from channel number 282 to channel number 377, except that it shall transmit codeword 501 (RQR) within 300 seconds ±30 seconds of its first transmission of codeword 501.

7.4.3.10.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| C- | B- | B- | C- | Prime |
| A- | C- | NULL | B- | Second |

1	-	At commencement of test.

2	- 	Upon first successful registration.

3	-	Upon timeout of TD.

4	-	Upon registering after successful registration timeout.

7.4.3.11	Temporary marking of undefined registration records

7.4.3.11.1	Test number

70411A

7.4.3.11.2	Purpose of test

To establish that an undefined registration record is marked as temporary when REG='1' is received in accordance with 10.4.1, 10.4.2.1 and 10.7 of MPT1343.

7.4.3.11.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 377.

The forward control channel on channel number 377 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001100001' and the value of the CCS field set to '1110101110001100'.

The radio unit shall be in the state prevailing at the end of test 70410A.

7.4.3.11.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')).

The Tester shall cause the General Test Assembly to change the SYS field value radiated in all CCSCs on the forward control channel number 377 to the value '101010001000001' (with corresponding change in CCS to '1100000001101010').

The Tester shall then cause the General Test Assembly to transmit codeword 705 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

After a suitable period has elapsed the Tester shall then cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

7.4.3.11.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) following the change in SYS field value and shall make no further transmissions until the General Test Assembly has transmitted codeword 704, when the radio unit shall again transmit codeword 501.

7.4.3.11.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| C- | J- | J1 | J0 | Prime |
| B- | C- | C- | C- | Second |

1	-	At commencement of test.

2	- 	Upon first successful registration.

3	- 	Upon receipt of codeword 705 (temporary registration state).

4	-	Upon receipt of codeword 704 (normal registration state) and after successful registration.

7.4.3.12	State of normal registration records after REG=1 (Home Zone)

7.4.3.12.1	Test number

70412A

7.4.3.12.2	Purpose of test

To establish that a normal registration record is not marked as temporary when REG='1' is received while the radiated value of the ZONE parameter matches the Home Zone of the radio unit in accordance with 10.4.1 and 10.7 of MPT1343.

7.4.3.12.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 377.

The forward control channel on channel number 377 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001000001' and the value of the CCS field set to '1100000001101010'.

The radio unit shall be in the state prevailing at the end of test 70411A.

7.4.3.12.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')).

The Tester shall then cause the General Test Assembly to transmit codeword 705 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

After a suitable period has elapsed the Tester shall then cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

7.4.3.12.5	Radio unit response

The radio unit shall make no transmissions for the duration of the tests.

7.4.3.12.6	States of radio unit registration records during test

(For information only).

| 1 |  |
| --- | --- |
| J0 | Prime |
| C- | Second |

1	-	A  commencement of, and for the duration of test.

7.4.3.13	State of normal registration records after REG=1 (Home Zone)

7.4.3.13.1	Test number

70413A

7.4.3.13.2	Purpose of test

To establish that a normal registration record is marked as temporary when REG='1' is received while the radiated value of the ZONE parameter does not match the Home Zone of the radio unit in accordance with 10.4.1 and 10.7 of MPT1343.  (Note that the AREA sub-field denoted 'A' has a zone value which does not match Home Zone of the radio unit).

7.4.3.13.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 377.

The forward control channel on channel number 377 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001000001' and the value of the CCS field set to '1100000001101010'.

The radio unit shall be in the state prevailing at the end of test 70412A.

7.4.3.13.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall cause the General Test Assembly to cease to radiate the control channel on channel number 377 and to replace this by a control channel on channel number 58 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall cause the General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001000100001' and the value of NA set to '10'.

After a suitable period has elapsed, the Tester shall cause the General Test Assembly to transmit codeword 705 (BCAST) with the value of the SYS field set to '101001000100001' and the value of NA set to '10'.

Finally, after a further suitable period has elapsed, the Tester shall cause the General Test Assembly again to transmit codeword 704 (BCAST) with the value of the SYS field set to '101001000100001' and the value of NA set to '10'.

7.4.3.13.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) following the change in control channel number.  No transmission from the radio unit shall follow the transmission of codewords 704 and 705 (BCAST) except for the final transmission of codeword 704 to which the radio unit shall react by transmitting codeword 501 (RQR).

7.4.3.13.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |  |
| --- | --- | --- | --- | --- | --- |
| J0 | A- | A0 | A1 | A0 | Prime |
| C- | J0 | J0 | J0 | J0 | Second |

1	-	At commencement of test.

2	-	After successful registration following change of channel number.

3	-	After receipt of first codeword 704.

4	-	After receipt of codeword 705.

5   -	After receipt of second codeword 704 and subsequent successful registration.

7.4.3.14	Undefined registration record marking

7.4.3.14.1	Test number

70414A

7.4.3.14.2	Purpose of test

To establish that an undefined registration record is marked as normal at some time between switch off and being made ready for a new session in accordance with 10.2.1, 10.2.2, 10.4.1 and 10.7 of MPT1343.  The resultant normal registration record is verified by showing that it is not marked as temporary by means of the appropriate BCAST message received whilst in the Home Zone.

7.4.3.14.3	Initialisation

The radio unit shall utilise the A personalisation.

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 58.

The forward control channel on channel number 58 shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001000100001' and the value of the CCS field set to '1000110100001001'.

The radio unit shall be in the state prevailing at the end of test 70413A.

7.4.3.14.4	Method

For the duration of the test any transmissions of the radio unit on the return control channel shall be monitored and the General Test Assembly shall respond to any transmissions of codeword 501 (RQR) from the radio unit with codeword 401 (ACK(QUAL='0')), in accordance with 7.2.4 and 8.2.1 of MPT1327.

The Tester shall cause the General Test Assembly to cease to radiate the control channel on channel number 58 and to replace this by a control channel on channel number 282 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101001001001001' and the value of the CCS field set to '1011110100010110'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall then cause the General Test Assembly to cease to radiate the control channel on channel number 282 and to replace this by a control channel on channel number 377 which shall carry codeword 102 as the CCSC where prescribed in 3.3.3.1 of MPT1327 with the value of the SYS field set to '101010001000001' and the value of the CCS field set to '1100000001101010'.

After the General Test Assembly has received codeword 501 from the radio unit and has acknowledged this request with codeword 401 the Tester shall switch the radio unit off in accordance with the manufacturer's instructions.

After a short time the Tester shall switch the radio unit on in accordance with the manufacturer's instructions.

After a suitable period has elapsed (to allow for channel hunting) the Tester shall cause the General Test Assembly to transmit codeword 705 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

Finally, after a further suitable period has elapsed, the Tester shall cause a General Test Assembly to transmit codeword 704 (BCAST) with the value of the SYS field set to '101010001000001' and the value of NA set to '10'.

7.4.3.14.5	Radio unit response

The radio unit shall transmit codeword 501 (RQR) subsequent to the change from channel number 58 to channel number 282 and subsequent to the change from channel number 282 to channel number 377.  The radio unit shall make no transmissions subsequent upon receiving codeword 704 or codeword 705.

7.4.3.14.6	States of radio unit registration records during test

(For information only).

| 1 | 2 | 3 | 4 |  |
| --- | --- | --- | --- | --- |
| A0 | B- | J- | J0 | Prime |
| J0 | A0 | B- | NULL | Second |

1	-	At commencement of test.

2	-	After first successful registration.

3	-	After second successful registration.

4	-	After switch on to end of test.

7.5	Security Number Checking

7.5.1	Attachment, removal and alteration of the security number device

7.5.1.1	Test number

70501A

7.5.1.2	Purpose of test

To verify that removal of the device containing the security number is likely to damage the radio unit such as to render it inoperative, in accordance with Section 7 of MPT1343.

To verify that adequate provision has been made to prevent alteration of the security number, in accordance with Section 7 of MPT1343.

7.5.1.3	Method

In order to avoid destructive testing of the radio unit, compliance with the requirements of 7.5.1.4 shall be determined by manufacturers declaration or by visual inspection.

7.5.1.4	Requirements

The Tester shall be satisfied that removal of the security number device is likely to render the radio unit inoperative.

The Tester shall be satisfied that adequate provision has been made to stop alteration of the security number device.

7.5.2	Security number marking

7.5.2.1	Test number

70502A

7.5.2.2	Purpose of test

To ensure that the security number marked on the radio unit is sufficiently permanent.

7.5.2.3	Initialisation

The radio unit shall be disassembled (in accordance to the manufacturers instructions) to the point that the security number can be accessed.

7.5.2.4	Method

The Tester shall lightly rub the security number mark for 15 seconds with a cloth soaked in water.

The Tester shall lightly rub the security number mark for 15 seconds with a cloth soaked in petroleum spirit.

7.5.2.5	Requirements

The security number mark shall still be legible after the two 15 second rubbing operations and adhesive labels (if used) shall not have become loose or become curled at the edges.

7.5.3	Security number value

7.5.3.1	Test number

70503A

7.5.3.2	Purpose of test

To ensure that the marked number and manufacturer's details supplied correspond to the security number used in the security check response message.

7.5.3.3	Initialisation

The Tester shall note that the security number details marked on the radio unit and shall then reassemble the radio unit.

The radio unit shall utilise A personalisation.  The tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.5.3.4	Method

The Tester shall cause the General Test Assembly to send codeword 601 on the forward control channel on channel number 282.  The General Test Assembly shall monitor the return control channel for a response from the radio unit to codeword 601 in the slot immediately following.

The General Test Assembly shall retain the content of any received codeword in that slot for later inspection and analysis by the Tester.

7.5.3.5	Radio unit response

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

The analysis of bits in codeword 801 shall confirm the following relationship to the security number information advised by the manufacturer under the provisions of 3.4 and marked on the unit.

Bits 2-9	shall carry the binary equivalent of the manufacturers code (bit 9 is LSB) as marked on the unit and advised under the provisions of 3.4.

Bits 10-13	shall carry the binary equivalent of the model code (bit 13 is LSB) as marked on the unit and advised under the provisions of 3.4.

Bits 14-21	shall carry the check bits as advised under the provisions of 3.4.

Bits 31-48	shall carry the unit serial number (bit 48 is LSB) as marked on the unit and advised under the provisions of 3.4.

7.6	Call Set Up

7.6.1	Introduction

The tests in the following sections are to be applied using the A personalisation only, see section 3.1 and 3.2 of this specification.

The tests in this section are designed to test the mobile procedure for making and receiving calls, in accordance with section 9 of MPT1327 and section 8 of MPT 1343.  The tests in this section cover the following:

a)	Requesting a call.

b)	Indication to user on ACK message.

c)	Go to Channel messages.

d)	Indication to user on AHY message.

The radio units tested will be in one of two categories:

Radio units of the first category (Category I) shall be those fitted with a numerical entry or selection device enabling the manual entry of numbers for the purpose of calling other parties or services available on the network.

Radio units of the second category (Category II) shall be those not fitted with any numerical entry or selection device, but which request calls to other parties using functionally specific control buttons, to select pre-programmed numbers.

The first test of this section determines the category for the radio unit, and then section 7.6.3 shall be employed for Category 1 radio units, while section 7.6.4 shall be employed for Category II radio units.

The Tester shall refer to the radio unit user's guide when observing the confidence indications given to the user.  These shall be unambiguous, in accordance with section 8.1 of MPT1343.

7.6.2	Establishing category of radio unit

7.6.2.1	Test number

70601A

7.6.2.2	Purpose of test

The test is to establish the category into which the radio unit should be placed, and determine the appropriate sequence of tests to be employed.

7.6.2.3	Initialisation

No initialisation is required.

7.6.2.4	Test method

The Tester shall carefully examine the radio unit for numerical entry or selection devices. A radio unit shall be deemed to provide numeric entry if any key is marked by a simple numeric character.

Alphabetic or Alphanumeric entry or selection devices are permissible provided they cannot be confused with the standard marking scheme.  (e.g.,M1, M2, CALL 1).

7.6.2.5	Limits

If the radio unit is equipped with any device for numerical entry or selection, the Tester shall employ tests 70602A to 70622A and 70634A onwards.  If the radio unit is not equipped for numerical entry or selection, the Tester shall employ tests 70623A onwards.

7.6.3	Requesting a call (Category I)

7.6.3.1	RQS generation (individual Call)

7.6.3.1.1	Test number

70602A   Individual, in-fleet call

70603A   Individual, inter-fleet call (same prefix,  small fleet)

70604A   Individual, inter-fleet call (same prefix,  large fleet)

70605A   Individual, inter-fleet call (inter-prefix, small fleet)

70606A   Individual, cross-fleet call (inter-prefix, large fleet)

7.6.3.1.2	Purpose of test

This test is designed to check the radio unit signalling and confidence indication in accordance with Sections 8.1 and 8.2 of MPT1343 and 9.2.1 of MPT1327.

7.6.3.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel number 282 which shall carry codeword 102 as the CCSC with SYS and CCS values as stated in Section 7.4.3.14.4.  The radio unit shall be switched on and in the idle state on this control channel immediately prior to the test.

7.6.3.1.4	Test method

The Tester shall key in each of the following numbers:

Test No:  70602A   275#

Test No:  70603A   3250 42#

Test No:  70604A   5725 223#

Test No:  70605A   320 2850 59#

Test No:  70606A   281 3369 220#

The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indications generated by the radio unit. No responses shall be generated by the General Test Assembly. After each number has been keyed in the radio unit shall be allowed to return to the idle state.

7.6.3.1.5	Radio unit responses

The radio unit shall generate CSUIP indication in accordance with the manufacturers instructions for use when the (#) key is pressed after a valid number.  When the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state.

The RQS messages generated shall be:

Test No:  70602A  Codeword 503

Test No:  70603A  Codeword 507 with IDENT1 = 2522

Test No:  70604A  Codeword 507 with IDENT1 = 7473

Test No:  70605A  Codeword 507 with IDENT1 = 8189

Test No:  70606A  Codeword 507 with IDENT1 = 8189

7.6.3.2	RQS generation (group call)

7.6.3.2.1	Test number

70607A

This test shall be carried out only if the radio unit is declared by the manufacturer to be capable of initialising group calls (see Section 3).

7.6.3.2.2	Purpose of test

This test is designed to check the radio unit signalling and confidence indications, in accordance with Sections 8.1 and 8.2 of MPT1343 and 9.2.1 of MPT1327.

7.6.3.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.6.3.2.4	Test method

The Tester shall key in the following number:

95#

The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indicators generated by the radio unit.

7.6.3.2.5	Radio unit response

The radio unit shall generate CSUIP indication in accordance with the manufacturers instruction for use when the (#) key is pressed.  When the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state.

The RQS message generated shall be codeword 507 with IDENT1 = 605.

7.6.3.3	Invalid number entry (individual calls)

7.6.3.3.1	Test numbers

70608A   Wrong number of digits

70609A   Out of fleet numbering range

7.6.3.3.2	Purpose of test

These tests check that invalid individual number entries are rejected by the radio unit.

7.6.3.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.3.3.4	Test method

The Tester shall key in the following numbers:

Test No:   70608A   35#

Test No:   70609A   455#

The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indications generated by the radio unit.

7.6.3.3.5	Radio unit response

After each number is entered the radio unit shall generate NU indication when the (#) key is pressed.  No transmission by the radio unit shall be recorded by the General Test Assembly.

7.6.3.4	Invalid number entry (group calls)

7.6.3.4.1	Test numbers

70610A	Wrong number of digits

70611A	Inter-fleet call when barred (two digit group)

70612A	Inter-fleet call when barred (three digit group)

70613A	Inter-prefix call when barred (two digit group)

70614A	Inter-prefix call when barred (three digit group)

7.6.3.4.2	Purpose of test

These tests are designed to check that a radio unit will respond correctly when invalid group numbers are entered.

7.6.3.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.3.4.4	Test method

The Tester shall key in the following numbers:

Test No:	70610A	902#

Test No:	70611A	2815 92#

Test No:	70612A	4195 910#

Test No:	70613A	227 5880 93#

Test No:	70614A	238 5360 909#

The tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indications generated by the radio unit.  After each number has been keyed in the radio unit shall be allowed to return to the idle state.

7.6.3.4.5	Radio unit response

After each number is entered the radio unit shall generate N.U indication when the (#) key is pressed.  No transmission by the radio unit shall be recorded by the General Test Assembly.

7.6.3.5	Non-prescribed data calls

7.6.3.5.1	Test number

70615A

This test shall be carried out only if the radio unit is declared by the manufacturer to be capable of initiating non-prescribed data calls (see Section 3).

7.6.3.5.2	Purpose of test

This test is designed to check the correct operation of the radio unit when a non-prescribed data call is requested, in accordance with 5.5.3.1.1 of MPT1327.

7.6.3.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.3.5.4	Test method

The Tester shall key in the following number:

*  31  *   275#

The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indication generated by the radio unit.

7.6.3.5.5	Radio unit response

The radio unit shall generate CSUIP indication when the (#) key is pressed.  After the RQS message has been sent 8 times the radio unit shall generate Call Fail indication and then return to the idle state.  The RQS message sent shall be Codeword 511 with the value of IDENT1 = 2079.

7.6.3.6	Priority calls

7.6.3.6.1	Test number

70616A

This test shall only be carried out if the radio unit is declared by the manufacturer to be capable of initiating priority calls (see Section 3).

7.6.3.6.2	Purpose of test

This test is designed to check that a suitably equipped radio unit is able to select priority for calls, in accordance with 5.5.3.1.1 of MPT1327.

7.6.3.6.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.3.6.4	Test method

The Tester shall key in the following message:

*  8  *275#

The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indication generated by the radio unit.

7.6.3.6.5	Radio unit response

The radio unit shall generate CSUIP indication when the (#) key is pressed.  After the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state.  The RQS message sent shall be codeword 513 with the value of IDENT1 set to 2079.

7.6.3.7	PABX calls

7.6.3.7.1	Test number

70617A	Call to single PABX operator

70618A	Call selected PABX operator

70619A	Call to single PABX extension

70620A	Selected PABX extension (single address codeword call)

70621A	Call to PABXI gateway (extended addressing)

70622A	Call to PABXI gateway (extended addressing)

This test sequence shall only be carried out if the radio unit is declared by the manufacturer to be capable of initiating PABX calls (see Section 3).

7.6.3.7.2	Purpose of test

This test sequence is designed to check that a radio unit generates the correct signalling for PABX calls, in accordance with Sections 8.2.5 of MPT1343 and 9.2.1 of MPT1327.

7.6.3.7.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to this test.

7.6.3.7.4	Test method

The Tester shall key in the following numbers:

Test No:	70617A	0#

Test No:	70618A	05#

Test No:	70619A	2815#

Test No:	70620A	43369#

Test No:	70621A	83369#

Test No:	70622A	788144621#

The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indications generated by the radio unit.  After each number has been keyed in the radio unit shall be allowed to return to the idle state.

7.6.3.7.5	Radio unit response

The radio unit shall generate CSUIP indication when the (#) key is pressed.  After the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state.  The RQS message sent shall be:

Test No:	70617A Codeword 508 with IDENT1 =	8000

Test No:	70618A Codeword 512 with IDENT1 =	8000, EXT='1', 
FLAG1='1', FLAG2='0'

Test No:	70619A Codeword 508 with IDENT1 =	1815

Test No:	70620A Codeword 512 with IDENT1 =	2369, EXT='1', 
FLAG1='0', FLAG2='1'

Test No:	70621A Codeword 512 with IDENT1 =	PABXI (8102), EXT='0', FLAG1='0', FLAG2='0'

Test No:	70622A Codeword 512 with IDENT1 =	PABXI, EXT='0', 
FLAG1='0', FLAG2='0'

7.6.4	Requesting a call (Category II)

The Tester shall program the addresses shown in this section into the radio unit, in accordance with the manufacturers instuctions for use, (see Section 3) so that each address can be requested by the functionally specific control buttons.  If there are more addresses than programmable buttons then the radio unit will have to be programmed more than once.

If bits EXT, FLAG1, FLAG2 and DT are not specified they shall be programmed, or default to '0'.  If LEVEL is not specified it shall be programmed, or default to '1'.

7.6.4.1	RQS generation (Individual Call)

7.6.4.1.1	Test number

70623A	Individual call

70624A	Inter-prefix call

7.6.4.1.2	Purpose of test

This test is designed to check the radio unit signalling and confidence indication in accordance with Sections 8.1 and 8.2 of MPT1343 and 9.2.1 of MPT1327.

7.6.4.1.3	Initialisation

The radio unit utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.4.1.4	Test method

The Tester shall program the two following addresses:

Test No:	70623A	39 2079

Test No:	70624A	120 1739

The Tester shall request each address in turn by use of the functionally specific control buttons.

The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indications generated by the radio unit.  After each address has been requested by the radio unit shall be allowed to return to the idle state.

7.6.4.1.5	Radio unit responses

The radio unit shall generate CSUIP indication in accordance with the manufacturers instructions for use when the functionally specific control button is pressed. When the RQS message has been sent 8 times and the radio unit timer TW has expired the Radio unit shall generate Call Fail indication and then return to the idle state.

The test RQS messages generated shall be:

Test No:	70623A 	Codeword 503

Test No:	70624A 	Codeword 507 with IDENT1 = 8189

7.6.4.2	RQS generation (Group Call)

7.6.4.2.1	Test number

70625A

This test shall be carried out only if the radio unit is declared by the manufacturer to be capable of initiating group calls (see Section 3).

7.6.4.2.2	Purpose of test

This test is designed to check the radio unit signalling and confidence indications, in accordance with Sections 8.1 of MPT1343 and 9.2.1 of MPT1327.

7.6.4.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.6.4.2.4	Test method

The Tester shall program the following address:

39  605

The Tester shall request the address by use of the functionally specific control buttons.  The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indications generated by the radio unit.

7.6.4.2.5	Radio unit response

The radio unit shall generate CSUIP indication in accordance with the manufacturers instruction for use when the functionally specific control button is pressed.  When the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state.

The RQS message generated shall be Codeword 507 with the value of IDENT1 set to 605.

7.6.4.3	Non-prescribed data calls

7.6.4.3.1	Test number

70626A

This test shall be carried out only if the radio unit is declared by the manufacturer to be capable of initiating non-prescribed data calls (see Section 3).

7.6.4.3.2	Purpose of test

This test is designed to check the correct operation of the suitably equipped radio unit when a non-prescribed data call is requested, in accordance with 5.5.3.1.1 of MPT1327.

7.6.4.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.4.3.4	Test method

The Tester shall program the following address:

39  2079 with DT = '1'

The Tester shall request the number by use of the functionally specific control buttons.  The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indications generated by the radio unit.

7.6.4.3.5	Radio unit response

The radio unit shall generate CSUIP indication when the functionally specific control button is pressed.  After the RQS message has been sent 8 times the radio unit shall generate Call Fail indication and then return to the idle state.  The RQS message sent shall be codeword 511 with the value of IDENT1 set to 2079.

7.6.4.4	Priority calls

7.6.4.4.1	Test number

70627A

This test shall only be carried out if the radio unit is declared by the manufacturer to be capable of initiating priority calls (see Section 3).

7.6.4.4.2	Purpose of test

This test is designed to check that a suitably equipped radio unit is able to select priority for calls, in accordance with 5.5.3.1.1 of MPT1327.

7.6.4.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.4.4.4	Test method

The Tester shall program in the following address:

39 2079 with LEVEL = '0'

The Tester shall request the address by use of the functionally specific control buttons.  The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indication generated by the radio unit.

7.6.4.4.5	Radio unit response

The radio unit shall generate CSUIP indication when the functionally specific control buton is pressed.  After the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state. The RQS message sent shall be codeword 513 with the value of IDENT1 set to 2079.

7.6.4.5	PABX calls

7.6.4.5.1	Test number

70628A	Call to single PABX operator

70629A	Call to selected PABX operator

70630A	Call to single PABX extension

70631A	Call to selected PABX extension (short addressing)

70632A	Call to PABXI gateway (extended addressing)

70633A	Call to PABXI gateway (extended addressing)

This test sequence shall only be carried out if the radio unit is declared by the manufacturer to be capable of initiating PABX calls (see Section 3).

7.6.4.5.2	Purpose of test

This test sequence is designed to check that a radio unit generates the correct signalling for PABX calls, in accordance with Sections 8.2.5 of MPT1343 and 9.2.1 of MPT1327.

7.6.4.5.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to this test.

7.6.4.5.4	Test method

The Tester shall program (or default values shall be assumed, see Section 7.6.4) the following addresses, EXT and FLAG bits.

IDEN	TEXT	FLAG 1	FLAG 2

Test No:	70628A	8000	'1'	'0'	'0'

Test No:	70629A	8000	'1'	'1'	'0'

Test No:	70630A	1815	'1'	'0'	'0'

Test No:	70631A	2369	'1'	'0'	'1'

Test No:	70632A	PABX 83369

Test No:	70633A	PABX 78814 4621

The Tester shall analyse the radio unit signalling using the General Test Assembly, and shall observe the confidence indications generated by the radio unit.  After each number has been keyed in the radio unit shall be allowed to return to the idle state.

7.6.4.5.5	Radio unit response

The radio unit shall generate CSUIP indication when the functionally specific control button is pressed.  After the RQS message has been sent 8 times and the radio unit timer TW has expired the radio unit shall generate Call Fail indication and then return to the idle state. The RQS message sent shall be:

Test No:	70628A Codeword  508 with IDENT1 =	8000

Test No:	70629A Codeword  512 with IDENT1 =	8000, EXT='1', 
FLAG1='1', FLAG2='0'

Test No:	70630A Codeword  508 with IDENT1 =	1815

Test No:	70631A Codeword  512 with IDENT1 =	2369, EXT='1', 
FLAG1='0', FLAG2='1'

Test No:	70632A Codeword  512 with IDENT1 =	PABXI (8102), EXT='0', 
FLAG1 '0', FLAG2='0'

Test No:	70633A Codeword  512 with IDENT1 =	PABXI, EXT='0', 
FLAG1='0', FLAG2='0'

7.6.5	Call request acknowledgement

7.6.5.1	ACKI(QUAL='1')

7.6.5.1.1	Test number

70634A

7.6.5.1.2	Purpose of test

This test is designed to check that the radio unit obeys the TW timeout, and generates the correct confidence indications in accordance with Section 8.1.3.2. of MPT1343.

7.6.5.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.5.1.4	Test method

The tester shall cause the radio unit to generate codeword 503.  The General Test Assembly shall be set to respond to codeword 503 with codeword 409 in the next slot.  The Tester shall observe the confidence indication generated by the radio unit.

7.6.5.1.5	Radio unit response

The radio unit shall generate CSUIP indications when the call is initiated.  The radio unit cease CSUIP and generate Call Fail indication 60 seconds ±6 seconds after the call is initiated.

7.6.5.2	Resetting timeout TW

7.6.5.2.1	Test number

70635A

7.6.5.2.2	Purpose of test

This test is designated to check that the radio unit will reset the appropriate timeout on receipt of an AHY message.

7.6.5.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The tester shall connect the radio unit of the General Test Assembly which shall be radiating a single control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.5.2.4	Test method

The Tester shall cause the radio unit to generate codeword 503.  The General Test Assembly shall be set to respond to codeword 503 with codeword 409 in the next slot.  30 seconds ±3 seconds after sending codeword 409 the General Test Assembly shall send codeword 610. The Tester shall analyse the radio unit signalling with the General Test Assembly and shall observe the confidence indications generated by the radio unit.

7.6.5.2.5	Radio unit response

The radio unit shall generate CSUIP indication when the call is initiated.  The radio unit shall respond to codeword 610 with codeword 418.  The radio unit shall cease CSUIP and generate Call Fail indications 60 seconds ±6 seconds after the General Test Assembly has sent codeword 610.

7.6.5.3	Inter-prefix call request

7.6.5.3.1	Test number

70636A

7.6.5.3.2	Purpose of test

This test is designed to check that the radio unit operates correctly when making an interprefix call.

7.6.5.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.5.3.4	Test method

The Tester shall cause the radio unit to generate a call to radio unit address 120 1739  (either by appropriate programming of a Category II mobile or by entering 320 2850 239 # on a Category I mobile).  The General Test Assembly shall be set to respond to the request with codeword 611 in the slot immediately following the request.

The Tester shall analyse the radio unit signalling using the General Test Assembly and shall observe the confidence indications generated by the radio unit.

7.6.5.3.5	Radio unit response

The radio unit shall generate CSUIP indication when the call is initiated.  The radio unit shall send codeword 803 in the slot immediately following codeword 611.  60 seconds ±6 seconds after the transmission of the call request the radio unit shall cease CSUIP and generate Call Fail indication.

7.6.5.4	Rejection of call request

7.6.5.4.1	Test number

70637A	Radio unit indication on receipt of ACKX(QUAL='0')

70638A	Radio unit indication on receipt of ACKX(QUAL='1')

70639A	Radio unit indication on receipt of ACKV(QUAL='0')

70640A	Radio unit indication on receipt of ACKV(QUAL='1')

7.6.5.4.2	Purpose of test

This test is designed to check that the radio unit generates the appropriate confidence indications in accordance with Section 8.1 of MPT1343 to acknowledgement messages which terminate the call signalling.

7.6.5.4.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single control channel to channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.5.4.4	Test method

The Tester shall cause the radio unit to generate codeword 503 either by dialling 275 # (Category I mobile) or use of a functionally specific control button (Category II mobile) programmed to call address 39 2079.

The General Test Assembly shall be set to respond to codeword 503 with the following acknowledgements:

Test No:	70637A	codeword 406	ACKX(QUAL='0')

Test No:	70638A	codeword 405	ACKX(QUAL='1')

Test No:	70639A	codeword 413	ACKV with QUAL = '0'

Test No:	70640A	codeword 413	ACKV with QUAL = '1'

The Tester shall observe the confidence indications generated by the radio unit.

7.6.5.4.5	Radio unit response

The radio unit shall generate the following indications in the named tests:

Test No:	70637A	N.U. Indication

Test No:	70638A	Call Fail     - 	Called Party Busy (or system busy,

if this option is implemented)

Test No:	70639A	Call Fail     -	Unavailable

Test No:	70640A	Call Fail     -	Called Party Busy

7.6.6	Assignment of radio unit to traffic channel

7.6.6.1	Called party channel changing and audio mute check

7.6.6.1.1	Test numbers

70641A	Individual Call (speech)

70642A	Individual Call (data)

70643A	Group Call (speech)

70644A	Group Call (data)

7.6.6.1.2	Purpose of test

This test sequence is designed to check the channel switching behaviour of the radio unit on receipt of a Go-To-Channel message when acting as a called party, and control of the audio mute for non-prescribed data calls.

7.6.6.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.6.1.4	Test method

The Tester shall cause the General Test Assembly to generate the following codewords for the named tests:

Test No:	70641A	Codeword 201

Test No:	70642A	Codeword 202

Test No:	70643A	Codeword 208 with D = '0'

Test No:	70644A	Codeword 208 with D = '1'

In each codeword the value of CHAN is set to '0010110110'.

At the appropriate time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 such that the first transmission on the forward traffic channel is codeword 601, with the first bit of the preamble preceding codeword 601 beginning at least 35ms after the last bit of the GTC codeword.

Following codeword 601 the forward traffic channel shall be modulated with a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation.

Approximately 10 seconds after codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702, with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.6.1.5	Radio unit response

On receipt of the GTC codeword the radio unit shall generate GTC indication in accordance with the manufacturer's instructions.  The radio unit shall respond to codeword 601 with codeword 801.  In tests 70641A and 70643A the Tester shall, in addition, hear the test tone from the radio unit.  In tests 70642A and 70644A the radio unit audio shall remain muted.

In all four tests, the radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.6.6.2	Calling party channel changing and audio mute check (individual call).

Test 70646A shall not be performed if the radio unit manufacturer has declared that the radio unit does not provide for the standard option to originate data calls (MPT 1343 section 11.5.5.3.1.1).

7.6.6.2.1	Test numbers

70645A	Speech call

70646A	Data call

7.6.6.2.2	Purpose of test

This test sequence is designed to check the channel switching behaviour of the  radio unit on receipt of a Go-To-Channel message when acting as a calling party, making an individual call, and control of the audio mute for non-prescribed data calls.

7.6.6.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.6.2.4	Test method

The Tester shall cause the radio unit to generate the following codewords in the named tests:

Test No:	70645A	Codeword 503

Test No:	70646A	Codeword 511 with IDENT1 = 2079

The General Test Assembly shall acknowledge the RQS codeword with codeword 409 in the next slot.  After approximately 15 seconds the Tester shall cause the General Test Assembly to generate codeword 408.

After a further 15 seconds (approximately), the Tester shall cause the General Test Assembly to generate the following codewords in the named tests:

Test No:	70645A	Codeword 209 with D = '0'

Test No:	70646A	Codeword 209 with D = '1'

In each codeword the value of CHAN is set to '0010110110'.

At the appropriate time the Tester shall cause the General Test Assembly to radiate on traffic channel number 239 such that the first transmission of the forward channel is codeword 601, with the first bit of the preamble preceeding codeword 601 beginning at least 35ms after the last bit of the GTC codeword.

Following codeword 601 the forward traffic channel shall be monitored with a 400 Hz tone set at a level giving a peak deviation of 60% of maximum permitted frequency deviation.

Approximately 10 seconds after codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.6.2.5	Radio unit response

When the call is initiated the radio unit shall generate CSUIP.  If the radio is declared by the manufacturer to be equipped with Called Party Ringing it shall generate the appropriate indication (as detailed in the manufacturer's information) when codeword 408 is sent by the General Test Assembly.  Otherwise the radio unit shall continue to indicate CSUIP.

When the GTC is sent the radio unit shall cease CSUIP or called party ringing indicator as appropriate and optionally may generate a GTC confidence indication.  The radio unit shall respond to codeword 601 with codeword 801.  In test 70645A the Tester shall, in addition, hear the test tone from the radio unit.  In test 70646A the radio unit audio shall remain muted.

In both tests, the radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.6.6.3	Calling party channel changing and audio mute check (group call)

Neither test 70647A nor test 70648A shall be performed if the radio unit manufacturer has declared that the radio unit does not provide for the standard option to originate calls to group addresses (MPT 1343 section 11.9). Test 70648A shall not be performed if the radio unit does not provide for the standard option to originate data calls (MPT 1343 section 11.5.5.3.1.1).

7.6.6.3.1	Test numbers

70647A	Speech call

70648A	Data call

7.6.6.3.2	Purpose of test

This test is designed to check the channel switching behaviour of the radio unit on receipt of a Go-To-Channel message when acting as a calling party making a group call and control of the audio mute for non-prescribed data calls.

7.6.6.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.6.3.4	Test method

The Tester shall cause the radio unit to generate the following codewords in the named tests:

Test No:	70647A	Codeword 511 with IDENT1 = 605, DT = '0'

Test No:	70648A	Codeword 511 with IDENT1 = 605, DT = '1'

The General Test Assembly shall acknowledge the RQS with the following codewords in the named tests:

Test No:	70647A	Codeword 210 with D = '0'

Test No:	70648A	Codeword 210 with D = '1'

In each codeword the value of CHAN is set to '0010110110'.

At some appropriate time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 such that the first transmission on the forward traffic channel is codeword 601, with the first bit of the preamble preceding codeword 601 beginning at least 35ms after the last bit of the GTC codeword.

Following codeword 601 the forward traffic channel shall be modulated with a 400 Hz tone at a level giving a peak deviation of 60% of maximum permitted frequency deviation.

Approximately 10 seconds after codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.6.3.5	Radio unit response

When the GTC codeword is sent the radio unit shall generate GTC indication.

The radio unit shall respond to codeword 601 with codeword 801.  In test 70647A the Tester shall, in addition, hear the test tone from the radio unit.  In test 70648A the radio unit audio shall remain muted.

In both tests, the radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.6.7	AHY messages

7.6.7.1	Response to AHY (CHECK = '0')

7.6.7.1.1	Test number

70649A

7.6.7.1.2	Purpose of test

This test is designed to check that the radio unit responds correctly to an availability check AHY in accordance with Section 9.2.2.2 of MPT1327.

7.6.7.1.3	Initialisation

The radio unit utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.7.1.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 612.

Within 10 seconds of sending codeword 612 the Tester shall cause the General Test Assembly to generate codeword 208 with D = '0' and with the value of CHAN set to '0010110110'.

At the appropriate time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 such that the first transmission on the forward traffic channel is codeword 601, with the first bit of the preamble preceding codeword 601 beginning at least 35ms after the last bit of the GTC codeword.

Within 20 seconds of sending codeword 208 the Tester shall cause the General Test Assembly to generate codeword 201 on the forward control channel number 282, with the value of CHAN set to '0010110110'.

Between 35 ms and five seconds of sending codeword 201 the Tester shall repeat the security check (codeword 601) on traffic channel number 239.

Approximately 10 seconds after the second codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.7.1.5	Radio unit response

The radio unit shall respond to codeword 612 with codeword 416 (ACK(QUAL='0')).

The radio unit optionally may generate GTC Indication on receipt of codeword 208, and shall not respond to the first codeword 601.

The radio unit shall generate GTC Indication on receipt of codeword 201 and shall respond to the second codeword 601 with codeword 801.

The radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.6.7.2	Response to AHY (CHECK='1')

7.6.7.2.1	Test number

70650A

7.6.7.2.2	Purpose of test

This test is designed to check that the radio unit responds correctly to the availability check, AHY (CHECK='1') in accordance with Section 9.2.2.2 of MPT1327.  Also to test that Alert Indication is generated in accordance with Section 8.1.3.9 of MPT1343, when the appropriate check is carried out.

7.6.7.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.7.2.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 609.

Within 10 seconds of sending codeword 609 the Tester shall cause the General Test Assembly to generate codeword 208 with D = '0' and the value of CHAN set to '0010110110'.

At the appropriate time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 with the first bit of the preamble preceding codeword 601 begininng at least 35ms after the last bit of the GTC codeword.

Within 20 seconds of codeword 208 the Tester shall cause the General Test Assembly to generate on control channel number 282 codeword 201, with the value of CHAN set to '0010110110'.

Between 35ms and five seconds after sending codeword 201 the Tester shall repeat the security check (codeword 601) on traffic channel number 239.

Approximately 10 seconds after the second codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.7.2.5	Radio unit response

The radio unit shall respond to codeword 609 with codeword 415 (ACKI(QUAL='0')) and shall generate the Alert indication in accordance with Section 8.1.3.9 of MPT1343.

The radio unit shall continue the Alert indication on receipt of codeword 208 and shall not respond to the first codeword 601.

The first unit shall cease Alert Indication on receipt of codeword 201, and optionally may generate GTC Indication.  The radio unit shall respond to the second codeword 601 with codeword 801.

7.6.7.3	Reset of Alert state timeout, TA

7.6.7.3.1	Test number

70651A

7.6.7.3.2	Purpose of test

This test is designed to check that the Alert timeout on a radio is reset by the appropriate AHY message, in accordance with Section 9.2.2.2 of MPT1327.

7.6.7.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.7.3.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 609.  30 seconds after generating codeword 609 the tester shall again cause the General Test Assembly to generate codeword 609.

The radio unit shall be allowed to time out and return to the idle state.

7.6.7.3.5	Radio unit response

The radio shall respond to codeword 609 with codeword 415 (ACKI(QUAL='0'), and shall generate the Alert indication.

When the radio unit receives the second 609 codeword, the radio unit shall continue with the Alert indication, and shall again respond with codeword 415.

60 seconds ±6 seconds after the second codeword 609 was sent, the radio unit shall cease the Alert indication and return to the idle state.

7.6.8	On-hook/Off-hook signalling

7.6.8.1	Incoming call acceptance

7.6.8.1.1	Test number

70652A

7.6.8.1.2	Purpose of test

This test is designed to check that a radio unit can correctly accept incoming calls from networks which send the AHY (CHECK='1') message, and give the appropriate confidence indications.

7.6.8.1.3	Initialisation

The radio unit utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.8.1.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 609.

Within 30 seconds from codeword 609, the Tester shall accept the call using the radio unit RFCC, as detailed in the radio unit user guide.  The General Test Assembly shall examine the messages generated by the radio unit, and shall respond to the 8th RQQ message (codeword 506) from the radio unit with codeword 411 (ACK(QUAL='0') with IDENT1 = TSCI).

The Tester shall then cause the General Test Assembly to generate codeword 201 with the value of CHAN set to '0010110110'.

At some time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 such that the first transmission on the forward traffic channel is codeword 601, with the first bit of the preamble preceeding codeword 601 beginning at least 35ms after the last bit of the GTC codeword.

Approximately 10 seconds after codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.8.1.5	Radio unit response

The radio unit shall respond to codeword 609 with codeword 415 in the next slot and shall initiate Alert indication.

When the Tester accepts the incoming call, the radio unit shall cease Alert indication, and generate CSUIP (called) in accordance with the manufacturer's user information.  The radio unit shall also transmit the random access codeword 506 (RQQ) with STATUS = '00000' eight times on the return control channel on channel number 282.  When codeword 411 is received the radio unit shall continue to indicate CSUIP.

On receipt of codeword 201 the radio unit shall cease CSUIP.  When the radio unit receives codeword 601 it shall respond with codeword 801.

The radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.6.8.2	Call accept signalling rejected by network

7.6.8.2.1	Test number

70653A

7.6.8.2.2	Purpose of test

This test is designed to check that the radio unit responds correctly in accordance with 13.1.2.7 of MPT1327, when the call accept signalling is rejected by the network.

7.6.8.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.8.2.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 609.  Within 30 seconds from codeword 609, the Tester shall accept the call using the radio unit RFCC.  On receipt of codeword 506 (RQQ) the General Test Assembly shall respond with codeword 613 (AHYX).

7.6.8.2.5	Radio unit response

The radio unit shall respond to codeword 609 with codeword 415 in the next slot and shall initiate the Alert indication.

When the Tester accepts the incoming call the radio unit shall cease the Alert indication, and generate CSUIP (called).  The radio unit shall also transmit the random access codeword 506 (RQQ) with STATUS = '00000'.

When codeword 613 is received the radio unit shall cease CSUIP and generate generate Call Fail-Unavailable indication.  The radio unit shall also transmit codeword 416 with QUAL='1'.

7.6.8.3	Alert state timeout reset after call accept

7.6.8.3.1	Test number

70654A

7.6.8.3.2	Purpose of test

This test is designed to check that the Alert state timeout on a radio unit is reset by the appropriate AHY messages in accordance with 9.2.2.2 of MPT1327.

7.6.8.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

7.6.8.3.4	Test method

The Tester shall cause the General Test Assembly to generate codeword 609.  Within 30 seconds of transmission of codeword 609, the Tester shall accept the call using the radio unit RFCC.  On receipt of the RQQ message (codeword 506) the General Test Assembly shall respond with codeword 411.

Approximately 45 seconds after sending codeword 609 the Tester shall cause the General Test Assembly to generate codeword 612.

Between 75 and 95 seconds after sending codeword 609 the Tester shall cause the General Test Assembly to generate codeword 201 with the value of CHAN set to '0010110110.

At the appropriate time the Tester shall cause the General Test Assembly to radiate a traffic channel on channel number 239 and that the first transmission on the forward traffic channel is codeword 601, with the first bit of the preamble preceding codeword 601 beginning not more than 35ms after the last bit of the GTC codeword.

Approximately 10 seconds after codeword 601 is sent the Tester shall cause the General Test Assembly to radiate codeword 702 with the value of CHAN set to '0010110110' and the value of CONT set to '0011100001'.

7.6.8.3.5	Radio unit response

The radio unit shall respond to codeword 609 with codeword 415 in the next slot and shall initiate Alert indication.

When the Tester accepts the incoming call, the radio unit shall cease Alert indication, and generate CSUIP indication.  The radio unit shall also transmit the random access codeword 506 (RQQ) with STATUS = '00000'.

When codeword 612 is received the radio unit shall respond with codeword 416 with QUAL = '0' and continue to indicate CSUIP.

When codeword 201 is received, the radio unit shall cease CSUIP.  The radio unit shall respond to codeword 601 with codeword 801 on the traffic channel.

The radio unit shall return to the idle state on control channel number 282 in response to codeword 702 on channel number 239.

7.7	Traffic Channel Procedures

7.7.1	General

The tests in this section are designed to check the correct operation of the radio unit once it has been assigned to a traffic channel.  The necessary radio unit behaviour is described in Section 9 of MPT1327.  The tests in this section cover the following:

a)	Generation of call maintenance messages.

b)	Operation of timeouts during traffic channel activity.

c)	Disabling user transmission.

d)	Procedures for clear down and return to a control channel.

The General Test Assembly is required for these series of tests to generate one simulated control channel and one simulated traffic channel, each on a different fixed frequency.

The tests cover operation of the radio unit with both the A personalisation and the B personalisation.  Sections 7.7.2 to 7.7.6 require the radio unit to utilise the A personalisation and section 7.7.7 repeats some of these tests with the radio unit utilising the B personalisation.  Corresponding tests in 7.7.2 to 7.7.6 and 7.7.7 are allocated the same test number with the corresponding letter suffix A or B (e.g., 70707A and 70707B).

7.7.2	Call maintenance

The following series of tests checks that the radio unit generates the correct call maintenance messages whilst on a traffic channel and that the generation of these messages conforms to the appropriate call maintenance parameters.

7.7.2.1	Default call maintenance behaviour

7.7.2.1.1	Test number

70701A

7.7.2.1.2	Purpose of test

To ensure that, when the radio unit has not received a BCAST (SYSDEF='00010') message since the start of the session, it sends call maintenance messages whilst transmitting on a traffic channel conforming to the default procedures specified in Section 9.2.2.6 of MPT1327.

7.7.2.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.2.1.4	Test method

The Tester shall switch off the radio unit in accordance with the manufacturer's instructions. After a short time the Tester shall switch on the radio unit in accordance with the manufacturer's instructions.

After a suitable period has elapsed (for channel hunting) the Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 15 and 20 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any messages received from the radio unit during the performance of the test and time the periods between received messages.

The Tester shall return the radio unit to the idle state on the control channel by signalling "call end request" with the RFCC in accordance with the manufacturer's instructions.

7.7.2.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 201 (GTC).  When the PTT is operated the radio unit shall transmit on the traffic channel and messages shall be sent by the radio unit in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel)  as follows:

If the radio unit manufacturer declares that the unit implements the option to send multiple messages then two codeword 706 (MAINT (OPER='000')) shall be sent by the radio unit at the start of the transmission on the traffic channel. Otherwise the radio unit shall send one codeword 706.

A minimum of two codewords 707 (MAINT(OPER='010')) shall be sent by the radio unit during the transmission on the traffic channel.

If the radio unit manufacturer declares that the unit implements the option to send multiple messages then three codeword 708 (MAINT(OPER='001')) shall be sent by the radio unit at the end of the transmission on the traffic channel. Otherwise the radio unit shall send one codeword 708.

The time period between any two adjacent received codewords shall not exceed 5.5 seconds.

7.7.2.2	Change of call maintenance parameters (disable periodic messages)

7.7.2.2.1	Test number

70702A

7.7.2.2.2	Purpose of test

To ensure that, when the radio unit has received a BCAST (SYSDEF='00010') message since the start of the session, it sends call maintenance messages whilst transmitting on a traffic channel conforming to the parameters specified in the last received BCAST (SYSDEF='00010') message in accordance with Sections 5.5.4.5 and 9.2.2.8 of MPT1327.

7.7.2.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.2.2.4	Test method

The Tester shall cause the General Test Assembly to transmit codeword 709 (BCAST) within the forward message stream on channel number 282.

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 15 and 20 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any messages received from the radio unit during the performance of the test.

The Tester shall return the radio unit to the idle state on the control channel by signalling "call end request" with the RFCC in accordance with the manufacturer's instructions.

7.7.2.2.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturers information supplied, upon receipt of codeword 201 (GTC).  When the PTT is operated the radio unit shall transmit on the traffic channel and messages shall be sent by the radio unit in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel) as   follows:

If the radio unit manufacturer declares that the unit implements the option to send multiple messages then two codeword 706 (MAINT (OPER='000')) shall be sent by the radio unit at the start of the transmission on the traffic channel. Otherwise the radio unit shall send one codeword 706.

If the radio unit manufacturer declares that the unit implements the option to send multiple messages then three codeword 708 (MAINT(OPER='001')) shall be sent by the radio unit at the end of the transmission on the traffic channel. Otherwise the radio unit shall send one codeword 708.

No codewords 707 shall be sent by the radio unit at any time during the transmission.

7.7.2.3	Change of call maintenance parameters (disable pressel on and change maximum period)

7.7.2.3.1	Test number

70703A

7.7.2.3.2	Purpose of test

To ensure that, when the radio unit has received a BCAST (SYSDEF='00010') message since the start of the session, it sends call maintenance messages whilst transmitting on a traffic channel conforming to the parameters specified in the last received BCAST(SYSDEF='00010') message in accordance with Sections 5.5.4.5 and 9.2.2.6 of MPT1327.

7.7.2.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.2.3.4	Test method

The Tester shall cause the General Test Assembly to transmit codeword 710 (BCAST) within the forward message stream on channel number 282.

The Tester shall then use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any messages received from the radio unit during the performance of the test and time the periods between received messages.

The Tester shall turn the radio unit off, (to reset call maintenance parameters to the default condition).

7.7.2.3.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturers information supplied, upon receipt of codeword 201 (GTC).  When the PTT is operated the radio unit shall transmit on the traffic channel and messages shall be sent by the radio unit in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel) as follows:

Codeword 706 shall not be sent by the radio unit at the start of the transmission on the traffic channel, or at any other time during the transmission.

A minimum of two codewords 707 shall be sent by the radio unit during the transmission on the traffic channel.

If the radio unit manufacturer declares that the unit implements the option to send multiple messages then three codeword 708 (MAINT(OPER='001')) shall be sent by the radio unit at the end of the transmission on the traffic channel. Otherwise the radio unit shall send one codeword 708.

The time period between any two adjacent received codewords shall not exceed 2.2 seconds.

7.7.3	User cleardown on a traffic channel

The following series of tests checks that the radio unit behaves correctly when the user initiates a "call end request" with the RFCC in accordance with Sections 8.3.3 of MPT1343 and 9.2.3.5 of MPT1327.

7.7.3.1	Calling party cleardown

7.7.3.1.1	Test number

70704A

7.7.3.1.2	Purpose of test

To ensure that, when the call originating user initiates a "call end request" with the radio unit RFCC the radio unit transmits the required number of disconnect requests and returns to a control channel in accordance with 8.3.3 of MPT1343 and 9.2.3.5 of MPT1327.

7.7.3.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.3.1.4	Test method

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (See Section 3.3).  After receipt of codeword 503 (RQS) from the radio unit on channel 282 the General Test Assembly shall respond with codeword 205 (GTC) in accordance with 7.2.4 and 9.1.1.1 of MPT1327.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The Tester shall then use the RFCC on the radio unit to signal "call end request" in accordance with the manufacurer's instructions and the General Test Assembly shall monitor any transmissions by the radio unit on channel 309.  Approximately 2 seconds after initiating the "call end request" the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.3.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 205 (GTC).

When the "call end request" is initiated by the Tester the radio unit shall transmit codeword 712 (MAINT(OPER='011')) five times in the format specified in Section 3.3.2 of MPT1327 (as appropriate to a traffic channel) and shall indicate "call clear" in accordance with the manufacturer's information supplied and operation of the PTT shall no longer cause the radio unit to transmit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.3.2	Called party cleardown (group call)

7.7.3.2.1	Test number

70705A

7.7.3.2.2	Purpose of test

To ensure that, when a called radio user in a group call initiates a "call end request" with the radio unit RFCC, the radio unit does not transmit disconnect messages prior to returning to a control channel in accordance with Sections 8.3.3 of MPT1343 and 9.2.3.5 of MPT1327.

7.7.3.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.3.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword 206 within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The Tester shall then use the RFCC on the radio unit to signal "call end request" in accordance with the manufacturer's instructions and the General Test Assembly shall monitor any transmissions by the radio unit on channel 309.

Approximately 2 seconds after initiating the "call end request" the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.3.2.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 206 (GTC).

When the "call end request" is initiated by the Tester the radio unit shall indicate "call clear" in accordance with the manufacturer's information supplied and operation of the PTT shall no longer cause the radio unit to transmit.

No disconnect messages shall be transmitted by the radio unit on channel 309 following initiation of "call end request" by the Tester.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.3.3	Called party cleardown (individual call)

7.7.3.3.1	Test number

70706A

7.7.3.3.2	Purpose of test

To ensure that, when a called user in an individual call initiates a "call end request" with the radio unit RFCC, the radio unit transmits the required number of disconnect messages and returns to a control channel in accordance with Sections 8.3.3 of MPT1343 and 9.2.3.5 of MPT1327.

7.7.3.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.3.3.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The Tester shall then use the RFCC on the radio unit to signal "call end request" in accordance with the manufacturer's instructions and the General Test Assembly shall monitor any transmissions by the radio unit on channel 309.

Approximately 2 seconds after initiating the "call end request" the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.3.3.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 201 (GTC).

When the "call end request" is initiated by the Tester the radio unit shall transmit codeword 712 (MAINT(OPER='011')) three times in the format specified in Section 3.3.2 of MPT1327 (as appropriate to a traffic channel) and shall indicate "call clear" in accordance with the manufacturer's information supplied and operation of the PTT shall no longer cause the radio unit to transmit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.4	Traffic channel timeouts

The following series of tests checks that the radio unit, whilst on a traffic channel, operates the timeouts specified in 9.2.3.6 of MPT1327.

7.7.4.1	Item duration timeout

7.7.4.1.1	Test number

70707A

7.7.4.1.2	Purpose of test

To ensure that, whilst on a traffic channel, the radio unit operates a timer following each Pressel On operation of the PTT and, in the event that a Pressel Off operation does not occur within a time period TT, ceases transmission and clears down from the traffic channel in accordance with Section 9.2.3.6 of MPT1327.

7.7.4.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.4.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit and, at the same time, initiate a timing device.  The Tester shall keep the PTT depressed for a period in excess of 70 seconds.

The General Test Assembly shall monitor the return traffic channel for any messages received from the radio unit during the performance of the test.

When the General Test Assembly receives codeword 708 on the traffic channel from the radio unit the Tester shall stop the timing device and note the elapsed time.

Approximately 2 seconds after observing "call clear" indication from the radio unit (in accordance with the manufacturer's instructions) the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.4.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 201 (GTC).

The radio unit shall transmit codeword 708 on the traffic channel within 54 to 66 seconds from the operation of the PTT, followed by codeword 712 three times.  These messages shall be in the format specified in Section 3.3.2 of MPT1327 (as appropriate to a traffic channel).

The radio unit shall also indicate "call clear" in accordance with the manufacturer's information supplied and operation of the PTT shall no longer cause the radio unit to transmit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.4.2	Inactivity timeout

7.7.4.2.1	Test number

70708A

7.7.4.2.2	Purpose of test

To ensure that, if whilst receiving on a traffic channel the radio unit encounters any period of inactivity on the forward traffic channel in excess of TN, then the radio unit shall clear down from the traffic channel in accordance with Sections 9.2.3.6 of MPT1327 and 8.1.3.8 of MPT1343.

7.7.4.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.4.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The Tester shall then cause the General Test Assembly to cease to radiate the forward traffic channel on channel number 309, but to continue to monitor the return channel for any messages from the radio unit.  The Tester shall initiate a timing device at the instant that radiation of the forward traffic channel ceases.

When the Tester observes the "call clear" indication from the radio unit (in accordance with the manufacturer's instructions) he shall stop the timing device and note the elapsed time.

Approximately 2 seconds after observing "call clear" indication from the radio unit the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.4.2.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 201 (GTC).

The radio unit shall generate a "call clear" indication within 6.3 to 7.7 seconds of the instant that the traffic channel ceases to radiate and following this operation of the PTT shall no longer cause the radio unit to transmit.

No disconnect messages shall be transmitted by the radio unit on channel 309 following generation of "call end".

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.5	Disabling user transmission

The following series of tests checks that receipt of a MAINT(OPER='111')) message by the radio unit on a traffic channel results in the radio unit transmitter being disabled for any subsequent activity on that traffic channel in accordance with Section 9.2.3.3 of MPT1327.

7.7.5.1	Radio unit is called unit

7.7.5.1.1	Test number

70709A

7.7.5.1.2	Purpose of test

To ensure that, if whilst on a traffic channel the radio unit receives an applicable MAINT(OPER='111')) message, the radio unit transmitter shall be disabled for the remainder of its activity on the traffic channel in accordance with Section 9.2.3.3 of MPT1327.

7.7.5.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.5.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 201 with the value of CHAN set to '0011111100' within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall cause the General Test Assembly to transmit codeword 713 (MAINT(OPER='111')) on the forward traffic channel in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel).  The Tester shall then operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any transmission by the radio unit during the performance of the test.

The Tester shall return the radio unit to the idle state on the control channel by signalling "call end request" with the RFCC in accordance with the manufacturer's instructions.

7.7.5.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 201 (GTC).

No transmissions shall be detected from the radio unit on the traffic channel throughout the test except for codeword 712 which shall be transmitted three times in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel) following initiation of "call end request" by the Tester.

7.7.5.2	Radio unit is calling unit (group call)

7.7.5.2.1	Test number

70710A

This test shall not be performed if the manufacturer has declared that the radio unit does not provide for the standard option to originate calls to group addresses (see Section 11.9 of MPT1343) or if the manufacturer has declared that the radio unit does not provide for the standard option to originate a simple call request (RQS) for a group call with FLAG1 set to '1' (i.e., broadcast calls, see Section 11.5.5.3.1.1 of MPT1343).

7.7.5.2.2	Purpose of test

To ensure that, if whilst on a traffic channel the radio unit receives a MAINT(OPER='111') message that is not applicable (due to the radio unit being the originator of a group call), then the radio unit transmitter shall not be disabled in accordance with Section 9.2.3.3 of MPT1327.

7.7.5.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.5.2.4	Test method

The Tester shall cause the radio unit to attempt a simple speech call request (broadcast mode) to group PFIX=39 IDENT=605 (see Section 3.3).  After receipt of codeword 504 (RQS) from the radio unit on channel 282 the General Test Assembly shall respond with codeword 207 (GTC) in accordance with Sections 7.2.4 and 9.1.1.1 of MPT1327.

Once the Tester has confirmed by observation the GTC indication (in accordance with the manufacturers instructions) that the radio unit is active on the traffic channel he shall cause the General Test Assembly to transmit codeword 714 (MAINT(OPER='111')) on the forward traffic channel in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel).

The Tester shall then operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any transmissions by the radio unit during the performance of the test.

The Tester shall return the radio unit to the idle state on the control channel by signalling "call end request" with the RFCC in accordance with the manufacturer's instructions.

7.7.5.2.5	Radio unit response

The radio unit shall generate "GTC" indication, in accordance with the manufacturers information supplied, upon receipt of codeword 207 (GTC).

The radio unit shall transmit upon operation of the PTT control and cease to transmit when the PTT control is released.  This transmission may include MAINT type codewords but these are not specified for the purposes of this test.

7.7.6	Network cleardown of radio units from a traffic channel

The following series of tests checks that the radio unit behaves correctly when it receives selective or general cleardown messages from the TSC on a traffic channel in accordance with Sections 9.2.3.7 and 9.2.3.8 of MPT1327.

7.7.6.1	Selective cleardown of unwanted unit

7.7.6.1.1	Test number

70711A

7.7.6.1.2	Purpose of test

To ensure that, if whilst on a traffic channel the radio unit receives a MAINT(OPER='110') message with an address not relating to the GTC message received which resulted in the radio unit acquiring a traffic channel, then the radio unit clears down from the traffic channel in accordance with Section 9.2.3.7 of MPT1327.

7.7.6.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.6.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 206 within the forward message stream on channel number 282.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any transmissions by the radio unit during the performance of the test.

Within 6 seconds of releasing the PTT control the Tester shall cause the General Test Assembly to transmit codeword 715 (MAINT(OPER='110')) with the value of IDENT1 set to 2048 on the forward traffic channel in the format specified in 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Approximately 2 seconds after the transmission of codeword 715 the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.6.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 206 (GTC).

The radio unit shall transmit upon operation of the PTT control and cease to transmit when the PTT control is released.  This transmission may include MAINT type codewords but these are not specified for the purposes of this test.

The radio unit shall indicate "call clear" in accordance with the manufacturer's information supplied in response to the receipt of codeword 715 and shall not transmit disconnect messages.  Operations of the PTT shall no longer cause the radio unit to transmit .

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.6.2	Selective cleardown not actioned by wanted unit

7.7.6.2.1	Test number

70712A

7.7.6.2.2	Purpose of test

To ensure that, if whilst on a traffic channel the radio unit receives a MAINT(OPER='110') message with an address relating to the GTC message received which resulted in the radio unit acquiring a traffic channel, then the radio unit remains on the traffic channel in accordance with Section 9.2.3.7 of MPT1327.

7.7.6.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.6.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 206 within the forward message stream on channel number 282.

Once the Tester has confirmed by observation the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

Within 6 seconds of releasing the PTT control the Tester shall cause the General Test Assembly to transmit codeword 715 (MAINT(OPER='110')) with the value of IDENT1 set to 605 on the forward traffic channel in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Within 6 seconds of the transmission of codeword 715 the Tester shall again operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any transmissions by the radio unit during the performance of the test.

The Tester shall return the radio unit to the idle state on the control channel by signalling "call end request" with the RFCC in accordance with the manufacturer's instructions.

7.7.6.2.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 206 (GTC).

The radio unit shall transmit upon each operation of the PTT control and cease to transmit when the PTT control is released.  This transmission may include MAINT type codewords but these are not specified for the purposes of this test.

7.7.6.3	General cleardown

7.7.6.3.1	Test number

70713A

7.7.6.3.2	Purpose of test

To ensure that, if whilst on a traffic channel the radio unit receives a valid CLEAR message, then the radio unit clears down from the traffic channel and acquires the control channel prescribed by the CONT field of the CLEAR message in accordance with Section 9.2.3.8 of MPT1327.

7.7.6.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.6.3.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 206 within the forward message stream on channel number 282.

Once the Tester has confirmed by observation the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The General Test Assembly shall monitor the return traffic channel for any transmissions by the radio unit during the performance of the test.

Within 6 seconds of releasing the PTT control the Tester shall cause the General Test Assembly to transmit codeword 702 (CLEAR) with the value of the CHAN field set to '0011111100' and the value  of the CONT field set to '0011100001' on the forward traffic channel in the format specified in Section 3.3.1 of MPT1327 (as appropriate to a traffic channel).

Approximately 2 seconds after the transmission of codeword 702 the Tester shall cause the General Test Assembly to transmit codeword 601 (AHYC) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 601 in the slot immediately following.

7.7.6.3.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 206 (GTC).

The radio unit shall transmit upon operation of the PTT control and cease to transmit when the PTT control is released.  This transmission may include MAINT type codewords but these are not specified for the purposes of this test.

Upon receipt of codeword 702 (CLEAR) the radio unit shall indicate "call clear" in accordance with the manufacturer's information supplied and shall not transmit disconnect messages.  Operation of the PTT shall no longer cause the radio unit to transmit.

The radio unit shall respond to codeword 601 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.7	B personalisation tests

The following is a repeat of some of the tests in Sections 7.7.2 to 7.7.6 but with values specified appropriate to the B personalisation.  Only those tests where the use of B personalisation parameters affect the radio unit behaviour in the tests (e.g., different timeout values) are repeated.

7.7.7.1	Item duration timeout

7.7.7.1.1	Test number

70707B

7.7.7.1.2	Purpose of test

To ensure that whilst on a traffic channel the radio unit operates a timer following each Pressel On operation of the PTT and, in the event that a Pressel Off operation does not occur within a time period TT, ceases transmission and clears down from the traffic channel in accordance with Section 9.2.3.6 of MPT1327.

7.7.7.1.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 281 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.7.1.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 281.

Once the Tester has confirmed by observation of the GTC indication (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit and, at the same time, initiate a timing device.  The Tester shall keep the PTT depressed for a period in excess of 15 seconds.

The General Test Assembly shall monitor the return traffic channel for any messages received from the radio unit during the performance of the test.

When the General Test Assembly receives codeword 716 (MAINT(OPER='001')) on the traffic channel from the radio unit the Tester shall stop the timing device and note the elapsed time.

Approximately 2 seconds after observing "call clear" indication from the radio unit (in accordance with the manufacturer's instructions) the Tester shall cause the General Test Assembly to transmit codeword 603 (AHYC) within the message stream on the forward control channel 281 and shall monitor the return channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.7.7.1.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 204 (GTC).

The radio unit shall transmit codeword 716 (MAINT(OPER='001')) on the traffic channel within 9 to 11 seconds from the operation of the PTT, followed by codeword 717 (MAINT(OPER='011')) three times.  These messages shall be in the format specified in Section 3.3.2 of MPT1327 (as appropriate to a traffic channel).

The radio unit shall also indicate "call clear" in accordance with the manufacturer's information supplied and operation of the PTT shall no longer cause the radio unit to transmit.

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.7.7.2	Inactivity timeout

7.7.7.2.1	Test number

70708B

7.7.7.2.2	Purpose of test

To ensure that, if whilst receiving on a traffic channel the radio unit encounters any period of inactivity on the forward traffic channel in excess of TN, then the radio unit shall clear down from the traffic channel in accordance with Sections 9.2.3.6 of MPT1327 and 8.1.3.8 of MPT1343.

7.7.7.2.3	Initialisation

The radio unit shall utilise the B personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 281 and a single traffic channel on channel number 309.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.7.7.2.4	Test method

The Tester shall use the General Test Assembly to send GTC codeword number 204 with the value of the CHAN field set to '0011111100' within the forward message stream on channel number 281.

Once the Tester has confirmed by observation of the GTC indication 107 (in accordance with the manufacturer's instructions) that the radio unit is active on the traffic channel he shall operate the PTT control on the radio unit, keeping the PTT operated for a period of between 5 and 10 seconds, and then release the PTT control.

The Tester shall then cause the General Test Assembly to cease to radiate the forward traffic channel on channel number 309, but to continue to monitor the return channel for any messages from the radio unit.  The Tester shall initiate a timing device at the instant that radiation of the forward traffic channel ceases.

When the Tester observes the "call clear" indication from the radio unit (in accordance with the manufacturer's instructions) he shall stop the timing device and note the elapsed time.

Approximately 2 seconds after observing "call clear" indication from the radio unit the Tester shall cause the General Test Assembly to transmit codeword 603 (AHYC) within the message stream on the forward control channel 281 and shall monitor the return channel for a response from the radio unit to codeword 603 in the slot immediately following.

7.7.7.2.5	Radio unit response

The radio unit shall generate GTC indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 204 (GTC).

The radio unit shall generate a "call clear" indication within 2.7 to 3.3 seconds of the instant that the traffic channel ceases to radiate and following this operation of the PTT shall no longer cause the radio unit to transmit.

No disconnect messages shall be transmitted by the radio unit on channel 309 following generation of "call end".

The radio unit shall respond to codeword 603 (AHYC) with codeword 801 (SAMIS) in the correct response slot.

7.8	System Parameter Calls

(Not included in this issue).

7.9	Emergency Calls

(Not included in this issue).

7.10	Call Cancellation

7.10.1	General

The tests in this section are designed to check the correct operation of the radio unit when call set up is aborted prior to a traffic channel being allocated.  The necessary radio unit behaviour is described in Sections 9 and 13 of MPT1327.  The tests in this section cover the following:

a)	Call cancellation by the calling user,

b)	incoming call reject by the called user,

c)	call cancellation by the called user after initial acceptance.

The General Test Assembly is required for this series of tests to generate one simulated control channel on a fixed frequency.

The tests cover operation of the radio unit with the A personalisation only.

7.10.2	Test procedures

7.10.2.1	Calling user cancels call

7.10.2.1.1	Test number

71001A

7.10.2.1.2	Purpose of test

To ensure that, when the calling user initiates a call set-up abort prior to allocation of a traffic channel, the calling radio unit terminates call set-up in accordance with the procedures specified in Sections 9.2.1.7 of MPT1327 and 8.1 and 8.3.3 of MPT1343.

7.10.2.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.10.2.1.4	Test method

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (see 3.3).  After receipt of codeword 503 (RQS) from the radio unit on channel 282 the General Test Assembly shall respond with codeword 409 (ACKI(QUAL='1')) in accordance with Sections 7.2.4 and 9.1.1.1 of MPT1327.

The Tester shall then use the radio unit RFCC to initiate "call set-up abort" in accordance with the manufacturer's instructions.

The General Test Assembly shall monitor the return control channel for any transmissions by the radio unit and shall respond to codeword 505 (RQX) from the radio unit with codeword 410 (ACK(QUAL='1')) in accordance with Sections 7.2.4 and 9.1.1.1 of MPT1327.

7.10.2.1.5	Radio unit response

The radio unit shall respond to the simple speech call request by generating "CSUIP (Calling)" indication in accordance with the manufacturer's information supplied and shall transmit codeword 503 (RQS) on the return control channel.

The radio unit shall not change its indication state nor make any transmissions in response to codeword 409 (ACKI(QUAL='1')).

When the Tester initiates "call set-up abort" the  radio unit shall transmit codeword 505 (RQX) on the return control channel.

Upon receipt of codeword 410 (ACK(QUAL='1')) the radio unit shall cease "call set-up in progress" indication.

7.10.2.2	Called user rejects call (prior to going off-hook)

7.10.2.2.1	Test number

71002A

This test shall not be peformed if the manufacturer has declared that the radio unit does not provide for the standard option "incoming call reject" (see Sections 8.3.3 of MPT1343 and 13.1.2.1 of MPT1327).

7.10.2.2.2	Purpose of test

To ensure that, when the called user initiates an "incoming call reject" in response to an incoming call, the called radio unit terminates call set-up in accordance with the procedures specified in Sections 13.1.2.1 of MPT1327 and 8.1 and 8.3.3 of MPT1343.

7.10.2.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.10.2.2.4	Test method

The Tester shall cause the General Test Assembly to transmit codeword 609 (AHY) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 609 in the slot immediately following.

Once the Tester has observed the radio unit "alert" indication (in accordance with the manufacturer's instructions) he shall initiate "incoming call reject" in accordance with the manufacturer's instructions.

The General Test Assembly shall monitor the return control channel for any transmissions by the radio unit.

The General Test Assembly shall respond to any transmissions of codeword 506 (RQQ) from the radio unit with codeword 411 (ACK(QUAL='0')) in accordance with Sections 7.2.4 and 13.1.1.1 of MPT1327.

The Tester shall observe any change in radio unit indications subsequent to the initiation of the "incoming call reject".

7.10.2.2.5	Radio unit response

The radio unit shall generate "alert" indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 609 (AHY) and shall respond to codeword 609 with codeword 415 (ACKI(QUAL='0')) in the correct response slot.

When the "incoming call reject" is initiated by the Tester the radio unit shall transmit codeword 506 (RQQ) with the value of the STATUS field set to '11111' in accordance with Section 7.3.4 of MPT1327.

Upon receipt of codeword 411 (ACK(QUAL='0')) by the radio unit it shall cease the "alert" indication.

7.10.2.3	Called user rejects call (after going off-hook)

7.10.2.3.1	Test number

71003A

This test shall not be performed if the manufacturer has declared that the radio unit does not provide for the standard option "cancel called party answer" (See Sections 8.3.3 of MPT1343 and 13.1.2.1 of MPT1327).

7.10.2.3.2	Purpose of test

To ensure that, when the called user initiates an "cancel called party answer" in response to an incoming call, the called radio unit terminates call set-up in accordance with the procedures specified in Sections 13.1.2.1 of MPT1327 and 8.1 and 8.3.3 of MPT1343.

7.10.2.3.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.10.2.3.4	Test method

The Tester shall cause the General Test Assembly to transmit codeword 609 (AHY) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 609 in the slot immediately following.

Once the Tester has observed the radio unit "alert" indication (in accordance with the manufacturer's instructions) he shall initiate "called party answer" in accordance with the manufacturer's instructions.

The General Test Assembly shall monitor the return control channel throughout the test for any transmissions by the radio unit.

The General Test Assembly shall respond to any transmissions of codeword 506 (RQQ) from the radio unit with codeword 411 (ACK(QUAL='0')) in accordance with Sections 7.2.4 and 13.1.1.1 of MPT1327.

Once the General Test Assembly has transmitted codeword 411 the Tester shall initiate "cancel called party answer" in accordance with the manufacturer's instructions.

The General Tests Assembly shall respond to any further transmissions of codeword 506 (RQQ) from the radio unit with codeword 411 (ACK(QUAL='0')) in accordance with Sections 7.2.4 and 13.1.1.1 of MPT1327.

7.10.2.3.5	Radio unit response

The radio unit shall generate "alert" indication, in accordance with the manufacturer's information supplied, upon receipt of codeword 609 (AHY) and shall respond to codeword 609 with codeword 415 (ACKI(QUAL='0')) in the correct response slot.

When the "called party answer" is initiated by the Tester the radio unit shall transmit codeword 506 (RQQ) with the value of the STATUS field set to '00000' and shall replace the "alert" indication by "CSUIP (called)" indication in accordance with the manufacturer's information supplied.

When the "cancel called party answer" is initiated by the Tester the radio unit shall transmit codeword 506 (RQQ) with the value of the status field set to '11111' in accordance with Section 7.3.4 of MPT 1327.

Upon receipt of codeword 411 (ACK I(QUAL='0')) by the radio unit it shall cease the "CSUIP (called)" indication.

7.11	Call Back (ACKB)

7.11.1	General

The tests in the section are designed to check the correct implementation of the call back procedures specified in Sections 9.2.1.4 and 9.2.2.2 of MPT1327.

All radio units are required to respond correctly to receipt of ACKB in response to a call request as prescribed in Section 9.2.14 of MPT1327.  In addition units may, as a standard option, respond to an incoming call request with ACKB in which case they shall conform to Section 9.2.2.2 of MPT1327.

The General Test Assembly is required for this series of tests to generate one simulated control channel on a fixed frequency.

The tests cover operation of the radio unit with the A personalisation only.

7.11.2	Test procedures

7.11.2.1	Response to a call request

7.11.2.1.1	Test number

71101A

7.11.2.1.2	Purpose of test

To ensure that a radio unit receiving a call back acknowledgement (ACKB) in response to a call request terminates call set-up in accordance with the procedures specified in 9.2.1.4 of MPT1327.

7.11.2.1.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.11.2.1.4	Test method

The Tester shall cause the radio unit to attempt a simple speech call request to unit PFIX=39 IDENT=2079 (See Section 3.3).  After receipt of codeword 503 (RQS) from the radio unit on channel 282 the General Test Assembly shall respond with codeword 409 (ACKI(QUAL='1')) in accordance with Sections 7.2.4 and 9.1.1.1 of MPT1327.

The Tester shall then cause the General Test Assembly to transmit codeword 404 (ACKB(QUAL='0')) within the forward message stream on channel number 282.

The Tester shall observe the confidence indications generated by the radio unit throughout the test.

7.11.2.1.5	Radio unit response

The radio unit shall respond to the simple speech call request by generating "CSUIP (Calling)" indication in accordance with the manufacturer's information supplied and shall transmit codeword 503 (RQS) on the return control channel.

The radio unit shall not change its indication state nor make any transmissions in response to codeword 409 (ACKI(QUAL='1')).

Upon receipt of codeword 404 (ACKB(QUAL='0')) the radio unit shall cease to generate "CSUIP (calling)" indication and shall generate "transaction confirmed" indication in accordance with the manufacturer's information supplied.

7.11.2.2	Response to an incoming call

7.11.2.2.1	Test number

71102A

This test shall not be performed if the manufacturer has declared that the radio unit does not provide for the standard option to accept calls for call back (See Sections 9.2.2.2 of MPT1327 and 11.9.2.2.2 of MPT1343).

7.11.2.2.2	Purpose of test

To ensure that, when a radio unit accepts an incoming call request for subsequent call back, it conforms to the procedures specified in Section 9.2.2.2 of MPT1327.

7.11.2.2.3	Initialisation

The radio unit shall utilise the A personalisation.  The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282.

The radio unit shall be in the idle state on the control channel immediately prior to the test.

7.11.2.2.4	Test method

The Tester shall configure the radio unit under test such that it will accept calls for call back in accordance with the manufacturer's instructions (Section 3.4).

The Tester shall cause the General Test Assembly to transmit codeword 609 (AHY) within the message stream on the forward control channel 282 and shall monitor the return channel for a response from the radio unit to codeword 609 in the slot immediately following.

The Tester shall observe the confidence indications generated by the radio unit throughout the test.

7.11.2.2.5	Radio unit response

The radio unit shall respond to codeword 609 (AHY) with codeword 412 (ACKB(QUAL='0')) in the correct response slot.

The radio units shall indicate that it has accepted a call for call back in accordance with the manufacturer's instructions.

7.12	Call Include

(Not included in this issue).

7.13	Call Diversion

(Not included in this issue).

7.14	RQC Calls

(Not included in this issue).

7.15	Generic Status Calls

7.16	Time-Shared Tests

7.16.1	Acquisition of a Time-Shared Control Channel

7.16.1.1	Test Number

71601E

7.16.1.2	Purpose of Test

To establish that the radio unit acquires a time-shared control channel.

7.16.1.3	Initialisation

The radio unit shall utilise the E personalisation.

The tester shall connect the radio unit to the General Test Assembly, which shall be radiating a continuous control channel on channel number 286.

The tester shall cause the General Test Assembly to radiate a discontinuous control channel on channel number 58. The level of this control channel shall be set to give a nominal value at the receiver of -103dBm to ensure that the radio unit carries out a pre-confirmation error check.  This control channel shall consist of a burst of carrier modulated by message 913. Each burst shall be separated with a gap equivalent to 49 codewords during which the General Test Assembly shall cease radiation.  The tester shall ensure that no bit or codeword synchronisation exists within the gap

The radio unit shall be in the idle state on control channel 286.

7.16.1.4	Method

The tester shall switch off control channel 286 and switch on discontinuous control channel 58.

The General Test Assembly shall monitor the return control channel 58 for codeword 501, from the radio unit, which shall be received within 20 seconds.  Upon receipt of codeword 501 from the radio unit, the General Test Assembly shall respond with codeword 401 in accordance with 7.2.4. and 8.2.1. of MPT 1327.

7.16.1.5.	Radio Unit Response

The correct acquisition of the control channel shall be confirmed by receipt of codeword 501 and by the "in service" indication on the radio unit (if so equipped).

7.16.2	Retention of a Time-Shared Control Channel

7.16.2.1	Test Number

71602E

7.16.2.2	Purpose of Test

To establish that the radio until retains a time-shared control channel without losing codeword synchronisation during a burst.

7.16.2.3	Initialisation

The radio unit shall utilise the E personalisation and shall have successfully completed test 71601E.

The Tester shall connect the radio unit to the General Test Assembly, which shall be radiating a discontinuous control channel on channel  number 58.

The discontinuous channel shall consist of bursts of carrier modulated with message 914.  Each message shall be separated with a gap equivalent to 49 codewords during which the GTA shall cease radiation.  The tester shall ensure that no codeword synchronisation exists between successive bursts

7.16.2.4	Method

Message 914 shall be repeated 100 times.  The General Test Assembly shall monitor the return control channel for codeword 801 from the radio unit.

7.16.2.5	Radio Unit Response

The radio unit shall respond with codeword 801 to 95% of all transmissions of codeword 601, and shall have a continuous "in service" indication (if so equipped).