6.	BASIC PROTOCOL TESTS

6.1	Introduction

This section checks that the radio unit conforms to the basic protocol rules as specified in MPT1343 and MPT1327.  The requirements tested are:

a)	Emission is confined to only the appropriate channel.

b)	Emission is confined to the appropriate time.

c)	Codeword sequences are correctly coded.

d)	Random access rules are observed.

e)	Muting of received audio is continuous whilst acquiring or active on a control channel, see MPT1343 5.3 and 9.3.1.

The tests are considered under two sections:

Those that are passive in nature and are monitoring the radio unit while connected to the General Test Assembly and those that are active in that they influence the data sent to the radio, and look for the appropriate results.

6.2	Passive Tests

These tests shall be carried out concurrently for the whole duration of the section 7 tests.

6.2.1	Emissions confined only to the relevant channel

6.2.1.1	Test number

60201Y

6.2.1.2	Purpose of Test

This test is to ensure that the radio unit confines its transmission to the relevant channel as specified for the test in progress.

6.2.1.3	Initialisation

The radio unit shall be connected to the General Test Assembly and undergoing the tests defined in Section 7.

6.2.1.4	Test method

The General Test Assembly shall monitor the output of the wide band power indicating receiver, as well as the output of the selective RSSI and record any occasion on which an output from the wide band receiver is not accompanied by a simultaneous indication from the RSSI.

6.2.1.5	Limits

No output shall be observed from the wide band receiver if not accompanied by a simultaneous output from the RSSI.

6.2.2	Emissions confined to appropriate times

6.2.2.1	Test numbers

60202Y Control channel response

60203Y Traffic channel response

6.2.2.2	Purpose of tests

This test is to check the timing of transmission from the radio unit, to ensure the rules for signal timing are obeyed, in accordance with 6.2.1.3 and 6.2.2.2 of MPT1327.

6.2.2.3	Initialisation

The radio unit shall be connected to the General Test Assembly, and undergoing the tests specified in section 7.

6.2.2.4	Test method

The General Test Assembly shall examine each response from a radio unit maintaining a bit count, where bit 1 is the start of the relevant slot for a control channel or the end of the last codeword in the received message for a traffic channel. Times refer to the start of the relevant numbered bit.

6.2.2.5	Limits

Test 60202Y

On a control channel:

a)	The RSSI shall not indicate earlier than bit 21.

b)	The SYNC sequence shall start not earlier than bit 46 nor later than bit 59.

c)	The RSSI shall stop indicating before bit ((128 x S) + 15) where S is the number of timeslots appropriate to the RU message.  For responses S = 1 unless the address codeword is 'HEAD' when S = 2 for LEN field values of '00' and '01' or S = 3 for LEN field values of '10' and '11'.

Test  60203Y

On a traffic channel:

a)	The RSSI shall not indicate earlier than bit 21.

b)	The SYNT sequence shall start not earlier than bit 52 nor later nor later than bit 65.

c)	The RSSI shall stop indicating within 6 bit periods after completion of the hangover bit.

6.2.3	Message format

6.2.3.1	Test numbers

60204Y Control channel response

60205Y Traffic channel response

6.2.3.2	Purpose of tests

These tests examine the content of received transmissions, for conformance with section 3 of MPT1327.

6.2.3.3	Initialisation

The radio unit shall be connected to the General Test Assembly, and undergoing the tests specified in section 7.

6.2.3.4	Test method

The General Test Assembly shall examine all received transmissions for the following conformity.

a)	The final preamble bit shall be '0'.

b)	SYNC or SYNT shall be error free.

c)	SYNC shall occur only on control channels and SYNT shall occur only on traffic channels.

d)	The hangover bit at the end of the message shall be present.

6.2.3.5	Limits

60204Y 60205Y all transmissions shall conform to a) to d).

6.2.4	Received audio muting whilst acquiring, or on, a control channel

6.2.4.1	Test number

60206Y

6.2.4.2	Purpose of test

This test is to check that received audio is muted in a radio unit unless the radio unit is on a traffic channel, as a result of receiving an appropriate GTC with the D field set to '0' in accordance with 5.4 of MPT1327.

6.2.4.3	Initialisation

The radio unit shall be connected to the General Test Assembly and undergoing the tests specified in Section 7.

6.2.4.4	Test method

The Tester shall aurally monitor the audio output of the radio unit under test, and shall record any occasions on which the audio is not muted, but the radio unit is not on a traffic channel. The test shall start from the instant that the radio unit is first switched on.

6.2.4.5	Limits

For the whole period of the tests the radio unit shall mute the received audio unless it is on a traffic channel, as a result of receiving an appropriate GTC with the D field set to '0'.

6.3	Active tests

These tests generate control channel signalling with specific properties, to allow the General Test Assembly to test particular MPT1327 parameters.

6.3.1		Randomness of access/singularity of access

6.3.1.1	Test numbers

60301A Randomness of access

60302A Singularity of access

60303A Access within a frame

6.3.1.2	Purpose of tests

Test 60301A tests for randomness of access, while test 60302A ensures that a radio unit only attempts one access in any given frame. Test 60303A tests that all accesses are within an access frame. These are in accordance with 7.3.3 of MPT1327.

6.3.1.3	Initialisation

The radio unit shall utilise the A personalisation. The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282. The radio unit shall be in the idle state on this control channel immediately prior to the test.

6.3.1.4	Test method

The Tester shall cause the General Test Assembly to send message 902 continuously on the forward control channel on channel number 282. Message 902 shall be sent with two alternate sets of parameters. In all transmissions of message 902 the value of CHAN 4 in codewords 301 and 303 shall be set to '0001' and the value of CHAN in codeword 203 shall be set to '0011100100'. Non-specified codewords,which will be sent as specified in4.2.2 but in the proportions indicated, shall be chosen from the following list of codewords: CCSC (50%), GTC (20%), Aloha (10%), ACKs (10%) and MARK (10%). In odd numbered transmissions of message 902 the value of N in codewords 303 and 402 shall be set to 6 and in codeword 203  to 3. In even numbered transmissions of message 902 the value of N shall be set to 0 in all N bearing codewords but the number of non-specified codewords shall be appropriate to a frame length of 6 slots. The Tester shall initiate a call from the radio unit and wait for the radio unit to time out. This shall be repeated 20 times with the radio unit being reset as appropriate.

The General Test Assembly shall record the positions of access attempts inside each frame and shall also record if an access is attempted in the periods when no frame is marked in the message (where N = 0). At the end of the test the General Test Assembly shall report on the number of attempts within frames and the number of attempts when no frame is marked.

6.3.1.5	Limits

Test  60301A

The number of attempts for each slot position should be approximately equal.

Test  60302A

The radio unit shall not attempt more than one access in any Random Access Frame.

Test  60303A

The radio unit shall always access within a frame.

6.3.2	Withdrawn slots

6.3.2.1	Test number

60304A

6.3.2.2	Purpose of test

This test checks that a radio unit does not attempt access in a slot that has been withdrawn by a codeword on the forward control channel in accordance with 7.3.6 of MPT1327.

6.3.2.3	Initialisation

The radio unit shall utilise the A personalisation. The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282. The radio unit shall be in the idle state on this control channel immediately prior to the test.

6.3.2.4	Test method

The Tester shall cause the General Test Assembly to send message 902 continuously on the forward control channel on channel number 282.  In codewords 301 and 303 the value of CHAN 4 shall be set to '0001' and in codeword 303 the value of N shall be set to 6.  In codeword 203 the value of CHAN shall be set to '0011100100' and the value of N shall be set to 3.

In codeword 402 the value of N shall be set to 6.  The Tester shall cause the radio unit to attempt to make a call. This shall be repeated 20 times with the radio unit being reset as appropriate. Throughout testing, slots shall be withdrawn randomly at an average rate of about 1 withdrawal per 4 slots.

The method of withdrawal shall be random and approximately evenly divided between:

a)	AHOY not addressed to the radio unit under test (codeword 604).

b)	ALHR M = 20 messages not addressed to the radio unit under test with N = 1; announcing a new frame at the completion of the previous one.

c)	Head followed by the data codewords with RSA = '0'.

d)	Spoilt transmission in second half of the slot, see 4.2.1 (c).

The General Test Assembly shall record random access and compare them with withdrawn slots.

6.3.2.5	Limits

No transmission from a radio unit shall be detected in a withdrawn slot.

6.3.3	Timeout parameters NR and TC

6.3.3.1	Test numbers

60305A	Maximum number of accesses

60306A )	Timeout for radio unit attempting random

60306B )	access.

6.3.3.2	Purpose of tests

These tests check that the radio unit is obeying the timing rules for accessing a control channel, and will cease access within the appropriate period in accordance with 7.3.8 of MPT1327.

6.3.3.3	Initialisation

The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282 for tests 60305A and 60306A and on channel number 281 for test 60307B. The radio unit shall be in the idle state on this control channel immediately prior to the test and shall employ the A or B personalisation  as specified in 6.3.3.4.

6.3.3.4	Test method

Test  60305A

The radio unit shall be set for A personalisation. The Tester shall cause the General Test Assembly to send message 905 continuously on the forward control channel on channel number 282. In codewords 301 and 303 the value of CHAN 4 shall be set to '0001' and in codeword 303 the value N shall be set to 10. The Tester shall initiate a call from the radio unit and wait for the radio unit to timeout. The Tester shall record the number of   transmissions as indicated on the selective power meter.

This shall be repeated 10 times.

Test  60306A

The radio unit shall be set for A personalisation.  The Tester shall cause the General Test Assembly to send message 905 continuously on the forward control channel on channel number 282. In codewords 301 and 303 the value of CHAN 4 shall be set to '0001' and in codeword 303 the value of N shall be set to 6. The Tester shall initiate a call from the unit and wait for the radio unit to transmit. As soon as a transmission from the radio unit

is detected, the Tester shall cause the General Test Assembly to send message 905 as before but the value of N in codeword 303 changed to 0.

After a time of 50 seconds ±1 second has elapsed from initiating the call attempt, the Tester shall cause the General Test Assembly to send a single message 905 with the value of N in codeword 303 set to 1, and then resume message 905 with the value of N in codeword 303 set to 0. The Tester shall record if the radio unit transmits as indicated on the selective power meter.

After a time of 70 seconds ±1 second has elapsed from initiating the call attempt, the Tester shall cause the General Test Assembly to send repeatedly message 905 with the value of N in codeword 303 set to 1. The Tester shall record if the radio unit transmits as indicated on the selective power meter.

Test  60306B

The radio unit shall be set for B personalisation.  The Tester shall cause the General Test Assembly to send message 905 continuously on the forward control channel number 281. In codewords 301 and 303 the value of CHAN 4 and shall set to '0000' and in codeword 303 the value of N shall be set to  6. The method of test 60306A shall be repeated, but using times of 25 ±1 and 35 ±1 seconds respectively rather than 50 and 70 seconds.

6.3.3.5  Limits

Test  60305A

The number of accesses for each call shall be 8.

Test  60306A

The radio unit shall respond at the 50 second time but shall not respond at the 70 second time.

Test  60306B

The radio unit shall respond at the 25 second time, but shall not respond at the 35 second time.

6.3.4	Address Modifier Parameter (M)

6.3.4.1	Test numbers

60307A

60308A

60309A

6.3.4.2	Purpose of tests

These tests check that the radio unit responds correctly to changes in the "M" parameter in accordance with 7.3.1 of MPT1327.

6.3.4.3	Initialisation

The unit shall utilise A personalisation. The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282. The radio unit shall be in the idle state on this control channel immediately prior to the test.

6.3.4.4	Test method

Test  60307A

The Tester shall cause the General Test Assembly to send message 903 on forward control channel number 282 continuously.  In codewords 301 and 304 the value of CHAN 4 shall be set to '0001' and in codeword 304 the value of M shall be set to 6.  A call shall be initiated from the radio unit but no response given and the radio unit allowed to timeout. The Tester shall record the number of accesses as indicated by the selective power meter.

Test  60308A

The method of test 60307A shall be repeated with the value of M in codeword 304 set to 7, and the number of accesses recorded.

Test  60309A

The method of test 60307A shall be repeated with the value of M in codeword 304 set to 19, and the number of accesses recorded.

6.3.4.5	Limits

Test 60307A shall result in 8 accesses, while tests 60308A and 60309A shall not produce any access attempts.

6.3.5	Wait parameter (WT)

6.3.5.1	Test numbers

60310A  WT  =  3

60311A  WT  =  6

60312A  WT  =  7

6.3.5.2	Purpose of tests

These tests check that the radio unit is obeying the WT parameter in ALH messages in accordance with 7.3.7 of MPT1327.

6.3.5.3	Initialisation

The radio unit shall utilise A personalisation. The Tester shall connect the radio unit to the General Test Assembly, which shall be radiating a single forward control channel on channel number 282.  The radio unit shall be in the idle state on this control channel immediately prior to the test.

6.3.5.4	Test method

Test  60310A

The Tester shall cause the General Test Assembly to send message 904 on the forward control channel on channel number 282. In codewords 301 and 305 the value of CHAN 4 shall be set to '0001' and in codeword 305 the value of WT shall be set to 3. In codeword 402 the value of N shall be set to 1. A call shall be initiated by the radio unit but no response given. The General Test Assembly shall record the positions of successive accesses.

Test 60311A

In codeword 305 WT shall be set to 6 and the method of test 60310A repeated.

Test  60312A

In codeword 305 WT shall be set to 7 and the method of test 60310A repeated.

6.3.5.5	Limits

For each call the number of unused slots between successive accesses shall be greater than:

60310A	3

60311A	10

60312A	15

6.3.6	Preamble bits

6.3.6.1	Test number

60313A

6.3.6.2	Purpose of test

This test looks at the signal received from the radio unit under test and checks that at least the minimum number of preamble bit reversals are transmitted in acordance with 3.1.2 of MPT1327.

6.3.6.3	Initialisation

The radio unit shall utilise the A personaliation. The Tester shall connect the radio unit to the General Test Assembly which shall be radiating a single forward control channel on channel number 282. The radio unit shall be in the idle state on this control channel immediately prior to the test. It shall be in the idle state and connected to the General Test Assembly.

6.3.6.4	Test method

The Tester shall cause the General Test Assembly to send message 901 on the forward control channel number 282. In codewords 301 and 302 the value of CHAN 4 shall be set to '0001'.

When the radio unit responds, the recovered audio shall be examined, with a storage oscilloscope or similar. The double mark at the start of the SYNC message shall be identified, and the number of transitions prior to this point recorded.

6.3.6.5	Limits

The number of transistions recorded shall be greater than 15 and transitions shall be 0.83 ms ± 10% apart.